3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2101973

25 January – 05 February 2021

Agenda item:	8.8.1

Source:	Nokia

Title:	Summary of [AT112-e][250][Slicing] LS replies to SA2 and RAN3 (Nokia)

WID/SID:	FS_NR_slice - Release 17

Document for:	Decision

# 1	Introduction

This document is the summary of the following email discussion:

[AT113-e][250][Slicing] LS replies to SA2 and RAN3 (Nokia)

Scope:

    - Ascertain which LS replies to SA2/RAN3 are needed (based on the LSs received so far), including what to answer to each required LS

Intended outcome:

    - Discussion summary in R2-2101973 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary):  2nd week Mon, UTC 1200

## Contact person(s) for each participating company

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Gyorgy Wolfner | gyorgy.wolfner@nokia.com |
| ZTE | Yuan Gao | gao.yuan66@zte.com.cn |

# 2	Discussion

The following document considered during this email discussion

R2-2100546	Discussion on slicing related reply LSs (R2-2008759 and R2-2010694)	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_slice

R2-2100766	Cell configuration within TA/RA to Support Allowed NSSAI	LG Electronics UK	discussion	Rel-17

R2-2100893	Discussion on SA2 LS	OPPO	discussion	Rel-17	FS_NR_slice

R2-2101061	Considerations on scenarios and solution space of RAN slicing enhancements	Lenovo, Motorola Mobility	discussion	Rel-17	FS_NR_slice	R2-2009669

R2-2101293	UE slice MBR enforcement in RAN	Ericsson	discussion	Rel-17	FS_NR_slice

R2-2101487	Rel-15/16 Status of Cell Configuration on Network Slicing	Futurewei	discussion	Rel-17	FS_NR_slice

R2-2101488	DRAFT Reply LS on Cell Configuration within TA/RA to Support Allowed NSSAI	Futurewei	LS out	Rel-17	FS_NR_slice, FS_eNS_Ph2	To:SA2, RAN3, CT1

R2-2101933	Draft reply LS on Cell Configuration within TARA to Support Allowed NSSAI	ZTE corporation, Sanechips	LS out	Rel-17	FS_NR_slice	To:SA2	Cc:CT1, RAN3

R2-2101700	Discussion on the SA2 incoming LS on Cell Configuration within TA/RA to Support Allowed NSSAI	Huawei, HiSilicon	discussion	Rel-17	FS_NR_slice

(moved from 8.8.2)

R2-2101294	Network slice support in cells	Ericsson	discussion	Rel-17	FS_NR_slice

(moved from 8.8.2)

Content from other contributions related to email discussion can also be considered in the discussion (as part of company feedback).

## 2.1	Reply LS for R2-2008759: LS on Cell Configuration within TA/RA to Support Allowed NSSAI

Question 1 of the LS is the following

In Rel-15 and 16, is it expected that each cell in the tracking area supports the same S-NSSAI(s)? (or, said otherwise, do all cells advertising the same TAC support the same set of S-NSSAIs?).

CT1 and RAN3 has already replied to the LS. CT1 reply LS states (R2-2010688.zip) that "CT1 has assumed that all S-NSSAIs in the allowed NSSAI are supported in all tracking areas of the registration area". RAN3 reply also states (R3-207147.zip) it is assumed in RAN3.

The answer for this question was discussed in RAN2#112e within email discussion "[AT112-e][250][Slicing] LS replies to SA2 and RAN3". The outcome of the email discussion was that about the same number of companies supported a "YES" answer as a "NO" answer (see R2-2011102).

Based on the input documents listed above the rapporteur's view is that the divergent view of the companies has not changed. In order to avoid the delay of SA2 progress a way forward may be to send a neutral answer to SA2 clarifying that the handling of TAs is not in the scope of RAN2, and there is no agreement in RAN2 on this assumption.

Some companies also discuss in their papers that even if they agree that there is this assumption in Rel-15/16, this should be removed in Rel-17. Rapporteur's view is that the question in the LS is clearly about Rel-15/16, and thus Rel-17 assumptions are out of the scope of the discussion of the reply LS.

Q1: Which type(s) of answer is acceptable and which type(s) of answer are not acceptable for Question 1 of the LS from the list below?

- YES, RAN2 confirms that it is assumed that each cell in a TA supports the same S-NSSAI(s) in R15/16.

- NO, from RAN2’s perspective, it is not expected that each cell in the tracking area supports the same S-NSSAI(s) in R15/R16.

- The handling of TAs is not in the scope of RAN2.

| Company | Acceptable | Not acceptable | Comment |
| --- | --- | --- | --- |
| ZTE | b) and c) | a) | (1) As we commented last meeting, we understand the target is that the allowed slice is available within the TA but can be achieved in various ways. One possible deployment is to let all the cells within a TA support the same slice. Another possible deployment is to deploy cells supporting different slices with overlapped coverage. For example, in the NG-ENDC scenario, some slices are supported via the NR SCG cells with same coverage as the LTE PCell but the LTE PCell itself does not support such a slice. If it is assumed that each cell in the same TA supports the same slice, it would not be possible to support some slice via NG SCG in NG-ENDC scenarios. As shown in the following figure, different cells on different frequency layer supporting different slices may be deployed with the same coverage. Since slice 1 and 2 can be supported within the coverage of TA#1, they can be configured as allowed NSSAI. Figure: Example of deployment scenario In such deployment scenario, the allowed NSSAI is available within the TA but not all the cells in this TA are required to support slice 1and 2. => Thus, option a) is not acceptable as we do not see the need to have such restriction in deployment and similar opinions have been expressed by operators last meeting. (2) To avoid the delay of SA2 progress a way forward (as the rapporteur of slice enhancement in SA2, our colleague is anxiously waiting for RAN2 feedback), we are also fine to provide a neutral answer saying there is no consensus in RAN2 on this aspect, which means both understanding a) and b) would be possible and allowed in cell deployment. |

Summary:

TBA

The other questions of LS are the following

If the answer is "no":

2a) Can RAN WGs and CT1 explain if it can happen that a UE, e.g. due to local radio conditions, can only use a cell in the TA where not all S-NSSAIs are present in the Allowed NSSAI it received (and that the TA supports), and can RAN WGs and CT1 explain how it is handled today in rel-15/16?

2b) If an S-NSSAI can be rejected depending on which cell the UE camps on even though it is supported in the TA, for the reason that it is not supported in the cell, is there in rel-15/16 a CT1 error code to handle this case (i.e. can a S-NSSAI be rejected, with a suitable cause code, depending on which cell of the TA the UE camps on, even though this S-NSSAI is known to be supported in the TA, for the reason that this S-NSSAI is actually not supported in the cell of the TA)? Is there any provisions in the RAN or CT1 specifications to handle this case?

As answers for these questions are only needed if RAN2 agrees sending a "NO" to question 1, therefore RAN2 should discuss the answer after concluding the answer to question 1.

## 2.2	Reply LS for R2-2010694: LS on restricting the rate per UE per network slice

The answer for this LS was discussed in RAN2#112e within email discussion "[AT112-e][250][Slicing] LS replies to SA2 and RAN3"(see R2-2011102). As there was no consensus that UL SMBR can be enforced without RAN2 specification impacts RAN2 provided the following reply to R2-2010694 on Solution #22 in R2-2011104:

1) Solution #22

In this solution RAN enforces uplink and downlink SMBR of UEs. This is a similar function as UE-AMBR enforcement at slice level. With proper configuration (LCG and LCH restrictions), the RAN is able to obtain and control the UL data volume of a slice. Therefore, many companies think the solution can be supported without changes to RAN2 specifications, but some companies do not agree, so RAN2 has no consensus on the matter and will continue to discuss.

As the text highlighted by yellow further feedback is expected from RAN2 on the feasibility of Solution#22, more specifically the enforcement of UL SMBR. Based on the input papers at least one company still have concerns on the UL SMBR enforcement (R2-2101293), while others clearly think that it is possible (e.g., R2-2100546, R2-2100893). The company having concerns does not say that it is not possible, just have concern on a specific method using configuration of existing LCG, PRB and LCH restrictions.

Rapporteur's proposal is to resolve the issue it is proposed to answer SA2 without clarifying the exact method to be used, and whether this has RAN2 specification impacts.

Q2: Is the following RAN2 feedback on Solution#22 acceptable?

"RAN2's view is that SMBR enforcement can be supported but the details of the UL SMBR may require further discussions in RAN2."

If not acceptable, please provide a wording suggestion to make it acceptable to all companies.

| Company | Answer | Comments and wording suggestions |
| --- | --- | --- |
| ZTE | Yes | The following options to enforce UL SMBR has been raised: - Option 1 (R2-2100546): With a maximum of 8 LCG, the data volumes of 8 different slices can be reported in uplink. With LCP restrictions, the mapping of a logical channel can be restricted to a subset of the configured cells, numerologies, PUSCH transmission durations, configured grant configurations. Thus, as long as the UE can be configured with isolated physical resources (e.g. cells), the gNB is able to control the UL data volume with a granularity equal to the number of such resources. - Option 2 (R2-2101293): Provide the slice-AMBR to UE and let UE shape the UL data rate, which is similar to the UL session MBR control in R15 and the slice-AMBR can be provided to UE via NAS signaling. With the option 1 and option 2 on table, I think it is quite clear that solution#22 is feasible and can be supported. If the slice-MBR in option 2 is sent to UE via NAS signaling, there would also be no RAN2 spec impact. To save our power and time for issues tasking RAN2 in the SID (e.g. slice specific cell (re)selection and RACH configuration), we would prefer to simply reply: “Solution#22 is feasible and can be supported from RAN2’s perspective.” Further discussion on the details in UL and DL slice MBR enforcement can be discussed in the WI phase if the solution#22 has been selected by SA2 and recommended for normative phase. |

Summary:

TBA

# 3	Conclusions