3GPP TSG-RAN WG2 Meeting #113-e	R2-2102100

Electronic, 25nd Jan– 05th Feb, 2021

Agenda Item:	8.11.3.2

Source:	Huawei, HiSilicon

Title:	Text proposal for IDLE and INACTIVE positioning

Document for: Discussion

Introduction

During RAN2#113e, the following email discussion is triggered after the meeting

[AT113-e][609][POS] Continued discussion of positioning in idle/inactive (Huawei)

Scope: Continue discussion of the issues from R2-2101230, and converge to an agreeable TP, taking as a baseline the principle that positioning in inactive is supported as recommended by RAN1.  R2-2101229 to be taken into account.

Intended outcome: Endorsable TP

Deadline:  Tuesday 2021-02-02 1200UTC

In this contribution, we propose a text proposal for the TR for the study of positioning enhancement in R17

Discussion

In this text proposal, we propose the following based on the current TR 38.857 and the progress in the email discussion

- Reorganize the sections for IDLE/INACTIVE positioning

- Adding the definition for IDLE/INACTIVE positioning

- Implementing RAN2 agreements for the discussions in [AT113-e][609]

Note that the part highlighted in yellow below are still up to further confirmation

Text proposal

================================FIRST CHANGE=====================================

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-193237: "new SID on NR Positioning Enhancements".

[3]	3GPP TR 38.855: "Study on NR Positioning (Release 16)".

[4]	R1-2009433	Evaluation results for Rel-16 positioning and Rel-17 enhancement	Huawei, HiSilicon

[5]	R1-2007665	Evaluation of NR positioning performance	vivo

[6]	R1-2007720	Evaluation of achievable positioning accuracy	BUPT

[7]	R1-2007754	Evaluation of achievable accuracy and latency	ZTE

[8]	R1-2007859	Discussion of evaluation of NR positioning performance	CATT

[9]	R1-2007908	NLOS Identification and Mitigation	FUTUREWEI

[10]	R1-2009390	Update of Evaluation Results for NR Positioning Performance in I-IoT Scenarios	Intel Corporation

[11]	R1-2007997	NR Positioning Latency Evaluations	Lenovo, Motorola Mobility

[12]	R1-2008225	Evaluation of NR positioning in IIOT scenario	OPPO

[13]	R1-2009555	Results on evaluation of achievable positioning accuracy and latency	Nokia, Nokia Shanghai Bell

[14]	R1-2009502 Discussion on Performance evaluation of Rel-17 positioning	Sony

[15]	R1-2008416	Discussions on evaluation of achievable positioning accuracy and latency for NR positioning	LG Electronics

[16]	R1-2008489	Evaluation of achievable positioning latency	InterDigital, Inc.

[17]	R1-2009708	Evaluation of achievable Positioning Accuracy & Latency	Qualcomm Incorporated

[18]	R1-2009428	Evaluation of positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

[19]	R1-2008720	Positioning evaluation results on potential enhancements for additional use cases	CeWiT

[20]	R1-2008764	Evaluation of achievable positioning accuracy and latency	Ericsson

[21]	R1-2008765	Potential positioning enhancements	Ericsson

[22]	R1-2007666	Discussion on potential positioning enhancements	vivo

[23]	R1-2005380	Evaluation of achievable positioning accuracy and latency	vivo

[24]	3GPP TS 22.261	Service requirements for the 5G system; Stage 1 (Release 17)

[25]	RP-202094	Revised SID: Study on NR Positioning Enhancements	CATT, Intel Corporation

[26]	3GPP TS 38.901	Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)

[xx]	3GPP TS 24.571	Control plane Location Services (LCS) procedures (Release 16)

======================================NEXT CHANGE===================================

- 10	Identified NR impacts in Rel-17

- 10.1	NR positioning for UEs in RRC_INACTIVE state

NR positioning for UEs in RRC_INACTIVE state is recommended for normative work, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

  - Support of UE positioning measurements for UEs in RRC_inactive state

    - Options that can be considered include DL-PRS or DL-PRS and SSB

  - Support of gNB positioning measurements for UEs in RRC_inactive state

The details of how to enable the UE positioning in RRC_ INACTIVE state can be further discussed during normative work. These details may include, but are not limited to the following aspects:

  - UL reference signals (e.g., SRS for positioning, PRACH preambles) for UL measurements

  - Signalling and procedures for support the assistance data delivery, DL-PRS configuration, UL reference signals for positioning resource configuration, measurement reporting, which may be developed based on the enhancements of existing signalling and procedures (e.g., existing 2-step and/or 4-step PRACH procedures, paging procedure, small data transmission).

The following procedures are recommended for normative work for DL positioning methods in RRC_INACTIVE:

  - Reporting of PRS measurement and/or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE.

    - The reporting of PRS measurement and/or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE is enabled by enhancing small data transmission in RRC_INACTIVE.

NOTE: The following procedures are considered to have already been supported by UE and can be reused for DL positioning in RRC_INACTIVE

    - On-demand SI request in RRC_INACTIVE for assistance data delivery by broadcast in RRC_INACTIVE

    - ProvideAssistanceData in RRC_CONNECTED for PRS configuration in RRC_INACTIVE downlink positioning

    - RequestLocationInformation can be sent in RRC_CONNECTED for PRS measurement or location estimate in RRC_INACTIVE

The following procedures are recommended for normative work for UL positioning methods in RRC_INACTIVE if SRS transmission is supported in RRC_INACTIVE.

  - Delivery of SRS configuration for UE SRS transmission in RRC_INACTIVE when the UE is in RRC_CONNECTED.

NOTE: The following procedures have already been supported for UE in RRC_CONNECTED and can be reused for UE in RRC_INACTIVE

    - NRPPa message for uplink positioning for UE in RRC_INACTIVE

The following procedures are recommended for normative work for NR E-CID in RRC_INACTIVE

  - Reporting of RRM measurement performed in RRC_INACTIVE in LPP for DL NR E-CID by the UE in RRC_INACTIVE

  - Reporting of RRM measurement performed in RRC_INACTIVE in RRC message for UL NR E-CID by the UE in RRC_INACTIVE.

NOTE: The following procedures have already been supported for NR E-CID in RRC_INACTIVE

  - Reporting of RRM measurement performed in RRC_INACTIVE in LPP/RRC by the UE in RRC_CONNECTED

- 10.9	DL positioning measurement in RRC_IDLE state

From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state.

  - Note: This does not imply that measurements have to be reported in RRC_IDLE state.

The following procedures are considered as feasible for DL positioning methods in RRC_IDLE:

  - Reporting of PRS measurement and/or location estimate performed in RRC_IDLE when the UE is in RRC_CONNETED.

NOTE: The following procedures are considered to have already been supported by UE and can be reused for positioning in RRC_IDLE

    - On-demand SI request in RRC_IDLE for assistance data delivery by broadcast in RRC_IDLE

    - ProvideAssistanceData can be sent in RRC_CONNECTED for PRS configuration in RRC_IDLE downlink positioning

    - RequestLocationInformation can be sent in RRC_CONNECTED for PRS measurement and/or location estimate in RRC_IDLE

NOTE: The following procedures have already been supported for NR E-CID in RRC_IDLE

  - Reporting of RRM measurement performed in RRC_IDLE in LPP/RRC by the UE in RRC_CONNECTED

- ================================NEXT CHANGE===================================

- 10.x Scope of RRC_IDLE/INACTIVE positioning

The following UE positioning procedures are under the scope of RRC_IDLE/RRC_INACTIVE positioning if any of them are performed when the UE is in RRC_IDLE/RRC_INACTIVE.

- NAS-transported positioning signalling

  - LCS messages defined in Clause 4.1.2 for location services in TS 24.571 [xx]

  - LPP signaling for positioning (e.g., Capability transfer, Assistance data transfer, Location information transfer)

- NRPPa

  - E-CID information transfer (UE-associated)

  - Positioning information transfer (UE-associated)

  - Measurement information transfer (non-UE-associated)

- Uu Signaling and procedure

  - RRC signaling for positioning (e.g.,  posSRS configuration)

  - MAC procedure/L1 signaling (e.g., activation/deactivation for semi-persistent/aperiodic posSRS)

  - Transmission of UL-PRS and reception of DL-PRS

  - Reception for assistance information broadcast

================================NEXT CHANGE=======================================

10.y RAT-Independent positioning

RAT-Independent positioning in RRC_IDLE/INACTIVE is recommended for normative work. The exact procedures that can be supported for RAT-Independent positioning in RRC_IDLE/INACTVE can be further studied.

- =======================================END OF CHANGES============================