3GPP TSG-RAN WG2 Meeting #113-e                                                                R2-2102915

Online, April 12 – April 20, 2021

Source:	CATT

Title:	Further Discussion on UE Onboarding and Provisioning for NPN

Agenda Item:	8.16.3

Document for:	Discussion and Decision

Introduction

At RAN2#113e meeting, the objective “Support UE onboarding and provisioning for NPN” was discussed and some agreement has been reached, the details of the agreement are as the following:

Broadcast a 1-bit indication for onboarding per O-SNPN. R2 assumes that the 1-bit indication for onboarding is in SIB1. The UE sends an indication for onboarding to the gNB at RRC Connection Establishment (intention to support AMF selection). Focus on the O-SNPN scenario. Wait for SA2 further conclusion on how a PLMN can be used as onboarding network. Will continue offline on the LS questions.

In this contribution, we discuss left issues on this objective.

Discussion

In reply to RAN2 LS according to [1], SA confirmed that onboarding can be enabled in only part of the SNPN network.

Question 3: Can RAN2 assume uniform support of onboarding in all cells in an O-SNPN? (I.e. can RAN2 assume that all cells of an O-SNPN broadcasts the support for onboarding or can some cells not set the ”onboardingEnabled” bit to e.g. control RAN congestion?) [SA2 answer] The ”onboardingEnabled” bit can be set/enabled per cell e.g. when onboarding is enabled in only part of the SNPN network and can also be used to avoid the load from onboarding UEs. The parameter is used to assist the UE in network selection.

According to above SA2 answer, cells of a onboarding SNPN may set the ”onboardingEnabled” bit to TRUE or FALSE. Hence, even a SNPN supporting onboarding has been selected by UE, UE performing onboarding should check additionally check whether the candidate cell supports onboarding before camp on a cell during cell selection/reselection.

Therefore the legacy SNPN suitable cell criteria need to be modified to reflect this new requirement.

Proposal 1: Enhancement to suitable cell criteria is needed for SNPN onboarding.

To align with SA2 conclusion, RAN2 has agreed that the UE sends an indication for onboarding to the gNB at RRC Connection Establishment (intention to support AMF selection).

Regarding how to indicate it to gNB, two options has been discussed but not concluded.

- Option A: RRCSetupComplete (i.e., msg5)

- Option B: RRCSetupRequest (i.e., msg3)

For the purpose of AMF selection, it is natural to use msg5 for the onboarding indication, as msg5 is the first message which triggers the AMF selection. Furthermore, there is already an IE “selectedPLMN-Identity” in MSG5, which is also used for AMF selection.

selectedPLMN-Identity Index of the PLMN or SNPN selected by the UE from the plmn-IdentityList or npn-IdentityInfoList fields included in SIB1.

And in R16 NPN WI, it has been agreed that if the selected network is SNPN, then index of the SNPN selected by the UE will be included in the MSG5.

Therefore, by using the “onboarding indication” and “selectedPLMN-Identity” in msg5 together, gNB can perform the AMF selection for onboarding UE.

Proposal 2: UE indicates the onboarding request in msg5 (i.e. RRCSetupComplete).

Given that onboarding is a one-shot procedure and rarely occurs, we do not see the need of a special UAC mechanism for onboarding. To avoid extra design on UE side, it can be kept simple by treating the onboarding as MO signaling, and then UE can follow the legacy UAC mechanism completely without extra design.

Proposal 3: legacy UAC is used for the access control of onboarding.

Based on SA2 conclusion，a SNPN can be used only for on-boarding service, or be used for both on-boarding service and normal service.

If the SNPN is only used for on-boarding service, only the UE for onboarding is allowed to camp on it. UE requires normal services should not be allowed to camp on it.

SA2 has concluded that Onboarding network should support functionality to restrict usage to only on-boarding service.

Onboarding network should support functionality to restrict usage to only on-boarding service

The mentioned functionality is located in AMF and be used when UE perform the registration after camp on the cell.

Even through it is not mentioned there is any NG-RAN impacts, we see benefit of a RAN level solution to prevent normal UE from camping on SNPN cell only for Onboarding. To facilitate this, RAN node may need to indicate whether a SNPN is only used for onboarding service or for both onboarding service and normal services.

Proposal 4: Discuss whether NG-RAN needs to indicate whether the SNPN only supports onboarding service, or supports both onboarding service and normal services.

Using PLMN as Onboarding Network is also supported according to SA conclusion in [2], clause 8.4 as following,

- Using PLMN credentials for UE onboarding and PLMN as Onboarding Network (ON) is already possible.

And onboarding is performed via the UP connectivity of PLMN.

Usage of a PLMN as Onboarding Network for a UE equipped with a USIM shall be possible. The SO-SNPN credentials can be transmitted to UE via UP connectivity.

It seems that SA2 is assuming any PLMN is able to provide UP connectivity to the provisioning server for credentials provisioning. With this, PLMN as Onboarding Network (ON) is transparent to NG-RAN. Therefore, from NG-RAN point of view, we do not need to indicate to UE whether onboarding is enabled by a specific PLMN, i.e. not like what is needed for onboarding SNPN.

Proposal 5: No enhancement is needed when PLMN is used as onboarding network.

SA2 has concluded that onboarding function could be used to PNI-NPN credentials provisioning and no enhancement to PLMN network selection is needed.

- No enhancement for the UE onboarding (component 1 of KI#4) with PLMN credentials used for primary authentication and PLMN network selection are needed for the case of PNI-NPN credentials provisioning.

Therefore we do not need to consider any enhancement in NG-RAN to support onboarding for PNI-NPN as well.

Proposal 6: No enhancement is needed to support onboarding for provisioning the PNI-NPN credentials to UE.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: Enhancement to suitable cell criteria is needed for SNPN onboarding.

Proposal 2: UE indicates the onboarding request in msg5 (i.e. RRCSetupComplete).

Proposal 3: legacy UAC is used for the access control of onboarding.

Proposal 4: Discuss whether NG-RAN needs to indicate whether the SNPN only supports onboarding service, or supports both onboarding service and normal services.

Proposal 5: No enhancement is needed when PLMN is used as onboarding network.

Proposal 6: No enhancement is needed to support onboarding for provisioning the PNI-NPN credentials to UE.

Reference

[1] S2-2101076, Reply LS on clarification request for eNPN features, SA2

[2] TR 23.700-07, Study on enhanced support of non-public networks,v1.2.0