3GPP TSG-RAN WG2 Meeting #113 bis electronic				R2-2103265
Online, April 12 – April 20, 2021

Agenda item:		8.6.5

Source:			Asia Pacific Telecom, FGI

Title:		CG-SDT based on beam operation

Document for:	 	Discussion and decision

Introduction

In RAN2#112e, we agreed that the association between the CG resource and the SSBs is need for CG-SDT. The UE should select one of the SSBs with SS-RSRP above the threshold and use the associated CG resource for SDT.

Agreements in RAN2#112 - From RAN2 point of view: An association between CG resources and SSBs is required for CG-based SDT. FFS up to RAN1 how the association is configured or provided to the UE. Send an LS to RAN1 to start the discussion on how the association can be made. Mention that one option RAN2 considered was explicit configuration with RRC Release message - A SS-RSRP threshold is configured for SSB selection. UE selects one of the SSB with SS-RSRP above the threshold and selects the associated CG resource for UL data transmission.

Based on this assumption, two more issues on beam operation should be discussed. The first is to discuss how to select the SSB, i.e., the UE should select the SSB for each CG new transmission or only needs to select the SSB for the very first CG new transmission in the CG-SDT procedure. The second is to discuss how to handle the beam failure condition, i.e., if none of SSBs with SS-RSRP above the RSRP threshold, for CG-SDT.

Discussion

CG-SDT procedure

Figure 1 shows that the NW can configure multiple SSBs and the corresponding CG occasions. While the UE performs small data transmission via CG-SDT, the UE should select a qualified SSB (i.e., the SSB with SS-RSRP above a RSRP threshold) first, then perform transmission via a specific CG occasion that associates with the selected SSB. Since the association between SSB and CG occasion is configured by NW, when the NW receives the UL data on the specific CG occasion, the beam alignment between UE and NW can be achieved.

Figure 1: Association between SSBs and CG occasions

Figure 2 illustrates a flow for CG-SDT procedure. In the beginning, the UE is in the RRC_CONNECTED state. The NW can send a RRC release message including the CG-SDT configuration to the UE. Then the UE moves to RRC_INACTIVE state. In the RRC_INACTIVE state, if there is SDT data arrival, and the criteria of the CG-SDT are satisfied, the UE will perform the CG-SDT procedure. During the SDT procedure, the UE should select the SSB and transmit the UL via the CG resource which is associated with the selected SSB. After the very first CG transmission, the period of the SDT procedure can be referred to subsequent data transmission period before receiving a RRC release message to terminate the SDT procedure or before the CG is released. Under the subsequent data transmission period, the UE should monitor the PDCCH for retransmission if the transmission fails or for a new transmission for subsequent UL transmission.

Figure 2: CG-SDT procedure

Beam selection

For the CG-SDT, one issue is about how to select the SSB for CG transmission. In our view, there are two options for the UE to select the SSB, as shown in Figure 3.

- Option 1: The UE only selects the SSB once for the very first CG new transmission (i.e., the UE does not select the SSB again in subsequent data transmission period)

- Option 2: The UE selects SSB for each CG new transmission (i.e., the UE should select SSB in subsequent data transmission period)

Figure 3: SSB selection for CG

For option 1, after the UE initiates a CG-SDT procedure, the UE will select the SSB only for the very first CG transmission. After that, the UE does not need to select SSB again, and the UE will use the selected SSB for the subsequent data transmission until the NW updates the beam for the UE. Thus, the maintenance of the beam for PDCCH and PDSCH should rely on NW. Moreover, in option 1, since the UE only selects the SSB once, the UE uses the same identity of the CG occasion for the CG configuration based on the SSB selected for the first CG transmission in the following CG transmissions.

For option 2, after the UE initiates a CG-SDT procedure, the UE will select the SSB for each CG new transmission. The benefit of option 2 is that the UE could provide the latest channel condition by repeatedly selecting SSB for each CG transmission. Option 2 is more similar to the beam operation in the RA procedure. Note that in the RA procedure, the UE selects the SSB for each RA preamble transmission. Moreover, in option 2, since the UE selects the SSB for each CG transmission, the UE may change to different identifies of the CG occasion for the CG configuration based on the SSB selected recently in the following CG transmissions.

In comparison with option 1 and option 2, option 2 can provide the latest information of qualified beam from UE side during the subsequent data transmission period without further impact. Thus, option 2 is preferred.

Proposal 1: UE performs SSB selection for each CG new transmission in the SDT procedure (including the subsequent data transmission period) and performs the CG new transmission via a CG occasion associated with the latest selected SSB.

Beam failure handling

In some situations, it is likely that the UE suffers from poor channel condition. That is, none of SSBs with SS-RSRP above the RSRP threshold. In the RA procedure, the UE selects any SSB to attempt the preamble transmission even if the quality of the SSB is lower than the RSRP threshold. Thus, Option 1 is to inherit the same method as RA procedure.

TS 38.321 v.16.3.0 2> if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available: 3> select an SSB with SS-RSRP above rsrp-ThresholdSSB. 2> else: 3> select any SSB.

However, there are some differences between legacy RACH transmission and CG-SDT transmission. For instance, the UE would transmit larger size of data via CG transmission than RA, e.g., Msg1/Msg3/MsgA, which needs better channel quality. Hence, if the DL channel condition is poor (i.e., no beam is qualified), it is very likely that the CG transmission will fail at the end. On top of that, the UE should avoid performing UL transmission via CG-SDT if none of qualified SSB can be detected. From this point of view, two options can be considered.

Option 2 is to switch to RA-SDT procedure from CG-SDT directly. That is, in case that none of SSBs with SS-RSRP above the RSRP threshold, the UE should release the CG-SDT resource and initiate a RA-SDT procedure. Nonetheless, option 2 may be too overkilled to directly abandon the CG-SDT only based on detecting the poor beam for once. In some conditions, the UE just suffers from the beam blockage temporarily, i.e., the UE may relieve this situation soon. To provide more opportunities for attempting the CG transmission, Option 3 is that the UE can skip the CG transmission in case none of SSBs with SS-RSRP above the RSRP threshold for SSB. However, for option 3, the UE may need to maintain a counter and count how many times that the UE skips the CG transmission due to poor beam in a period of time. If the value of the counter reaches to a maximum time, the UE should release the CG resource and initiates a RA-SDT procedure. This mechanism is somehow similar to beam failure detection and recovery procedure. The drawback is to increase the specification complexity.

Proposal 2: RAN2 down selects one of the following options for CG-SDT if none of SSBs with SS-RSRP above the RSRP threshold:

- Option 1: UE selects any SSB and performs the CG transmission based on the selected SSB.

- Option 2: UE releases the CG-SDT resource and initiates a RA-SDT procedure.

- Option 3: UE skips the CG transmission and increments a counter by 1. If the counter reaches to a maximum value, UE releases the CG-SDT resource and initiates a RA-SDT procedure.

Conclusion

In summary, we have the following proposals.

Proposal 1: UE performs SSB selection for each CG new transmission in the SDT procedure (including the subsequent data transmission period) and performs the CG new transmission via a CG occasion associated with the latest selected SSB.

Proposal 2: RAN2 down selects one of the following options for CG-SDT if none of SSBs with SS-RSRP above the RSRP threshold:

- Option 1: UE selects any SSB and performs the CG transmission based on the selected SSB.

- Option 2: UE releases the CG-SDT resource and initiates a RA-SDT procedure.

- Option 3: UE skips the CG transmission and increments a counter by 1. If the counter reaches to a maximum value, UE releases the CG-SDT resource and initiates a RA-SDT procedure.

Reference

[1] Report from [POST113-e][504][SDT] CG open issues (Huawei)