3GPP TSG-RAN WG2 Meeting #113bis e                  	R2-2103378

E-Meeting, 12th – 20th April, 2021

Source:	vivo

Title:	QoE measurement handling

Agenda Item:	8.14.2.2

Document for:	Discussion and Decision

- Introduction

At RAN#91 meeting, a new WI on QoE management and optimizations for diverse services has been approved, with the following objective [1]:

The detailed objectives of the work item are as follows: - Specify the support for QoE measurement collection in NR standalone mode. [RAN2, RAN3] - Specify configuration, activation, and deactivation procedures for both signalling-based and management-based QoE measurement collection and reporting, taking LTE QoE solutions as baseline, as defined in TR 38.890. - Specify configuration and reporting for multiple simultaneous QoE measurements at a UE. - Specify QoE measurement handling at RAN overload, including pause and resume of QoE measurement reporting. - Specify QoE measurement handling in RRC_INACTIVE, i.e. keeping the QoE measurement configuration without measuring and reusing the same configuration upon transition from RRC_INACTIVE to RRC_CONNECTED. NOTE: RRC segmentation may be needed for transmission of QoE reports, and any potential solutions need detailed technical specification of the procedures (if time allows in RAN2). - Specify the support for QoE measurement collection and reporting continuity in intra-system intra-RAT mobility scenario for signaling based QoE. Mobility support for management based QoE measurements is pending input from SA5. [RAN3, RAN2] - To support RAN visible QoE, evaluate and specify an initial relevant set of RAN-visible QoE parameters, then specify configuration and reporting. [RAN3, RAN2] - Specify the support for per-slice QoE measurement. [RAN3, RAN2] - Specify the necessary mechanism to support alignment of radio-related measurement (i.e. MDT) and QoE measurement. [RAN3, RAN2] Note: If needed, co-operate with other working groups, e.g. SA4/SA5/CT1.

RAN3 has begun discussion on QoE management and optimizations and sent Ls to RAN2 [2] to inform RAN2 about RAN3 progress.

This contribution further discusses some QoE management and optimizations issues.

- Discussion

  - QoE measurement handling

RAN3 has agreed that an NG-RAN node can issue a release of QoE measurement configuration for UEs previously configured for QoE measurement reporting, provided that the session for which the QoE measurements are reported is completed, but whether RAN is allowed to release an ongoing QoE measurement reporting is not clear yet.

From our understanding, RAN node may be overloaded. If RAN node is overloaded and the UE continue reporting QoE measurements, RAN node would be further overloaded. As a consequence, RAN node may drop some QoE measurement report or discard all UE QoE measurement report altogether. Thus,

- UE QoE reported to an overloaded RAN node may be dropped or discarded by the overloaded RAN node.

UE reporting QoE measurements that would be dropped or discarded by the network, this would eventually result to UE and RAN unnecessary waste of network resources. In such RAN node overloaded scenario, RAN node should be able to pause or release one or some of the existing QoE measurement configuration.

Therefore,

- In case of RAN node overloaded scenario, RAN node should be able to pause or release one or some of the existing QoE measurement configuration

QoE measurement triggering and stopping can be realized using time-based and/or threshold-based criteria, configured by the OAM. Time-based QoE measurement triggering and stopping in NR is achieved by reusing mechanisms specified in LTE for the start and stop of QoE measurements. Threshold-based QoE measurement triggering and stopping allows to start and stop QoE measurement when given thresholds are passed.

Therefore,

- Time-based and/or threshold criteria can be used for start or stopping QoE measurements configuration.

QoE measurement configuration can be released for UEs previously configured for QoE measurement reporting. If the releasing is done before QoE measurements are completely reported, some collected measurement will be lost. To avoid losing already collected QoE measurements, the QoE measurement configuration releasing should be done after the QoE measurements are completely reported.

Therefore,

- Releasing QoE measurement configuration is done only after the session for which the QoE measurements reporting is completed

- Conclusion

This paper further discussed QoE management configuration and handling. The paper concludes with:

Observation 1:	UE QoE reported to an overloaded RAN node may be dropped or discarded by the overloaded RAN node.

Proposal 1：	In case of RAN node overloaded scenario, RAN node should be able to pause or release one or some of the existing QoE measurement configuration

Proposal 2：	Time-based and/or threshold criteria can be used for start or stopping QoE measurements configuration.

Proposal 3：	Releasing QoE measurement configuration is done only after the session for which the QoE measurements are reported reporting is completed

- References

- RP-210913, New WID on NR QoE management and optimizations for diverse services