3GPP TSG-RAN WG2 Meeting#113bis-e		       					  R2-2104067

Electronic, 12th - 20th April 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the RAN2 impacts of introducing DL 1024QAM for NR

Agenda item:		8.17

Document for: 	Discussion and Decision

# Introduction

RAN1 has finished their work on introduction of DL 1024QAM for NR FR1 with an LS sent to RAN2 [1] to inform their agreements. RAN2 is tasked to specify the corresponding RRC signaling and UE capabilities with RAN1 input taken into consideration. In this paper, we share consideration on the RAN2 impacts, including the required RRC signaling and the UE capabilities.

# Discussion

## RRC signaling

The following agreements have been made in RAN1:

- Introduce new RRC signalling to indicate use of 1024-QAM MCS table for at least DCI format 1_1

  - FFS : support of 1024-QAM MCS table for DCI format 1_2

    - Note: If 1024-QAM MCS table for DCI format 1_2 is supported, separate RRC signalling is used for each of the two DCI formats 1_1 and 1_2, respectively

  - FFS : whether the RRC signalling is only introduced in PDSCH-Config or it can also be separately configured in SPS-Config

- RRC signaling (mcs-Table-r17) to indicate use of 1024-QAM MCS table for DCI format 1_1 is present only in PDSCH-config

- Introduce new RRC signaling to indicate use of 1024-QAM CQI table.

- Introduce separate RRC signaling to indicate use of 1024-QAM MCS table for DCI format 1_2.

Based on the above agreements, we understand the following RRC signaling needs to be specified in RAN2:

- Introduce mcs-Table-v17xy in PDSCH-config with value “qam1024” and clarify that value “qam1024” only applies to DCI formats 1_1.

- Introduce mcs-TableDCI-1-2-v17xy in PDSCH-config with value “qam1024”.

- Introduce cqi-Table-v17xy with value “table4” in CSI-ReportConfig.

Proposal 1: Introduce mcs-TableDCI-1-2-v17xy and mcs-Table-v17xy in PDSCH-config with value “qam1024”. Clarify that value “qam1024” in mcs-Table-v17xy only applies to DCI formats 1_1.

Proposal 2: Introduce cqi-Table-v17xy with value “table4” in CSI-ReportConfig.

## UE capabilities

As highlighted in the WID [2], the DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability.

Firstly, we understand that pdsch-1024QAM-FR1-r17 should be introduced in BandNR as a per-band UE capability to indicate whether UE support DL PDSCH1024QAM for FR.

Proposal 3: Introduce pdsch-1024QAM-FR1-r17 in BandNR as a per-band UE capability to indicate whether UE support DL PDSCH1024QAM for FR1.

In addition, as shown below, the supportedModulationOrderDL indicates the maximum supported modulation order to be applied for DL in the carrier in the max data rate calculation. With the 1024 QAM introduced for FR1 DL, UE may indicate 1024QAM for NW to calculate the max date rate.

| supportedModulationOrderDL Indicates the maximum supported modulation order to be applied for downlink in the carrier in the max data rate calculation as defined in 4.1.2. If included, the network may use a modulation order on this serving cell which is higher than the value indicated in this field as long as UE supports the modulation of higher value for downlink. If not included: - for FR1, the network uses the modulation order signalled in pdsch-256QAM-FR1. - for FR2, the network uses the modulation order signalled per band i.e. pdsch-256QAM-FR2 if signalled. If not signalled in a given band, the network shall use the modulation order 64QAM. In all the cases, it shall be ensured that the data rate does not exceed the max data rate (DataRate) and max data rate per CC (DataRateCC) according to TS 38.214 [12]. | FSPC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Thus, a supportedModulationOrderDL-v17xy with value “qam1024” should be introduced in FeatureSetDownlinkPerCC. And for FR1, if the supportedModulationOrderDL is not included, network uses the modulation order signaled per band i.e. pdsch-1024QAM-FR1-r17, if signaled. If not signaled in a given band, the network uses the modulation order signaled in pdsch-256QAM-FR1.

Proposal 4a: Introduce supportedModulationOrderDL-v17xy with value “qam1024” in FeatureSetDownlinkPerCC to allow UE to indicate the maximum supported modulation order as 1024QAM to be applied for DL in the carrier in the max data rate calculation.

Proposal 4b: For FR1, if the supportedModulationOrderDL is not included, network uses the modulation order signaled per band i.e. pdsch-1024QAM-FR1-r17, if signaled. If not signaled in a given band, the network uses the modulation order signaled in pdsch-256QAM-FR1.

# Conclusion and proposals

With the above analysis, we have the following conclusions and proposals:

Proposal 1: Introduce mcs-TableDCI-1-2-v17xy and mcs-Table-v17xy in PDSCH-config with value “qam1024”. Clarify that value “qam1024” in mcs-Table-v17xy only applies to DCI formats 1_1.

Proposal 2: Introduce cqi-Table-v17xy with value “table4” in CSI-ReportConfig.

Proposal 3: Introduce pdsch-1024QAM-FR1-r17 in BandNR as a per-band UE capability to indicate whether UE support DL PDSCH 1024QAM for FR1.

Proposal 4a: Introduce supportedModulationOrderDL-v17xy with value “qam1024” in FeatureSetDownlinkPerCC to allow UE to indicate the maximum supported modulation order as 1024QAM to be applied for DL in the carrier in the max data rate calculation.

Proposal 4b: For FR1, if the supportedModulationOrderDL is not included, network uses the modulation order signaled per band i.e. pdsch-1024QAM-FR1-r17, if signaled. If not signaled in a given band, the network uses the modulation order signaled in pdsch-256QAM-FR1.

# References

- R1-2102088_LS on Introduction of DL 1024QAM for NR

- RP-202886_Introduction of DL 1024QAM for NR FR1