3GPP TSG-RAN WG2 Meeting #113-bis Electronic	R2-2104086

# Taiwan, 12 – 20 April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | <CR#> | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SUL switch |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-02-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN2#100, it was agreed that The UE shall not perform SUL switch while having an ongoing RA procedure and captured in S5.16. Although the intention is to keep RA procedure running by ignoring the switching indication (DCI) from the network, “SUL switch” is undefined and can be misleading to refer only the switch from NUL to SUL. As a result, wrong implementaion can exist that the switching indication from SUL to NUL is not ignored even while RA procedure is ongoing on SUL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In S5.16, ‘an SUL switch’ is changed to ‘switching between the NUL carrier and SUL carrier’ by resuing the existing text in the same subclause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Impact analysis Impacted functionality: SUL operation, RA procedure Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the network may assume the UE is not switched from SUL to NUL if RA procedure was on-going on SUL but the UE is switched to NUL, hence, further scheduling lost may happen. . 2. If the UE is implemented according to the CR and the network is not, the network may assume the UE is switched from SUL to NUL even if RA procedure was on-going on SUL but the UE stays on SUL, hence, further scheduling lost may happen. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Modified Subclause

5.16	SUL operation

The Supplementary UL (SUL) carrier can be configured as a complement to the normal UL (NUL) carrier. Switching between the NUL carrier and the SUL carrier means that the UL transmissions move from one carrier to the other carrier, which is done by:

-	an indication in DCI;

-	the Random Access procedure as specified in clause 5.1.1.

If the MAC entity receives a UL grant indicating switching between the NUL carrier and the SUL carrier while a Random Access procedure is ongoing, the MAC entity shall ignore the UL grant.

The Serving Cell configured with supplementaryUplink belongs to a single TAG.

End of the modification