3GPP TSG RAN WG2#113bis-e                                                	 R2-2104292

Electronic meeting, 12th April – 20th April, 2021

Agenda Item:	8.13.2.2

Source:	CATT

Title:	Summary of AI 8.13.2.2

Document for:	Discussion and Decision

Introduction

This document provides the summary of the contributions submitted to agenda item 8.13.2.2 of #113bis-e meeting [1-9]. The topic has also been discussed during the post meeting email, as summarized in [10], which is also submitted to the same agenda item.

Taking the company proposals as well as summary in [10] into account, section 3 provides sets of proposals for easy agreement, as well as for further discussions.

Discussion

Fallback information and switching information in 2-step RA report

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2103421 | Oppo | Proposal 2: Introduce an explicit fallback indication per 2-step RA attempt. Proposal 3: Introduce a type switching indication in 2-step RA report. The indication can be implicit by introducing a sperate IE to record 2-step RA related information. |
| R2-2103711 | CMCC | Proposal 2: UE reports the information that can distinguish 2-step RA from 4-step RA in the granularity of per-RA procedure. |
| R2-2103942 | Ericsson | Proposal 2 Include a 4-step fallback indication per RA attempt. Proposal 3 Switch from 2-step RA to 4-step RA can be deduced from the RA-Report when the UE includes both 2-step and 4-step RA related information in the same RA-Report. Hence no need for explicit indication for switch to 4-step RA. |
| R2-2104055 | Samsung | Proposal 3: Based on the agreeable ASN.1 structure, RAN2 discusses the following indicators: - how to indicate the random access type, i.e. 2SRA or 4SRA, - whether to indicate if switching to 4SRA happened during random access procedure and - whether to receive ‘fallbackRAR’ during random access procedure. |
| R2-2104057 | SHARP | Proposal 1: introduce an explicit fallback information by (1) 1 bit fallback indication per RA attempt, or (2) the number of fallbacks per RA procedure. Proposal 2: the configured maximum msgA transmission number is not needed in the 2-step RA report. |

Rapporteur Summary:

1) fallback information

Explicit indication: Oppo, Ericsson, SHARP

Implicit indication:

2) switching information

Explicit indication: CMCC

Implicit indication: Oppo, Ericsson, SHARP

For the fallback indication, three companies support the explicit fallback indication. One company proposes that fallback information can be discussed based on the agreeable ASN.1 structure.

This topic was also discussed and summarized in [10], where majority of the companies prefer explicit fallback indication in 2-step RA Report.

As there seems to be no new reasoning nor change to the majority’s position, Rapporteur suggests to follow the conclusion made in [10], which is copied as the following proposal (i.e., Proposal 1 in [10]).

Proposal 1	The RA report includes an explicit indication that enables the network to know that the fallback from 2-step RA to 4-step RA was performed by the UE.

For the switching information, three companies support implicit indication, if a separate IE is introduced to record 2-step RA related information. One company mentions that the explicit switching indication might be needed if further extension of ra-InformationCommon-r16 IE is applied [2]. One company proposes to report the information that can distinguish 2-step RA from 4-step RA in the granularity of per-RA procedure. And one company proposes that switching information can be discussed based on the agreeable ASN.1 structure.

From [10], majority of the companies supported network knows implicitly whether the switching is performed. And it seems knowing this from the signalling structure is also one potential realization of implicit way. Give these, Rapporteur suggests to follow the conclusion in [10], and puts the detailed mechanism to further study (i.e., Proposal 3 in [10] copied here as proposal 2, and one more proposal 3 on detailed mechanism).

Proposal 2	Network knows implicitly whether switching from 2-step RA to 4-step RA is performed by UE due to ‎reaching a configured MSGA transmission times.

Proposal 3	RAN2 to discuss the details for implicit switching information that enables the network to know whether switching from 2-step RA to 4-step RA is performed by UE due to reaching a configured MSGA transmission times, based on the progress of RA report signalling structure.

RA type indication in RA Report

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2103551 | Nokia | Proposal 1: We propose to use explicit indication of the RACH Type in the RACH Report (2-step or 4-step RACH). Proposal 2: RACH Type in the RACH report is indicated according to the RACH Type that initiated the RACH procedure (2-step versus 4-step RACH attempt). |
| R2-2103711 | CMCC | Proposal 1: UE reports the RA types configured by network, e.g. 2-step RA, 4-step RA, or even 2-step CBRA, 2-step CFRA, 4-step CBRA, 4-step CFRA, in the RA-Report. |
| R2-2104055 | Samsung | Proposal 3: Based on the agreeable ASN.1 structure, RAN2 discusses the following indicators: - how to indicate the random access type, i.e. 2SRA or 4SRA, - whether to indicate if switching to 4SRA happened during random access procedure and - whether to receive ‘fallbackRAR’ during random access procedure. |

Rapporteur Summary:

Two companies propose to use explicit indication of the RACH Type in the RACH Report, one company further proposes to report the contention type related information. One company proposes that RA type indication can be discussed based on the agreeable ASN.1 structure.

Again there seems to be no new reasoning or change of position based on what has been summarized in [10]. So Rapporteur suggests following the proposal 2 as in [10] for RA type indication in RA report. This is as in the following.

Proposal 4	RA type is implicitly indicated by 2-step RA specific information, e.g. the measured RSRP of DL pathloss reference or PRACH resource information.

Signalling model and RA report structure

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2103165 | Vivo | Proposal 1: RAN2 to choose one of the following options as the signalling model of 2-step RA report: - enhance the legacy field ra-InformationCommon to include 2-step RA related frequency parameters; - adopt a new field ra-InformationCommon-2step to include all 2-step specific information. |
| R2-2103421 | Oppo | Proposal 1: RAN2 to discuss which option shall be selected to introduce 2-step RACH related information in RACH report for SON purpose. - Option1: A new field, quoted as ra-InformationCommnon-2step, is defined dedicatedly for 2-step RACH - Option2: 2-step RA related information is included in ra-InformationCommon-r16 |
| R2-2103942 | Ericsson | Proposal 1 RAN2 to introduce a 2-stepRAReport-r17 to represent the 2-step RA attempts within the existing RA-Report. |
| R2-2104055 | Samsung | Proposal 1: RAN2 discusses ASN.1 structure of RA Report before introducing new indicators. Proposal 2: RAN2 discusses if information on only single type of RA can be included in a RA Report entry or not. |
| R2-2104193 | ZTE | Proposal 3: It is proposed to use one RA entry for one RA procedure, and the same IE (RA-InformationCommon) in the RA entry to include both 2step RA and 4step RA information of the same RA procedure. |

Rapporteur Summary:

From the contributions, there are two options for RA type signalling structure, i.e., option 1 is to enhance the legacy field ra-InformationCommon to include 2-step RA related frequency parameters, and option 2 is to adopt a new field ra-InformationCommon-2step to include all 2-step specific information. One company proposes to discuss the ASN.1 structure of RA Report before introducing new indicators.

Rapporteur understands this is signalling design and may require further discussions. Also it is noted that as in proposal 3, the detailed mechanism of switching indication from 2-step to 4-step RACH may also depend on the conclusion of signalling design. We therefore have the following proposal for further discussions and potential agreement in this meeting.

Proposal 5	RAN2 to discuss which option should be selected to introduce 2-step RACH related information in RACH report

- Option 1: enhance the legacy field ra-InformationCommon to include 2-step RA related frequency parameters;

- Option 2: introduce a new field ra-InformationCommon-2step to include all 2-step specific information.

MSGA PUSCH related information

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2103551 | Nokia | Proposal 5: Include in the UE RACH Report the payload size transmitted in MSGA for a 2-step RACH attempt. Additionally, the group type of a preamble (group A or group B) for the 2-step RACH attempt can be logged in the RACH Report. Proposal 6: Log the preamble ID used in the RACH transmission in the RACH Report. Proposal 7: UE logs the type of scrambling identity (PCI or msgA-dataScramblingIndex) per MSGA PUSCH transmission in the RACH Report. In case msgA-dataScramblingIndex is configured, the UE can further include the index in the RACH Report. |
| R2-2103711 | CMCC | Proposal 3: Include the PUSCH resource allocated for msgA in the RA-Report. Proposal 5: Include the indication whether the RO is shared in the RA-Report. |
| R2-2103942 | Ericsson | Proposal 4 Include information to allow the network to retrieve the preamble group used by the UE. Proposal 5 RAN2 to consider including indication of whether the payload size is above or below the ra-MsgA-SizeGroupA threshold, and indication of whether the pathloss is above or below the pathloss threshold for group A/B selection. |
| R2-2104193 | ZTE | Proposal 1: To include PUSCH related configuration in 2step RA report. Proposal 2: It is proposed to include the following PUSCH configuration of the PUSCH resource used in 2step RA report: -  the MCS index , -  the number of PRB per PO of of the PUSCH resource, -  the combination of start symbol and length and PUSCH mapping type, -  PUSCH group information, -  the padding size of the transmitted PUSCH payload -  Offset of lowest PUSCH occasion in frequency domain with respect to PRB 0 -  The number of msgA PUSCH occasions FDMed in one time instance |

Rapporteur Summary:

Two companies propose to include MSGA PUSCH resources in RA Report. Two companies propose to report the preamble group related information to optimize the preamble group A/B configuration. One company further proposes to include the preamble ID used by the UE.

As these aspects haven’t been discussed by most of the companies, Rapporteur suggests further discussions for them. This is as the following.

Proposal 6	RAN2 to discuss which MSGA PUSCH related information should be included in RA report:

- MSGA PUSCH resources

- Preamble group related information

- Preamble ID used by the UE

Other aspects

Response information after msgA transmission and the reason for selecting 4-step RA type

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2103711 | CMCC | Proposal 4: For the scenario that both 2-step RA and 4-step RA are configured, include the reason of 4-step RA in the RA-Report, e.g. RSRP measurement is lower than the RSRP threshold for 2-step RA, or UE has transmitted N times MSGA but 2-step RA is not completed or fallback from 2-step RA. Proposal 6: Include the response information after msgA transmission, i.e. contention resolution/ fallback indication / backoff indication in the RA-Report. |

2-step and 4-step CFRA, 2-step and 4-step CBRA

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2103551 | Nokia | Proposal 3: Log in the RACH Report every time there is a change in the RACH type and contention method (2-step CBRA to 4-step CBRA, 2-step CFRA to 2-step CBRA, 4-step CFRA to 4-step CBRA) to a different one. Include the reason for the change (e.g., switching after N attempts, fall back, no suitable beam found) for the RACH attempt when the change happens. Proposal 4: UE logs in the RACH Report the events of using CBRA RACH resources whenever CFRA resources were configured but not used. |

DL beam quality

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2103732 | Huawei | Proposal 1: In addition to the initial RSRP, the measured DL beam RSRP at the time of success is recorded in cases 1, 2, 4, 5, 6, 8. Proposal 2: In addition to the initial RSRP, the best measured DL beam RSRP during the RA procedure is recorded in cases 3, 7. |

RACH prioritization

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2103551 | Nokia | Proposal 8: Consider RACH prioritization information logging in the RACH Report. |

UE Power Limits in MR-DC

| TDoc | Company name | Proposals |
| --- | --- | --- |
| R2-2104055 | Samsung | Proposal 4: The UE indicates whether the UE could not transmit a PRACH due to the power limitation arising from the power allocation related to MR-DC (e.g., EN-DC, NE-DC, or NR-DC). The UE indicates whether the UE had to reduce its PRACH transmission power due to the power limitation arising from the power allocation related to MR-DC. |

Rapporteur Summary:

Since the above topics are proposed by only few company, and it is unclear what is the majority view for them. Therefore, Rapporteur proposes to postpone these to further discuss.

Proposal 7	FFS on other potential content in RA Report.

Conclusion

Based on summary of [1]-[10], we have the following proposals. The proposals are highlighted as such

- potentially easy agreement

- for discussion and potential agreement in this meeting, and

- for further discussions

Fallback information and switching information

Proposal 1 (for easy agreement, also proposed by [10])	The RA report includes an explicit indication that enables the network to know that the fallback from 2-step RA to 4-step RA was performed by the UE.

Proposal 2 (for easy agreement, also proposed by [10])	Network knows implicitly whether switching from 2-step RA to 4-step RA is performed by UE due to ‎reaching a configured MSGA transmission times.

Proposal 3 (for further discussions)	RAN2 to discuss the details for implicit switching information that enables the network to know whether switching from 2-step RA to 4-step RA is performed by UE due to reaching a configured MSGA transmission times, based on the progress of RA report signalling structure.

‎

RA type indication in RA Report

Proposal 4 (for easy agreement, also proposed by [10])	RA type is implicitly indicated by 2-step RA specific information, e.g. the measured RSRP of DL pathloss reference or PRACH resource information.

‎Signalling model and RA report structure

Proposal 5 (for discussion and potential agreement)	RAN2 to discuss which option should be selected to introduce 2-step RACH related information in RACH report

- Option 1: enhance the legacy field ra-InformationCommon to include 2-step RA related frequency parameters;

- Option 2: introduce a new field ra-InformationCommon-2step to include all 2-step specific information.

‎

MSGA PUSCH related information

Proposal 6 (for further discussions)	RAN2 to discuss which MSGA PUSCH related information should be included in RA report:

- msgA PUSCH resources

- Preamble group related information

- Preamble ID used by the UE

‎Other aspects

Proposal 7 (for further discussions)	FFS on other potential content in RA Report.

Reference

- R2-2103165  Discussion on signalling model of 2-step RACH report	vivo	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

- R2-2103421	  Discussion on 2-step RACH reporting in SON	OPPO	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

- R2-2103551	  Remaining Issues and New Aspects in 2-step NR UE Report	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

- R2-2103711  SON Enhancement for 2-step RA	CMCC	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

- R2-2103732	  Discussion on 2 step RA related SON aspects	Huawei, HiSilicon	discussion	Rel-17

- R2-2103942   2-Step RA information for SON purposes	Ericsson 	discussion	NR_ENDC_SON_MDT_enh-Core

- R2-2104055	  SON Enhancements for 2SRA	Samsung	discussion	NR_ENDC_SON_MDT_enh-Core

- R2-2104057   Discussion on RA information for 2-step RA	SHARP Corporation	discussion	NR_ENDC_SON_MDT_enh-Core

- R2-2104193	  Remaining issues on RA related enhancements	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2103093	  Report of [Post113-e][852][NR17 SON/MDT] 2 step RA and other SON changes (CATT)	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core