3GPP TSG-RAN WG2 Meeting #113bis-e	R2-210xxxx

Online, 12–20 April 2021

# Agenda item:	5.3.1

# Source:	Samsung

# Title:	Report of [AT113bis-e][003][NR15] MAC (Samsung)

# Document for:	Discussion and Agreement

# 1	Introduction

This is to report the result of the following email discussion in RAN2#113bis-e Meeting [1].

[AT113bis-e][003][NR15] MAC (Samsung)

Scope: Treat R2-2102683, R2-2102684, R2-2103848, R2-2104053, R2-2104091, R2-2104092, R2-2103448, R2-2104086,

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed-in-principle CRs.

Deadline: Schedule A

# 2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| Samsung | Jaehyuk JANG (jack.jang@samsung.com) |

# 3	Discussion

## 3.1	Correction to DRX active time criteria with CSI masking

R2-2102683	Correction to DRX active time criteria with CSI masking	Qualcomm Incorporated	CR	Rel-15	38.321	15.12.0	1063	-	F	NR_newRAT-Core

R2-2102684	Correction to DRX active time criteria with CSI masking	Qualcomm Incorporated	CR	Rel-16	38.321	16.4.0	1064	-	F	NR_newRAT-Core

| Company | Agree as is; Agree with changes; Disagree | Detailed Comments |
| --- | --- | --- |
| Samsung | Agree as is (Rel-15) Rel-16 CR should be Cat.A | We are fine with the change. The error came from Rel-11 LTE text, and can be corrected in NR from Rel-15. The category of Rel-16 CR should be Cat. A. |

Conclusion:

TBD

## 3.2	Error handling of MAC PDU with invalid order of MAC subPDUs.

R2-2103848	Error handling of invalid MAC PDU formats	Apple	discussion	Rel-15	NR_newRAT-Core

The discussion paper includes the following proposals and also the curresponding TP for the proposal 1:

Proposal 1: RAN2 to specify the intended error behavior in clause 5.13 of TS 38.321. Proposal 2: RAN2 to discuss the intended behavior including whether it can be up to implementation.

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing a MAC CE placed at an invalid order within the MAC PDU, the MAC entity shall at least: 1> discard the received MAC CE and any remaining subPDUs in the MAC PDU.

| Company | Do you agree with Proposal 1 in R2-2103848? | Detailed Comments |
| --- | --- | --- |
| Samsung | No; can be up to implementation | Even though a transmitter (i.e. either UE or gNB) must follow the procedures as defined in TS 38.321 subclause 6.1.2, the proposed behaviour seems overkill, and to define a new behaviour according to the Proposal 1 would even cause unexpected interoperability issue. Hence, we think it can be left to UE/network implementation. |

Conclusion:

TBD

## 3.3	Whether to have further clarification on reporting multiplexed CSI on PUCCH in DRX

R2-2104053	Clarification on reporting multiplexed CSI on PUCCH in DRX	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

The discussion paper includes the following proposal:

Observation 1: Based on the current NOTE in TS 38.321, in DRX non-Active Time, the UE may still report the CSI which is originally configured on a PUCCH resource outside DRX Active Time on PUCCH. Proposal 1: Clarify that the NOTE relevant to CSI multiplexed with other UCI(s) in subclause 5.7 of TS 38.321 only refers to the case that the UE performs CSI multiplexing in DRX Active Time or in the on-duration period if CSI masking is setup but would report the multiplexed CSI on a PUCCH resource outside DRX Active Time or outside on-duration period if CSI masking is setup.

| Company | Do you agree with Observation 1 and Proposal 1 in R2-2104053? | Detailed Comments |
| --- | --- | --- |
| Samsung | No | From the condition at the beginning of the sentence (i.e. "If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] clause 9.2.5"), sensible implementation would not consider the Case 2 in R2-2104053, and thus no further changes would be needed. |

Conclusion:

TBD

## 3.4	Clarification on DL HARQ process number

R2-2104091	Clarification on DL HARQ process number	Huawei, HiSilicon	CR	Rel-15	38.321	15.12.0	1092	-	F	NR_newRAT-Core

R2-2104092	Clarification on DL HARQ process number	Huawei, HiSilicon	CR	Rel-16	38.321	16.4.0	1093	-	A	NR_newRAT-Core

| Company | Agree as is; Agree with changes; Disagree | Detailed Comments |
| --- | --- | --- |
| Samsung | Disagree | We do not see any ambiguity with the sentence "The dedicated broadcast HARQ process is used for BCCH.", and thus the CR is not needed. |

## 3.5	Correction on Truncated BSR

R2-2103448	Correction on Truncated BSR	ASUSTeK	CR	Rel-16	38.321	16.4.0	1088	-	F	NR_newRAT-Core

| Company | Agree as is; Agree with changes; Disagree | Detailed Comments |
| --- | --- | --- |
| Samsung | Disagree | RAN2 had discussed the issue (several times), and the current text captures the intention correctly: the text in subclause 5.4.5 is about which LCGs would be included but does not specify the actual order of LCG in the MAC CE, while the text in subclause 6.1.3.1 is about the actual order in the MAC CE. Hence, the CR is not needed. |

## 3.6	Clarification on SUL switch

R2-2104086	Clarification on SUL switch	LG Electronics UK	CR	Rel-16	38.321	16.4.0	1091	-	F	TEI16

| Company | Agree as is; Agree with changes; Disagree | Detailed Comments |
| --- | --- | --- |
| Samsung | Disagree | We do not see ambiguity, as 'SUL switch' here should be interpreted as both switching from NUL to SUL and switching from SUL to NUL. Hence, the CR is not needed. |

# 4	Conclusion

TBD

# 5	References

[1]	R2-113bise Chairman notes 2021-04-11.docx