3GPP TSG-RAN2 #113bis-e	R2-210xxxx

# Electronic meeting, April 12 – April 20, 2021

Agenda item:	5.3.2

Source:	LG Electronics (Rapporteur)

Title: 	Report of [AT113bis-e][004][NR15] PDCP SDAP

Document for:	Discussion and Decision

1.	Introduction

This document is to report the result of the following email discussion in RAN2#113bis-e Meeting.

[AT113bis-e][004][NR15] PDCP SDAP (LGE)

Scope: Treat R2-2103301, R2-2103302, R2-2103303, R2-2104201, R2-2104202, R2-2104293

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed-in-principle CRs.

Deadline: Schedule A

2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| LG Electronics | SeungJune Yi (seungjune.yi@lge.com) |
| vivo | Zhangyanxia(yanxia.zhang@vivo.com) |

3.	Discussion

3.1 	PDCP re-establishment after RRC re-establishment

R2-2103301	Discussion on the issue of PDCP re-establishment after RRC re-establishment	NEC	discussion	Rel-15	NR_newRAT-Core

R2-2103302	Correction on PDCP re-establishment after RRC re-establishment	NEC	CR	Rel-15	38.323	15.7.0	0066	-	F	NR_newRAT-Core

R2-2103303	Correction on PDCP re-establishment after RRC re-establishment	NEC	CR	Rel-16	38.323	16.3.0	0067	-	A	NR_newRAT-Core

Reason for change

According to the current spec, the DRBs are suspended when the NW configure the UE to re-establish PDCP after RRC re-establishment, thus the unconfirmed PDCP SDUs of AM DRBs before re-establishment will be treated as new packets from upper layer for transmission.

However, to support lossless transmission and in-order delivery at RAN side, the COUNT of the unconfirmed PDCP SDUs before PDCP re-establishment should be reused for retransmission or transmission in the new cell after successful RRC re-establishment (corresponding to the behavior of AM DRBs which are not suppsended).

Q1: Do you agree to the CR?

| Company | Agree/Disagree/ Agree with modification | Detailed Comments |
| --- | --- | --- |
| LG | Disagree | DRB suspend and PDCP suspend are different events. The text “for suspended AM DRBs” in PDCP specification is referring to the case when “PDCP suspend” was performed before. Thus, the unconfirmed PDCP SDUs of AM DRBs are treated as new packets only when the PDCP suspend was performed before. Note that, in RRC specification, “PDCP suspend” is performed only when the UE receives RRCRelease message. |
| vivo | Disagree | Agree with LG’s understanding. UE additionally indicate PDCP suspend to lower layers of all DRBs when RRCRelease message with suspendConfig. When upper layers request a PDCP entity suspend, the receiving PDCP entity shall set TX_NEXT to the initial value. This is why unconfirmed PDCP SDUs of AM DRBs are treated as new packets when RRC resume successfully. For RRC reestablishment case, no PDCP suspend is indicated to PDCP layer. For all DRBs, the corresponding transmitting PDCP entity will maintain the transmitting status when the RRC re-establishment procedure is initiated. Thus, PDCP can re-transmit unconfirmed PDCP SDUs of AM DRBs with PDCP SN which are allocated before. |

3.2 	Integrity check for interspersed ROHC feedback

R2-2104201	Integrity check for interspersed ROHC feedback	LG Electronics Inc. (PDCP rapporteur)	CR	Rel-15	38.323	15.7.0	0068	-	F	NR_newRAT-Core	Late

R2-2104202	Integrity check for interspersed ROHC feedback	LG Electronics Inc. (PDCP rapporteur)	CR	Rel-16	38.323	16.3.0	0069	-	A	NR_newRAT-Core	Late

Reason for change

The interspersed ROHC feedback is transmitted via PDCP Control PDU, and the PDCP Control PDU is neither ciphered nor integrity protected. However, in the current PDCP specification, it is specified that ciphering is not applied but not specified that integrity protection is not applied. It could be misled that integrity protection is applied to PDCP Control PDU including interspersed ROHC feedback.

Q2: Do you agree to the CR?

| Company | Agree/Disagree/ Agree with modification | Detailed Comments |
| --- | --- | --- |
| LG | Agree | This is rapporteur CR. Note that similar change is proposed for EHC in Rel-16. |
| vivo | Disagree | In TS 38.323 clause 5.9, it is specified clearly that “The integrity protection is not applicable to PDCP Control PDUs.” Thus, we think the change is not necessary. |

3.3 	Change of PDU session ID

R2-2104293	Clarification on the change of PDU session ID	Samsung	CR	Rel-15	38.313	15.13.0	2568	-	F	NR_newRAT-Core	Late

Reason for change

From the current specification, it is unclear whether PDU session ID (i.e. pdu-Session under SDAP-Config in DRB-ToAddMod) can be changed using DRB modification procedure.

Even though the latest specification does not restrict such configuration, such scenario was never discussed in RAN2, and thus it should not be allowed to avoid any malfunction.

Note that TS 37.324 (only) defines QoS flow remapping scenario based on the RRC reconfiguration, but from our understanding, the scenario assumes that it can occur within the same PDU session.

Q3: Do you agree to the CR?

| Company | Agree/Disagree/ Agree with modification | Detailed Comments |
| --- | --- | --- |
| LG | Agree | We have the same understanding that PDU session ID can be changed only by release/addition of the DRB. |

4.	Conclusions

To be filled later..