3GPP RAN WG2 Meeting #113bis-e	R2-210xxxx

eMeeting April 12th – April 20th, 2021

Title:	[Draft] LS to CT1 on Small data transmission

Release:	Release 17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	Intel Corporation [to be RAN2]

To:	CT1

CC:	SA2

Contact Person:

Name:	Marta Martinez Tarradell

E-mail Address:	marta.m.tarradell@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 is working on small data transmission in RRC_INACTIVE where multiple UL and DL packets can be exchanged between the network and the UE without UE transitioning to RRC_CONNECTED with WID (RP-210870).

RAN2 agreed to the following points and would like to ask CT1 to inform RAN2 of any feedback:

- SDT is transparent to NAS layer (i.e. NAS generates one of the existing resume causes and AS decides SDT vs non-SDT access)

- Small data transmission (SDT) with RRC message is supported as baseline.

  - For small data when the UE receives RRC release with Suspend config, the UE at least performs the following actions (i.e. same action as in legacy): SRBs and DRBs are suspended except SRB0.

  - Upon initiating RESUME procedure for SDT initiation (i.e. for first SDT transmission), the UE shall re-establish at least the SDT PDCP entities and resume the SDT RBs that are configured for small data transmission (along with the SRB1).

  - The first UL SDT message (i.e. MSG3 for 4-step RACH, MSGA payload for 2-step RACH and the CG transmission for CG) may contain at least the following contents (depending on the size of the message): CCCH message (needs to be included); data from one or more RBs which are configured by the network for small data transmission.

  - Support configuring of SRB1 and SRB2 for small data transmission for carrying RRC and NAS messages.

  - Small data transmission is configured by the network on a per DRB basis.

  - RAN2 design assumes that RRCRelease message is sent at the end to terminate the SDT procedure from RRC point of view.

  - When UE is in RRC_INACTIVE, it should be possible to send multiple UL and DL packets as part of the same SDT mechanism and without transitioning to RRC_CONNECTED.

- The UE behaviour for handling of non-SDT data arrival after sending the first UL data packet is fully specified (i.e. not left to UE implementation)

  - Non-SDT radio bearers are only resumed upon receiving RRCResume (same as today)

  - Switching from SDT to non-SDT is supported. UE receive indication from network to switch to non-SDT procedure. Network can send RRCResume to transit the UE to RRC_CONNECTED during an ongoing SDT session.

  - RAN2 also have an additional question:

RAN2 agreed that only radio bearers configured for SDT are resumed and additional UL and DL data can be exchanged between UE and network as part of a given SDT session while the UE is still in RRC_INACTIVE (i.e. without transition to RRC_CONNECTED). In this case, if new UL data or NAS message becomes available for non-SDT radio bearers (which are suspended), would it be possible that NAS triggers another request to transition into RRC_CONNECTED and provides access category, access identities and resume cause.

2. Actions:

To CT1:

ACTION:	RAN2 kindly asks CT1 to provide feeback, if any, on the RAN2 agreed points above and provide a response to the question.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#114- e	 	May 19th – 27th, 2020		Online meeting