3GPP TSG-RAN WG2 Meeting #113bis Electronic	R2-210xxxx

Elbonia, Apr 12 – 20 2021

Agenda item:	6.6.3

Source:	CATT

Title:	Report of	[AT113bis-e][607][POS] LPP proposals (CATT)

WID/SID:	NR_pos-Core - Release 16

Document for:	Discussion and Agreement

# 1	Introduction

This document is the report of the following email discussion:

- [AT113bis-e][607][POS] LPP proposals (CATT)

Scope: Discuss the proposals in R2-2103129 and conclude on which are agreeable.

Intended outcome: Report to comeback session, in R2-2104411

Deadline:  Tuesday 2021-04-20 0800 UTC

In this email discussion the following contributions are discussed to decide if these contributions or proposals in the contributions can be agreed. Please see R2-2103129 for a summary of these contributions and for Rapporteur’s comments/suggestions. Please also check the contribution themselves before answering the questions in this email discussion.

- R2-2102920	Corrections on the field description of NR-AdditionalPathList and DL-PRS positioning frequency layer related parameters, CATT

- R2-2102921	Corrections on NR-Multi-RTT-RequestAssistanceData, CATT

- R2-2102987	Considerations on missing need codes in LPP, Lenovo, Motorola Mobility

- R2-2103921	LPP Layer interaction with lower layers for Positioning Frequency layer and Measurement Gap, Ericsson

- R2-2103923	Need of compact expirationTime Indication, Ericsson

- R2-2103924	Correction of field description name, Ericsson

- R2-2104049	Correction to PRS configuration, Huawei, HiSilicon

- R2-2104050	Correction to the uplink LPP message, Huawei, HiSilicon

- R2-2104051	Correction to DL-PRS capability, Huawei, HiSilicon

- R2-2104052	Correction on positioning error reporting, Huawei, HiSilicon

- R2-2104269	Correction on the field description of additionPaths, ZTE Corporation, Sanechips

- R2-2102786     37.355 Draft CR on timestamp reference in NR positioning measurement report, vivo

# 2	Contact Information

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Contact: Name (E-mail) |
| --- | --- |
| Huawei, HiSilicon | yinghaoguo@huawei.com |
| Lenovo | Hyung-Nam Choi (hchoi5@lenovo.com) |

# 3	Discussion

## 3.1	Field description of NR-AdditionalPathList and PFL related parameters

R2-2102920 addresses the problem related with the field description of IE nr-RelativeTimeDifference and positioning frequency layer related parameters configured by NR-DL-PRS-PositioningFrequencyLayer, i.e., dl-PRS-SubcarrierSpacing, dl-PRS-CyclicPrefix, dl-PRS-PointA, dl-PRS-CombSizeN, dl-PRS-ResourceBandwidth and dl-PRS-StartPRB. And the following changes are proposed:

- Add a description for the mapping of reported value and the measured negative value in the field description of nr-RelativeTimeDifference-r16.

- Add a restriction that “all DL PRS resource sets belonging to the same positioning frequency layer have the same value of the parameters configured by NR-DL-PRS-PositioningFrequencyLayer” in the field description of the following parameters configured by NR-DL-PRS-PositioningFrequencyLayer:

- dl-PRS-SubcarrierSpacing,

- dl-PRS-CyclicPrefix,

- dl-PRS-StartPRB

Rapporteur’s comments: This is an essential correction. For the 1st correction, it makes the NW and UE behaviour clear. For the 2nd correction, the corresponding changes are related with the definition of the positioning frequency layer, which makes the definition of the DL-PRS frequency layer clear and aligned with RAN1’s spec. So, rapporteur proposes to agree on this CR.

Proposal 1: RAN2 to agree adding a description for the mapping of reported value and the measured negative value, and if it is agreeable to add clarification that all DL PRS resource sets belonging to the same positioning frequency layer have the same value of the parameters dl-PRS-SubcarrierSpacing, dl-PRS-CyclicPrefix and dl-PRS-PointA.

Question 1: please provide your views on proposal 1 of whether to add a description for the mapping of reported value and the measured negative value, and to add clarification for the DL-PRS frequency related parameters.

| Company | Agree/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon | Agree, but can be merged to another CR with similar issues | The structure of the PRS signalling already implicitly that these fields are applicable for all the PRS resources under this positioning frequency layer. Not quite essential |
| Qualcomm | Agree with modification | The 2nd change should be consistent: "DL PRS" "DL-PRS" "positioning frequency layer" "Positioning Frequency layer" |

Summary 1: TBD.

Proposal 1: TBD.

## 3.2	nr-AdType field in NR-Multi-RTT-RequestAssistanceData IE

R2-2102921 points out an issue related with the required assistance data for NR Multi-RTT positioning. NR-Multi-RTT-RequestAssistanceData IE used to request assistance data for NR Multi-RTT positioning. This IE has a field (nr-AdType) that indicates the type of assistance data requested. One of the codepoints for this field is ‘ul-srs’ which indicates SRS related information is being requested. However, since SRS related information is not provided as part of the assistance data from LMF to UE in the case of multi-RTT positioning, the nr-AdType in the request for assistance data for multi-RTT should not have the ‘ul-srs’ codepoint. Thus, the following changes are proposed in R2-2102921:

- Add a field description for the IE ul-srs that this IE should not be included in this version of the protocol.

- Add a filed description for the IE nr-AdType in the NR-Multi-RTT-RequestAssistanceData.

Rapporteur’s comments: This is an essential correction and with backward compatible changes. Besides, this question has been discussed in the last meeting RAN2#113e, and most of companies indicate the change can be OK if it is done in a backward compatible way. Thus, Rapporteur proposes to agree the CR.

Proposal 2: RAN2 to agree adding a field description for nr-AdType and clarifying in the field description of that the codepoint ‘ul-srs’ is not used in this release.

Question 2: please provide your views on proposal 2 to add a field description for nr-AdType and clarifying in the field description of that the codepoint ‘ul-srs’ is not used in this release.

| Company | Agree/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon | Agree | During R16, we have agreed that for multi-RTT, SRS confifguration is by gNB instead of LMF. We don’t consider the field is useful in the request assistance data message. |
| Qualcomm | Disagree | The SRS configuration is provided by the gNB, but an LMF would still have to instigate the procedure to deliver the SRS to the UE. Or what should be the expected UE behaviour in the case the UE received a Multi-RTT location request but has no SRS configured/activated? Or if the UE received a Multi-RTT location request but has neither DL-PRS nor UL-PRS assistance data. Would a request for ad-type='dl-prs' trigger the LMF to always provide both, DL-PRS and UL-PRS? |

Summary 2: TBD.

Proposal 2: TBD.

## 3.3	Missing need codes

R2-2102987 proposes several overviews of the optional fields and conditional fields for which need codes are missing and the following changes are proposed:

Proposal 1: For Rel-14 and earlier, it is recommended not to add the missing need codes for the time being but to check the existing implementations carefully and decide afterwards whether there is a need to make any changes or not. Proposal 2: For Rel-15, it is recommended to add the missing need codes but may need to be decided case-by-case (i.e. feature-based) depending on whether there are already existing implementations of UE and network in the field or not. Details of the need codes can be discussed separately. Proposal 3: For Rel-16, it is recommended to add the missing need codes. Details of the need codes can be discussed separately.

Rapporteur’s comments: The discussion paper makes a general analysis for the optional fields and conditional fields for which need codes are missing. RAN2 can first discuss whether to agree to add the missing need codes and the corresponding version of the specifications based on this discussion paper. Since the details of the need codes to be modified have already been covered by email discussion [Offline-601][POS], thus, if agreed to make such changes, companies can further check these parameters case by case in email discussion [Offline-601][POS].

Proposal 3: RAN2 to discuss whether to agree to add the missing need codes in principle first and which corresponding version of the specifications need to be modified. And if agreed, companies can further check these parameters case by case in another email discussion [Offline-601][POS].

Question 3: please provide your views on proposal 3 of whether to add the missing need codes in principle first, and if yes, please provide the corresponding version of the specification need to be modified.

| Company | Agree/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon (proponent) |  | Already discussed in 601 |
| Lenovo |  | Proponent; as this contribution is discussed already in [Offline-601][POS] we propose not to duplicate discussion here. |
| Qualcomm |  | I agree that we have been a bit "sloppy" with the need codes in the past. However, we are not aware of any issues in real deployments. We agree with a fix for Rel-16, but not with open up deployed/deeply-frozen Releases. |

Summary 3: TBD.

Proposal 3: TBD.

## 3.4	LPP and RRC interaction for NR DL PRS measurements

R2-2103921[4] is a revised resubmission of CR in R2-2102123 addressing an issue impacting the NR DL PRS measurements requiring measurement gaps. And the following changes are proposed in R2-2103921:

- The LPP interaction with RRC and lower layers has been captured 6.4.3.

Rapporteur’s comments: We have the following description in LTE which is missing in NR under IE OTDOA-ReferenceCellInfo: “If earfcnRef of this assistance data reference cell is different from that of the serving cell, the LPP layer shall inform lower layers to start performing inter-frequency RSTD measurements with this cell and provide to lower layers the information about this assistance data reference cell, e.g. EARFCN and PRS positioning occasion information”. It is worth considering a similar clarification for NR DL PRS measurements also. This seems to be an essential correction in Rel-16.

Proposal 4: RAN2 to discuss if it is agreeable to add a clarification about the LPP layer to RRC layer interaction when measurement gap is required for NR DL PRS measurements.

Question 4: please provide your views on proposal 4 of whether to add a clarification about the LPP layer to RRC layer interaction when measurement gap is required for NR DL PRS measurements.

| Company | Agree/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon | No strong view | With the current spec, we cannot see what ambiguity can exist. It is obvious that the lower layer needs to perform measurement based on the PRS configuration provisioned by the LPP layer. And, according to RAN4 spec, the UE starts the measurement only when the UE receives request location information for the UE-assisted positioning If this is purely editorial, maybe can be merged with the other CRs with the same editorial issues. |
| Qualcomm | Agree with modification | The "e.g. ARFCN" should be modified to "e.g., DL-PRS PointA" since there are two different ARFCNs in the assistance data: The ARFCN of the CD-SSB/PCI and the ARFCN of the DL-PRS. |

Summary 4: TBD.

Proposal 4: TBD.

## 3.5	ExpirationTime Indication

R2-2103923 discusses the need of granular expirationTime rather than the UTC time and the following changes are proposed:

Observation 1 The ValueTag is not efficient since the posSIB still needs to be decoded. and ExpirationTime in the current form is also difficult to use because it is per SIB and consumes 12 Bytes. Observation 2 Significant signalling savings can be done for broadcast with Light weight ignalling mechanism. Proposal 1 RAN2 to agree to include updateRateTimeUnit and updateRateTime as substitute of expirationTime for some of the posSIBs and in addition to the expirationTime for some of the other posSIBs.

Rapporteur’s comments: It seems an enhancement on broadcast positioning assistance data instead of essential correction in Rel-16.

Proposal 5: RAN2 to discuss whether to include updateRateTimeUnit and updateRateTime as substitute of expirationTime or in addition to the expirationTime for some posSIBs.

Question 5: please provide your views on proposal 5 of whether to include updateRateTimeUnit and updateRateTime as substitute of expirationTime or in addition to the expirationTime for some posSIBs.

| Company | Agree/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon | Disagree | This is not a correction, but addition of a new feature. |
| Qualcomm | Disagree | Not backwards compatible. Expiration time provides the same information as update rate, but expiration time is more flexible. |

Summary 5: TBD.

Proposal 5: TBD.

## 3.6	Field description name of nr-PositionCalculationAssistance

R2-2103924 points out different names exist for the field description nr-PositionCalculationAssistance. In 37.355, the IE nr-PositionCalculationAssistance is defined to provides position calculation assistance data for UE-based mode. However, in the corresponding field description of assistance data related parameters for DL-TDOA and DL-AOD methods, the wrong IE names of nr-PositionCalculationAssistanceData is used. Based on this,  the following changes are proposed in R2-2103924:

- The field name has been changed from nr-PositionCalculationAssistanceData to nr-PositionCalculationAssistance.

Rapporteur’s comments: This is an essential correction which is a typo. The rapporteur proposes a CR to include all of such typo corrections.

Proposal 6: RAN2 to agree the correction to change the field name from nr-PositionCalculationAssistanceData to nr-PositionCalculationAssistance.

Question 6: please provide your views on proposal 6 to change the field name from nr-PositionCalculationAssistanceData to nr-PositionCalculationAssistance.

| Company | Agree/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon | Agree, but | Can be merged to the other CRs with similar editorial corrections |
| Lenovo | Agree |  |
| Qualcomm | Agree | Can probably be merged into an "editorial CR". |

Summary 6: TBD.

Proposal 6: TBD.

## 3.7	Corrections to DL PRS configuration related IEs/fields

R2-2104049 addresses corrections related with the PRS configuration relevant parameters and the following changes are proposed:

- Accept the changes regarding DL-PRS related IEs.

- Clarify that the numbering space for NR-DL-PRS-ResourceSetID is per TRP across multiple frequency layers

- Modify the sentence "qcl-DL-PRS-ResourceSetID specifies the DL-PRS Resource Set ID" to "qcl-DL-PRS-ResourceSetID specifies DL-PRS Resource Set configured for the same TRP whose DL-PRS resource serve as the source reference signal for the DL-PRS"

- Change the name nrMaxSetsPerTRP to nr-MaxSetsPerTRP-PerFrequencyLayer

- In the sentence "The IE NR-SelectedDL-PRS-IndexList is used by the location server to provide the selected Frequency Layer index of nr-DL-PRS-AssistanceDataList to the target device.", it should be the index of PRS resources

- Accept the following changes regarding the associated-DL-PRS-ID.

- In the IE NR-DL-PRS-BeamInfo

  - In the field description of associatedDL-PRS-ID, remove the sentence "The beam information from the associated TRP is considered to be in GCS if the lcs-gcs-translation-parameter field is not provided, and to be in LCS if the lcs-gcs-translation-parameter field is provided."

  - In the field description of associatedDL-PRS-ID, clarify that when the field is present, the fields lcs-GCS-TranslationParameter and dl-PRS-BeamInfoSet shall be absent.

  - In the field desctiption for lcs-GCS-TranslationParameter, clarify that the field’s fucntion for the current TRP is applicable only when the field associatedDL-PRS-ID is absent

- In the IE NR-TRP-LocationInfo

  - In the field description of associatedDL-PRS-ID, clarify that when the field is present, the field trp-Location shall be absent.

Proposal 7: RAN2 to discuss whether to agree the following corrections proposed by R2-2104049.

Question 7: please provide your views on proposal 7 of whether to agree the above corrections proposed by R2-2104049.

| Company | Ageee/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon (proponent) | Agree |  |
| Qualcomm | Agree with modification | The CR should be based on the latest version of the spec on the server. dl-PRS-QCL-Info: Not clear what "configured under the same TRP" means. The qcl-DL-PRS-ResourceSetID specfies the DL-PRS Resource Set ID of the qcl-DL-PRS-ResourceID. Not clear why the proposed change is needed. New Table entry for nr-DL-PRS-ResourceSetID should be the first row of the field description Table. "DL-PRS resource set ID""DL-PRS Resource Set ID "DL-PRS source set" "DL-PRS Resource Set" "under the same TRP""of the TRP"? "selected DL-PRS resource" "selected DL-PRS Resource" |

Summary 7: TBD.

Proposal 7: TBD.

## 3.8	Need code and conditional presence tags in fields in UL messages

R2-2104050 points out that according to the description in Section 6.1 of the LPP spec 37.355, the conditional presence tag should not be used in the uplink LPP message. However, there are several uplink LPP messages, within which conditional presence tag was used. Thus,  the following changes are proposed in R2-2104050:

- Add field description for the field nr-dl-tdoa-LocationInformation, remove the conditonal presence tag UEB and its explanation. Move the explanation to the field description.

- Add field description for the field nr-dl-AoD-LocationInformation, remove the conditonal presence tag UEB and its explanation. Move the explanation to the field description.

- Remove the condition presence tag SameRx for the field nr-DL-AoD-AdditionalMeasurements and nr-DL-PRS-RxBeamIndex and remove the explanation for SameRx. Add the description to the field description of the field nr-DL-PRS-RxBeamIndex

Rapporteur’s comments: The motivation for the corrections is supported. Moreover, the similar problems also exist in LTE spec and these conditional presence tags handling for LTE/NR has been there since Rel-9/Rel-15. Besides, except the conditional tags proposed to be changed in R2-2104050, there still other similar problems in current 37.355, i.e., conditional presence tag NB-IoT for NB-IOT positioning specific IEs, or the tag segmentation for the common IEs CommonIEsProvideLocationInformation, CommonIEsRequestAssistanceData and CommonIEsProvide Capabilities.

Proposal 8: RAN2 to discuss whether it is OK to replace the conditional presence tags for fields used in uplink messages with field description explained the conditions under which the field is present.

Question 8: please provide your views on proposal 8 of whether to replace the conditional presence tags for fields used in uplink messages with field description explained the conditions under which the field is present.

| Company | Agree/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon (proponent) | Agree |  |
| Lenovo | Disagree | The proposed changes are technically correct but not critical as the changes don’t impact UE implementations. Therefore, we see no stringent need to fix it. We can take it as lessons learned and for the future we should be more careful not to define conditional tags in UL. |
| Qualcomm | Agree with modification | The nr-dl-tdoa-LocationInformation etc. should not be connected with the LocationSource. The LocationSource was originally introduced for standalone/device-based-hybrid to inform the LMF which positioning method(s) have been used for calculating the fix. However, for UE-based this is not always needed since a e.g. NR-DL-TDOA-RequestLocationInformation results in a NR-DL-TDOA-ProvideLocationInformation anyhow, and therefore, the locationSource is implicit. If a field description is needed, it should be added for all fields in the NR-DL-TDOA-ProvideLocationInformation-r16, etc. E.g., we don't have a field description for A-GNSS-ProvideLocationInformation either. Maybe: "This field provides location information for UE-based DL-TDOA." or similar should be sufficient – if not obvious. But the other field descriptions should then also be added. I don't know if this is a Word issue or if the wrong version of the spec is used: When I open the specification on the server, it looks different compared to the version used for the CR (i.e, the ASN.1 alignments are muddled). |

Summary 8: TBD.

Proposal 8: TBD.

## 3.9	Corrections to DL-PRS capability related IEs/fields

R2-2104051 addresses the corrections related with the DL-PRS capability and the following changes are proposed:

- Clarify that NR-DL-PRS-ProcessingCapabilityPerBand is defined for a single positioning frequency layer on a certain band

- In the field description for NR-DL-TDOA/DL-AoD-ProvideCapabilities

- For the field nr-DL-TDOA/AoD-mode, clarify that the bit for standalone should always be set to “0”

- In the field description for periodical reporting, clarify that the bit for standalone should be set to 0 and what are the meanings for the other bits for ue-based and ue-assisted

- remove the sentence” If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.”

- In NR-DL-AoD-MeasurementCapability, remove “srs-PosResource” and clarify that it is “UE Multi-RTT measurements”

Proposal 9: RAN2 to discuss whether it is OK to make above corrections proposed by R2-2104051.

Question 9: please provide your views on proposal 9 of whether to agree above corrections proposed by R2-2104051.

| Company | Agree/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon (proponent) | Agree |  |
| Qualcomm | Agree with modification | The meaning of the bits for PositioningModes is not needed in all field descriptions, since described in the Common IEs anyhow. The change for the periodicalReporting description is not needed, since this is currently aligned with all other methods. I don't know if this is a Word Issue or if the wrong version of the spec is used: When I open the specification on the server, it looks different compared to the version used for the CR (i.e, the ASN.1 alignments are muddled). For example, the ASN.1 of the version used for the CR cannot compile. There is not even a space between e.g., nr-DL-TDOA-MeasurementCapability-r16NR-DL-TDOA-MeasurementCapability-r16. So it seems the CR is not based on the spec version on the server. |

Summary 9: TBD.

Proposal 9: TBD.

## 3.10	Clarifications for positioning error reporting

R2-2104052 points out the problem about the error cause for E-CID, DL-AOD, DL-TDOA and multi-RTT methods. Since the error for E-CID is only reported in provideLocationInforamtion for this version of LPP spec sent from device to the server. Hence, the following changes are proposed in R2-2104052:

- Add in the IE description of NR-ECID-Error that in this version of specification, only NR-ECID-TargetDeviceErrorCauses can be chosed

- Add in the IE description of NR-DL-TDOA-Error, NR-DL-AoD-Error, NR-Multi-RTT-Error that when DL-TDOA-Error, DL-AoD-Error, Multi-RTT-Error is included in provideAssistanceData, locationServerErrorCauses is chosen; while under ProvideLocationInformation, targetDevidceErrorCauses is chosen

Rapporteur’s comments: Except the proposed changes in R2-2104052, the same problems also exist in other positioning methods in both NR and LTE (NR since Rel-15, LTE since Rel-9), too much corrections but not essential need to be introduced if agreed. Besides, according to rapporteur’s view, there is no need to do such clarification since nothing is unclear or broken according to the current spec.

Proposal 10: RAN2 to discuss whether need to further clarify the cases under which the two error types (locationServerErrorCauses, targetDevidceErrorCauses) should be included.

Question 10: please provide your views on proposal 10 of whether need to further clarify the cases under which the two error types (locationServerErrorCauses, targetDevidceErrorCauses) should be included.

| Company | Agree/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon (proponent) | Agree |  |
| Qualcomm | Disagree | The current text is used for all positioning methods and the CHOICE is obvious. |

Summary 10: TBD.

Proposal 10: TBD.

## 3.11	Add field description of additionalPaths

In R2-2104269 the following changes are proposed:

- Add the field description of additionalPaths in NR-DL-TDOA-RequestLocationInformation and NR-Multi-RTT-RequestLocationInformation.

Proposal 11: RAN2 to discuss whether it is agreeable to add the field description of additionalPaths.

Question 11: please provide your views on proposal 11 of whether it is agreeable to add the field description of additionalPaths.

| Company | Agree/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon | Agree, but | Not really useful. The values of the field “requested” is already self-explanatory. Can be merged to the other CRs with similar editorial corrections |
| Qualcomm | Agree with modification | The description should be aligned with other similar fields. E.g., This field, if present, indicates that the target device is requested to provide … The CR should be based on the spec on the server. |

Summary 11: TBD.

Proposal 11: TBD.

## 3.12	Timestamp reference

R2-2102786 points out that the reference for generation of timestamp in NR positioning measurement report is based on information provided nr-DL-PRS-ReferenceInfo. However, the clarification is missing in TS37.355. Thus, R2-2102786 proposes to make the following changes:

- Add description of the construction of timestap and clarify these parameters come from reference cell.

Rapporteur’s comments: nr-TimeStamp is reported not only in DL-TDOA, but also in DL-AoD and Multi-RTT measurement which has no reference cell for measurement report. nr-TimeStamp is a common IE which doesn’t not only come from reference cell. So it seems no need to add this clarification.

Proposal 12: The CR to add description of the construction of timestamp and clarify these parameters comes from reference cell is not agreed, since nr-TimeStamp is also reported in DL-AoD and/or Multi-RTT measurement which has no reference cell for measurement report.

Question 12: please provide your views on proposal 12 to not add description of the construction of timestamp and clarify these parameters come from reference cell.

| Company | Agree/Disagree | Technical Arguments/Suggested Text Changes/CR cover issues |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong view, the reference in nr-DL-PRS-ReferenceInfo seems already clear. |
| Lenovo | Disagree | Agree with Rapporteur’s comments. |
| Qualcomm | Disagree | The IE NR-TimeStamp is a common IE. If there are restrictions/requirements on setting the fields, it should be described in the IEs where the NR-TimeStamp is used. |

Summary 12: TBD.

Proposal 12: TBD.

# 4	Conclusion

TBD