3GPP TSG-RAN WG2 Meeting #114 electronic	R2-2105057

# Online, May 19  -  27 , 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1810 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the capability of eutra-IdleInactiveMeasurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2021-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS36.331, when UEs indicating support of RRC connection suspension is in RRC_CONNECTED state, network can request the CRS-based RRM measurements performed in IDLE state via UEInformationRequest message, and then UE can provide its CRS-based RRM measuremens performed in IDLE state via UEInformationResponse. However, the above features is lacked in TS36.306, where UE indicaing support of RRC connection suspension can report the CRS-based RRM measurements performed in IDLE state only when resuming from RRC_IDLE state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a clarification in the descriptions of eutra-IdleInactiveMeasurements that UEs indicaing support of RRC connection suspension can report the CRS-based RRM measurements performed in IDLE state when UE is in RRC_CONNECTED Impact analysis Impacted 5G architecture options: EN-DC Impacted functionality: Idle measurements report Inter-operability: - If the network is implemented according to the CR and the UE is not, When UEs indicating support eutra-IdleInactiveMeasurements-r16 and the UE also indicates support of RRC connection suspension, network may request the idle measurements from UE by UEInformationRequest message, however the UE doesn’t support to report the measurement result via UEInformationResponse. - If the UE is implemented according to the CR and the network is not, the network will never request the UE to report the idle mesurements by UEInformationRequest message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When UEs indicating support of eutra-IdleInactiveMeasurements-r16and the UE also indicates support of RRC connection suspension, network cannot request the UE report the idle measurements by UEInformationRequest/UEInformationResponse. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.40 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 4.3.6.40	eutra-IdleInactiveMeasurements-r16

This field defines whether the UE supports:

-	(if the UE also indicates support of inactiveState-r15), performing eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_INACTIVE, including reporting them when requested by the eNB while resuming from RRC_INACTIVE or in RRC_CONNECTED, as specified in TS 36.331 [5];

-	(if the UE also indicates support of RRC connection suspension), reporting eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE while resuming the RRC connection from RRC_IDLE or in RRC_CONNECTED, as specified in TS 36.331 [5];

A UE that indicates support of this feature shall also indicate support of ca-IdleModeMeasurements-r15.

END OF CHANGE