3GPP TSG RAN WG2 #114-e			R2-2105120

e-Meeting, May 19th – May 28th, 2021

Agenda Item:	8.10.3.3

Source:	Apple

Title:	On Connected Mode Issues for NR NTN

Document for:	Discussion/Decision

# Introduction

One of the offshoots of RAN2-lead objectives in the NTN WID [1], [2] for NTN architectures is the use of earth fixed and earth mobile cells/beams being discussed in [3]. In this contribution, we provide our views on the topic of conditional handovers as discussed in [4].

# Discussion

## Conditional Handovers as a way to mitigate frequent handover load

In [4], 3GPP RAN2 discussed five different conditional handover enhancements that are useful to improving the overall signaling performance in connected mode of the NTN system in scenarios of constant change especially related to the LEO, MEO and HAPS architectures. Table 7.3.2.2.2-1 in [4] describes the pros and cons of these measurement-based, location-based, timer-based, TA-based and elevation-angle based CHO mechanisms in explicit detail.

In this section, we re-analyze these mechanisms based on architectural choices of GEO, MEO, LEO and HAPS and try to gather more information that could be potentially useful in making additional judgements about the use of CHO process. The summary of this analysis is presented in Table 1.

| CHO Enhancement | Potentially useful for configurations | Potentially Not useful for configurations |
| --- | --- | --- |
| Measurement-based | GEO, LEO, HAPS | - |
| Location-based | LEO, HAPS | GEO |
| Timer-based | LEO |  |
| TA-based | All configurations but the requirement is primarily for pre-compensation. |  |
| Elevation-angle based | LEO, HAPS | GEO |

Table 1. Comparison of CHO enhancements for different satellite configurations

Based on the table we have the following observation.

Observation 1: The new conditional handover enhancements proposed for NTN are not suitable to tackle all different satellite configurations.

Further, with the proposals in R2-2105117[5], there is no further need for the timer-based or location-based enhancements for connected mode enhancements in NTN.

Observation 2: With the availability of ephemeris at the UE and a potential mapping of satellite cell IDs to terrestrial cell IDs as explained in R2-2105117[5], there is no further need of using location based or timer-based CHO and regular measurement based CHOs can be utilized.

We, therefore, have the following proposal.

Proposal 1: Measurement based CHO is prioritized over other mechanisms by RAN2 for NTN.

# Conclusion

In this contribution, we provided our views on connected mode enhancements for NTN. Our observations and proposals are as follows:

Observation 1: The new conditional handover enhancements proposed for NTN are not suitable to tackle all different satellite configurations.

Observation 2: With the availability of ephemeris at the UE and a potential mapping of satellite cell IDs to terrestrial cell IDs as explained in R2-2105117[5], there is no further need of using location based or timer-based CHO and regular measurement based CHOs can be utilized.

Proposal 1: Measurement based CHO is prioritized over other mechanisms by RAN2 for NTN.

# References

- RP-193234, Solutions for NR to support non-terrestrial networks (NTN), Sitges, Spain, Dec. 2019.

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Jun. 2020.

- RAN2 Email discussions, [Post111-e#911][NTN] Connected Mode Aspects (ZTE)

- 3GPP TR 38.821, Solution for NR to support non terrestrial networks (NTN), v. 16.2.0, Dec. 2019.

- R2-2105117, “Satellite Cell ID Mapping to Earth Fixed Locations for efficient cell selection and Cell Re-selection in NTN”, Apple, May. 2021.