3GPP TSG-RAN WG2 Meeting #114 electronic	R2-2105328

# Online, 19th – 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0374 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the support for RACH Optimization solutions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RACH information report consists of: - perRASSBInfoList; - perRACSI-RSInfoList, But currently the description of CSI-RS related information (csi-RS-Index/ numberOfPreamblesSentOnCSI-RS) is missing in clause 15.5.3. #### TS 38331 – UEInformationResponse PerRAInfo-r16 ::= CHOICE { perRASSBInfoList-r16 PerRASSBInfo-r16, perRACSI-RSInfoList-r16 PerRACSI-RSInfo-r16 } PerRASSBInfo-r16 ::= SEQUENCE { ssb-Index-r16 SSB-Index, numberOfPreamblesSentOnSSB-r16 INTEGER (1..200), perRAAttemptInfoList-r16 PerRAAttemptInfoList-r16 } PerRACSI-RSInfo-r16 ::= SEQUENCE { csi-RS-Index-r16 CSI-RS-Index, numberOfPreamblesSentOnCSI-RS-r16 INTEGER (1..200) } PerRAAttemptInfoList-r16 ::= SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16 PerRAAttemptInfo-r16 ::= SEQUENCE { contentionDetected-r16 BOOLEAN OPTIONAL, dlRSRPAboveThreshold-r16 BOOLEAN OPTIONAL, ... }. |
| Summary of change: |  | Add the description of CSI-RS related information in clause 15.5.3. Impact analysis Impacted functionality: SON and MDT Inter-operability: - If UE implements according to the CR and the network does not, there is no impact foreseen. - If the network implements according to the CR and the UE does not, there is no impact foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The contents between different specifications are not aligned, which cause further confusions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.5.3 Support for RACH Optimization |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

15.5.3	Support for RACH Optimization

RACH optimization is supported by UE reported information made available at the NG RAN node and by PRACH parameters exchange between NG RAN nodes.

The contents of the RACH information report comprise of the following:

-	Contention detection indication per RACH attempt;

-	Indexes of the SSBs and number of RACH preambles sent on each tried SSB listed in chronological order of attempts;

-	Indication whether the selected SSB is above or below the rsrp-ThresholdSSB threshold per RACH attempt;

-	The CSI-RS index corresponding to the Random Access attempt and number of Random Access Preambles transmitted for each tried CSI-RS in chronological order of attempts.

END OF CHANGES