3GPP TSG-RAN WG2 Meeting #114-e	R2-2105410

Electronic, 19 – 27 May 2021

Agenda item:	8.16.3

Source: 	Qualcomm Incorporated

Title:  	UE onboarding and provisioning

Document for:	Discussion and decision

# Introduction

One of the objectives of the Rel-17 WI Enhancement of Private Network Support for NG-RAN [1] is the support of UE onboarding captured in the WID as follows:

- Support UE onboarding and provisioning for NPN including:

  - The UE onboarding relevant parameter broadcast from SIB [RAN2]

  - The associated cell selection/reselection, cell access control and the connected mode mobility support [RAN2/RAN3]

This topic was discussed in RAN2#113e and the following essential elements were agreed [2]:

- Broadcast a 1-bit indication for onboarding per O-SNPN.

- R2 assumes that the 1-bit indication for onboarding is in SIB1.

- The UE sends an indication for onboarding to the gNB at RRC Connection Establishment (intention to support AMF selection).

- Focus on the O-SNPN scenario. Wait for SA2 further conclusion on how a PLMN can be used as onboarding network.

RAN2#113E has also sent an LS to SA2 (R2-2102489 [3]), asking for feedback regarding the granularity of onboarding support in the network.

RAN2#113bis-e has made further progress, by also taking into account the LS response by SA2 [4] as follows:

UE AS forwards the onboarding indication (and Group IDs if Proposal#1 is agreed) per SNPN to UE NAS for onboarding network selection.

No UE impact on connected mode mobility for onboarding.

A new onboarding indication is included in RRCSetupComplete message.

R2 assumes that no enhancement is needed to support onboarding for provisioning the PNI-NPN credentials to UE.

There is no need to introduce an onboarding request indication in RRC messages for UEs in RRC_INACTIVE.

Group IDs per SNPN for onboarding purpose is broadcast in the SIB. FFS whether the Group IDs for onboarding purpose and for credential by separate entity are different.

R2 assumes that onboarding will not impact cell reselection.

In this contribution, we discuss the remaining issues for UE onboarding and provisioning for NPN.

# Discussion

SA2 response LS [4] has the following responses to RAN2 questions on onboarding and provisioning for NPN:

Question 3: Can RAN2 assume uniform support of onboarding in all cells in an O-SNPN? (I.e. can RAN2 assume that all cells of an O-SNPN broadcasts the support for onboarding or can some cells not set the ”onboardingEnabled” bit to e.g. control RAN congestion?)

[SA2 answer] The ”onboardingEnabled” bit can be set/enabled per cell e.g. when onboarding is enabled in only part of the SNPN network and can also be used to avoid the load from onboarding UEs. The parameter is used to assist the UE in network selection.

Even if there is no uniform support and a UE moves to a cell in an O-SNPN not supporting onboarding, SA2 foresees no impact to mobility procedures as remote provisioning can continue in the target cell. Once the PDU session for remote provisioning has been activated existing 5GS functionality applies for mobility.

Question 1 was brought up in the RAN2#113e email discussion [5] where it was argued by a few companies that non-uniform support of the these new parameters can require changes to cell selection and re-selection.

As confirmed by SA2 LS, the onboarding indication can be per cell. On the other hand, since this is a one-shot procedure, it should not have any impact on cell re-selection. This was also emphasized in the SA2 LS.

Based on the SA2 response, RAN2#113bis-e has already agreed that “R2 assumes that onboarding will not impact cell reselection”. There is no technical reason not to confirm this.

Proposal 1: RAN2 to confirm that there is no impact to cell re-selection procedures for SNPNs for onboarding support.

For cell selection, UE AS will report the new parameters, as confirmed in RAN2#113e agreements, to NAS which will in turn perform the SNPN selection.

Some companies argued that the UE should consider the onboarding support by the cell in cell suitability criteria. Even though onboarding indication can be at per cell level as confirmed by SA2, this does not mean that how the UE chooses a cell for onboarding should be specified. In fact, the baseline SA2 CR [7] for 23.501 has the following description:

##### 5.30.2.x.2.5 Network selection This clause applies only when the UE is in SNPN access mode. When the UE wants to perform UE onboarding using an SNPN, the UE shall perform Onboarding SNPN (ON-SNPN) selection as described below. An ON-SNPN is an SNPN broadcasting the Onboarding enabled indication. NOTE 1: The trigger for the UE to initiate the UE Onboarding procedure is UE implementation dependent (e.g. the trigger can be that the UE, which has not yet been provisioned with credentials other than Default UE credentials, is powered on, or the trigger can be based on UE interaction with the user). For automatic or manual selection, the UE may select and attempt to register to an ON-SNPN which broadcasts information matches the pre-configured information (if available) as described in clause 5.30.2.x.2.6 according to the UE implementation-specific logic. If the registration cannot be achieved the UE may select and attempt to register to a different ON-SNPN in accordance with the pre-configured information.

In existing LTE and NR, cell selection is mostly up to the UE implementation. How the UE considers the onboarding information for cell selection should also be treated in the same way, instead of defining new UE logic in AS specifications.

Proposal 2: How the onboarding indication is used in cell selection is left to the UE implementation.

Per RAN2#113e agreement, the parameter “a 1-bit indication for onboarding per O-SNPN” will be broadcast in SIB1. The UE will need to check this indication before performing access procedure. Therefore, the natural place in SIB1 would be CellAccessRelatedInfo which contains other cell access related parameters. Furthermore, as agreed in RAN2#113e, the new parameter will be per SNPN and thus it should be placed in NPN-IdentityInfoList along with other NPN related parameters.

Proposal 3: The optional 1-bit indication for onboarding is introduced in NPN-IdentityInfo-r16.

The SA2 conclusion [6] also has a Note shown above on handling of RAN-level congestion for onboarding. The existing UAC mechanism can already control restricting or reducing the number of attempts for a given Access Identiy and Access Category. The simplest solution to handle congestion for onboarding attempts is to define a new Access Category.

Proposal 4: Use existing UAC framework to handle congestion due to connections attempts for onboarding. A new Access Category can be introduced by CT1 and SA1. RAN2 to send an LS to SA1 and CT1 for confirmation.

In RAN2#113bis-e, some companies proposed adding the SO-SNPN in msg5 so that the gNB can select the appropriate AMF for UE’s SO-SNPN. However, this is not how the system is expected to work. Any AMF which supports onboarding should be able to communicate with the supported SO-SNPNs. Therefore, RAN2 should not make new assumptions on the functionality of CN nodes unless explicitly indicated by other SA/CT groups.

Proposal 5: For onboarding support, no additional information other than the already agreed onboarding indication is sent in RRCSetupComplete message.

# Conclusion

In this document, we discussed the remaining issues for onboarding support for SNPNs and propose the following:

Proposal 1: RAN2 to confirm that there is no impact to cell re-selection procedures for SNPNs for onboarding support.

Proposal 2: How the onboarding indication is used in cell selection is left to the UE implementation.

Proposal 3: The optional 1-bit indication for onboarding is introduced in NPN-IdentityInfo-r16.

Proposal 4: Use existing UAC framework to handle congestion due to connections attempts for onboarding. A new Access Category can be introduced by CT1 and SA1. RAN2 to send an LS to SA1 and CT1 for confirmation.

Proposal 5: For onboarding support, no additional information other than the already agreed onboarding indication is sent in RRCSetupComplete message.

# References

[1] RP-202363, Revised WID: Enhancement of Private Network Support for NG-RAN, China Telecom

[2] RAN2#1113e Chair Notes

[3] R2-2102489, Clarification request for eNPN features, Nokia

[4] S2-2101076, Reply LS on clarification request for eNPN features, Nokia

[5] R2-2102363, Summary of [AT113-e][032][eNPN] UE onboarding and provisioning for NPN, Ericsson

[6] 23.700-07 V2.0.0, Study on enhanced support of non-public networks, Rel-17