3GPP TSG-RAN WG2 Meeting #114 Electronic	R2-2105556

# Elbonia, 19 – 27 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | 076 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC connection re-establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson, Nokia Shanghai Bell, Sequans Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As a result of the agreed CR 025 [link] the behaviour intended for RLF recovery is now bypassed. Indeed RRC requires the DRBs to be suspended in case of RRC connection re-establishment (5.3.7.2 “suspend all RBs, except SRB0”), but the behaviour given for “suspended AM DRBs” in PDCP does not allow lossless RRC connection re-establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Refer to suspended PDCP entity as originally itended, and not to suspended DRB. Impact analysis Impacted functionality: RRC connection re-establishment. Inter-operability: - If the network is implemented according to the CR and the UE is not, RRC connection re-establishment cannot be guaranteed as lossless. - If the UE is implemented according to the CR and the network is not, no issues are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC connection re-establishment is not always lossless. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 5.1.2	PDCP entity re-establishment

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause for Uu or PC5 interface. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the ROHC protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the EHC protocol for uplink if drb-ContinueEHC-UL is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers, and;

-	for AM DRBs for Uu interface and belonging to a PDCP entity which is suspended (see clause 5.1.4), from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, for each PDCP SDU already associated with a PDCP SN:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer, as specified in clause 5.2.1;

-	for AM DRBs belonging to a PDCP entity which is not suspended (see clause 5.1.4), from, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the clause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for SRBs and UM DRBs, if t-Reordering is running:

-	stop and reset t-Reordering;

-	for UM DRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	for AM DRBs for Uu interface, perform header decompression using ROHC for all stored PDCP SDUs if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for AM DRBs for PC5 interface, perform header decompression using ROHC for all stored PDCP IP SDUs;

-	for AM DRBs for Uu interface, perform header decompression using EHC for all stored PDCP SDUs if drb-ContinueEHC-DL is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the ROHC protocol for downlink and start with NC state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the EHC protocol for downlink if drb-ContinueEHC-DL is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set RX_NEXT and RX_DELIV to the initial value;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

NOTE:	After PDCP re-establishment on a sidelink ‎SRB/DRB, UE determines when to transmit and receive with the new key and discard the old key as specified in TS ‎‎33.536 [14].‎

End of Changes