3GPP TSG-RAN WG2 Meeting #114 electronic	   R2-2105712

Online, May 19 – May 27, 2021

Title:	[Draft] Reply LS on half-duplex operation

Response to:	R1-2104122

Release:	Rel-16

Work Item:	TEI16

Source:	Huawei, HiSilicon [to be RAN2]

To:	RAN1

Cc:

Contact Person:

Name: Yiru Kuang

E-mail Address: kuangyiru@huawei.com

Attachments:	N/A

1. Overall Description:

RAN2 would like to thank RAN1 on the LS on half-duplex operation in R1-2104122.

RAN2 has discussed and agreed to update the RAN2 specifications as below.

- For TS 38.306

half-DuplexTDD-CA-SameSCS-r16 Indicates whether the UE supports directional collision handling between reference and other cell(s) for half-duplex operation in TDD CA with same SCS. The UE can include this field for an intra-band only band combination, and include this field for a mixed intra-band and inter-band NR CA band combination if simultaneousRxTxInterBandCA is not present.

- For TS 38.331

directionalCollisionHandling Indicates that this serving cell is using directional collision handling between a reference and other cell(s) for half-duplex operation in TDD CA with same SCS as specified in TS 38.213 [13], clause 11.1. The half-duplex operation only applies within the same frequency range and cell group. The network only configures this field for TDD serving cells that are using the same SCS. The UE does not expect any directional collision among the serving cells that the UE is not capable of simultaneous transmission and reception after the UE applies the directional collision handling within the set of cell(s).

2. Actions:

To RAN1: RAN2 respectfully asks RAN1 to take above information into account.

3. Date of Next TSG WG RAN2 Meetings:

TSG-RAN2 Meeting#115-e 	                   16 – 27 August      2021                               Online

TSG-RAN2 Meeting#116-e 	                   01 – 12 November 2021	                        Online