3GPP TSG-RAN WG2 Meeting #114 electronic	R2-2105887

Online, May 19 – May 27, 2021	Revision of R2-2103434

Agenda item:	8.6.5

Source:	Qualcomm Incorporated

Title:	Discussion on open issues for CG based SDT

WID/SID:	NR_SmallData_INACTIVE-Core – Release 17

Document for:	Discussion and Decision

# Introduction

RAN2 #113e meeting has made several agreements related to CG based SDT.

Agreements

- CG-SDT resource configuration is provided to UEs in RRC_Connected only within the RRCRelease message, i.e. no need to also include it in RRCReconfiguration message

- CG-PUSCH resources can be separately configured for NUL and SUL.  FFS if we allow them at the same time.  This depends on the alignments CRs for Rel-16.

- RRCRelease message is used to reconfigure or release the CG-SDT resources while UE is in RRC_INACTIVE

- For CG-SDT the subsequent data transmission can use the CG resource or DG (i.e dynamic grant addressed to UE’s C-RNTI). Details on C-RNTI, can be the same as the previous C-RNTI or may be configured explicitly by the network can be discussed in stage 3

- TAT-SDT is started upon receiving the TAT-SDT configuration from gNB, i.e. RRCrelease message, and can be (re)started upon reception of TA command.

- From RAN2 point of view, assume similar to PUR, that we introduce a TA validation mechanism for SDT based on RSRP change, i.e.  RSRP-based threshold(s) are configured.  Ask RAN1 to confirm.  FFS on how to handle CG configuration when TA expires or when is invalid due to RSRP threshold.  Details of the TA validation procedure can be further discussed.

- As a baseline assumption, it’s a network configuration issue whether to support multiple CG-SDT configurations per carrier in RRC_INACTIVE (i.e. we will not restrict network configuration for now).

- FFS Discuss further in stage 3 how to specify the agreement that CG-SDT resources are only valid in one cell (i.e. cell in which RRCRelease is received)

- UE releases CG-SDT resources when TAT expires in RRC_Inactive state

RAN2 #113bis-e meeting further made following agreements related to CG based schemes.

Agreements:

1	CG-SDT resources can be configured at the same time on NUL and SUL

2	Implicit release of CG-SDT resource is not supported

3	UE start a window after CG/DG transmission for CG-SDT.   FFS whether to design a new timer or to reuse an existing timer.

4	Support retransmission by dynamic grant for CG-SDT.

5	Support multiple HARQ processes for uplink CG-SDT.

6	CG resource availability delay is not considered as a criterion for CG validation.

7	UL carrier selection is performed before CG-SDT selection

8	FFS CG-SDT resource can be configured on BWPs other than initial BWP

In this paper, we would like to discuss more details specific to the CG based SDT and provide our view.

# Discussion

CG resource request message

In the RAN2 #112e and #113e post email discussion, whether UE should send CG resource request message for CG-SDT resource while being in RRC_CONNECTED has been extensively discussed. In the LTE PUR, the PUR resource configured for a UE could be triggered either by UE sending a PUR Configuration Request message or by network side information such as, subscription information and/or traffic statistics by network itself. Based on UE request, network may configure PUR to the UE.

For NR small data transmission, the similar mechanism could be adopted which will be quite efficient for network to configure the appropriate CG resource for small data transfer. Even the Subscription UE Differentiation Information is known by network, UE may have different traffic requirements in various scenarios. The QoS information cannot always accurately reflect the user requirement. Even for the same type of IoT device, the uplink traffic pattern could not be always the same.

Observation 1: The UE uplink traffic has different traffic profile even for the same device type.

As the analysis in our companion paper [1], the BSR could only reflect the current amount of data UE may already have but could not represent the future traffic profile.

Observation 2: The buffer status report information only reflects the amount of data that UE may already have. It is difficult for network to know the future traffic profile.

If UE is allowed to send CG resource request message, it can be observed that it is beneficial at least in the following scenarios.

- UE requests CG resource configuration to the network by sending CG resource request message when the UE is in RRC_CONNECTED, and network provides the appropriate CG resource configuration in RRC Release message.

- UE sends CG resource request message in MSGA/Msg3 to indicate UE buffer and traffic status so that network can decide whether to move UE to RRC_CONNECTED (if potential large amount of data is reported), or keep UE in RRC_INACTIVE and providing the CG resource configuration in the RRC Release message. Noted that in the last meeting, RAN2 has confirmed the RRC Release message sent at the end of the SDT may contain the CG resource.

- The UE reported CG resource request message could be under the UE assistance information framework and the serving gNB may forward this message via Xn to anchor gNB, which is helpful for the anchor gNB to make decision on the UE RRC state transition or additional UL resource allocation for small data.

Observation 3: It is helpful and beneficial for network to make decision on UE RRC state transition or providing appropriate CG resource for UE if UE sends CG resource request message.

The following contents could be considered in the CG resource request message.

- Buffer status information, to indicate the amount of data UE may already have in the buffer.

- Small data traffic pattern, to indicate the type of small data, e.g., one-shot or multi-shot. The traffic pattern can also optionally include the periodicity of traffic and the amount of data for each traffic occasion.

The CG request message could be a new RRC message or under the UE assistance framework, and the details on prevention redundant request from UEs and whether needs prohibit mechanisms can be for further study. Thus, we have the following proposal.

Proposal 1: UE is allowed to send the CG resource request message to request for appropriate CG configuration from network.

Proposal 2: The CG resource request message contains at least the buffer status, traffic pattern indication with one-shot or multi-shot, periodicity of traffic and estimated amount data of each shot.

Configuration of CG resources for SDT

In the previous RAN2 meeting, it has been agreed that an association between CG resources and SSBs is required for CG-based SDT, and a SS-RSRP threshold is configured for SSB selection. UE selects one of the SSBs with SS-RSRP above the threshold and selects the associated CG resource for UL data transmission.

Considering the agreements above related to the CG resource configuration in multi-beam operation, the network should configure UE with the following information together with the CG resource. (1) a list of candidate SSBs for UE; (2) mapping between candidate SSBs and CG resources for each CG configurations; (3) RSRP threshold for SSB; (4) The search space(s) in which UE should monitor network’s response. Thus, we make the following proposal.

Proposal 3: Network should configure at least the following information within the CG resource configuration. (1) a list of candidate SSBs for UE; (2) mapping between candidate SSBs and CG resources for each CG configurations; (3) RSRP threshold for SSB; (4) The search space(s) in which UE should monitor network’s response.

Since network should configure a list of candidate SSBs with an association of CG resources, it should be similar to the mechanism about SSB mapping to RACH occasions. For each CG resource, it consists of multiple CG occasions, and a subset of candidate SSBs is associated with each CG occasions. After selecting the suitable SSBs, UE transmits the small data via the first available CG occasion associated with the selected SSB. In addition, one CG resource configuration should include one or multiple CG resources in order to support subsequent small data transmission. Fig.1 shows an example.

Figure 1 The candidate beam associated with the configured grant (CG) occasion

Proposal 4: Each CG resource should consist of multiple CG occasions associating with the configured candidate SSBs. One CG resource configuration includes one or multiple CG resources.

If there are no qualified beam detected, i.e., none of the RSRP of the configured SSBs are above the RSRP threshold, the UE behavior should be defined. In the RACH resource selection associated with beam, UE may select any SSB if none of the SSB is qualified.

Regarding the CG resource selection, we think it is the different case. If there is no qualified beam found it implies that current channel quality is not good enough for UE to transfer data via the CG resources. UE should perform RACH based scheme instead.

Proposal 5: When none of RSRP of the configured SSB is above the RSRP threshold for CG-SDT resource selection, UE should perform RACH based small data transmission.

After UE transmits the small data via the CG resource, UE should monitor network’s response and may also monitor the dynamic scheduling for retransmission. UE monitors PDCCH in the configured search space over monitoring occasions that are QCL-ed with its selected SSB. A new UE specific-RNTI is preferred for UE to monitor PDCCH for the response of the transmitted uplink small data or dynamic scheduling for retransmission. It is much clearer for the specification that C-RNTI is only configured and used in RRC_CONNECTED state.

Proposal 6: UE should monitor PDCCH in a configured search space with a new UE specific-RNTI after transmitting the uplink small data in the CG resources.

Regarding the search space configured in the CG resource configuration, network may not be able to preconfigure the TCI-state for the CG resource search space, because network can’t know in advance which beam UE will use. If UE-specific search space is used in CG-SDT, how to configure the TCI-state for the CORESET associated to UE-specific search space might be an issue. The only way is that network preconfigures a set of TCI states, and UE can choose to monitor the occasion(s) whose TCI state is QCL-ed with the beam UE has selected. The whole procedure could be similar to type 1 or type 2 common search space design.

Proposal 7: The configured search space in CG resource for SDT should be the common search space.

Initial BWP and configured BWP for CG-SDT resource

In the past RAN2 meetings, whether CG-SDT resource can be configured on initial BWP or a separate BWP has been extensively discussed. In our view, we observe several issues on the separate BWP configuration for CG-SDT resource.

Firstly, UE in RRC_INACTIVE should receive the Short Message (SI modification and PWS notification) in CORESET #0 which is important for UE to not miss them during SDT. So, for a TDD system, the associated DL BWP should fully contain the CORESET #0 even if the CG-SDT resource is configured in a separate UL BWP.

In addition, since the use case is ‘small data’, the traffic profile should be the small size of packets with a short period. Considering power consumption and network resource allocation, we don’t see there is a strong need to configure CG resource in the separate BWP.

Finally, considering SDT is a typical traffic for RedCap, there is a parallel discussion in RedCap WI on configuring initial or dedicated BWP for RedCap. We think the BWP configuration should be aligned with these two features.

Observation 4: The SDT user expects to receive the Short Message, such as SI modification and/or PWS notifications, in CORESET #0 by paging during SDT.

Proposal 8: The CG resource for SDT should be configured on initial BWP. If CG resource has to be configured on a separate BWP, the associated DL BWP should fully contain CORESET #0 in TDD system.

Release of CG-SDT resource

In the previous RAN2 meetings, it has been agreed that CG-SDT resources are only valid in one cell in which the RRC release message is received, and FFS on how to specify it. In our understanding, the intention of this agreement only asks UE to make the CG resource invalid when UE is not in this serving cell. It does not imply UE has to implicitly release the CG resource when it moves to the new cell. In fact, UE may move back to the previous cell after re-selecting a new cell sometime later. It is beneficial for UE to use the previous configured CG resources to avoid additional configuration signaling. Thus, we have the following proposals.

Proposal 9: The CG resource for SDT should be released when UE initiates RRC resume in a new cell.

# Conclusion

We have the following observations:

Observation 1: The UE uplink traffic has different traffic profile even for the same device type.

Observation 2: The buffer status report information only reflects the amount of data that UE may already have. It is difficult for network to know the future traffic profile.

Observation 3: It is helpful and beneficial for network to make decision on UE RRC state transition or providing appropriate CG resource for UE if UE sends CG resource request message.

Observation 4: The SDT user expects to receive the Short Message, such as SI modification and/or PWS notifications, in CORESET #0 by paging during SDT.

We’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: UE is allowed to send the CG resource request message to request for appropriate CG configuration from network.

Proposal 2: The CG resource request message contains at least the buffer status, traffic pattern indication with one-shot or multi-shot, periodicity of traffic and estimated amount data of each shot.

Proposal 3: Network should configure at least the following information within the CG resource configuration. (1) a list of candidate SSBs for UE; (2) mapping between candidate SSBs and CG resources for each CG configurations; (3) RSRP threshold for SSB; (4) The search space(s) in which UE should monitor network’s response.

Proposal 4: Each CG resource should consist of multiple CG occasions associating with the configured candidate SSBs. One CG resource configuration includes one or multiple CG resources.

Proposal 5: When none of RSRP of the configured SSB is above the RSRP threshold for CG-SDT resource selection, UE should perform RACH based small data transmission.

Proposal 6: UE should monitor PDCCH in a configured search space with a new UE specific-RNTI after transmitting the uplink small data in the CG resources.

Proposal 7: The configured search space in CG resource for SDT should be the common search space.

Proposal 8: The CG resource for SDT should be configured on initial BWP. If CG resource has to be configured on a separate BWP, the associated DL BWP should fully contain CORESET #0 in TDD system.

Proposal 9: The CG resource for SDT should be released when UE initiates RRC resume in a new cell.

# References

- R2-2105885, Discussion on open issues of SDT, Qualcomm.

# Annex