3GPP TSG-RAN WG2 Meeting #114 electronic	R2-2105936

Online, May 19 – 27, 2021

Agenda Item:	8.10.3.3

Source:	Ericsson

Title:	Connected mode aspects for NTN

Document for:	Discussion, Decision

# 1	Introduction

A new WI on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86, with an updated WID approved at RAN#88 [1]. The WI aims to specify the following control plane enhancements:

- Idle mode:

  - Definition of additional assistance information for cell selection/reselection (e.g. using UE location information, satellite Ephemeris information)

  - Definition of NTN (satellite/HAPS) cell specific information in SIB

- Connected mode

  - Enhancement necessary to take into account location information (UE & Satellite/HAPS) and/or ephemeris in determining when to perform hand-over, in order to have a high degree of hand-over control for hand-over robustness and coverage management.

  - Enhancement to existing measurement configurations to address absolute propagation delay difference between satellites (e.g. SMTC measurement gap adaptation to the SSB/CSI-RS measurement window) [RAN2/4].

- Service continuity for mobility from TN to NTN and from NTN to TN systems (to be addressed when connected mode mobility has sufficiently progressed)

- Identify potential issues associated to the use of the existing Location Services (LCS) application protocols to locate UE in the context of NTN and specify adaptations if any [RAN2/3]

RAN2#113bis agreements for connected mode are as follows and the list of all connected mode agreements can be found in Appendix B.

Agreements:

1.	Timing information in CHO execution triggering for NTN describes the time after which the UE is allowed to execute CHO to the candidate target cell.

2.	Working assumption: the timing information for CHO execution triggering in NTN is defined in the form of a timer/timers. This can be revised and a solution based on UTC/system frame number can be considered if problems are found (e.g. if the timer lacks accuracy due to RTT in NTN).

3.	The location in location-based CHO execution triggering for NTN describes the distance between the UE and the reference location of the cell (serving cell or the target cell). FFS what the reference location of the cell is (e.g cell center or other) and how this is provided to the UE

Agreements:

- For Rel-17 NTN, Rel-17 NR operation is enhanced (e.g. the SMTC configuration and UE measurement gap onfiguration) aiming to address the issues associated with the different/larger propagation delays, and the satellites (considering e.g. their deployment, mobility, height, minimum elevation and prioritizing typical NTN scenarios).

- Rel-17 NTN will not rely only on network implementation to address the issue explained in agreement 1.

- Enhancements of the SMTC configuration is supported for Rel-17 NTN.

- Optional new UE assistance is defined in Rel-17 NTN for network to properly (re)configure the SMTC and/or measurement gap

Agreements - via email (from offline [106])

- For Rel-17 NTN, one or more SMTC configuration(s) associated to one frequency can be configured. FFS solution details.

-	The SMTC configuration can be associated with a set of cells (e.g., per satellite or any other suitable set per gNB determination).

-	The multiple SMTC configurations are enabled by introducing different new offsets in addition to the legacy SMTC configuration. FFS how the offsets will be managed/signalled.

FFS the following open questions:

(a) can the UE be configured with multiple SMTCs per carrier and use them all in parallel?

(b) How the NW knows which SMTC (incl. offsets/periodicity, etc.) is relevant for a particular UE?

(c) Is there any validity: in time or for certain location only, foreseen in such multiple SMTC configuration?

(d) What is the potential impact on the signalling, assuming this delay is a dynamic value?

(e) What about the feeder link delay? Is it considered anywhere?

- The configuration of one or multiple offsets is left up to the network implementation.

- It is up to network to update the SMTC configuration of the UE to accommodate the different propagation delays.

Agreements online:

- Measurement gaps enhancements should be supported. FFS on the details

In this paper we discuss further details of CHO, location reporting, STMC and measurement gaps details and provide arguments why RACH-less HO is very much needed for NTN operation and shoud be considered in Release 17 NR NTN.

# 2	CHO and location based measurement events for RRM

RAN2 has agreed to introduce three new trigger events for CHO when configured in NTN. A time/timer based and a location based CHO trigger event, and a CHO trigger event based on the existing A4 event (Neighbour becomes better than threshold).

The most obvious scenario where the time/timer based CHO trigger event is expected to be useful is at the feeder and service link switch in the (quasi) Earth-fixed cell scenario. The time of the cell/satellite change will typically affect the vast majority of the UEs located in the cell coverage area regardless of their position within the cell. Only UEs with high velocity close to the cell border may need to be configured with an additional CHO trigger event as for example the A4 event.

## 2.1	Time/timer based CHO trigger event

According to the agreement from the RAN2#113bis meeting, the timing information in the time/timer based CHO trigger event describes the time after which the UE is allowed to perform CHO to a given candidate target cell. The timing information for the candidate target cell can be provided to each UE in the CHO configuration informing the UE about when the target cell is available and thus after which CHO to that cell may be considered. However, this does not as such trigger CHO.

Current agreement gives information about when a target cell becomes available and after which UE can consider CHO to that cell. However, this does not as such trigger CHO.

This information is basically telling when it is worth for the UE to start detecting and measuring a given candidate target cell. Agreement says:

Timing information in CHO execution triggering for NTN describes the time after which the UE is allowed to execute CHO to the candidate target cell.

The CHO configuration as defined in ASN1 includes per candidate target cell two MeasId’s which lead to the configured CHO event in ReportConfigNR and RRCReconfiguration that gives information about the target candidate cell e.g. the PCI of the candidate target PCell. This timing info could be given in the IE CondReconfigToAddMod or in the RRCReconfiguration in SpCellConfig. Both options are shown here and the running CR submitted to RAN2#114 implements the latter.

------------------TP1--------------------------

#### –	CondReconfigToAddModList

The IE CondReconfigToAddModList concerns a list of conditional reconfigurations to add or modify, with for each entry the condReconfigId and the associated condExecutionCond and condRRCReconfig.

CondReconfigToAddModList information element

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Cond condReconfigAdd

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

... ,

candidateTargetCellTimingInfo-r17   FFS                                                     OPTIONAL,   -- Need R

}

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

CondReconfigToAddMod field descriptions

condExecutionCond The execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject.

condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config.

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

... ,

candidateTargetCellTimingInfo-r17   FFS                                                     OPTIONAL,   -- Need R

}

--------------TP1__________________________________________________

As stated in the agreement, after this time the actual CHO event is evaluated which is then with current framework one or two events (MeasID) configured for that candidate target cell. Note that UE can be configured with up to 8 candidate target cells that may be same or different as per current signalling structure. For the purpose of informing UE about candidate cell being available a timer is good enough.

For the purpose of informing the UE when a candidate target cell becomes available and thus can be detected and measured, a timer is good enough.

In a feeder link switch scenario, once replacing cell is available, potentially a large number of UEs will attemt the HO at same time with this timer information about candidate target cell and RSRP related event the CHO

If we consider the feeder/service link switch in the (quasi) Earth-fixed cell scenario when a large number of UEs perform handover to the same cell within a very short time period, the reliance to accurate timing information may be crucial.

That is, to allow CHO distribution over time, and by that not exceeding the RACH capacity in the target cell, each UE need to perform the handover to the target candidate cell at the time as indicated in the timing information. CHO also needs to be performed before the candidate cell stops serving the area in order to avoid service interruptions

Due to limited RACH capacity in the cell taking over the area, it was concluded in the SI that CHO need to be distributed over time.

UE configured with CHO need to perform HO before serving cell stops serving the area in order to avoid service interruption

For this purpose, the accuracy of a timer is not good enough.

In order to distribute the Conditional Handover over time and by that avoid peak signaling load in the target cell, the network would need to determine when each UE performs the HO. By configuring an exact time when the UE shall perform the CHO, this can be achieved.

As an alternative solution, an indication in the CHO configuration may request the UE to (autonomously) subtract a random value from the remaining serving cell time whereby the UE performs the Conditional Handover to the indicated candidate target cell.

Likely there are many different alternative solutions to this issue and pros and cons to the different options can be discussed in due time.

Define an  event with the enter condition a time expressed as absolute time, or in system frame number, when the UE is to perform the CHO to the target cell.

In fact, when a target cell is going to replace the same coverage area as the serving cell (as in the (quasi) Earth-fixed cell scenario) the majority of the UEs do not need to perform RSRP measurements on that cell. Actually, demanding RSRP measurements may delay the CHO and cause timing ambiguity on when UEs are accessing the RACH resources in the candidate target cell. However, if the network prefers to configure also RSRP measurements it is possible even time only event would be allowed. Further we propose that the event does not inlcude  hysteresis or TTT.

The time event does not include TTT or hysteresis.

The time event has entry condition only.

## 2.2	Location based CHO trigger event

Location based CHO trigger events have been discussed at the last two RAN2 meetings and a number of potential trigger definitions have been identified whereas the following have gained most support:

- The distance between the UE and the centre of the serving cell. A radius defines a circle around the serving cell.

- The distance between the UE and the centre of a candidate target cell. A radius defines a circle around the target candidate cell.

- The difference in distance between “the UE and the centre of the serving cell” and “the UE and the centre of the candidate target cell”. A hexagonal cell area is defined.

The agreement taken at the RAN2#113bis meeting “The location in location-based CHO execution triggering for NTN describes the distance between the UE and the reference location of the cell (serving cell or the target cell).” then only in practice exclude the last option above.

An elliptical shape around a given reference point will of course describe the cell area in a more accurate way compared to a circle (or a hexagonal cell area), although it would require a bit more data to describe the area. As the shape of the cell area also changes with the satellite movement (elevation angle change), the information provided to the UE describing the cell area (reference point, radius etc.), also need to be constantly updated.

Parts of the information describing the cell area can likely be provided in RRC configuration (e.g. in system information), but this very much depends on what information will be included in ephemeris.

If the reference for the location trigger is serving cell center, it should be places in MeasConfig and if it is with respect to candidate target cell it should be in ReportConfigNR. In the running CR submitted to Ran2#114, it is placed for now in ReportConfigNR.

Further, if the reference point is either serving or candidate target is available in Ephemeris data then pointers there should be used.

Decide if  the location based CHO trigger is with respect to the centre of the serving cell or the target cell.

What information to be provided in CHO configuration, system information etc need to await further progress in ephemeris discussions.

In addition, the event definition in TS 38.331 Clause 5.5.4 needs to consider whether TTT and hysteresis are implemented and whether a threshold with offset is used. Our proposal is to use TTT and hysteresis in order not to trigger the measurement report or the CHO if UE does not stay beyond the threshold for a defined amount of time and to use hysteresis in order to avoid the ping pong effect. Further, in expressing the location, a threshold with offset should be used. That is to use threshold to give the distance from the reference location with a certain accuracy and then use offset to fine tune value.

The location event includes hysteresis and TTT

In addition to event based trigger, the network may configure a UE to report its location to the network periodically. If this period is sparse enough it avoids UE to keep constant up to date of its location. Disadvantage is that it consumes uplink resources. Regardless, it provides an option for the UE capability with respect to maintaining up to date location.

Periodic reporting of UE’s location may provide an option where UE does not need to maintain location info constantly.

RAN2 to discuss the feasibility of periodic location reporting as an addition to the event triggered based.

# 3 	SMTC and measurement gaps

RAN2 has agreed to configure the UE with SMTC configurations that are per list of cells: This as such exists in TS 38.331 enabling up to 4 different SMTC periodicities:

SSB-MTC3List-r16::=                 SEQUENCE (SIZE(1..4)) OF SSB-MTC3-r16

SSB-MTC3-r16 ::=                    SEQUENCE {

periodicityAndOffset-r16            CHOICE {

sf5-r16                                     INTEGER (0..4),

sf10-r16                                    INTEGER (0..9),

sf20-r16                                    INTEGER (0..19),

sf40-r16                                    INTEGER (0..39),

sf80-r16                                    INTEGER (0..79),

sf160-r16                                   INTEGER (0..159),

sf320-r16                                   INTEGER (0..319),

sf640-r16                                   INTEGER (0..639),

sf1280-r16                                  INTEGER (0..1279)

},

duration-r16                        ENUMERATED {sf1, sf2, sf3, sf4, sf5},

pci-List-r16                        SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                   OPTIONAL,  -- Need M

ssb-ToMeasure-r16                   SetupRelease { SSB-ToMeasure }                                          OPTIONAL   -- Need M

}

RAN2 had the following open questions to discuss further:

FFS the following open questions:

(a) can the UE be configured with multiple SMTCs per carrier and use them all in parallel?

(b) How the NW knows which SMTC (incl. offsets/periodicity, etc.) is relevant for a particular UE?

(c) Is there any validity: in time or for certain location only, foreseen in such multiple SMTC configuration?

(d) What is the potential impact on the signalling, assuming this delay is a dynamic value?

(e) What about the feeder link delay? Is it considered anywhere?

Question a) seems to be answered as this exists already in Rel-16. Question b) is not about the SMTC configuration but about e.g. location reports from the UE. Note that earlier RAN2 agreed network does not have to keep track on UE location. Regardless, reported UE location does help the network to configure nthe SMTC and measurement gaps correctly. Thus, should further discuss which are the different ways UE location can be received by the network. There could for example be request and response type of location reporting.

The reported UE location is important in order the network to configure SMTC and measurement gap correctly. Note that this is inline with earlier agreement that does not require network to keep track on UE location.

RAN2 to discuss request and response type of UE location reporting to assist SMTC and measurement gap configuration.

For question c) and d) our view is that it would be good to configure a predetermined offset scaling for the UE with certain timing rule per RRC. Then, if the UE moves more than e.g. 100km network should change the rule. That is, UE move that causes propagation delay to change 0.5-1ms then updated signalling is needed.RAN2 should agree that network can configure a predetermined offset scaling for the UE with certain timing rule per RRC.

# 4	RACH-less Handover

RACH-less handover was introduced as part of the mobility enhancements in LTE Rel-14 and consists of that Msg1 and Msg2 are skipped in the target cell. The UE’s first transmission in the target cell is instead the message confirming the completion of the handover, i.e. RRCConnectionReconfigurationComplete in LTE and RRCReconfigurationComplete in NR, i.e. the RRC message which is normally included in Msg3 in a regular RACH-based handover.

UL transmission resources for UE’s first message in the target cell are allocated either through pre-allocated UL grants or UL grants dynamically allocated through PDCCH in the target cell without preceding scheduling request due to the missing random access procedure in the target cell. If pre-allocated UL grants are configured, these are periodic UL transmission resources allocated in the RRCConnectionReconfiguration or RRCReconfiguration message constituting the HO Command. The pre-allocated UL grants potentially achieves shorter interruption, since the PDCCH allocation adds some additional delay, but PDCCH allocation is more flexible.

Since the random access procedure, and therefore also the Random Access Response message is skipped in the target cell, the target gNB cannot determine a precise timing advance (TA) for the UE. Hence, RACH-less HO is limited to scenarios where the TA is known in advance. In practice, this means scenarios where the TA in the target cell is the same as in UE’s source cell or when the target cells is so small that the TA can be assumed to always be 0. Since the coverage area of a satellite is likely to include several cells, all intra satellite HO will experience the same TA value and thus even in this case the RACH-less HO procedure could be used. The assumption that all NTN UE´s are GNSS capable could allow the UE to estimate its TA to the source satellite but also the TA to the target cell in an inter satellite HO and even in this case, the use of RACH-less HO with a TA based on UE estimates should be considered for NTN.

Even with a PRACH configuration that maximizes the available PRACH opportunities in a cell, the collision rate will be high. Adding attach attempts from other UEs than the ones that are doing HO, the collision rate will increase even further. This means that in order to support mobility, the signaling needs to be reduced.

It is believed that the RACH capacity will be limited even in sparsely populated areas due to the large supported cells in NTN. RACH-less will offload the PRACH resources and thus reduce the collision rate. It also has the potential to reduce the interruption time since it is possible to use dense pre-allocated grants in the HO command.

Our companion contribution [8] discusses further details of the RACH-less HO and it is preferred that detailed discussion takes place under UP agenda rather than CP agenda.

The assumption that all NTN UE´s are GNSS capable could allow the UE to estimate its TA to the source satellite but also the TA to the target cell in an inter satellite HO and even in this case, the use of RACH-less HO with a TA based on UE estimates seems feasible.

The PRACH collision rate will be high in NTN systems and ways to limit this especially for cases where there is expected to be large number of HOs like feeder/service link switch, ways to limit this is definitely needed.

RACH capacity will be limited as seen during SI phase and the available RACH capacity needs to consider, initial access, L3 mobility as well as L1 beam management(BFR) and RLF caused access attempts.

RAN2 should support RACH-less HO in Rel-17 WI for NTN.

Further discuss the details under user plane agenda item.

# 5	Conclusion

In the previous sections we made the following observations:

Observation 1	Current agreement gives information about when a target cell becomes available and after which UE can consider CHO to that cell. However, this does not as such trigger CHO.

Observation 2	For the purpose of informing the UE when a candidate target cell becomes available and thus can be detected and measured, a timer is good enough.

Observation 3	In a feeder link switch scenario, once replacing cell is available, potentially a large number of UEs will attemt the HO at same time with this timer information about candidate target cell and RSRP related event the CHO

Observation 4	Due to limited RACH capacity in the cell taking over the area, it was concluded in the SI that CHO need to be distributed over time.

Observation 5	UE configured with CHO need to perform HO before serving cell stops serving the area in order to avoid service interruption

Observation 6	Periodic reporting of UE’s location may provide an option where UE does not need to maintain location info constantly.

Observation 7	The reported UE location is important in order the network to configure SMTC and measurement gap correctly. Note that this is inline with earlier agreement that does not require network to keep track on UE location.

Observation 8	The assumption that all NTN UE´s are GNSS capable could allow the UE to estimate its TA to the source satellite but also the TA to the target cell in an inter satellite HO and even in this case, the use of RACH-less HO with a TA based on UE estimates seems feasible.

Observation 9	The PRACH collision rate will be high in NTN systems and ways to limit this especially for cases where there is expected to be large number of HOs like feeder/service link switch, ways to limit this is definitely needed.

Observation 10	RACH capacity will be limited as seen during SI phase and the available RACH capacity needs to consider, initial access, L3 mobility as well as L1 beam management(BFR) and RLF caused access attempts.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Define an  event with the enter condition a time expressed as absolute time, or in system frame number, when the UE is to perform the CHO to the target cell.

Proposal 2	The time event does not include TTT or hysteresis.

Proposal 3	The time event has entry condition only.

Proposal 4	Decide if  the location based CHO trigger is with respect to the centre of the serving cell or the target cell.

Proposal 5	What information to be provided in CHO configuration, system information etc need to await further progress in ephemeris discussions.

Proposal 6	The location event includes hysteresis and TTT

Proposal 7	RAN2 to discuss the feasibility of periodic location reporting as an addition to the event triggered based.

Proposal 8	RAN2 to discuss request and response type of UE location reporting to assist SMTC and measurement gap configuration.

Proposal 9	For question c) and d) our view is that it would be good to configure a predetermined offset scaling for the UE with certain timing rule per RRC. Then, if the UE moves more than e.g. 100km network should change the rule. That is, UE move that causes propagation delay to change 0.5-1ms then updated signalling is needed.RAN2 should agree that network can configure a predetermined offset scaling for the UE with certain timing rule per RRC.

Proposal 10	RAN2 should support RACH-less HO in Rel-17 WI for NTN.

Proposal 11	Further discuss the details under user plane agenda item.

# 6	References

RP-201256, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #88e, June 2020.

R2-21xxxxx, “On Random Access in NTNs” Ericsson

R2-2102045, Report of [AT113-e][106][NTN] CHO aspects (Ericsson)

# Appendix A

------------------------------------ start TP 1------------------------------------------------------------

##### 5.3.5.13.4	Conditional reconfiguration evaluation

The UE shall:

1>	for each condReconfigId within the VarConditionalReconfig:

2>	consider the cell which has a physical cell identity matching the value indicated in the ServingCellConfigCommon included in the reconfigurationWithSync in the received condRRCReconfig to be applicable cell;

2>	for each measId included in the measIdList within VarMeasConfig indicated in the condExecutionCond associated to condReconfigId:

3>	if the entry condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

4>	consider the event associated to that measId to be fulfilled;

3>	if the leaving condition(s) applicable for this event associated with the condReconfigId, i.e. the event corresponding with the condEventId(s) of the corresponding condTriggerConfig within VarConditionalReconfig, is fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding timeToTrigger defined for this event within the VarConditionalReconfig:

4>	consider the event associated to that measId to be not fulfilled;

2>	if the condTriggerConfig within the VarConditionalReconfig includes the EventEvaluationConfig:

3>	if the EventEvaluationConfig is set to anyEvent:

4>	if one event associated to any measId within condTriggerConfig for a target candidate cell within the stored condRRCReconfig is fulfilled:

5>	consider the target candidate cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell;

5>	initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;

2>	else if event(s) associated to all measId(s) within condTriggerConfig for a target candidate cell within the stored condRRCReconfig are fulfilled:

3>	consider the target candidate cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell;

3>	initiate the conditional rconfiguration execution, as specified in 5.3.5.13.5;

NOTE:	Up to 2 MeasId can be configured for each condReconfigId. The conditional reconfiguration event of the 2 MeasId may have the same or different event conditions, triggering quantity, time to trigger, and triggering threshold.

------------------------------------ end TP 1------------------------------------------------------------

------------------------------------ start TP 2------------------------------------------------------------

#### 5.5.4.xx	Event L1 (The UE location is beyond a threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition L1-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition L1-2, as specified below, is fulfilled;

Inequality L1-1 (Entering condition)

Inequality L1-2 (Leaving condition)

The variables in the formula are defined as follows:

Ms is the UE location, not taking into account any offsets.

Hys is the hysteresis parameter (i.e. l1-Hysteresis as defined within ReportConfigNR) for this event.

Thresh is the reference threshold parameter for this event given in MeasConfig(i.e. locationThreshRef as defined within MeasConfig).

Offset is the offset value to locationThreshRef to obtain the absolute threshold for this event. (i.e. l1-ThresholdOffset as defined within ReportConfigNR)

Ms is expressed in meters.

Thresh is expressed in the same unit as Ms.

NOTE 2:	The definition of Event L1 also applies to CondEvent L1.

#### 5.5.4.14	 CondEvent T1 (Absolute time has passed a threshold time)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition T1-1, as specified below, is fulfilled;

Inequality T1-1 (Entering condition)

Definition FFS

------------------------------------ end TP 2------------------------------------------------------------

------------------------------------ start TP 3------------------------------------------------------------

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEASCONFIG-START

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                                              OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                                              OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList                                            OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList                                            OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList                                                  OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList                                                  OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                                       OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                                                      OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                                                       OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                                                OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                                                   OPTIONAL    -- Need R

]],

[[		locationThreshRef-r17	FFS		OPTIONAL	--Need ON

]]

}

MeasObjectToRemoveList ::=              SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=                  SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=            SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEASCONFIG-STOP

-- ASN1STOP

MeasConfig field descriptions

interFrequencyConfig-NoGap-r16 If the field is set to true, UE is configured to perform SSB based inter-frequency measurement without measurement gaps when the inter-frequency SSB is completely contained in the active DL BWP of the UE, as specified in TS 38.133 [14], clause 9.3. Otherwise, the SSB based inter-frequency measurement is performed within measurement gaps.

locationThreshRef Reference location threshold for eventL1 in reportConfigNR. The locationThreshRef is a distance from a cell center reference location(definition FFS) --Editors note: definition and for cell center reference location is FFS and it is assumed it is given is SI or elsewhere thus not added here--

measGapConfig Used to setup and release measurement gaps in NR.

measIdToAddModList List of measurement identities to add and/or modify.

measIdToRemoveList List of measurement identities to remove.

measObjectToAddModList List of measurement objects to add and/or modify.

measObjectToRemoveList List of measurement objects to remove.

reportConfigToAddModList List of measurement reporting configurations to add and/or modify.

reportConfigToRemoveList List of measurement reporting configurations to remove.

s-MeasureConfig Threshold for NR SpCell RSRP measurement controlling when the UE is required to perform measurements on non-serving cells. Choice of ssb-RSRP corresponds to cell RSRP based on SS/PBCH block and choice of csi-RSRP corresponds to cell RSRP of CSI-RS.

measGapSharingConfig Specifies the measurement gap sharing scheme and controls setup/ release of measurement gap sharing.

#### –	ReportConfigNR

The IE ReportConfigNR specifies criteria for triggering of an NR measurement reporting event or of a CHO or CPC event. For events labelled AN with N equal to 1, 2 and so on, measurement reporting events and CHO or CPC events are based on cell measurement results, which can either be derived based on SS/PBCH block or CSI-RS.

Event A1:	Serving becomes better than absolute threshold;

Event A2:	Serving becomes worse than absolute threshold;

Event A3:	Neighbour becomes amount of offset better than PCell/PSCell;

Event A4:	Neighbour becomes better than absolute threshold;

Event A5:	PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2;

Event A6:	Neighbour becomes amount of offset better than SCell;

Event L1:	UE location becomes further than threshold;

CondEvent A3: Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell;

CondEvent A5: PCell/PSCell becomes worse than absolute threshold1 AND Conditional reconfiguration candidate becomes better than another absolute threshold2;

CondEvent T1: Absolute time is more than configured threshold;

For event I1, measurement reporting event is based on CLI measurement results, which can either be derived based on SRS-RSRP or CLI-RSSI.

Event I1:	Interference becomes higher than absolute threshold.

ReportConfigNR information element

-- ASN1START

-- TAG-REPORTCONFIGNR-START

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16

}

}

ReportCGI ::=                     SEQUENCE {

cellForWhichToReportCGI          PhysCellId,

...,

[[

useAutonomousGaps-r16            ENUMERATED {setup}                OPTIONAL   -- Need R

]]

}

ReportSFTD-NR ::=                 SEQUENCE {

reportSFTD-Meas                  BOOLEAN,

reportRSRP                       BOOLEAN,

...,

[[

reportSFTD-NeighMeas             ENUMERATED {true}                                OPTIONAL,   -- Need R

drx-SFTD-NeighMeas               ENUMERATED {true}                                OPTIONAL,   -- Need R

cellsForWhichToReportSFTD        SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellId   OPTIONAL    -- Need R

]]

}

CondTriggerConfig-r16 ::=        SEQUENCE {

condEventId                      CHOICE {

condEventA3                      SEQUENCE {

a3-Offset                        MeasTriggerQuantityOffset,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

condEventA5                      SEQUENCE {

a5-Threshold1                    MeasTriggerQuantity,

a5-Threshold2                    MeasTriggerQuantity,

hysteresis                       Hysteresis,

timeToTrigger                    TimeToTrigger

},

...,

[[

condEventL1-r17							SEQUENCE {

l1-ThresholdOffset-r17			MeasTriggerLocation,

l1-Hysteresis-17			    FFS,

timeToTrigger                   TimeToTrigger

},

condEventT1-r17							SEQUENCE {

t1XXX-r17			FFS

}

]]

},

rsType-r16                       NR-RS-Type,

...

}

EventTriggerConfig::=                       SEQUENCE {

eventId                                     CHOICE {

eventA1                                     SEQUENCE {

a1-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA2                                     SEQUENCE {

a2-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger

},

eventA3                                     SEQUENCE {

a3-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA4                                     SEQUENCE {

a4-Threshold                                MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA5                                     SEQUENCE {

a5-Threshold1                               MeasTriggerQuantity,

a5-Threshold2                               MeasTriggerQuantity,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

eventA6                                     SEQUENCE {

a6-Offset                                   MeasTriggerQuantityOffset,

reportOnLeave                               BOOLEAN,

hysteresis                                  Hysteresis,

timeToTrigger                               TimeToTrigger,

useWhiteCellList                            BOOLEAN

},

...,

[[

eventL1-r17							SEQUENCE {

l1-ThresholdOffset-r17			MeasTriggerLocation,

l1-Hysteresis-17			    FFS,

timeToTrigger                   TimeToTrigger

]]

},

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                     MeasReportQuantity                                            OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

reportAddNeighMeas                          ENUMERATED {setup}                                             OPTIONAL,   -- Need R

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

useT312-r16                                 BOOLEAN                                                        OPTIONAL,   -- Need M

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL    -- Need M

]]

}

PeriodicalReportConfig ::=                  SEQUENCE {

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                    MeasReportQuantity                                             OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

useWhiteCellList                            BOOLEAN,

...,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL,   -- Need M

ul-DelayValueConfig-r16                     SetupRelease { UL-DelayValueConfig-r16 }                       OPTIONAL,   -- Need M

reportAddNeighMeas-r16                      ENUMERATED {setup}                                             OPTIONAL    -- Need R

]]

}

NR-RS-Type ::=                              ENUMERATED {ssb, csi-rs}

MeasTriggerQuantity ::=                     CHOICE {

rsrp                                        RSRP-Range,

rsrq                                        RSRQ-Range,

sinr                                        SINR-Range

}

MeasTriggerQuantityOffset ::=               CHOICE {

rsrp                                        INTEGER (-30..30),

rsrq                                        INTEGER (-30..30),

sinr                                        INTEGER (-30..30)

}

MeasReportQuantity ::=                      SEQUENCE {

rsrp                                        BOOLEAN,

rsrq                                        BOOLEAN,

sinr                                        BOOLEAN

}

MeasRSSI-ReportConfig-r16 ::=               SEQUENCE {

channelOccupancyThreshold-r16               RSSI-Range-r16         OPTIONAL   -- Need R

}

CLI-EventTriggerConfig-r16 ::=              SEQUENCE {

eventId-r16                                 CHOICE {

eventI1-r16                                 SEQUENCE {

i1-Threshold-r16                            MeasTriggerQuantityCLI-r16,

reportOnLeave-r16                           BOOLEAN,

hysteresis-r16                              Hysteresis,

timeToTrigger-r16                           TimeToTrigger

},

...

},

reportInterval-r16                          ReportInterval,

reportAmount-r16                            ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

maxReportCLI-r16                            INTEGER (1..maxCLI-Report-r16),

...

}

CLI-PeriodicalReportConfig-r16 ::=          SEQUENCE {

reportInterval-r16                          ReportInterval,

reportAmount-r16                            ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCLI-r16                       MeasReportQuantityCLI-r16,

maxReportCLI-r16                            INTEGER (1..maxCLI-Report-r16),

...

}

MeasTriggerQuantityCLI-r16 ::=              CHOICE {

srs-RSRP-r16                                SRS-RSRP-Range-r16,

cli-RSSI-r16                                CLI-RSSI-Range-r16

}

MeasReportQuantityCLI-r16 ::=               ENUMERATED {srs-rsrp, cli-rssi}

MeasTriggerLocation-r17 ::=                 SEQUENCE {

cellReferenceLocation                     FFS,

distanceFromReference                     FFS

}

-- TAG-REPORTCONFIGNR-STOP

-- ASN1STOP

CondTriggerConfig field descriptions

a3-Offset Offset value(s) to be used in NR conditional reconfiguration triggering condition for cond event a3. The actual value is field value * 0.5 dB.

a5-Threshold1/ a5-Threshold2 Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR conditional reconfiguration triggering condition for cond event a5. In the same condeventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

condEventId Choice of NR conditional reconfiguration event triggered criteria.

timeToTrigger Time during which specific criteria for the event needs to be met in order to execute the conditional reconfiguration evaluation.

t1XXX

ReportConfigNR field descriptions

reportType Type of the configured measurement report. In EN-DC, network does not configure report of type reportCGI using SRB3. The condTriggerConfig is used for CHO or CPC configuration.

ReportCGI field descriptions

useAutonomousGaps Indicates whether or not the UE is allowed to use autonomous gaps in acquiring system information from the NR neighbour cell. When the field is included, the UE applies the corresponding value for T321.

EventTriggerConfig field descriptions

a3-Offset/a6-Offset Offset value(s) to be used in NR measurement report triggering condition for event a3/a6. The actual value is field value * 0.5 dB.

aN-ThresholdM Threshold value associated to the selected trigger quantity (e.g. RSRP, RSRQ, SINR) per RS Type (e.g. SS/PBCH block, CSI-RS) to be used in NR measurement report triggering condition for event number aN. If multiple thresholds are defined for event number aN, the thresholds are differentiated by M. The network configures aN-Threshold1 only for events A1, A2, A4, A5 and a5-Threshold2 only for event A5. In the same eventA5, the network configures the same quantity for the MeasTriggerQuantity of the a5-Threshold1 and for the MeasTriggerQuantity of the a5-Threshold2.

channelOccupancyThreshold RSSI threshold which is used for channel occupancy evaluation.

eventId Choice of NR event triggered reporting criteria.

l1-ThresholdOffset An offset value to locationThreshRef to obtain the threshold to be used in for event L1 and CondEvent L1

maxNrofRS-IndexesToReport Max number of RS indexes to include in the measurement report for A1-A6 events.

maxReportCells Max number of non-serving cells to include in the measurement report.

reportAddNeighMeas Indicates that the UE shall include the best neighbour cells per serving frequency.

reportAmount Number of measurement reports applicable for eventTriggered as well as for periodical report types.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a cell in cellsTriggeredList, as specified in 5.5.4.1.

reportQuantityCell The cell measurement quantities to be included in the measurement report.

reportQuantityRS-Indexes Indicates which measurement information per RS index the UE shall include in the measurement report.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

ul-DelayValueConfig If the field is present, the UE shall perform the actual PDCP queueing delay measurement per DRB as specified in TS 38.314 [53] and the UE shall ignore the fields reportQuantityCell and maxReportCells. The applicable values for the corresponding reportInterval are (one of the) {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30}. The reportInterval indicates the periodicity for performing and reporting of UL PDCP Delay per DRB measurement as specified in TS 38.314 [53].

useT312 If value TRUE is configured, the UE shall use the timer T312 with the value t312 as specified in the corresponding measObjectNR. If value FALSE is configured, the timer T312 is considered as disabled. Network configures value TRUE only if reportType is set to eventTriggered.

useWhiteCellList Indicates whether only the cells included in the white-list of the associated measObject are applicable as specified in 5.5.4.1.

CLI-EventTriggerConfig field descriptions

i1-Threshold Threshold value associated to the selected trigger quantity (e.g. SRS-RSRP, CLI-RSSI) to be used in CLI measurement report triggering condition for event i1.

eventId Choice of CLI event triggered reporting criteria.

maxReportCLI Max number of CLI measurement resource to include in the measurement report.

reportAmount Number of measurement reports.

reportOnLeave Indicates whether or not the UE shall initiate the measurement reporting procedure when the leaving condition is met for a CLI measurement resource in srsTriggeredList or rssiTriggeredList, as specified in 5.5.4.1.

timeToTrigger Time during which specific criteria for the event needs to be met in order to trigger a measurement report.

CLI-PeriodicalReportConfig field descriptions

maxReportCLI Max number of CLI measurement resource to include in the measurement report.

reportAmount Number of measurement reports.

reportQuantityCLI The CLI measurement quantities to be included in the measurement report.

------------------------------------ end TP 3------------------------------------------------------------

# Appendix B

Agreements related to connected mode from RAN2#102:

Agreements:

- RAN2 to consider the case where gNB is co-located at the GW with higher priority.

- RAN2 will continue working with the assumption that service link switch implies L3 mobility (meaning that at least in case the SSBs are on the same sync raster point the PCIs need to be different). Check if an LS to RAN1 asking for feasibility of having same PCI as well can be agreed

Agreements

- Reconfiguration with sync is the baseline for connected mode mobility in NTN (the use of legacy RLF and re-establishment mechanism are not excluded)

- The CHO can be used in NTN for both moving cell and fixed cell scenarios, and the CHO procedure and execution condition defined in Rel-16 is the baseline for NTN CHO.

3.	NTN specific CHO execution condition can be further discussed.

4.	The existing measurement framework (e.g. measurement configuration, execution and reporting) is the baseline, and all the existing measurement criteria and event can be used in NTN. Support for new measurement is not excluded.

5.	Legacy SSB periods (as in TN) shall be supported in NTN

Agreements via email - offline 105:

- Time or timer based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario.  FFS on how to configure the time or timer based CHO triggering event. Also FFS how to consider the feeder/service link switch timing.

- DAPS HO for NTN is de-prioritized in this release.

- Location based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario. FFS on how to configure the location based CHO triggering event. FFS if location based CHO triggering event only (not in combination with other events) can also be considered.

- The Location-based measurement event, in combination with the existing measurement event in NR, should be supported in NTN for both moving cell and fixed cell scenarios. FFS on how to configure the location based measurement event.

Agreements via email - offline 106:

- RAN2 understanding that UE shall not be forced to detect the SSB burst outside the corresponding configured SMTC window in NTN, just like the principle in TN.

Agreements:

- SMTC and gap configuration in NTN are configured based on the timing of PCell

- RAN2 can first identify the scenarios and discuss how serious the impact is before addressing any enhancement for SMTC configuration in NTN.

- RAN2 can’t assume that the network will always have UE accurate location info for SMTC window configuration in NTN

- UE along with the network in NTN should also have the same understanding of the timing, including the timing for measurement gap, to avoid any un-synchronized scheduling between UE and the network, just like the way we have in TN

RAN2#113 agreements:

Agreements:

- Support A4 event for NTN CHO. FFS whether other triggers need to be combined with this.

RAN2#113bis agreements:

Agreements:

1.	Timing information in CHO execution triggering for NTN describes the time after which the UE is allowed to execute CHO to the candidate target cell.

2.	Working assumption: the timing information for CHO execution triggering in NTN is defined in the form of a timer/timers. This can be revised and a solution based on UTC/system frame number can be considered if problems are found (e.g. if the timer lacks accuracy due to RTT in NTN).

3.	The location in location-based CHO execution triggering for NTN describes the distance between the UE and the reference location of the cell (serving cell or the target cell). FFS what the reference location of the cell is (e.g cell center or other) and how this is provided to the UE

Agreements:

- For Rel-17 NTN, Rel-17 NR operation is enhanced (e.g. the SMTC configuration and UE measurement gap onfiguration) aiming to address the issues associated with the different/larger propagation delays, and the satellites (considering e.g. their deployment, mobility, height, minimum elevation and prioritizing typical NTN scenarios).

- Rel-17 NTN will not rely only on network implementation to address the issue explained in agreement 1.

- Enhancements of the SMTC configuration is supported for Rel-17 NTN.

- Optional new UE assistance is defined in Rel-17 NTN for network to properly (re)configure the SMTC and/or measurement gap

Agreements - via email (from offline [106])

- For Rel-17 NTN, one or more SMTC configuration(s) associated to one frequency can be configured. FFS solution details.

-	The SMTC configuration can be associated with a set of cells (e.g., per satellite or any other suitable set per gNB determination).

-	The multiple SMTC configurations are enabled by introducing different new offsets in addition to the legacy SMTC configuration. FFS how the offsets will be managed/signalled.

FFS the following open questions:

(a) can the UE be configured with multiple SMTCs per carrier and use them all in parallel?

(b) How the NW knows which SMTC (incl. offsets/periodicity, etc.) is relevant for a particular UE?

(c) Is there any validity: in time or for certain location only, foreseen in such multiple SMTC configuration?

(d) What is the potential impact on the signalling, assuming this delay is a dynamic value?

(e) What about the feeder link delay? Is it considered anywhere?

- The configuration of one or multiple offsets is left up to the network implementation.

- It is up to network to update the SMTC configuration of the UE to accommodate the different propagation delays.

Agreements online:

- Measurement gaps enhancements should be supported. FFS on the details