3GPP TSG-RAN WG2 #114-e	Tdoc R2-2105983

Electronic meeting, May 19 – 27, 2021

Agenda Item:	5.4.3

Source:	Ericsson

Title:	Allowed bandwidth in BWP configuration

Document for:	Discussion, Decision

# 1	Introduction

When configuring BWPs, each configured BWP should be within the limits of the channel bandwidth supported by the UE. But it is not clear whether the union of the configured BWPs’ frequency domain resources must be within the channel bandwidth supported by the UE. This contribution discusses this scenario and the impact to BWP configuration.

# 2	Discussion

To further describe this case, we can take an example:

A gNB operates a 30 MHz wide carrier with an initial BWP of 20 MHz and a CORESET#0 of 10 MHz (see Figure 1). Upon initial access, a UE supporting 20 MHz channel bandwidth (but not 30 MHz) reads MIB and SIB1 and determines based on the information therein that it may access the cell using the BWP#0. Based on the UE capabilities the gNB knows that it may not configure more than 20 MHz. But to spread the load on the carrier it may attempt to configure a BWP#1 (also 20 MHz wide and with the same SCS as BWP#0) spanning over the 10 MHz that are within the carrier but outside the BWP#0.:

Figure 1 – Both BWP#0 and BWP#1 are 20 MHz wide.

Hence, despite both BWP#0 and BWP#1 being configured according to what the UE supports in terms of channel bandwidth, it is not clear whether those BWPs should be further bounded by the union of the frequency region from BWP#0 and BWP#1.

Considering the changes agreed in principle in [1][2], the definition of BWPs in TS 38.306 should capture the following limitations:

bwp-DiffNumerology Indicates whether the UE supports BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer. Except for SUL, the UE only supports the same numerology for the active UL and DL BWP. For the UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell and PSCell (if configured). For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s).

bwp-SameNumerology Indicates whether UE supports BWP adaptation (up to 2/4 BWPs) with the same numerology, via DCI and timer. Except for SUL, the UE only supports the same numerology for the active UL and DL BWP. For the UE capable of this feature, the bandwidth of a UE-specific RRC configured DL BWP includes the bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB for PCell and PSCell (if configured). For SCell(s), the bandwidth of the UE-specific RRC configured DL BWP includes SSB, if there is SSB on SCell(s).

The configuration depicted in Figure 1 fulfils the requirements highlighted in the field description. From that perspective, the configuration described in the example above is allowed.

Confirm that the union of the bandwidths of the configured (initial + dedicated) BWPs may exceed the maximum channel bandwidth supported by the UE.

According to 38.331 the UE determines the applicable channel/carrier bandwidth ...

- either from ServingCellConfig-> downlinkChannelBW-PerSCS-List

- or from ServingCellConfigCommon(SIB)-> downlinkConfigCommon-> frequencyInfoDL (if the former is not provided)

In the example shown above, SIB1 indicates a carrierBandwidth of 30 MHz. When the UE accesses the cell, it applies the procedural text in section “5.2.2.4.2 Actions upon reception of the SIB1”:

4>	apply a supported uplink channel bandwidth with a maximum transmission bandwidth which

-	is contained within the carrierBandwidth indicated in uplinkConfigCommon for the SCS of the initial uplink BWP, and which

-	is wider than or equal to the bandwidth of the initial BWP for the uplink;

4>	apply a supported downlink channel bandwidth with a maximum transmission bandwidth which

- is contained within the carrierBandwidth indicated in downlinkConfigCommon for the SCS of the initial downlink BWP, and which

- is wider than or equal to the bandwidth of the initial BWP for the downlink;

That means, the UE will, even though SIB1 indicates that the carrier is 30 MHz wide, apply a channel bandwidth of 20 MHz and position its filters so that it can acquire the DL- and UL initial BWP.

When the gNB configures the above-mentioned BWP#1, the channel configuration previously determined by the UE would prevent the UE from acquiring the DL BWP#1 and from transmitting on the UL BWP#1. Of course, the UE could apply a similar rule as defined for the initial access, i.e., apply a transmission bandwidth that covers the BWP#1 for UL and DL. However, since there is no such procedural text in 38.331 and since there is the downlinkChannelBW-PerSCS-List field in ServingCellConfig common, it seems reasonable that the gNB should configure the appropriate channel bandwidth and position therein.

The network could provide the downlinkChannelBW-PerSCS-List in the RRCReconfiguration message in which it configures the BWP#1 and in which it commands the UE to switch to BWP#1.

When configuring a UE with a dedicated BWP that is not within the channel bandwidth that the UE applied when acquiring SIB1, the network may (and probably should) configure the downlinkChannelBW-PerSCS-List and/or uplinkChannelBW-PerSCS-List appropriately.

However, when the UE switches to another BWP (here to BWP#0) by means of a timer or triggered by a DCI the previously configured channel bandwidth would prevent the UE from receiving or transmitting on that non-overlapping BWP#0. This raises the question whether timer- and DCI- based BWP switching must be avoided in such cases or whether the UE is required to autonomously release the configured downlinkChannelBW-PerSCS-List and uplinkChannelBW-PerSCS-List parameters.

Discuss whether and how a UE supports switching to a BWP which is not within the configured channel bandwidth (down-/uplinkChannelBW-PerSCS-List).

If RAN2 concludes that switching by DCI or timer is still possible, RAN2 should also discuss whether the adjustment of the channel bandwidth and position has also an impact on the switching delay.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	When configuring a UE with a dedicated BWP that is not within the channel bandwidth that the UE applied when acquiring SIB1, the network may (and probably should) configure the downlinkChannelBW-PerSCS-List and/or uplinkChannelBW-PerSCS-List appropriately.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Confirm that the union of the bandwidths of the configured (initial + dedicated) BWPs may exceed the maximum channel bandwidth supported by the UE.

Proposal 2	Discuss whether and how a UE supports switching to a BWP which is not within the configured channel bandwidth (down-/uplinkChannelBW-PerSCS-List).

# 4	References

R2-2104573, Correction to BWP capabilities, Nokia, Nokia Shanghai Bell, Rel-15 CR, RAN2#113bis-e, 12 - 20 April, 2021.

R2-2104574, Correction to BWP capabilities, Nokia, Nokia Shanghai Bell, Rel-16 CR, RAN2#113bis-e, 12 - 20 April, 2021.