3GPP TSG-RAN WG2 Meeting #114-e	Tdoc R2-2106169

Electronic Meeting, May 19th – 27th, 2021

Agenda Item:	9.2.3

Source:	Ericsson

Title:	Connection density evaluation for IoT NTN devices

Document for:	Discussion, Decision

- Introduction

In RAN#86, a SI was approved to determine and evaluate the minimum necessary specifications to introduce NB-IoT/eMTC support for non-terrestrial networks (NTN). The description for the SI was updated in RAN#90 [1] and it was agreed to use the existing work on NR NTN captured in TR 38.821 [2] as a baseline.

After RAN2#113e an e-mail discussion was initiated to discuss the assumptions and how to evaluate the performance, which are captured in [5].

In this contribution we follow up on the e-mail discussion outcome and provide a text proposal for paging and random access capacity and provide some preliminary results for connection density evaluation.

- Paging capacity

In the e-mail discussion [5], a set of suggested methods, assumptions and agreements were brought up. Using the outcome of the e-mail discussion as guidance, we have come up with the following parameters to calculate the paging capacity:

- number of paging occasions per paging frame determined by the RRC parameter nB. This has a maximum value of 4.

- number of paging frames per second, determined by the paging cycle configured.

- number of carriers, determined by the RRC parameter paging-narrowBands-r13 for LTE-M and maxNonAnchorCarriers-NB-r14 for NB-IoT. In LTE-M, the maximum number of paging narrowbands is 16, while in NB-IoT the maximum number of non-anchor carriers is 15, which means that the maximum number of paging carriers is 16 including the anchor carrier. We have assumed that paging weights do not affect the paging capacity.

- number of records, where the maximum number of records is 16. It should be noted that having 16 records may not be advantageous from a link budget perspective, thus for UEs in bad coverage, the number of records would probably be less in practice.

- paging area, which is composed of 1) the spotbeam on earth, 2) the spotbeam-to-PCI mapping ratio and 3) the number of cells that make up a tracking area.

- number of average paging attempts on per user. This is given by 45.820 as fractions of UEs with specific inter-arrival times: 40% of UEs have 1 day inter-arrival time, 40% - 2 hours, 15% - 1 hour and 5% 30 minutes. This works out to an average of 1.2963*10-4 pages per UE per second.

- UE density per km2. When calculating the paging load, 400 UEs per km2 is used, as defined in Table Table B.2-1 of [6].

A further set of parameters that should be considered, but not included are:

- number of paging MPDCCH/NPDCCH, determined by the RRC parameter mpdcch-NumRepetition-Paging-r13 for LTE-M and npdcch-NumRepetitionPaging-r13 for NB-IoT. When configuring a set of repetitions for NPDCCH/MPDCCH, care needs to be taken so that there are not too many repetitions. For instance, configuring 4 paging occasions per paging frame, it would not be possible to configure 128 M/NPDCCH repetitions per narrowband or paging carrier. In other words, the repetition configuration needs to be reflected in the number of paging configurations and number of paging frames per second.

Capture the text proposal on paging capacity in Annex 7.1.

- Connection density

One important aspect of a study item is to assess the use cases and performance at a system level. In this section we look at the connection density that can be achieved in an NTN. In short, the evaluation looks at the connection density achievable under the traffic assumption that the UE shall be able to deliver a 32-byte packet in the uplink under 10 seconds with an outage probably of less than 1%.

To evaluate the connection density for NTN we have chosen 2 LEO scenarios: Case 9 and Case 14 [2] which have similar characteristics with the difference that Case 9 is at 600 km altitude and Case 14 is at 1200 km altitude. These scenarios target handheld devices. The UE characteristics follow that of Section 6.2.1 in 36.763, which correspond to the IoT devices.

Some aspects to consider, as outlined in the e-mail discussion, are that, to perform the evaluation in acceptable computational times, the number of cells simulated has been selected as 19 cells with the statistics counted only for the inner 7 cells, as shown in Figure 1. Furthermore, UEs will only have 20 seconds to deliver the packet before the delivery attempt is cancelled, meaning that no further re-transmissions will be attempted after 20 seconds. This is not to be confused with the 10 seconds that determines the outage rate, in other words a UE may deliver the uplink packet after 10 seconds, but this will be counted as an outage based on the assumed criteria.  Not being able to deliver the packet at all within 20s is also counted as an outage.

Figure 1. Simulating 19 cells but only accounting for the inner 7 cells.

In order to be able to observe how uplink and downlink SINR get worse with increasing load, i.e., arrival rate, and PUSCH SINR are given in Figure 2 and PDSCH SINR percentiles are given in Figure 3.

Figure 2. PDSCH SINR percentiles as a function of the arrival rate.

Figure 3. PUSCH SINR percentiles as a function of the arrival rate.

Figure 4. The outage rate as a function of the arrival rate.

The outage rate as a function of the arrival rate can be observed for cases 9 and 14 in Figure 4. For computing the achievable connection density, we can see that the maximum arrival rate where the outage rate is below 1% is 60 and 70 UE/sec/cell for Case 9 and Case 14 respectively. In [4], it was considered that a UE would send 1 message per 2 hours. For the area of a cell in a satellite scenario we do not have a specific cell size, as we only define beam separation from the point of view of the satellite. However, from observing the satellite antenna pattern on the ground, we can estimate the area on the ground as ~1385 km2 and 5543 km2. The connection density per narrowband can thus be computed as:

Connection density = 1st percentile arrival rate (UE/s/cell) * UE traffic pattern [s] / cell area [km2]

We present the results for LTE-M in Table 1. Comparing Case 9 and Case 14 we can see that the achievable number of devices supported for Case 14 is significantly less than Case 9, owing to the much larger cell size associated with having the satellites at 1200 km versus 600 km altitude.

Table 1.

| Scenario | LTE-M, NTN, Case 9 [2] | LTE-M NTN, Case 14 [2] |
| --- | --- | --- |
| Cell-size | A=1385 km2 | A=5543 km2 |
| # of devices supported per km2 with 6 PRBs | 364 devices/km2 | 78 devices/km2 |

The achievable connection density for LTE-M is 364 UEs/km2 in Case 9 and 78 UEs/km2 in Case 14 for a single narrowband.

To capture the results, a text proposal is given in Annex 7.3:

Capture the text proposal on connection density in Annex 7.3.

- Random access capacity evaluation

An important consideration when deploying a terrestrial deployment is to make sure that the capacity for random access is sufficient for the expected QoS that is to be provided coupled with the expected user activity – in other words the traffic and the density.

In RAN2#112bis-e the following was agreed:

RAN2 assumes that PRACH capacity in eMTC/NB-IoT over NTN will be evaluated to check whether it can support the large cell size of GEO/LEO. However, RAN2 believes this is more of a RAN1 topic and thus recommends companies to submit their contributions in RAN1.

This evaluation is to be presented also in RAN1 WG following the recommendation above, however we think that it would be beneficial to discuss this contribution also in RAN2 to gain some insights on random access capacity considering that such evaluation was made in RAN2 during Release 16 NTN SI [2].

For evaluating the achievable random-access capacity, we have to consider a number of different configurations. Here we mainly focus on random access capacity for baseline (Rel-13) LTE-M/NB-IoT.

For LTE-M, the configurations are:

- number of PRACH configurations (provided in the PRACH-ConfigIndex as shown in Table 6.3.3.2-2 to Table 6.3.3.2-4 in TS 36.211),

- number of preambles per PRACH opportunity per CE level (preambleMappingInfo),

- number of repetitions per CE level (numRepetitionPerPreambleAttempt)

Furthermore, it is also possible that the repetitions are either configured by sharing the PRACH opportunities through preambles, or separate preambles or a mix of them.

For NB-IoT the configurations are:

- number of NPRACH opportunities per CE level (given by NPRACH-Periodicity),

- number of subcarriers per NPRACH opportunity per CE level (NPRACH-NumSubcarriers)

- number of repetitions per CE level (numRepetitionsPerPreambleAttempt)

In NB-IoT the repetitions the random-access preambles are orthogonal, meaning NPRACH resources will be configured separately. The method for calculating the PRACH capacity is given in the Annex, which serves as a good baseline for the random access capacity evaluation.

Capture the text proposal on random access capacity in the TR.

- Conclusion

In this contribution we have discussed the evaluation for connection density for IoT NTN devices. In the previous sections we made the following observation:

Observation 1	The achievable connection density for LTE-M is 364 UEs/km2 in Case 9 and 78 UEs/km2 in Case 14 for a single narrowband.

Based on the discussion above we propose the following:

Proposal 1	Capture the text proposal on paging capacity in Annex 7.1.

Proposal 2	Capture the text proposal on connection density in Annex 7.3.

Proposal 3	Capture the text proposal on random access capacity in the TR.

- References

RP-202689, Study on NB-IoT/eMTC support for Non-terrestrial Network, RAN#90, Dec 2020.

TR 38.821, Solutions for NR to support Non-terrestrial Networks (NTN), 3GPP, V16.0.0, Jan 2016.

R2-1901404, IoT Device Density Models for Various Environments, Vodafone, RAN2#105, Athens 2019.

IMT-2020 self evaluation: mMTC non-full buffer connection density for LTE-MTC and NB-IoT, Ericsson, Gothenburg, August 2018.

R2-2104033, Summary of [Post113-e][055][IoT NTN] Performance evaluation, Ericsson, RAN2#113-e, online, 2021.

3GPP TS 36.763, ”Study on Narrow-Band Internet of Things/enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”, Release 17, V0.1.0, March 2021.

- Annex

## Paging capacity

Text proposal for 36.763.

----------------------------

###### x.y.z	IoT paging capacity evaluation

For determining the paging capacity, where we have extracted the configuration possibilities for LTE-M and NB-IoT:

- NPO, number of paging occasions per paging frame determined by the RRC parameter nB. This has a maximum value of 4.

- NPF, number of configured paging frames per second, determined by the paging cycle configured.

- Ncarriers, number of carriers, determined by the RRC parameter paging-narrowBands-r13 for LTE-M and maxNonAnchorCarriers-NB-r14 for NB-IoT.

- Nrecords, number of records, where the maximum number of records are 16.

- Apaging=Aspotbeam×NspotbeamNPCI×M, where the Apaging is the paging area,  Aspotbeam is the spotbeam area, NspotbeamNPCI is the spotbeam to PCI ratio and M is the number of cells in a tracking area. The area of a spotbeam can roughly be calculated as Aspotbeam= 332R2 where R is the larger radius of the hexagonal area.

- Npages, number of average paging attempts on per user.

- DUE, UE density per sq. km.

Although there are some differences in terms of how LTE-M and NB-IoT would work in practice, for paging capacity based on what is configurable by the standard, they can be seen to have the same parameters except for some corner cases. In the evaluation we only consider the average UE in terms of coverage and thus do not include factors such as percentage of UEs in deep coverage.

The supported number of pages messages per second for the LTE-M/NB-IoT cell is computed as:

Cpaging= Ncarrier×NPF×NPO×Nrecords

The paging channel load is given as:

Paging channel load= Npages×DUE×ApagingCpaging

The achievable UE density is given as:

Achievable UE density= CpagingNpages×Apaging,

For the number of pages Npages, we consider the traffic model given in [TR 45.820, E.2.3], that indicates that the periodic inter-arrival time is distributed as 40% of UEs have 1 day inter-arrival time, 40% - 2 hours, 15% - 1 hour and 5% 30 minutes. On average per UE, this means Npages=1.2963*10-4.

To evaluate the paging capacity, Table x-1 gives a number of examples. The rationale the cases are:

- Case 1: IoT dense paging configuration at 600 km altitude Set 1 [2, Section 6.1], considering UEs are in good radio conditions not requiring any repetitions and thus more paging occasions can be used.

- Case 2: IoT sparse paging configuration at 600 km altitude Set 1 [2, Section 6.1], considering somewhat more UEs being in worse radio conditions requiring more repetitions for the paging occasions.

- Case 3: IoT sparse paging configuration (to allow for repetitions) for GEO altitude Set 1 [2, Section 6.1] considering UEs in decent radio conditions.

- Case 4: IoT sparse paging configuration (to allow for repetitions) for Set 4 (see Table in Section 6.xxx (36.763)) with repetitions configured for paging occasions to overcome link budgets thus requiring more sparse paging.

Table x-1: The details of the cases

| Case | Paging Parameters |  |  |  | Paging area |
| --- | --- | --- | --- | --- | --- |
|  | NPO | NPF | Ncarriers | Nrecords |  |
| Paging Case 1 | 1 | 100 | 16 | 2 | R=25 km, M=1, Apaging=1623 km2 |
| Paging Case 2 | 1 | 50 | 16 | 1 | R=25 km, M=1, Apaging=1623 km2 |
| Paging Case 3 | 1 | 100 | 16 | 1 | R=125 km, M=1, Apaging=40595 km2 |
| Paging Case 4 | 1 | 50 | 16 | 1 | R=850 km, M=1, Apaging=1877110 km2 |

The results can be found in the following Table x-2 and Table x-3. For Table x-2 we have assumed a UE density of 400 UE/km2 following [3]. In Table x-3 we evaluate the achievable UE density.

Table x-2: Paging channel load for a given and UE density.

| Case | UE density [UE/km2] | Paging channel load |
| --- | --- | --- |
| Paging Case 1 | 400 | 2.63% |
| Paging Case 2 | 400 | 10.52% |
| Paging Case 3 | 400 | 131.6% |
| Paging Case 4 | 400 | 12166% |

Table x-3: Supported UE density.

| Case | Achievable UE density [UE/km2] |
| --- | --- |
| Paging Case 1 | 15210 |
| Paging Case 2 | 3803 |
| Paging Case 3 | 304 |
| Paging Case 4 | 3.29 |

----------------------------

## Connection density evaluation

Text proposal for 36.763.

-------------------------------

###### x.y.z	Connection density evaluation

To determine the achievable connection density for an NTN mMTC IoT, the evaluation looks at the connection density achievable under the traffic assumption that the UE shall be able to deliver a 32 byte packet in the uplink within 10s with an outage probably of less than 1%.

To evaluate the connection density for NTN, we have chosen 2 LEO scenarios: Case 9 and Case 14 [2] which have similar characteristics with the difference that Case 9 is at 600 km altitude and Case 14 is at 1200 km altitude. These scenarios target handheld devices. The UE characteristics follow that of Section 6.2.1 in [6], which correspond to the IoT devices.

Some aspects to consider, as outlined in the e-mail discussion, are that, to reduce the computational complexity, the number of cells simulated have been selected as 19 cells with the statistics counted only for the inner 7 cells, as shown in Figure 2. Furthermore, the UEs will only have 20 seconds to deliver the packet before the delivery attempt is cancelled, meaning that no further re-transmissions will be attempted after 20 seconds. This is not to be confused with the 10 seconds that determines the outage rate. In other words, a UE may deliver the uplink packet after 10 seconds, but this will be counted as an outage.  Not being able to deliver the packet at all within 20s is also counted as an outage.

Figure 5. Simulating 19 cells but only accounting for the inner 7 cells.

In order to be able to observe how the uplink and downlink SINR gets worse with increasing load, i.e., arrival rate, the PUSCH SINR are shown in Figure 6 and PDSCH SINR percentiles are shown in Figure 7.

Figure 6. PDSCH SINR percentiles as a function of the arrival rate.

Figure 7. PUSCH SINR percentiles as a function of the arrival rate.

Figure 8. The outage rate as a function of the arrival rate.

The outage rate as a function of the arrival rate in these scenarios can be observed in Figure 8. For computing the achievable connection density, we can see that the maximum arrival rate where the outage rate is below 1% is 60 and 70 UE/sec/cell for Case 9 and Case 14 respectively. In [4], it was considered that a UE would send 1 message per 2 hours. For the area of a cell in a satellite scenario we do not have a specific cell size, as we only define beam separation from the point of view of the satellite. However, from observing the satellite antenna pattern on the ground, we can estimate the area on the ground as ~1385 km2 and 5543 km2. The connection density per narrowband can thus be computed as:

Connection density = 1st percentile arrival rate (UE/s/cell) * UE traffic pattern [s] / cell area [km2]

We present the results for LTE-M in Table x-y. Comparing Case 9 and Case 14 we can see that the achievable number of devices supported for Case 14 is significantly less than Case 9, owing to the much larger cell size associated with having the satellites at 1200 km versus 600 km altitude.

Table x-y. Connection density of LTE-M for different scenarios.

| Scenario | LTE-M, NTN, Case 9 [2] | LTE-M NTN, Case 14 [2] |
| --- | --- | --- |
| Cell-size | A=1385 km2 | A=5543 km2 |
| # of devices supported per km2 with 6 PRBs | 364 devices/km2 | 78 devices/km2 |

-------------------------------------

## Random access capacity

###### x.y.z	IoT RACH capacity evaluation

For calculating the LTE-M and NB-IoT RACH capacity we have the following parameters that are configurable by the network, for CE level r:

- The number of (N)PRACH configurations (decided by the PRACH-ConfigIndex in LTE-M and NPRACH-Periodicity in NB-IoT) per second denoted as ρr,

- Number of preambles per (N)PRACH opportunity per CE level (preambleMappingInfo in LTE-M and NPRACH-NumSubcarriers in NB-IoT) pr,

- Number of repetitions per CE level (numRepetitionPerPreambleAttempt in LTE-M and numRepetitionsPerPreambleAttempt in NB-IoT) R

Using the above, we get the following number of configured random access opportunities per CE level is computed as:

Mr=ρr*prR,   for LTE-Mρr*pr, for NB-IoT

For all CE levels, the configured number of random access opportunities are:

M=r=0CE-levelsMr,

The Physical Random Access Channel (PRACH) in LTE-M is largely inherited from LTE and uses Slotted Aloha as access method. For NB-IoT the NPRACH is also a slotted Aloha access method. The PRACH preamble collision probability between contending system access attempts on a PRACH radio resource is calculated as:

P(collision)=1-e-γM

Where M is the number of configured access opportunities per second, and γ is the random access arrival rate per second. The number of the random access arrival rate per second supported is thus:

γsupported=-ln1-Pcollision*M

where the collision rate Pcollision is typically aimed at 10% for optimum performance – which is what assumed hereon.

The supported user densities UE density is thus given by:

supported UE density= γsupportedcoverage*RACH per second per UE

In table y.z examples of LTE-M RACH capacity is shown and in table y.a examples of NB-IoT RACH capacity is shown for a number of different cases.

Table Y.Z: Supported UE density for LTE-M for typical GEO and LEO cell

| Cell Radius | PRACH-ConfigIndex | Repetition config, preamble config | RACH per second per UE | Supported UE density |
| --- | --- | --- | --- | --- |
| LEO 30 km | 6 | CE1: R=1, 58 preambles | 1 time per hour per UE | 1555 UE/km2 |
| LEO 30 km | 6 | CE1: R=1, 50 preambles CE2: R=2, 8 preambles | 1 time per hour per UE | 1448 UE/km2 |
| LEO 30 km | 6 | Separate PRACH opportunities CE1: R=1, 58 preambles CE2: R=2, 58 preambles | 1 time per hour per UE | 2827 UE/km2 |
| GEO 120 km | 1 | CE1: R=1, 58 preambles | 1 time per hour per UE | 24 UE/km2 |
| GEO 120 km | 1 | CE1: R=1, 50 preambles CE2: R=2, 8 preambles | 1 time per hour per UE | 23 UE/km2 |
| GEO 120 km | 1 | Separate PRACH opportunities CE1: R=1, 58 preambles CE2: R=2, 58 preambles | 1 time per hour per UE | 36 UE/km2 |

Table Y.A: Supported UE density for NB-IoT for typical GEO and LEO cell

| Cell Radius | NPRACH-Periodicity | Repetion config, preamble config | RACH per second per UE | Supported UE density |
| --- | --- | --- | --- | --- |
| LEO 30 km | CE1: 80 | CE1: R=1, 48 subcarriers | 1 time per day per UE | 1932 UE/km2 |
| LEO 30 km | CE1: 80 CE2: 160 | CE1: R=1, 24 subcarriers CE2: R=4, 12 subcarriers | 1 time per day per UE | 2174 UE/km2 |
| GEO 120 km | 40 | CE1: R=1, 48 subcarriers | 1 time per day per UE | 242 UE/km2 |
| GEO 120 km | CE1: 80 CE2: 640 CE2: 640 | CE1: R=1, 24 subcarriers CE2: R=1, 24 subcarriers CE3: R=1, 12 subcarriers | 1 time per day per UE | 71 UE/km2 |

----------------------------