3GPP TSG-WG2 Meeting #114 electronic    	     R2-xxx

Online, May, 2021

Source: 			CMCC

Title: Handling of non-SDT data arriving

Agenda item:	8.6.3

Document for: 	Discussion

Introduction

How to handling the non-SDT data during small data transmission has been discussed in the previous meetings and the email discussion #502. Here we list some basic agreements for this issue:

Agreements：

RAN2#113bis-e

- Non-SDT radio bearers are only resumed upon receiving RRCResume (same as today)

- Down-scope to two solutions (CCCH or DCCH) and ask SA3 about security issues (explain that CCCH message will be repeated in same cell and ask if there is a question)

- Switching from SDT to non-SDT is supported.

- UE switches from SDT to non-SDT in following cases:

Case 1 (27/0): UE receive indication from network to switch to non-SDT procedure.

Network can send RRCResume. FFS whether network can send indication in RAR/fallbackRAR/DCI to switch to non-SDT procedure.

FFS Case 2 (18/9): Initial UL transmission (in msgA/Msg3/CG resources) fails configured number of times

In this contribution, we will further discuss the handling the non-SDT data arriving.

Discussion

In RAN2#111e, the data volume threshold has been agreed as one of the criterions for selection between SDT and non-SDT, and the SDT is configured on a per DRB basic. So for SDT, whilst data arrives at UE, UE can aware whether the arrived data is SDT data or non-SDT data.

As to whether and how to inform such information to MAC entity, some more discussion is needed on internal AS modeling issue. In our understanding, data arriving on the non-SDT DRBs can be visible to the MAC layer regardless whether the non-SDT DRB can be resumed. With that assumption, MAC entity can report buffer status information to network for UL scheduling. Comparing another RACH procedure triggered by non-SDT data arriving during ongoing RA-SDT procedure, informing the arriving of non-SDT to network along with small data is more reasonable and effective. That’s to say, UE reports the arriving of non-SDT to network with indicator or some other BSR like MAC CE and monitor the behavior indication from network to resume to RRC_CONNECTED state.

Proposal 1: Non-SDT data arriving can be visible for MAC layer.

Proposal 2: UE informs the arriving information of non-SDT DRBs to network for decision.

The solution for notifying network the arriving of non-SDT data is under discussion. We share more considerations on DCCH based solutions.

According to current agreements, data volume threshold is configured by network for UE to decide whether to do SDT or not. If the data volume of coming data is larger than the configured threshold, legacy RA is triggered (RRC resume). In another words, if non-SDT data arrives at the initial stage of SDT procedure, SDT will not be triggered any more. So the case we discussed is mainly about non-SDT data arrives after the MSGA or MAS3 is transmitted. For supporting subsequent data transmission, network may allocate sufficient or redundant UL resource to UE to fulfill applied buffered data or potential subsequent uplink data. Of course, it is always based on network implementation. However, since the subsequent data transmission is supported, it is an effective way for network assign additional UL resource for BSR to avoid UL resource scheduling request. So, concerns on UL grant for DCCH transmission can be ignored and DCCH based solution can be supported as solution for informing the non-SDT data. Furthermore, BSR like MAC CE to indicate the non-SDT data is direct and reasonable.

Proposal 3: Support DCCH based indication to network informing the non-SDT data.

It has been supported for UE to switch from SDT to non-SDT in last meeting. To avoid the case that UE does not get any response from network and the SDT access criteria are always fulfilled, UE triggered switch from SDT to non-SDT should been supported. During initiation data transmission phase, when UL transmission (MSGA/Msg3/CG resources) fails configured number of times, non-SDT based RA procedure can be triggered. The maximum transmission time can be same among 2-step, 4-step RA based SDT and CG based SDT. Proposal 4: Non-SDT based RA procedure can be triggered when UL transmission (MSGA/Msg3/CG resources) fails configured number of times.

As agreed in last meeting, UE can switch to non-SDT when it receives RRCResume message. As to the FFS that including switch indication in RAR/fallbackRAR/DCI, we do not think it is necessary and efficient. The motivation for network to send switch indication is to inform UE to initial non-SDT RA and transmit RRCResumeRequest. For the trigger condition that non-SDT data arriving, the UL transmission carrying non-SDT data arriving information shall be the subsequent UL packet. So, RAR/fallbackRAR can not be transmitted obviously. In our understanding, one possible case that network informs switch with RAR/fallbackRAR is that at least preamble has been successfully received for RA-SDT, which is a rare case.

Proposal 5: Exclude the non-SDT switch solution that using indicator in RAR/fallbackRAR/DCI.

Conclusions

According the above discussion we have following proposals:

Proposal 1: Non-SDT data arriving can be visible for MAC layer.

Proposal 2: UE informs the arriving information of non-SDT DRBs to network for decision.

Proposal 3: Support DCCH based indication to network informing the non-SDT data.

Proposal 4: Non-SDT based RA procedure can be triggered when UL transmission (MSGA/Msg3/CG resources) fails configured number of times.

Proposal 5: Exclude the non-SDT switch solution that using indicator in RAR/fallbackRAR/DCI.

References

[1] R2-2103022, Summary of General and other control plane open issues for SDT (email: [Post 113-e][502]),	ZTE