3GPP TSG-RAN WG2 Meeting #114-e	R2-2106506

# Electronic Meeting, May 19th – 27th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2534 | rev | 3 | Current version: | 16.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Misc corrections for Rel-16 DCCA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2021-05-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Miscellaneous minor corrections related to the enhanced DCCA work item - In incoming LS R2-2100059, RAN4 informed RAN2 about changed requirements for idle/inactive measurements, in which idle/inactive mode measurement became rependent on SnonIntraSearchP and SnonIntraSearchQ. This is in conflict with current RAN2 specification note in section 5.7.8.2a. This inconsistency was discussed in RAN2#113e, but the conclusion was not to send LS to RAN4 regarding the changed requirement. Still, the inconsistency between RAN4 and RAN2 specs need to be corrected by removing the corresponding sentence from RAN2 spec. - Idle/inactive measurements are not covered in UE behavior description in section 4.2.1. - According to current specification, the field description of pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup is “This is applicable to both CA and none CA operation”. It is obovious that it is not applicable to non-CA case since secondary PUCCH group can not be configured for UE in non-CA case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In section 4.2.1, Add idle/inactive measurements to the UE behavior description. - In section 5.7.8.2a, remove text inconsistent with RAN4 specifications regarding the serving cell measurements affecting idle/inactive mode measurements. - In 6.3.2, correct the applicability of pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup. Impact Analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC,NR-DC Impacted functionality: Idle/inactive measurements Secondary PUCCH group Inter-operability: If the network is implemented according to the CR and the UE is not, there will not be inter-operability problems. If the UE is implemented according to the CR and the network is not, there will not be inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miscellaneous minor errors remain in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 5.7.8.2a, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2103803, R2-2104342, R2-2106333 |  |  |  |  |  |  |  |  |

START OF CHANGE

### 4.2.1	UE states and state transitions including inter RAT

A UE is either in RRC_CONNECTED state or in RRC_INACTIVE state when an RRC connection has been established. If this is not the case, i.e. no RRC connection is established, the UE is in RRC_IDLE state. The RRC states can further be characterised as follows:

-	RRC_IDLE:

-	A UE specific DRX may be configured by upper layers;

-	UE controlled mobility based on network configuration;

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5);

-	Monitors a Paging channel for CN paging using 5G-S-TMSI;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Acquires system information and can send SI request (if configured);

-	Performs logging of available measurements together with location and time for logged measurement configured UEs;

-	Performs idle/inactive measurements for idle/inactive measurement configured UEs.

-	RRC_INACTIVE:

-	A UE specific DRX may be configured by upper layers or by RRC layer;

-	UE controlled mobility based on network configuration;

-	The UE stores the UE Inactive AS context;

-	A RAN-based notification area is configured by RRC layer;

The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5);

-	Monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Performs RAN-based notification area updates periodically and when moving outside the configured RAN-based notification area;

-	Acquires system information and can send SI request (if configured);

-	Performs logging of available measurements together with location and time for logged measurement configured UEs;

-	Performs idle/inactive measurements for idle/inactive measurement configured UEs.

-	RRC_CONNECTED:

-	The UE stores the AS context;

-	Transfer of unicast data to/from UE;

-	At lower layers, the UE may be configured with a UE specific DRX;

-	For UEs supporting CA, use of one or more SCells, aggregated with the SpCell, for increased bandwidth;

-	For UEs supporting DC, use of one SCG, aggregated with the MCG, for increased bandwidth;

-	Network controlled mobility within NR and to/from E-UTRA;

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5), if configured;

-	Monitors control channels associated with the shared data channel to determine if data is scheduled for it;

-	Provides channel quality and feedback information;

-	Performs neighbouring cell measurements and measurement reporting;

-	Acquires system information;

-	Performs immediate MDT measurement together with available location reporting.

Figure 4.2.1-1 illustrates an overview of UE RRC state machine and state transitions in NR. A UE has only one RRC state in NR at one time.

Figure 4.2.1-1:	UE state machine and state transitions in NR

Figure 4.2.1-2 illustrates an overview of UE state machine and state transitions in NR as well as the mobility procedures supported between NR/5GC E-UTRA/EPC and E-UTRA/5GC.

Figure 4.2.1-2:	UE state machine and state transitions between NR/5GC, E-UTRA/EPC and E-UTRA/5GC

NEXT CHANGE

#### 5.7.8.2a	Performing measurements

When performing measurements on NR carriers according to this clause, the UE shall derive the cell quality as specified in 5.5.3.3 and consider the beam quality to be the value of the measurement results of the concerned beam, where each result is averaged as described in TS 38.215 [9].

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the VarMeasIdleConfig includes the measIdleCarrierListEUTRA and the SIB1 contains idleModeMeasurementsEUTRA:

3>	for each entry in measIdleCarrierListEUTRA within VarMeasIdleConfig:

4>	if UE supports NE-DC between the serving carrier and the carrier frequency indicated by carrierFreqEUTRA within the corresponding entry:

5>	perform measurements in the carrier frequency and bandwidth indicated by carrierFreqEUTRA and allowedMeasBandwidth within the corresponding entry;

5>	if the reportQuantitiesEUTRA is set to rsrq:

6>	consider RSRQ as the sorting quantity;

5>	else:

6>	consider RSRP as the sorting quantity;

5>	if the measCellListEUTRA is included:

6>	consider cells identified by each entry within the measCellListEUTRA to be applicable for idle/inactive mode measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive measurement results for the measurement quantities indicated by reportQuantitiesEUTRA;

5>	store the derived measurement results as indicated by reportQuantitiesEUTRA within the measReportIdleEUTRA in VarMeasIdleReport in decreasing order of the sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThresholdEUTRA is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThresholdEUTRA;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

2>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and the SIB1 contains idleModeMeasurementsNR:

3>	for each entry in measIdleCarrierListNR within VarMeasIdleConfig that contains ssb-MeasConfig:

4>	if UE supports carrier aggregation or NR-DC between serving carrier and the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry:

5>	perform measurements in the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry;

5>	if the reportQuantities is set to rsrq:

6>	consider RSRQ as the cell sorting quantity;

5>	else:

6>	consider RSRP as the cell sorting quantity;

5>	if the measCellListNR is included:

6>	consider cells identified by each entry within the measCellListNR to be applicable for idle/inactive measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive cell measurement results for the measurement quantities indicated by reportQuantities;

5>	store the derived cell measurement results as indicated by reportQuantities for cells applicable for idle/inactive measurement reporting within measResultsPerCarrierListIdleNR in the measReportIdleNR in VarMeasIdleReport in decreasing order of the cell sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThreshold is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

5>	if beamMeasConfigIdle is included in the associated entry in measIdleCarrierListNR and if UE supports idleInactiveNR-MeasBeamReport for the FR of the carrier frequency indicated by carrierFreq within the associated entry, for each cell in the measurement results:

6>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

6>	if the reportQuantityRS-Indexes is set to rsrq:

7>	consider RSRQ as the beam sorting quantity;

6>	else:

7>	consider RSRP as the beam sorting quantity;

6>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

7>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

6>	if the includeBeamMeasurements is set to true:

7>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

2>	if, as a result of the procedure in this subclause, the UE performs measurements in one or more carrier frequency indicated by measIdleCarrierListNR or measIdleCarrierListEUTRA:

3>	store the cell measurement results for RSRP and RSRQ for the serving cell within measResultServingCell in the measReportIdleNR in VarMeasIdleReport.

3>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and it contains an entry with carrierFreq set to the value of the serving frequency:

4>	if beamMeasConfigIdle is included in that entry, and if the UE supports idleInactiveNR- MeasBeamReport for the FR of the serving cell:

5>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

5>	if the reportQuantityRS-Indexes is set to rsrq:

6>	consider RSRQ as the beam sorting quantity;

5>	else:

6>	consider RSRP as the beam sorting quantity;

5>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

6>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included in SIB2 of serving cell, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

5>	if the includeBeamMeasurements is set to true:

6>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

NOTE 1:	How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting.

NOTE 2:	The UE is not required to perform idle/inactive measurements on a given carrier if the SSB configuration of that carrier provided via dedicated signaling is different from the SSB configuration broadcasted in the serving cell, if any.

NOTE 3:	How the UE prioritizes which frequencies to measure or report (in case it is configured with more frequencies than it can measure or report) is left to UE implementation.

NEXT CHANGE

### 6.3.2	Radio resource control information elements

[…]

#### –	PhysicalCellGroupConfig

The IE PhysicalCellGroupConfig is used to configure cell-group specific L1 parameters.

PhysicalCellGroupConfig information element

-- ASN1START

-- TAG-PHYSICALCELLGROUPCONFIG-START

PhysicalCellGroupConfig ::=         SEQUENCE {

harq-ACK-SpatialBundlingPUCCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

harq-ACK-SpatialBundlingPUSCH       ENUMERATED {true}                                               OPTIONAL,   -- Need S

p-NR-FR1                            P-Max                                                           OPTIONAL,   -- Need R

pdsch-HARQ-ACK-Codebook             ENUMERATED {semiStatic, dynamic},

tpc-SRS-RNTI                        RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUCCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

tpc-PUSCH-RNTI                      RNTI-Value                                                      OPTIONAL,   -- Need R

sp-CSI-RNTI                         RNTI-Value                                                      OPTIONAL,   -- Need R

cs-RNTI                             SetupRelease { RNTI-Value }                                     OPTIONAL,   -- Need M

...,

[[

mcs-C-RNTI                          RNTI-Value                                                      OPTIONAL,   -- Need R

p-UE-FR1                            P-Max                                                           OPTIONAL    -- Cond MCG-Only

]],

[[

xScale                              ENUMERATED {dB0, dB6, spare2, spare1}                           OPTIONAL    -- Cond SCG-Only

]],

[[

pdcch-BlindDetection                SetupRelease { PDCCH-BlindDetection }                           OPTIONAL    -- Need M

]],

[[

dcp-Config-r16                      SetupRelease { DCP-Config-r16 }                                 OPTIONAL,   -- Need M

harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup-r16    ENUMERATED {enabled, disabled}             OPTIONAL,   -- Cond twoPUCCHgroup

harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup-r16    ENUMERATED {enabled, disabled}             OPTIONAL,   -- Cond twoPUCCHgroup

pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup-r16          ENUMERATED {semiStatic, dynamic}           OPTIONAL,   -- Cond twoPUCCHgroup

p-NR-FR2-r16                                              P-Max                                     OPTIONAL,   -- Need R

p-UE-FR2-r16                                              P-Max                                     OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR1-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

nrdc-PCmode-FR2-r16                ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic}       OPTIONAL,   -- Cond MCG-Only

pdsch-HARQ-ACK-Codebook-r16            ENUMERATED {enhancedDynamic}                                 OPTIONAL,   -- Need R

nfi-TotalDAI-Included-r16              ENUMERATED {true}                                            OPTIONAL,   -- Need R

ul-TotalDAI-Included-r16               ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedback-r16     ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackNDI-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackCBG-r16  ENUMERATED {true}                                            OPTIONAL,   -- Need R

downlinkAssignmentIndexDCI-0-2-r16     ENUMERATED { enabled }                                       OPTIONAL,   -- Need S

downlinkAssignmentIndexDCI-1-2-r16     ENUMERATED {n1, n2, n4}                                      OPTIONAL,   -- Need S

pdsch-HARQ-ACK-CodebookList-r16        SetupRelease {PDSCH-HARQ-ACK-CodebookList-r16}               OPTIONAL,   -- Need M

ackNackFeedbackMode-r16                ENUMERATED {joint, separate}                                 OPTIONAL,   -- Need R

pdcch-BlindDetectionCA-CombIndicator-r16 SetupRelease { PDCCH-BlindDetectionCA-CombIndicator-r16 }  OPTIONAL,   -- Need M

pdcch-BlindDetection2-r16                SetupRelease { PDCCH-BlindDetection2-r16 }                 OPTIONAL,   -- Need M

pdcch-BlindDetection3-r16                SetupRelease { PDCCH-BlindDetection3-r16 }                 OPTIONAL,   -- Need M

bdFactorR-r16                          ENUMERATED {n1}                                              OPTIONAL    -- Need R

]]

}

PDCCH-BlindDetection ::=                INTEGER (1..15)

DCP-Config-r16 ::=                  SEQUENCE {

ps-RNTI-r16                         RNTI-Value,

ps-Offset-r16                       INTEGER (1..120),

sizeDCI-2-6-r16                     INTEGER (1..maxDCI-2-6-Size-r16),

ps-PositionDCI-2-6-r16              INTEGER (0..maxDCI-2-6-Size-1-r16),

ps-WakeUp-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need S

ps-TransmitPeriodicL1-RSRP-r16      ENUMERATED {true}                                               OPTIONAL,   -- Need S

ps-TransmitOtherPeriodicCSI-r16     ENUMERATED {true}                                               OPTIONAL    -- Need S

}

PDSCH-HARQ-ACK-CodebookList-r16 ::=     SEQUENCE (SIZE (1..2)) OF ENUMERATED {semiStatic, dynamic}

PDCCH-BlindDetectionCA-CombIndicator-r16 ::= SEQUENCE {

pdcch-BlindDetectionCA1-r16                  INTEGER (1..15),

pdcch-BlindDetectionCA2-r16                  INTEGER (1..15)

}

PDCCH-BlindDetection2-r16 ::=                INTEGER (1..15)

PDCCH-BlindDetection3-r16 ::=                INTEGER (1..15)

-- TAG-PHYSICALCELLGROUPCONFIG-STOP

-- ASN1STOP

PhysicalCellGroupConfig field descriptions

ackNackFeedbackMode Indicates which among the joint and separate ACK/NACK feedback modes to use within a slot as specified in TS 38.213 [13] (clause 9).

bdFactorR Parameter for determining and distributing the maximum numbers of BD/CCE for mPDCCH based mPDSCH transmission as specified in TS 38.213 [13] Clause 10.1.

cs-RNTI RNTI value for downlink SPS (see SPS-Config) and uplink configured grant (see ConfiguredGrantConfig).

downlinkAssignmentIndexDCI-0-2 Indicates if "Downlink assignment index" is present or absent in DCI format 0_2. If the field "downlinkAssignmentIndexDCI-0-2" is absent, then 0 bit for "Downlink assignment index" in DCI format 0_2. If the field "downlinkAssignmentIndexDCI-0-2" is present, then the bitwidth of "Downlink assignment index" in DCI format 0_2 is defined in the same was as that in DCI format 0_1 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.1).

downlinkAssignmentIndexDCI-1-2 Configures the number of bits for "Downlink assignment index" in DCI format 1_2. If the field is absent, then 0 bit for "Downlink assignment index" in DCI format 1_2. Note that 1 bit and 2 bits are applied if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic. 4 bits is applied if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook is set to dynamic (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.1).

harq-ACK-SpatialBundlingPUCCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUCCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling of PUCCH HARQ ACKs for the primary PUCCH group is disabled (see TS 38.213 [13], clause 9.1.2.1). If the field harq-ACK SpatialBundlingPUCCH-secondaryPUCCHgroup is present, harq-ACK-SpatialBundlingPUCCH is only applied to primary PUCCH group. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUCCH-secondaryPUCCHgroup Indicates whether spatial bundling of PUCCH HARQ ACKs for the secondary PUCCH group is enabled or disabled. The field is only applicable when more than 4 layers are possible to schedule (see TS 38.213 [13], clause 9.1.2.1). When the field is absent, the use of spatial bundling of PUCCH HARQ ACKs for the secondary PUCCH group is indicated by harq-ACK-SpatialBundlingPUCCH. See TS 38.213 [13], clause 9.1.2.1. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUSCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUSCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling of PUSCH HARQ ACKs for the primary PUCCH group is disabled (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2). If the field harq-ACK SpatialBundlingPUSCH-secondaryPUCCHgroup is present, harq-ACK-SpatialBundlingPUSCH is only applied to primary PUCCH group. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

harq-ACK-SpatialBundlingPUSCH-secondaryPUCCHgroup Indicates whether spatial bundling of PUSCH HARQ ACKs for the secondary PUCCH group is enabled or disabled. The field is only applicable when more than 4 layers are possible to schedule (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2). When the field is absent, the use of spatial bundling of PUSCH HARQ ACKs for the secondary PUCCH group is indicated by harq-ACK-SpatialBundlingPUSCH. See TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2. Network does not configure for a UE both spatial bundling of HARQ ACKs and codeBlockGroupTransmission within the same cell group.

mcs-C-RNTI RNTI to indicate use of qam64LowSE for grant-based transmissions. When the mcs-C-RNTI is configured, RNTI scrambling of DCI CRC is used to choose the corresponding MCS table.

nfi-TotalDAI-Included Indicates whether the NFI and total DAI fields of the non-scheduled PDSCH group is included in the non-fallback DL grant DCI (see TS 38.212 [17], clause 7.3.1). The network configures this only when enhanced dynamic codebook is configured (pdsch-HARQ-ACK-Codebook is set to enhancedDynamic).

nrdc-PCmode-FR1 Indicates the uplink power sharing mode that the UE uses in NR-DC in frequency range 1 (FR1) (see TS 38.213 [13], clause 7.6).

nrdc-PCmode-FR2 Indicates the uplink power sharing mode that the UE uses in NR-DC in frequency range 2 (FR2) (see TS 38.213 [13], clause 7.6).

pdcch-BlindDetection, pdcch-BlindDetection2, pdcch-BlindDetection3 Indicates the reference number of cells for PDCCH blind detection for the CG. Network configures the field for each CG when the UE is in NR DC and sets the value in accordance with the constraints specified in TS 38.213 [13]. The network configures pdcch-BlindDetection only if the UE is in NR-DC. The network configures pdcch-BlindDetection2 only if the UE is in NR-DC with at least one downlink cell using Rel-16 PDCCH monitoring capability. The network configures pdcch-BlindDetection3 only if the UE is in NR-DC with at least one downlink cell using Rel-15 PDCCH monitoring capability.

pdcch-BlindDetectionCA-CombIndicator Configure one combination of pdcch-BlindDetectionCA1 (for R15) and pdcch-BlindDetectionCA2 (for R16) for UE to use for scaling PDCCH monitoring capability if the number of serving cells configured to a UE is larger than the reported capability, and if UE reports more than one combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 as UE capability. The combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2) configured by pdcch-BlindDetectionCACombIndicator is from the more than one combination of pdcch-BlindDetectionCA1 and pdcch-BlindDetectionCA2 reported by UE (see TS 38.213 [13], clause 10).

p-NR-FR1 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 1 (FR1). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR1 (configured total for all serving cells operating on FR1).

p-NR-FR2 The maximum total transmit power to be used by the UE in this NR cell group across all serving cells in frequency range 2 (FR2). The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-UE-FR2 (configured total for all serving cells operating on FR2). This field is only used in NR-DC.

ps-RNTI RNTI value for scrambling CRC of DCI format 2-6 used for power saving (see TS 38.213 [13], clause 10.1).

ps-Offset The start of the search-time of DCI format 2-6 with CRC scrambled by PS-RNTI relative to the start of the drx-onDurationTimer of Long DRX (see TS 38.213 [13], clause 10.3). Value in multiples of 0.125ms (milliseconds). 1 corresponds to 0.125 ms, 2 corresponds to 0.25 ms, 3 corresponds to 0.375 ms and so on.

ps-WakeUp Indicates the UE to wake-up if DCI format 2-6 is not detected outside active time (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not wake-up if DCI format 2-6 is not detected outside active time.

ps-PositionDCI-2-6 Starting position of UE wakeup and SCell dormancy indication in DCI format 2-6 (see TS 38.213 [13], clause 10.3).

ps-TransmitPeriodicL1-RSRP Indicates the UE to transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start.

ps-TransmitOtherPeriodicCSI Indicates the UE to transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start.

p-UE-FR1 The maximum total transmit power to be used by the UE across all serving cells in frequency range 1 (FR1) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR1 (configured for the cell group).

p-UE-FR2 The maximum total transmit power to be used by the UE across all serving cells in frequency range 2 (FR2) across all cell groups. The maximum transmit power that the UE may use may be additionally limited by p-Max (configured in FrequencyInfoUL) and by p-NR-FR2 (configured for the cell group).

pdsch-HARQ-ACK-Codebook The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to both CA and none CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3). If pdsch-HARQ-ACK-Codebook-r16 is signalled, UE shall ignore the pdsch-HARQ-ACK-Codebook (without suffix). If the field pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup is present, pdsch-HARQ-ACK-Codebook is applied to primary PUCCH group. Otherwise, this field is applied to the cell group (i.e. for all the cells within the cell group).

pdsch-HARQ-ACK-CodebookList A list of configuration for at least two simultaneously constructed HARQ-ACK codebooks. Each configuration in the list is defined in the same way as pdsch-HARQ-ACK-Codebook (see TS 38.212 [17], clause 7.3.1.2.2 and TS 38.213 [13], clauses 7.2.1, 9.1.2, 9.1.3 and 9.2.1). If this field is present, the field pdsch-HARQ-ACK-Codebook is ignored for the case at least two HARQ-ACK codebooks are simultaneously constructed. If this field is present, the value of this field is applied for primary PUCCH group and for secondary PUCCH group (if configured).

pdsch-HARQ-ACK-Codebook-secondaryPUCCHgroup The PDSCH HARQ-ACK codebook is either semi-static or dynamic. This is applicable to CA operation (see TS 38.213 [13], clauses 9.1.2 and 9.1.3). It is configured for secondary PUCCH group.

pdsch-HARQ-ACK-OneShotFeedback When configured, the DCI_format 1_1 can request the UE to report A/N for all HARQ processes and all CCs configured in the PUCCH group (see TS 38.212 [17], clause 7.3.1).

pdsch-HARQ-ACK-OneShotFeedbackCBG When configured, the DCI_format 1_1 can request the UE to include CBG level A/N for each CC with CBG level transmission configured. When not configured, the UE will report TB level A/N even if CBG level transmission is configured for a CC. The network configures this only when pdsch-HARQ-ACK-OneShotFeedback is configured.

pdsch-HARQ-ACK-OneShotFeedbackNDI When configured, the DCI_format 1_1 can request the UE to include NDI for each A/N reported. The network configures this only when pdsch-HARQ-ACK-OneShotFeedback is configured.

sizeDCI-2-6 Size of DCI format 2-6 (see TS 38.213 [13], clause 11.5).

sp-CSI-RNTI RNTI for Semi-Persistent CSI reporting on PUSCH (see CSI-ReportConfig) (see TS 38.214 [19], clause 5.2.1.5.2). Network always configures the UE with a value for this field when at least one CSI-ReportConfig with reportConfigType set to semiPersistentOnPUSCH is configured.

tpc-PUCCH-RNTI RNTI used for PUCCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-PUSCH-RNTI RNTI used for PUSCH TPC commands on DCI (see TS 38.213 [13], clause 10.1).

tpc-SRS-RNTI RNTI used for SRS TPC commands on DCI (see TS 38.213 [13], clause 10.1).

ul-TotalDAI-Included Indicates whether the total DAI fields of the additonal PDSCH group is included in the non-fallback UL grant DCI (see TS 38.212 [17], clause 7.3.1). The network configures this only when enhanced dynamic codebook is configured (pdsch-HARQ-ACK-Codebook is set to enhancedDynamic).

xScale The UE is allowed to drop NR only if the power scaling applied to NR results in a difference between scaled and unscaled NR UL of more than xScale dB (see TS 38.213 [13]). If the value is not configured for dynamic power sharing, the UE assumes default value of 6 dB.

| Conditional Presence | Explanation |
| --- | --- |
| MCG-Only | This field is optionally present, Need R, in the PhysicalCellGroupConfig of the MCG. It is absent otherwise. |
| SCG-Only | This field is optionally present, Need S, in the PhysicalCellGroupConfig of the SCG in (NG)EN-DC as defined in TS 38.213 [13]. It is absent otherwise. |
| twoPUCCHgroup | This field is optionally present, Need R, if secondary PUCCH group is configured. It is absent otherwise. |

END OF CHANGE