3GPP TSG-RAN WG2 Meeting #114-e	R2-21xxxxx

E-meeting, 19th – 27th May 2021

Agenda Item:	5.4.1.3

Source: 	OPPO

Title:  	Report of [AT114-e][009][NR15] System Information (OPPO)

Document for:	Discussion and decision

# Introduction

This document is to kick off the following email discussion:

- [AT114-e][009][NR15] System Information (OPPO)

Scope: Treat R2-2105367, R2-2105368, R2-2104952, R2-2104953, R2-2104954, R2-2104955, R2-2104956,

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed CRs.

Deadline: Schedule A

- Collect companies’ view. Deadline for comments Friday May 21 1000 UTC to settle scope what is agreeable etc (phase 1).;

- Deadline for any functional and/or scope comments Wednesday May 26 1200 UTC. At this point, non-agreeable parts shall be removed/excluded. (phase 2)

Contact Information

| Company | Email |
| --- | --- |
| Qualcomm | mambriss@qti.qualcomm.com |
| OPPO | wangshukun@oppo.com |

# Discussion

Companies are requested to add their comments for each of the treated CRs of this email discussion in the boxes below.

## Search space SIB1

[1] R2-2104952	Discussion on RMSI reception based on non-zero search space	OPPO, CMCC	discussion	LTE_NR_DC_CA_enh-Core

[2] R2-2104953	38331 R15 RMSI reception based on non-zero search space-option 1	OPPO	CR	Rel-15	38.331	15.13.0	2591	-	F	LTE_NR_DC_CA_enh-Core

[3] R2-2104954	38331 R16 RMSI reception based on non-zero search space-option 1	OPPO	CR	Rel-16	38.331	16.4.1	2592	-	A	NR_newRAT-Core

[4] R2-2104955	38331 R15 RMSI reception based on non-zero search space-option 2	OPPO	CR	Rel-15	38.331	15.13.0	2593	-	F	NR_newRAT-Core

[5] R2-2104956	38331 R16 RMSI reception based on non-zero search space-option 2	OPPO	CR	Rel-16	38.331	16.4.1	2594	-	A	NR_newRAT-Core

In [1], the company mentions that mapping between RMSI PDCCH occasion and SSBs for non-zero search space for searchSpaceSIB1 on dedicated BWP is missing.

In R15, the dedicated BWP can be configured for the UE and the common search space can be optionally configured for the BWP. If common search space is configured for the dedicated BWP, the UE will receive the SIB1/OSI/paging via common search space monitoring.

In R15, the beam sweeping is supported for SIB1/OSI/paging and the mapping between the SIB1/OSI/paging PDCCH monitoring occasions and SSBs is specified in RAN1 Spec and RAN2 Spec. However, after checking RAN1 spec and RAN2 spec, the case for non-zero search space for searchSpaceSIB1 is missing.

In [1], company summarize 4 observations:

Observation 1: Dedicated BWP can be configured with common search space for SIB 1, i.e. searchSpaceSIB1 IE and UE will follow 38.213 to receive SIB1.

Observation 2: The searchSpaceSIB1 IE can be set to non-zero except initial BWP.

Observation 3: For OSI, if searchSpaceOtherSystemInformation IE is set to 0, TS 38.213 will specify the mapping between OSI PDCCH monitoring occasions and SSBs. Otherwise, the TS 38.331 will specify.

Observation 4: RAN1 spec does not specify the mapping between RMSI PDCCH monitoring occasions and SSBs when searchSpaceSIB1 is set to non-zero.

Q1: Do companies agree the observations in [1]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| QCom | Yes |  |
| OPPO | Yes |  |

Q2: Do companies agree the issue that mapping between RMSI PDCCH occasion and SSBs for non-zero search space for searchSpaceSIB1 on dedicated BWP is missing?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| QCom | Yes |  |
| OPPO | Yes |  |

If companies agree the issue in Q2, there are 2 options to fix the issue in [1].

Option 1: RAN2 clarify that the searchSpaceSIB1 IE can only be set to zero for both initial DL BWP and dedicated BWP if configured.

Option 2: RAN2 capture text for the mapping between RMSI PDCCH occasion and SSBs as OSI did if searchSpaceSIB1 is set to non-zero.

the changes in TS 38.331 are provided in [1] for option 1 and option 2.

Q3: Which option do companies prefer to fix the issue in R15 and R16 respectively?

| Company | Option for R15 and R16? | Comments |
| --- | --- | --- |
| QCom | No strong view |  |
| OPPO | Option 1 for R15 and R16; Option 2 for R17 |  |

Q4: Do companies agree to send LS to RAN1 for information?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| QCom | Neutral | Seems not needed |
| OPPO | Yes | It is up to majority view. |

## Stored SI

[6] R2-2105367	Clarification of cell Identity for SIB validity	vivo	CR	Rel-15	38.331	15.13.0	2621	-	F	NR_newRAT-Core

[7] R2-2105368	Clarification of cell Identity for SIB validity	vivo	CR	Rel-16	38.331	16.4.1	2622	-	A	NR_newRAT-Core

In [5][6], company thinks there is an ambiguity on cellIdentity when checking the validity of the stored version of a SIB in RAN sharing scenario.

Q5: Do companies agree the changes of the CR in [5][6]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| QCom | Yes |  |
| OPPO | Yes |  |

# Conclusions

Based on the discussion above, we propose:

# Reference