3GPP TSG-RAN WG2 Meeting #114-e	R2-21xxxxx

E-meeting, 19th – 27th May 2021

Agenda Item:	5.4.1.3

Source: 	OPPO

Title:  	Report of [AT114-e][009][NR15] System Information (OPPO)

Document for:	Discussion and decision

# Introduction

This document is to kick off the following email discussion:

- [AT114-e][009][NR15] System Information (OPPO)

Scope: Treat R2-2105367, R2-2105368, R2-2104952, R2-2104953, R2-2104954, R2-2104955, R2-2104956,

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed CRs.

Deadline: Schedule A

- Collect companies’ view. Deadline for comments Friday May 21 1000 UTC to settle scope what is agreeable etc (phase 1).;

- Deadline for any functional and/or scope comments Wednesday May 26 1200 UTC. At this point, non-agreeable parts shall be removed/excluded. (phase 2)

Contact Information

| Company | Email |
| --- | --- |
| Qualcomm | mambriss@qti.qualcomm.com |
| OPPO | wangshukun@oppo.com |
| Apple | zhibin_wu@apple.com |
| MediaTek | Chun-fan.tsai@mediatek.com |
| Ericsson | martin.van.der.zee@ericsson.com |
| Nokia | amaanat.ali@nokia.com |
| vivo | panxiang@vivo.com |
| Huawei | Brian.alexander.martin@huawei.com |

# Discussion

Companies are requested to add their comments for each of the treated CRs of this email discussion in the boxes below.

## Search space SIB1

[1] R2-2104952	Discussion on RMSI reception based on non-zero search space	OPPO, CMCC	discussion	LTE_NR_DC_CA_enh-Core

[2] R2-2104953	38331 R15 RMSI reception based on non-zero search space-option 1	OPPO	CR	Rel-15	38.331	15.13.0	2591	-	F	LTE_NR_DC_CA_enh-Core

[3] R2-2104954	38331 R16 RMSI reception based on non-zero search space-option 1	OPPO	CR	Rel-16	38.331	16.4.1	2592	-	A	NR_newRAT-Core

[4] R2-2104955	38331 R15 RMSI reception based on non-zero search space-option 2	OPPO	CR	Rel-15	38.331	15.13.0	2593	-	F	NR_newRAT-Core

[5] R2-2104956	38331 R16 RMSI reception based on non-zero search space-option 2	OPPO	CR	Rel-16	38.331	16.4.1	2594	-	A	NR_newRAT-Core

In [1], the company mentions that mapping between RMSI PDCCH occasion and SSBs for non-zero search space for searchSpaceSIB1 on dedicated BWP is missing.

In R15, the dedicated BWP can be configured for the UE and the common search space can be optionally configured for the BWP. If common search space is configured for the dedicated BWP, the UE will receive the SIB1/OSI/paging via common search space monitoring.

In R15, the beam sweeping is supported for SIB1/OSI/paging and the mapping between the SIB1/OSI/paging PDCCH monitoring occasions and SSBs is specified in RAN1 Spec and RAN2 Spec. However, after checking RAN1 spec and RAN2 spec, the case for non-zero search space for searchSpaceSIB1 is missing.

In [1], company summarize 4 observations:

Observation 1: Dedicated BWP can be configured with common search space for SIB 1, i.e. searchSpaceSIB1 IE and UE will follow 38.213 to receive SIB1.

Observation 2: The searchSpaceSIB1 IE can be set to non-zero except initial BWP.

Observation 3: For OSI, if searchSpaceOtherSystemInformation IE is set to 0, TS 38.213 will specify the mapping between OSI PDCCH monitoring occasions and SSBs. Otherwise, the TS 38.331 will specify.

Observation 4: RAN1 spec does not specify the mapping between RMSI PDCCH monitoring occasions and SSBs when searchSpaceSIB1 is set to non-zero.

Q1: Do companies agree the observations in [1]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| QCom | Yes |  |
| OPPO | Yes |  |
| Apple | No | Regarding observation 4, we think for the active DL BWP, if SIB1 is in CORESET#0 (of initial BWP), then it need follow RAN1 spec. If searchSpaceSIB1 points to a non-zero CORESET, then UE need follow RRC configuration, and there is no need to specify the mapping between RMSI PDCCH monitoring occasions and related QCL information in RAN1 spec for this case. |
| MediaTek | No for O4 | In TS 38.213 Section 10, it says that RMSI SS (=Type0-PDCCH CSS) can be provided by searchSpaceSIB1 in PDCCH-ConfigCommon. And it further specifies how to monitor PDCCH candidates if the searchSpaceID is zero or non-zero. Therefore, we think that probably there is no issue at all. We would suggest at least ask RAN1 before concluding there is a problem to fix. |
| Ericsson | Not sure | Our reading of O4 is that it only observes that RAN1 does not specify the mapping when search space is non-zero, not that RAN1 should specify this use case. RAN1 specifies perhaps how to monitor the PDCCH candidates, but not the mapping issue, e.g. to exclude SSBs that overlap with UL symbols? We are still checking with RAN1 colleagues though. |
| Nokia | No | We fail to see what the issue is here. Using non-zero SS ID for SIB1 (search space), would mean we are looking at non-cell defining SSB (i.e. there is no Type0-PDCCH SS or CORESET#0 config in MIB). i.e. there would not be any IDLE UEs for this SSB. Thus, no definition needed from the IDLE UE perspective. For CONNECTED mode, as per 38.213, the UE monitors as set by the search space like normally. Whether it is possible to give TCI state to CORESET#0 we have following in 38.213 defining the QCL for CORESET#0: ” For a CORESET with index 0, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with: - the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or - a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure.” So, we are not actually sure if anything is broken. |
| Huawei, HiSilicon | Yes/No | For Observation 1, it is true that dedicated BWPs can be configured with a common search space for SIB1, but it may not be necessary for RAN1 to specify mapping. In case of non-zero search space, the UE can just monitor the PDCCH occasions of the configured search space, and nothing is broken we think. |

Q2: Do companies agree the issue that mapping between RMSI PDCCH occasion and SSBs for non-zero search space for searchSpaceSIB1 on dedicated BWP is missing?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| QCom | Yes |  |
| OPPO | Yes |  |
| Apple | NO |  |
| MediaTek | No | See our comment above and suggest to check with RAN1 first. |
| Ericsson | Not sure | See Q1. |
| Huawei, HiSilicon | No | At least for R15/16, the mapping is not essential and the UE can monitor all PDCCH occasions of the configured search space. We are fine to enhance this in Rel-17 if needed. |

If companies agree the issue in Q2, there are 2 options to fix the issue in [1].

Option 1: RAN2 clarify that the searchSpaceSIB1 IE can only be set to zero for both initial DL BWP and dedicated BWP if configured.

Option 2: RAN2 capture text for the mapping between RMSI PDCCH occasion and SSBs as OSI did if searchSpaceSIB1 is set to non-zero.

the changes in TS 38.331 are provided in [1] for option 1 and option 2.

Q3: Which option do companies prefer to fix the issue in R15 and R16 respectively?

| Company | Option for R15 and R16? | Comments |
| --- | --- | --- |
| QCom | No strong view |  |
| OPPO | Option 1 for R15 and R16; Option 2 for R17 |  |
| MediaTek | Wait for RAN1. | If needed, we could only accept option 1 in R15/R16. |
| Ericsson | Option 2 from Rel-15 | If this is already captured in RAN1 spec, then this is already supported, and we do not need to do anything, but we are not sure about that. If option 2 from Rel-15 is not acceptable for other companies, we can consider option 2 for a later release (and introduce an IOT capability for this). |
| Huawei, HiSilicon | No need to fix in Rel15/16, can consider enhancement in R17 | In R15/16, it is not necessary to define the mapping and the UE can monitor all PDCCH occasions of the configured search space. We are fine to enhance this in Rel-17 if needed. |

Q4: Do companies agree to send LS to RAN1 for information?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| QCom | Neutral | Seems not needed |
| OPPO | Yes | It is up to majority view. |
| Apple | No | No need for LS to RAN1 |
| MediaTek | No strong view | If needed, we suggest simply check with RAN1 on whether there is an issue if RMSI SS is non-zero in a dedicate BWP. |
| Ericsson | No | Only if needed |
| Huawei, HiSilicon | Not for corrections | We can send the LS in TEI17 scope. |

## Stored SI

[6] R2-2105367	Clarification of cell Identity for SIB validity	vivo	CR	Rel-15	38.331	15.13.0	2621	-	F	NR_newRAT-Core

[7] R2-2105368	Clarification of cell Identity for SIB validity	vivo	CR	Rel-16	38.331	16.4.1	2622	-	A	NR_newRAT-Core

In [5][6], company thinks there is an ambiguity on cellIdentity when checking the validity of the stored version of a SIB in RAN sharing scenario.

Q5: Do companies agree the changes of the CR in [5][6]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| QCom | Yes |  |
| OPPO | Yes |  |
| Apple | Maybe not | Each PLMN-identify is only associated with one cell-identity, once the first PLMN is chosen, then there is no ambiguity of the cell identity. I do not think the current text is ambiguous. [vivo]: we agree with you that the cellIdentity should be the one associate with the PLMN-identity, however, there is no clear description about the relation between them in current spec. That’s why we want to clarify. |
| MediaTek | Yes with comment | We suggest to change the term “the first PLMN-Identity and the first cellIdentity in the PLMN-IdentityInfoList” with “the first PLMN-Identity and corresponding cellIdentity in PLMN-IdentityInfoList”. The reason is that it is unclear what does “first” cell Id mean. It is actually the cell ID that is included in the first PLMN. |
| Ericsson | No | We have not understood why "if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag" is unclear? There is only one cellIdentity associated with the first PLMN-Identity in the PLMN-IdentityInfoList (i.e. there is only one cellIdentity in PLMN-IdentityInfo): SIB1 ::= SEQUENCE { … cellAccessRelatedInfo CellAccessRelatedInfo, … CellAccessRelatedInfo ::= SEQUENCE { plmn-IdentityList PLMN-IdentityInfoList, … PLMN-IdentityInfoList ::= SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo PLMN-IdentityInfo ::= SEQUENCE { plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity, trackingAreaCode TrackingAreaCode OPTIONAL, -- Need R ranac RAN-AreaCode OPTIONAL, -- Need R cellIdentity CellIdentity, [vivo]: yes, we agree with you that there is only one cellIdentity associated with the first PLMN-Identity in the PLMN-IdentityInfoList. However, there are more than one cellIdentity in the PLMN-IdentityInfoList. Further, the current text does not mention “the cellIdentity associated with the first PLMN-Identity in the PLMN-IdentityInfoList”, right? |
| Nokia | No | Existing text looks fine to me. Not essential. Once the PLMN ID is unambiguously identified, the cell identity should be associated with that PLMN only. Why would the UE compare against one PLMN identity and then use a different cell identity of an unrelated PLMN? [vivo]: in the RAN sharing case, the UE may use the CellIdentity associate with the selected PLMN. |
| vivo | Yes | In SIB validity checking, the text in the specification doesn’t clarify which PLMN identity information the cellIdentity corresponding to. The “first” in the text may only aim at PLMN-Identity, which would cause a misunderstanding, some may think the cellIdentity corresponds to the first PLMN, while others may think it corresponds to the selected PLMN. if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB For instance, in RAN sharing case, the structure of the PLMN-IdentityInfoList may be like this: PLMN-IdentityInfolist. PLMN-IdentityInfo (operator 1); PLMN-IdentityInfo (operator 2). PLMN-Identity 1; PLMN-Identity 2. cellIdentity 1; cellIdentity 2. For users of operator2, the PLMN-Identity 1 is chosen for validity check as the spec is clear. While for cellIdentity, it may choose the cellIdentity 2 as it is associate with the selected PLMN (PLMN-Identity 2) and the current spec has not clearly specified it. I think we all have reached a consensus that the cellIdentity is the one associate with the first PLMN-Identity in PLMN-IdentityInfoList, we think the clarification is essential and we are fine with MTK’s suggestion. |
| Huawei, HiSilicon | No | We think this is not an essential change, while the proposed wording seems more accurate it also looks difficult to misinterpret the existing wording. |

# Conclusions

Based on the discussion above, we propose:

# Reference