3GPP TSG-RAN WG2 Meeting #114-e	R2-21xxxxx

Online, 19 – 27th of May 2021

Agenda item:	5.4.4

Source:	ZTE corporation, Sanechips

Title:	Report of [AT114-e] [013] [NR15] Idle Inactive mode (ZTE)

Document for:	Discussion and Decision

- Introduction

This document reflects the content and outcome of the following email discussion:

[AT114-e] [013][NR15] Idle Inactive mode (ZTE)

Scope: Treat R2-2105751, R2-2105744, R2-2105745, R2-2105752, R2-2105753, R2-2105754, R2-2105755, R2-2106196,

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed CRs.

Deadline: Schedule A

R2-2105751	Discussion on PO determination for UE in inactive state	ZTE corporation, Sanechips, Ericsson	discussion	Rel-15	NR_newRAT-Core

R2-2105744	Correction on PO determination for UE in inactive state-38.304	ZTE corporation, Sanechips, Ericsson	CR	Rel-16	38.304	16.4.0	0208	-	F	NR_newRAT-Core

R2-2105745	Correction on PO determination for UE in inactive state-38.306	ZTE corporation, Sanechips, Ericsson	CR	Rel-16	38.306	16.4.0	0592	-	F	NR_newRAT-Core

R2-2105752	Correction on PO determination for UE in inactive state-38.331	ZTE corporation, Sanechips, Ericsson	CR	Rel-16	38.331	16.4.1	2646	-	F	NR_newRAT-Core

R2-2105753	Correction on PO determination for UE in inactive state-36.331	ZTE corporation, Sanechips, Ericsson	CR	Rel-16	36.331	16.4.0	4663	-	F	LTE_5GCN_connect-Core

R2-2105754	Correction on PO determination for UE in inactive state-36.304	ZTE corporation, Sanechips, Ericsson	CR	Rel-16	36.304	16.3.0	0826	-	F	LTE_5GCN_connect-Core

R2-2105755	Correction on PO determination for UE in inactive state-36.306	ZTE corporation, Sanechips, Ericsson	CR	Rel-16	36.306	16.4.0	1815	-	F	LTE_5GCN_connect-Core

R2-2106196	Discussion on PO misalignment for INACTVIE and IDLE states	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

R2-2104907	Discussion on paging DRX cycle determination for inactive mode	vivo	discussion	Rel-15	NR_newRAT-Core	Late

2	Discussion

2.1	PO misalignment for RAN paging and CN paging

The following agreements have been reached in RAN2 NR Adhoc 201701:

Agreements:

1	A UE in INACTIVE is reachable via RAN-initiated notification and CN-initiated Paging. RAN and CN paging occasions overlap and same paging/notification mechanism used.

2	A RAN node can configure a UE in INACTIVE with a RAN configured paging DRX cycle (which could be UE specific configuration).

UE in inactive state has to monitor both CN-initiated paging and RAN-initiated paging and it has been expected in 38.300 that “The POs of a UE for CN-initiated and RAN-initiated paging are based on the same UE ID, resulting in overlapping POs for both.” so that UE in inactive can monitor CN-initiated paging and RAN-initiated paging in the overlapping POs.

However, the actual situation does not meet our expectations and it is possible that NW send CN paging and RAN paging in different POs and UE in active state may only monitor the PO calculated for RAN paging and miss CN paging.

The PF and PO for paging are determined by the following formula [1]:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N) * (UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

The following parameters are used for the calculation of PF and i_s above:

T: DRX cycle of the UE (T is determined by the shortest of the UE specific DRX value(s), if configured by RRC and/or upper layers, and a default DRX value broadcast in system information. In RRC_IDLE state, if UE specific DRX is not configured by upper layers, the default value is applied).

N: number of total paging frames in T(configured by nAndPagingFrameOffset with value T, T/2, T/4, T/8, or T/16)

Ns: number of paging occasions for a PF

PF_offset: offset used for PF determination

UE_ID: 5G-S-TMSI mod 1024

| Example of configuration | RAN paging cycle: 32rf Default paging cycle: 64rf N: T/16 Ns: 4 PF_offset: 14 UE_ID:5G-S-TMSI mod 1024 = 3 |
| --- | --- |
| RRC_INACTIVE | PF:2 34 66 98130 162 194 226 258 290 322 354 386 418 450 |
|  | i_s =1 |
| RRC_IDLE | PF:34 98 162 226 290 354 418 |
|  | i_s=0 |

For a UE with same UE ID, as shown in the above example, it is possible that the T used in inactive state is different from the T used in idle mode as NW is allowed to configure a RAN paging cycle different from the UE specific paging cycle configured by upper layer or the default value in system information while the N used in calculation is still the one broadcast in SIB1 with value T, T/2, T/4, T/8, or T/16 and will turn into different values when the T changes.

As a result, the index of the PO (i.e. the i_s) would be different for inactive state and idle state as the N is a value related to the T while the T has different value in idle and inactive state, which deviates from the intention that the POs of a UE for CN-initiated and RAN-initiated paging should be overlapped and inactive UE can monitor CN paging and RAN paging in the overlapped POs. Under this circumstance, it is worth considering what is the expected UE behavior for UE in RRC_INACTIVE. If UE in RRC_INACTIVE only monitors the RAN paging PO, CN paging failure would happen.

Question 1:  Do companies agree with the observation that “For a UE, the index of the PO calculated based on the same UE ID may be different in inactive state and idle state. If a UE in inactive state only monitors the PO derived for inactive state, CN paging failure would happen in both NR and eLTE”?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| Nokia | Yes |  |
| MediaTek | Yes |  |
| LGE | Yes |  |
| OPPO | Yes |  |

2.2	Potential Solutions

The following solutions can be considered to address the issue described above:

- Solution 1: It is up to NW implementation to ensure RAN and CN paging occasions overlap.

- Solution 2: UE in RRC_INACTIVE shall monitor both RAN and CN PO, in case RAN and CN PO are not overlapped.

- Solution 3: UE in RRC_INACTIVE should use the same i_s to determine PO as for RRC_IDLE and both CN paging and RAN paging will be sent on such POs.

Question 2:  Which solution do companies prefer to address the PO misalignment issue?

| Company | Preferred Solution | Comments |
| --- | --- | --- |
| Samsung | Solution 1 | We think that it is a corner case i.e. to be paged by CN paging for RRC_INACTIVE state due to RNAU. Besides, the concerned scenario seems only valid in case the value of RAN paging cycle is less than that of CN paging cycle. If any solution is required other than Solution 1, we prefer to go for Solution 3 from Rel-17. |
| Nokia | Solution 1 | Any other solution is not feasible in release 15 and having other solutions in later releases does not help too much as anyway UEs not supporting this would require to follow legacy paging procedure. |
| MediaTek | Solution 1 | As indicated by Samsung, this is a corner case. The paging for UE in INACTIVE mode is mainly from RAN paging. Monitoring CN paging in INACTIVE mode is something like error handling (Note that UE will transit to IDLE while receiving CN paging). So, hope that no CN paging reception is needed in most case. Considering that, we would prefer to have solution 1 in R15/R16. The simple way of solution 1 is to have same paging cycle for both CN and RAN paging. If anything is needed, we also prefer solution 3 from Rel-17. |
| LGE | Solution 1 | We do not support Sol 2. because it requires additional power consumption.. NW should ensure RAN and CN paging occasions overlap. If not, legacy UEs may miss the CN paging in INACIVE. |
| OPPO | Solution 1 | For R15/R16, no much benefit we can get, so solution1 is sufficient. For R17, we can consider both solution1 and solution3. |

Further details for solution 1

Question 2-1: If solution 1 is selected, is there a need to capture anything, e.g. It is up to NW implementation to ensure RAN and CN paging occasions overlap, in specs to reflect the agreement?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No | We think capturing it in the minutes seems enough. |
| Nokia | No | Same as Samsung |
| MediaTek | No | Same view as Samsung |
| LGE | No | Same as Samsung |
| OPPO | No | Same as Samsung |

Further details for solution 2

Question 2-2:  If solution 2 is selected, is there a need to capture anything, e.g. UE in RRC_INACTIVE shall monitor both RAN and CN PO, in case RAN and CN PO are not overlapped, in specs to reflect the agreement?

| Company | Yes/No | Comments |
| --- | --- | --- |
| LGE |  | We don’t support sol. 2, but if this solution is agreed, it can be specified like “UE in RRC_INACTIVE shall monitor both RAN and CN PO”. |

Further details for solution 3

If solution 3 is selected, a UE capability should be introduced to indicate UE support for using the same i_s in PO determination in RRC_INACTIVE state as in RRC_IDLE state so that NW can identify such UE and send both CN paging and RAN paging in the same POs.

Question 2-3-1:  If solution 3 is selected, is there a need to introduce a UE capability to indicate UE support for using the same i_s in PO determination in RRC_INACTIVE state as in RRC_IDLE state?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Without a UE capability, NW has no idea whether UE can support it. |
| MediaTek | Yes |  |
| LGE | Yes |  |
| OPPO | Yes |  |

For solution 3, the following options can be considered on the signaling details:

- Option 1: If UE indicates support for such behaviour, NW send an indication (e.g. useIdlePO) in RRCRelease message when release UE from RRC_CONNECTED to RRC_INACTIVE state and ensure that both CN paging and RAN paging will be sent in the same POs (i.e. the idle POs) within the configured RNA.

- The useIdlePO in RRCRelease message can only be configured when all the gNBs within the RNA support to send both CN paging and RAN paging in the same POs, i.e. the idle POs.

- The indication (e.g. useIdlePO) is also sent to the neighbour RAN nodes for Xn-paging.

- Option 2: Broadcast an indication (e.g. ranPagingInIdlePO) in system information to show if network supports to send both CN paging and RAN paging in the idle POs.

- UE monitor CN paging and RAN paging in idle POs when both of the anchor cell and serving cell broadcast ranPagingInIdlePO.

- To assist Xn-paging, the UE capability for using the same i_s in PO determination in RRC_INACTIVE state as in RRC_IDLE state will be sent to neighbour RAN nodes.

Question 2-3-2:  If solution 3 is selected, which option do companies prefer on the signaling details?

| Company | Option | Comments |
| --- | --- | --- |
| Samsung | Option 1 |  |
| MediaTek | Option 1 |  |
| LGE | Option 2 |  |
| OPPO | either |  |

Question 2-3-3: Which option do companies prefer to reflect the changes needed for solution 3?

- Option 1: Support solution 3 since Rel-16 and agree on Rel-16 CRs with magic sentence to support early implementation.

- Option 2: Support solution 3 since Rel-17.

| Company | Option | Comments |
| --- | --- | --- |
| Samsung | Option 2 | See our previous comments on Question 2. |
| MediaTek | Option 2 |  |
| LGE | Option 2 |  |
| OPPO | Option 2 |  |

3	Conclusions

Based on the views expressed in the previous sections, we propose the following:

To be added

Contact information

| Company | Delegate contact |
| --- | --- |
| Samsung | Sangyeob Jung (sy0123.jung@samsung.com) |
| MediaTek | Felix Tsai (chun-fan.tsai@mediatek.com) |
| LGE | HyunJung Choe (Stella.choe@lge.com) |
| OPPO | fanjiangsheng@oppo.com |