3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2107841

Online, 16th – 27th Aug 2021

Source:	vivo

Title: 	UAC enhancements for minimization of service interruption when disaster condition applies

Agenda Item:	8.22

Document for:	Discussion and Decision

# Introduction

CT1 has discussed solutions to minimize service interruption when disaster condition applied. CT1 would like to have RAN2 feedback on two solutions that may require some changes to UAC procedure [1] as follows:

# 1 Overall description Based on the stage 1 requirements on minimization of service interruption described in clause 6.31 of 3GPP TS 22.261, CT1 has been performing stage 2 study (see 3GPP TR 24.811 for further details). Among the solutions that CT1 has not excluded to progress during the normative phase, there are two solutions impacting UAC after a disaster inbound roamer selects a PLMN without disaster condition: Solutions #38 and #40. These solutions require changes in the barring configuration in addition to introducing Access Identity 3 (see 3GPP TS 22.261), which are in the remit of RAN2. # 2 Actions To RAN2 ACTION: CT1 kindly request RAN2 provide feedback on Solutions #38 and #40 described in 3GPP TR 24.811.

This contribution analyzes theses two solutions from RAN2 perspective and proposes to respond CT1 in [2]

# Discussion

## Solution#38

Solution#38 is described in TR 24.811 as follows:

Within UAC-BarringInfoSet, an NG-RAN node can include barring factor for Access Identity 3. During the access barring check, if the UE NAS layer provides Access Identity 3 to the UE RRC layer together with an access category, the UE RRC layer decides whether the access attempt is allowed or not based on the value of the barring factor for Access Identity 3 associated with the access category and a random number drawn if none of the bit(s) for other access identity(ies) in uac-BarringForAccessIdentity is set to zero. NOTE: The change in the UAC-BarringInfoSet proposed in this solution is subject to RAN2 agreement. The 5GSM level congestion can be prevented by properly setting the values of the barring factor for Access Identity 3 each of which is associated with an access category. For example, if access attempts of disaster inbound roamers related to DNN X should be reduced, the barring factor for Access Identity 3 associated with an operator-defined access category for DNN X can be adjusted.

SA1 access identity requirement for potential disaster configuration is highlight below:

Table 6.22.2.2-1: Access Identities Access Identity number; UE configuration. 0; UE is not configured with any parameters from this table. 1 (NOTE 1); UE is configured for Multimedia Priority Service (MPS).. 2 (NOTE 2); UE is configured for Mission Critical Service (MCS).. 3; UE for which Disaster Condition applies (note 4). 4-10; Reserved for future use. 11 (NOTE 3); Access Class 11 is configured in the UE.. 12 (NOTE 3); Access Class 12 is configured in the UE.. 13 (NOTE 3); Access Class 13 is configured in the UE.. 14 (NOTE 3); Access Class 14 is configured in the UE.. 15 (NOTE 3); Access Class 15 is configured in the UE.. NOTE 1: Access Identity 1 is used by UEs configured for MPS, in the PLMNs where the configuration is valid. The PLMNs where the configuration is valid are HPLMN, PLMNs equivalent to HPLMN, and visited PLMNs of the home country. Access Identity 1 is also valid when the UE is explicitly authorized by the network based on specific configured PLMNs inside and outside the home country. NOTE 2: Access Identity 2 is used by UEs configured for MCS, in the PLMNs where the configuration is valid. The PLMNs where the configuration is valid are HPLMN or PLMNs equivalent to HPLMN and visited PLMNs of the home country. Access Identity 2 is also valid when the UE is explicitly authorized by the network based on specific configured PLMNs inside and outside the home country. NOTE 3: Access Identities 11 and 15 are valid in Home PLMN only if the EHPLMN list is not present or in any EHPLMN. Access Identities 12, 13 and 14 are valid in Home PLMN and visited PLMNs of home country only. For this purpose, the home country is defined as the country of the MCC part of the IMSI. NOTE 4: The configuration is valid for PLMNs that indicate to potential Disaster Inbound Roamers that the UEs can access the PLMN. See clause 6.31..

This solution works as follows:

-	The UE needs to be able to read information related to Access identity 3 in the barring information including barring factor for Access Identity 3.

-	The UE needs to determine whether an access attempt associated with Access Identity 3 is allowed based on the value of the barring factor for Access Identity 3

When network configures some specific access categories access identity barring factor, disaster UEs in the network would use same barring factor value. And those disaster UEs would be treated with same UAC.

This solution has the advantage that it can work and does not require any change to RRC ASN.1, only UE NAS may be impacted as NAS would have to apply the AI = 3. But, as UAC parameters are per cell, UEs with AI different from 3 may be allowed. However, other UEs with different AI may be barred.

- Solution#38 has to just comply with SA1 requirement configuration for AI =3 to work

- With just some NAS enhancement, Solution#38 would require a new Access Identity to work without any additional RRC enhancement and mostly can follow existing mechanisms.

## Solution#40

Solution#40 is described in TR 24.811 as follows:

It is important for the PLMN providing disaster roaming to avoid degradation of service levels to its own subscribers due to the activities of incoming UEs. For this reason, the network needs a method by which it can set differential access barring levels for own subscribers and disaster roamers. The usage of new Access Identity 3 allows network to differentiate inbound roamers from own subscribers. But this alone would not be useful to mitigate congestion caused by a sudden inflow of inbound roamers. A new offset value is introduced to the unified access control barring information. A UE which is registered or attempting registration in a PLMN which is on the forbidden PLMN list or on the list of "forbidden tracking areas for roaming", but which is broadcasting "disaster roaming active", shall apply a uac-DisasterOffsetToBarringFactor to the uac-BarringFactor. The uac-DisasterOffsetToBarringFactor indicates to the disaster roaming UEs the offset value by which the BarringFactor must be reduced when evaluating the access barring condition for that access category. The uac-DisasterOffsetToBarringFactor is set per access category. The uac-DisasterOffsetToBarringFactor is defined as a range of s5 till s95 in steps of 5. A disaster roaming UE computes the uac-BarringFactor for its access category as uac-BarringFactor = max (p00, (uac-BarringFactor - uac-DisasterOffsetToBarringFactor)) NOTE: The addition of a new parameter to UAC Barring information is subject to RAN2 agreement.

Beside SA1 access identity requirement of AI = 3, solution#40 also requires an additional offset as a combination with AI = 3 to work.

- In addition to SA1 requirement configuration for AI number, Solution#40 would also require an offset for BarringFactor to work

This solution works by defining a unified access control: of a new offset for BarringFactor.

When network configures some specific access categories access identity barring factor, disaster UEs in the network may use different barring factor value. And those disaster UEs would be may be treated with different UAC. During UAC procedure, Solution#40 reject some disaster UE accessing the network, i.e., some disaster UE may be barred and would be denied access.

For this solution to work, some significant RRC enchantment may be required as current RRC Access barring check procedure would not comply with the introduction of the new offset uac-DisasterOffsetToBarringFactor. And the required RRC enhancement may not be backward compatible. Thus, this solution needs a new Access identity and an offset, which has huge impact of RRC.

- Besides NAS enhancement, Solution#40 would also require a new Access Identity and an offset which may require some significant RRC protocol enchantment to work.

# Conclusion

The observations and proposals are the following:

- Solution#38 has to just comply with SA1 requirement configuration for AI =3 to work

- With just some NAS enhancement, Solution#38 would require a new Access Identity to work without any addition RRC enhancement and mostly can follow existing mechanisms.

- In addition to SA1 requirement configuration for AI number, Solution#40 would also require an offset for BarringFactor to work

- Beside NAS enhancement, Solution#40 would also require a new Access Identity and an offset which may require some significant RRC protocol enchantment to work.

- RAN2 to reply to CT1 with draft LS reply in [2]

- Reference

- R2-2106902, LS on UAC enhancements for minimization of service interruption when disaster condition applies

- R2-2107840, Draft LS reply on UAC enhancements for minimization of service interruption when disaster condition applies