3GPP TSG-RAN WG2 #115-e	R2-2108368

Online, Aug 16 – Aug 27, 2021

Agenda Item:	5.4.3

Source:	ZTE Corporation, Sanechips

Title:	Discussion on BWP switch for TDD

Document for:	Discussion, Decision

1	Introduction

In spec 38321 section 5.15, there is the following description coloured as below on BWP switch for TDD:

------------------

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id for SpCell or activation of an SCell, the DL BWP and/or UL BWP indicated by firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id respectively (as specified in TS 38.331 [5]) is active without receiving PDCCH indicating a downlink assignment or an uplink grant. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

--------------------

In this paper, we discuss some ambiguities on BWP switch for TDD.

2	Discussion

For the description ‘BWP switching is common for both UL and DL’ above, we think it means that DL BWP switching and UL BWP switching are simultaneous for TDD.

Observation 1: DL BWP switching and UL BWP switching are simultaneous for TDD.

But how to achieve the simultaneous DL and UL BWP switching, we need to discuss further including DCI-based and RRC-based BWP switching.

DCI-based BWP switching:

For TDD, when a UE receives a DCI1_1 indicating a DL BWP switching, the UE shall switch the current DLBWP and ULBWP simultaneously to the target DLBWP and ULBWP with the same BWP-ID indicated by the DIC1_1. Similarly, when a UE receives a DCI0_1 indicating a UL BWP switching, the UE shall switch the current ULBWP and DLBWP simultaneously to the target ULBWP and DLBWP with the same BWP-ID indicated by the DIC0_1. That is for TDD when a UE receives a DCI1_1 indicating a DL BWP switching or a DCI0_1 indicating a UL BWP switching, the UE shall switch the current DLBWP and ULBWP simultaneously to the target DLBWP and ULBWP with the same BWP-ID indicated by the DCI.

Observation 2: For TDD, when a UE receives a DCI1_1 indicating a DL BWP switching or a DCI0_1 indicating a UL BWP switching, the UE shall switch the current DLBWP and ULBWP simultaneously to the target DLBWP and ULBWP with the same BWP-ID indicated by the DCI.

RRC-based BWP switching:

For TDD RRC-based BWP switching, how to achieve the simultaneous DL and UL BWP switching, there are two comprehensions.

- Comprehension 1: UE ensures the simultaneous DL and UL BWP switching for TDD. That is when a UE receives a RRC message only including the field firstActiveDownlinkBWP-Id that indicates a DL BWP switching, the UE shall switch the current DLBWP and ULBWP simultaneously to the target DLBWP and ULBWP with the same BWP-ID indicated by firstActiveDownlinkBWP-Id. Similarly, when a UE receives a RRC message only including the field firstActiveUplinkBWP-Id that indicates a UL BWP switching, the UE shall switch the current ULBWP and DLBWP simultaneously to the target ULBWP and DLBWP with the same BWP-ID indicated by firstActiveUplinkBWP-Id. For this comprehension, NW can only include firstActiveDownlinkBWP-Id or firstActiveUplinkBWP-Id in one RRC message for TDD BWP switching.

- Comprehension 2: NW ensures the simultaneous DL and UL BWP switching for TDD. That is when NW wants to switch the DLBWP and/or UL BWP by RRC, NW should include the fields firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id simultaneously in a same RRC message.

We kindly request companies to check your comprehensions and product’s implementation. Considering that NW needs more robust performance, we prefer comprehension 2. So the following proposals are proposed.

Proposal 1: For TDD, when NW wants to switch the DLBWP and/or UL BWP by RRC, NW should include the fields firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id simultaneously in a same RRC message.

Proposal 2: Based on proposal 1, agree the CRs in [2][3].

3	Summary

Based on the discussion above we propose the following proposals:

Observation 1: DL BWP switching and UL BWP switching are simultaneous for TDD.

Observation 2: For TDD, when a UE receives a DCI1_1 indicating a DL BWP switching or a DCI0_1 indicating a UL BWP switching, the UE shall switch the current DLBWP and ULBWP simultaneously to the target DLBWP and ULBWP with the same BWP-ID indicated by the DCI.

Proposal 1: For TDD, when NW wants to switch the DLBWP and/or UL BWP by RRC, NW should include the fields firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id simultaneously in a same RRC message.

Proposal 2: Based on proposal 1, agree the CRs in [2][3].

4 Reference

[1] spec 38321-fc0

[2] R2-2108369 Correction on firstActiveBWP-Id for TDD        ZTE Corporation, Sanechips

[3] R2-2108370 Correction on firstActiveBWP-Id for TDD(R16)   ZTE Corporation, Sanechips