3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108680

Online, August 09 – August 27, 2021

Source:	CATT

Title:	Consideration on PDCP protocol in SDT

Agenda Item:	8.6.2

Document for:	Discussion and Decision

Introduction

In RAN2 previous meetings, various agreements were made on PDCP [1]-[2].

RAN2#112 meeting 2 For both RACH and CG based solutions, upon initiating RESUME procedure for SDT initiation (i.e. for first SDT transmission), the UE shall re-establish at least the SDT PDCP entities and resume the SDT DRBs that are configured for small data transmission (along with the SRB1).

RAN#113bis meeting 1 The UE performs PDCP re-establishment implicitly, i.e. without explicit indication for PDCP re-establishment, when the UE initiates SDT procedure. 2 As in legacy, whether to support ROHC continuity is explicitly configured by the network. 3 PDCP duplication is not supported for SDT

However, there are some issues left in PDCP. In this document, we discuss the remaining issues in PDCP and provide our preferences. A TP for PDCP is also provided.

Discussion

- PDCP status report

In RAN2#113bis meeting, whether the UE implicitly disables PDCP status report was discussed in Summary of UP open issues [2].

If PDCP status report is disabled explicitly by the network indication, statusReportRequired should be set to false before UE enters RRC Inactive state. One possible procedure is that:

- Step 1: Before the UE enters into RRC Inactive state, the network sends RRCReconfig with statusReportRequired set to false to the UE. And UE maintains this configuration.

- Step 2: The network sends RRCRelease message to indicate the UE goes into RRC Inactive state.

- Step 3: the UE suspends PDCP.

- Step 4: When the UE initiates SDT procedure, it resumes the stored PDCP configuration and finds that PDCP status report is disabled by statusReportRequired. Then, the PDCP status report is disabled.

Apparently, extra signaling, i.e. RRCReconfig is needed to make sure that the PDCP status report is disabled. Furthermore, it should be avoided that both RRCReconfig and RRCRelease are transmitted together to make sure that RRCReconfig works before RRCRelease. The disabling of PDCP status report explicitly may result in both signaling overhead and latency.

On the other hand, disabling of the PDCP status report implicitly by the UE would bring some specification impacts. However, considering performance impacts, we slightly prefer the disabling of PDCP status report implicitly by the UE.

Proposal 1: The UE implicitly disables PDCP status report when SDT is initiated.

The corresponding TP is provided in Annex.

- Data volume calculation in PDCP

In RAN2#111 meeting, it has been agreed that data volume will be taken into account during SDT initiation procedure [3].

8	Data volume threshold is used for the UE to decide whether to do SDT or not.

However, it has not been decided how to perform the data volume for SDT. And there are 4 options provided in [4].

- Option 1: Data volume used for SDT selection criteria is calculated same as BS, i.e. PDCP data volume + RLC data volume, without considering RLC and MAC headers.

- Option 2: Data volume used for SDT selection criteria is the size of MAC PDU, i.e. PDCP data volume + RLC data volume + MAC/RLC/PDCP/SDAP/RRC overhead.

- Option 3: Data volume used for SDT selection criteria is the PDCP data volume.

- Option 4: Data volume used for SDT selection criteria is left up to UE implementation.

The MAC/RLC PDUs have not been built at the SDT initiation procedure. Therefore, it is not accurate to estimate the RLC data volume as well as RLC headers/MAC headers. Therefore, we support:

Proposal 2: Data volume used for SDT selection criteria is the PDCP data volume (Option 3).

The corresponding TP is provided in Annex.

- ROHC continuity

In RAN2#113bis meeting, it has been agreed that ROHC continuity is supported [2].

2	As in legacy, whether to support ROHC continuity is explicitly configured by the network.

The ROHC continuity can be performed when the anchor is not relocated. But if the ROHC continuity is configured, but the anchor is relocated during SDT initiation, ROCH decompression may fail. Some solutions addressing this issue are provided in [5]-[6]. However, it has not been decided which node and when to decide whether the anchor relocation is performed. A candidate solution can be that the anchor node decides whether to perform anchor relocation after receiving indication, e.g. first SDT packet or signalling request from the serving cell. Obviously, the UE can’t figure out whether anchor relocation will happen or not in SDT initiation. Hence, the UE can’t decide whether to perform ROHC continuity by checking the cell ID or whether UE stays within the same RNA area. Therefore, a simple way is to leave the decision to network implementation, i.e. when ROHC continuity is configured but the anchor relocation happen, whether to continue ROHC is or not is left to the network implementation.

Proposal 3: It is up to network implementation to deal with the issue that ROHC continuity is configured but anchor relocation happens.

Conclusion

This contribution analyzed some issues left in PDCP. We conclude the following proposals:

Proposal 1: The UE implicitly disables PDCP status report when SDT is initiated.

Proposal 2: Data volume used for SDT selection criteria is the PDCP data volume (Option 3).

Proposal 3: It is up to network implementation to deal with the issue that ROHC continuity is configured but anchor relocation happens.

Reference

- RAN2#112 meeting report;

- RAN2#113bis meeting report;

- RAN2#111 meeting report;

- R2-2104395, Summary of UP SDT open issues, LG;

- R2-2103018, User plane open issues for SDT, ZTE Corporation, Sanechips;

- R2-2104784	User Plane Common Aspects of RACH and CG based SDT	Samsung Electronics Co., Ltd;

- Annex

The first change

### 5.4.1	Transmit operation

For AM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired in TS 38.331 [3]), the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a PDCP entity re-establishment which is not triggered by SDT;

-	upper layer requests a PDCP data recovery;

-	upper layer requests a uplink data switching;

-	upper layer reconfigures the PDCP entity to release DAPS and daps-SourceRelease is configured in TS 38.331 [3].

For UM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired in TS 38.331 [3]), the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a uplink data switching.

For AM DRBs in the sidelink, the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a PDCP entity re-establishment.

If a PDCP status report is triggered, the receiving PDCP entity shall:

-	compile a PDCP status report as indicated below by:

-	setting the FMC field to RX_DELIV;

-	if RX_DELIV < RX_NEXT:

-	allocating a Bitmap field of length in bits equal to the number of COUNTs from and not including the first missing PDCP SDU up to and including the last out-of-sequence PDCP SDUs, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the bitmap field as '0' for all PDCP SDUs that have not been received, and optionally PDCP SDUs for which decompression have failed;

-	setting in the bitmap field as '1' for all PDCP SDUs that have been received;

-	submit the PDCP status report to lower layers as the first PDCP PDU for transmission via the transmitting PDCP entity as specified in clause 5.2.1 for Uu interface and in clause 5.2.3 for PC5 interface.

The Second change

## 5.6	Data volume calculation

For the purpose of MAC buffer status reporting or SDT initiation evaluation, the transmitting PDCP entity shall consider the following as PDCP data volume:

-	the PDCP SDUs for which no PDCP Data PDUs have been constructed;

-	the PDCP Data PDUs that have not been submitted to lower layers;

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

If the transmitting PDCP entity is associated with at least two RLC entities, when indicating the PDCP data volume to a MAC entity for BSR triggering and Buffer Size calculation (as specified in TS 38.321 [4] and TS 36.321 [12]), the transmitting PDCP entity shall:

-	if the PDCP duplication is activated for the RB:

-	indicate the PDCP data volume to the MAC entity associated with the primary RLC entity;

-	indicate the PDCP data volume excluding the PDCP Control PDU to the MAC entity associated with the RLC entity other than the primary RLC entity activated for PDCP duplication;

-	indicate the PDCP data volume as 0 to the MAC entity associated with RLC entity deactivated for PDCP duplication;

-	else (i.e. the PDCP duplication is deactivated for the RB or the RB is a DAPS bearer):

-	if the split secondary RLC entity is configured; and

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the primary RLC entity and the split secondary RLC entity is equal to or larger than ul-DataSplitThreshold:

-	indicate the PDCP data volume to both the MAC entity associated with the primary RLC entity and the MAC entity associated with the split secondary RLC entity;

-	indicate the PDCP data volume as 0 to the MAC entity associated with RLC entity other than the primary RLC entity and the split secondary RLC entity;

-	else, if the transmitting PDCP entity is associated with the DAPS bearer:

-	if the uplink data switching has not been requested:

-	indicate the PDCP data volume to the MAC entity associated with the source cell;

-	else:

-	indicate the PDCP data volume excluding the PDCP Control PDU for interspersed ROHC feedback associated with the source cell to the MAC entity associated with the target cell;

-	indicate the PDCP data volume of PDCP Control PDU for interspersed ROHC feedback associated with the source cell to the MAC entity associated with the source cell;

-	else:

-	indicate the PDCP data volume to the MAC entity associated with the primary RLC entity;

-	indicate the PDCP data volume as 0 to the MAC entity associated with the RLC entity other than the primary RLC entity.