3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108831

Online, August, 2021

Agenda item:	10.1

Source: 	Vice Chairman (Nokia)

Title:	Report on LTE legacy, 71 GHz, DCCA, Multi-SIM and RAN slicing

Document for:	Approval

# Organizational

List of offline email discussions:

NOTE: the email discussion deadlines are meant to allow at least all regions to have one day to comment (other than weekend) and also give rapporteurs time to update their proposals before the meeting)

Organizational

[AT115-e][200] Organizational – LTE legacy, 71 GHz, DCCA, Multi-SIM and RAN slicing (RAN2 VC)

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions

    - Share meetings notes and agreements for review and endorsement

    - Flag LSs and in-principle agreed CRs for discussion

Intended outcome (for LS discussion):

    - General information sharing about the sessions

Deadline for providing comments to LSs:

    - Deadline: 2nd week Mon, UTC 0900

LTE Legacy (kicked off at meeting start)

[AT115-e][201][LTE] Miscellaneous LTE CRs (Samsung)

Scope:

    - Discuss LTE CRs marked for this discussion (if needed)

Intended outcome:

    - Discussion report in R2-2108851

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Thu, UTC 1700

    - Deadline for CR finalization: 2nd week Wed, UTC 0900

LTE Rel-17 (only started after online session, all still TBD)

[AT115-e][202][LTE/NR] Inclusive language (Ericsson)

Scope:

    - Draft LS (To: SA5) according to RAN2 decisions on inclusive language alignment between WGs and TSGs

Intended outcome:

    - Discussion report in R2-2108852 (if needed)

    - Agreeable LS in R2-2108853

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  2nd week Thu, UTC 0900

    - Deadline for final LS: 2nd week Thu, UTC 1200

[AT115-e][203][LTE] Further details of TEI17 topics (NN)

Scope:

    - Discuss details of selected TEI17 topics (as per online session agreements)

Intended outcome:

    - Discussion report in R2-2108854

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Wed, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Wed, UTC 1700

NR Extension to 71 GHz (only started after online session, all still TBD)

[AT115-e][210][71 GHz] Capability differentiation between FR2-1 and FR2-2 (NN)

Scope:

    - Discuss which RAN2 capabilities require differentiation between FR2-1 and FR2-2.

Intended outcome:

    - Discussion summary in R2-2108858 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  2nd week Wed, UTC 1200

    - Initial deadline (for rapporteur summary):  2nd week Thu, UTC 1200

NR Rel-17 DCCA (only started after online session, all still TBD)

[AT115-e][220][R17 DCCA] UP aspects of SCG deactivation (NN)

Scope:

    - Discuss the UP aspects (bearer handling, RLC) of SCG (de)activation based on online discussion FFSs

Intended outcome:

    - Discussion summary in R2-2108862 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur summary):  2nd week Mon, UTC 1000

[AT115-e][221][R17 DCCA] UE measurements when SCG is deactivated (NN)

Scope:

    - Discuss further details on UE measurements when SCG is deactivated (based on online discussion)

Intended outcome:

    - Discussion summary in R2-2108863 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur summary):  2nd week Mon, UTC 1000

[AT115-e][222][R17 DCCA] Activation of deactivated SCG (NN)

Scope:

    - Discuss further details on deactivated SCG activation (based on online discussion)

Intended outcome:

    - Discussion summary in R2-2108864 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur summary):  2nd week Mon, UTC 1000

NR Rel-17 Multi-SIM

[AT115-e][230][MUSIM] Discussion on AS vs. NAS-based busy indication (Intel)

Scope:

    - Discuss details required to reply to SA2/CT1 and draft the reply LS

Intended outcome:

    - Draft LS to SA2/CT1 in R2-2108856 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Wed, UTC 0900

    - Initial deadline (for draft LS):  1st week Wed, UTC 1700

NR Rel-17 Multi-SIM (only started in 2nd week, all still TBD)

[AT115-e][231][MUSIM] Paging with service indication (NN)

Scope:

    - Discuss remaining open issues for paging with service indication and try to have draft TPs to illustrate the necessary modifications

Intended outcome:

    - Discussion summary in R2-2108857 (by email rapporteur, including TPs)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  2nd week Mon, UTC 1000

    - Initial deadline (for rapporteur summary):  2nd week Tue, UTC 1000

[AT115-e][232][MUSIM] LS to RAN4 on gap handling (NN)

Scope:

    - Draft LS to RAN4 on RAN2 decisions for gap handling

Intended outcome:

    - LS to RAN4 in R2-2108861 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  2nd week Tue, UTC 0900

    - Initial deadline (for draft LS):  2nd week Tue, UTC 1700

NR Rel-17 RAN Slicing

[AT115-e][240][Slicing] Reply LS to SA2 on band-specific slices in cell reselection (Nokia)

Scope:

    - Draft reply LS to SA2 LS R2-2106972 (S2-2105158).

Intended outcome:

    - Draft LS to SA2/CT1 in R2-2108860 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 1000

    - Initial deadline (for final draft LS):  1st week Fri, UTC 0900

Dates and deadlines – Elections

August 9th 0900 UTC	Elections Start. If there is only one nominee per position voting will not be needed.

Dates and deadlines – Technical Meeting

August 5th 23.59 PDT	(August 6th 06.59 UTC) Tdoc number allocation deadline.
Tdoc Submission Deadline. Kick off, summaries.  Deadline long Post114-e email discussions (hopefully the report can be available at the deadline or not long after).

August 12th 0700 UTC	Tdocs submission deadline for Summaries

August 16th 0700 UTC	e-Meeting Start (by email) (August 17th 0700 UTC is first possible email deadline).

August 20th 1000 UTC 	Weekend break, Suspend decision making in email discussions (= no deadlines etc)

It should be possible for a delegate to take the weekend off, rejoin and not miss decisions.

August 23rd 0800 UTC	Resume after weekend. Resume decision making in email discussions.

August 27th 1000 UTC	e-Meeting Stop, no more email comments for AT-meeting email discussions. Decision confirmations 
			announced within 24h. Session notes for email checking.

September 3rd		Deadline Short Post115-e email discussions (note that the time to RP is short).

Web Conference Schedule

Note that this schedule is indicative and can change. After Week 1 the schedule for Week 2 will be updated. No Overtime, Hard stop at UTC 15.55 and UTC 05:10

WEEK 1:

| Time Zone UTC | Web Conference R2 - Main | Web Conference R2 - BO1 | Web Conference R2 - BO2 |
| --- | --- | --- | --- |
| Monday |  |  |  |
| 12:15-13:05 | Main session (Johan) [1] Announcement (2 min) [3] Inc LS. CP corrections: [6.1.4.1.1] CHO, [6.1.4.1.5] CandidateBeamRSList, [6.1.4.3] n77, [6.1.4.5] RRM relax [5.4.1.1] Common Fields, NR17 Other: [8.22] BCS5/4 (if time) UP corrections: [6.1.3] UL skip, UCI pdu handling. | NR16 Pos (Nathan) [6.3.2] [6.3.3] [6.3.1] | NR17 NTN, non-pos aspects (Sergio) - [8.10.1] - [8.10.2.1] - [8.10.2.2] - [8.10.2.3] |
| 13:05-14:25 |  | NR17 Multi-SIM (Tero) - [8.3.3]: Outcome of [Post114-e][243][MUSIM] Gap handling (ZTE) - [8.3.3]: Outcome of [Post114-e][242][MUSIM] Switching message details (vivo) - 8.3.4: Short online discussion, will continue in offline discussion - [8.3.1] SA2 LS on busy indication (S2-2105150), will continue in offline discussion (for LS draft) | NR17 NTN (Sergio) - [8.10.3.1] - [8.10.3.2] - [8.10.3.3] |
| 14:25-15:45 | TEI17 (Johan) Clocked presentations and initial comments. | 14:25-14:55: NR17 Tero Early Items - MUSIM: [8.3.x] overflow (if necessary for SA2 LS discussion) - RAN slicing [8.8.1]: discussion on whether SA2 proposal on band-specific slices in cell reselection has impacts on the RAN (cv. SA2 LS R2-2106972) 14:55 – 15:45: NR17 SL enh (Kyeongin) [POST114-e][704] [POST114-e][705] (if time allows) | LTE17 IoT (Brian) |
| Tuesday |  |  |  |
| 12:15-13:05 | NR17 QoE (Johan) | NR17 RAN Slicing (Tero) - [8.8.2]: At least outcome of [Post114-e][251][Slicing] Solution direction details for slice priorities in cell reselection (Lenovo) - [8.8.3]: At least outcome of [Post114-e][252][Slicing] RACH partitioning details for slicing (CMCC) - [8.8.2]: SA2 LS reply (if further discussion needed) | NR17 Small Data Enh (Diana) |
| 13:05-14:25 | NR17 eIAB (Johan) | NR16 V2X (Kyeongin) 6.2.1, 6.2.2, 6.2.3 (if time allows) | NR17 Small Data Enh (Diana) |
| 14:25-15:45 | NR17 ePowSav (Johan) | NR17 DCCA (Tero) - [8.2.2.3]: outcome of [Post114-e][231][R17 DCCA] SCG activation/deactivation options (Huawei) - [8.2.2.1]/[8.2.2.3]: UP issues (may continue in offline discussion) - [8.2.2.3]: UE request for SCG activation and other activation details - [8.2.2.2] UE measurements for deactivated SCG (may continue in offline discussion) - [8.2.3.1] Inter-node message design and reply to RAN3 LS - [8.2.3.2] outcome of [Post114-e][233][R17 DCCA] Uu Message design for CPAC (CATT) (if time allows) | NR17 SL enh (Kyeongin) [POST114-e][705] [POST114-e][706] (if time allows) |
| Wednesd |  |  |  |
| 12:15-13:05 | NR17 eNPN (Johan) | 12:15-13:35: NR17 RedCap (Sergio) - [8.12.1] - [8.12.2.1]: including outcome of [Post114-e][105] - [8.12.2.2]: - [8.12.3.1] - [8.12.3.2] | NR17 SL Relay (Nathan) [8.7.1] [8.7.2.2] |
| 13:05-14:25 | NR17 Multicast (Johan) | 13:35-14:25: NR17 CovEnh (Sergio) - [8.19.1] - [8.19.2] | NR17 Pos (Nathan) [8.11.1] [8.11.2] [8.11.3] |
| 14:25-15:45 | NR17 Multicast (Johan) | NR17 SONMDT (HuNan) | NR17 IIOT URLLC (Diana) |
| Thursday |  |  |  |
| 04:00-05:00 | NR17 feMIMO (Johan) | NR17 SL Relay (Nathan) [8.7.2.3] [8.7.2.1] | LTE16e IoT (Emre, Brian) [4.2] [7.2.1] [7.2.2] |
| Friday |  |  |  |
| 04:00-05:00 | NR17 Other (Johan) | NR17 SL Relay (Nathan) [8.7.2.4] [8.7.3.1] [8.7.3.2] | LTE All releases Misc (Tero) LTE17 (Tero) - Inclusive language (SA5 LS + discussion,LS reply drafting to continue in offline [202]) LTE15/16 (Tero) - Outcome of LTE offline [201] LTE17 (Tero) - TEI17 topics (timed presentations with short time for comments, some may continue in offline discussion [203]) IF NEEDED (TBC at least 24h before the session): MUSIM/RAN slicing LS replies to SA2 |

WEEK 2:

| Time Zone UTC | Web Conference R2 - Main | Web Conference R2 - BO1 | Web Conference R2 - BO2 |
| --- | --- | --- | --- |
| Monday |  |  |  |
| 12:15-13:05 | NR17 Other (Johan) | NR17 up to 71 GHz (Tero) - [8.20.1] Work plan and LSs - [8.20.2] UE capability differentiation for FR2-1 and FR2-2 - [8.20.2] UP impacts (RACH, RLC) - [8.20.2] Other topics (if time allows) | NR16 SONMDT (HuNan) |
| 13:05-14:25 | NR17 IoT NTN (Johan) | CB Tero NR17 DCCA - [8.2.3.2] outcome of [Post114-e][233][R17 DCCA] Uu Message design for CPAC (CATT) (if not time earlier) - Offline discussion outcomes (if any) NR17 MUSIM - Offline discussion outcomes (if any) | CB Kyeongin 6.2.3 (if needed) [POST114-e][706] |
| 14:25-15:45 | NR15 NR16 Main session (Johan) | NR17 RACH indication / partitioning (Diana) | NR17 Pos (Nathan) [8.11.4] [8.11.5] [8.11.6] |
| Tuesday |  |  |  |
| 12:15-13:05 | CB eNPN, ePowsav, QoE if needed (Johan) | CB Sergio NTN CB session, including offline discussion outcomes | CB Nathan |
| 13:05-14:25 | CB eIAB, TEI17 (Johan) | CB Sergio Redcap CB session, including offline discussion outcomes CE offline discussion outcome (if any) CB Diana | CB Brian Emre |
| 14:25-15:45 | CB Multicast, IoT NTN (Johan) | CB Diana | CB Kyeongin Come-back issues from 6.2.2 and 6.2.3 [POST114-e][706] (if needed) Offline discussion outcomes (if time allows) |
| Wednesd |  |  |  |
| 04:00-05:00 | CB Multicast, feMIMO (Johan) | CB Tero NR17 RAN slicing - Offline discussion outcomes (if any) NR17 DCCA - Offline discussion outcomes (if any) NR17 Multi-SIM - Offline discussion outcomes (if any) | CB Nathan |
| Thursday |  |  |  |
| 04:00-05:00 | CB NR16 NR15 (Johan) | CB HuNan | CB Kyeongin Offline discussion outcomes |
| Friday |  |  |  |
| 04:00-05:00 | CB TBD (Johan) | CB Sergio Diana | CB Tero |

# 4	EUTRA corrections Rel-15 and earlier

See Appendix A for reference to Work items, work item codes and WIDs.

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.5	Other LTE corrections Rel-15 and earlier

Documents in this agenda item will be handled in a break out session.

Purely editorial corrections should be avoided, text enhancements may be deprioritized. Corrections should be taken up with the specification editor before submitting to avoid CR duplication. If this is not done, the contribution may not be treated.

By Email [201] (3)

R2-2108312	On T330 resetting	Ericsson, ZTE Corporation, Sanechips	CR	Rel-15	36.331	15.14.0	4712	-	F	LTE_5GCN_connect-Core

R2-2108634	Minor changes collected by Rapporteur for Rel-15	Samsung	CR	Rel-15	36.331	15.14.0	4718	-	F	LTE_eMTC4-Core, LTE_sTTIandPT, LTE-L23

R2-2108635	Minor changes collected by Rapporteur for Rel-16	Samsung	CR	Rel-16	36.331	16.5.0	4719	-	A	LTE_eMTC4-Core, LTE_sTTIandPT, LTE-L23

Email discussions ([201])

[AT115-e][201][LTE] Miscellaneous LTE CRs (Samsung)

Scope:

    - Discuss LTE CRs marked for this discussion (if needed)

Intended outcome:

    - Discussion report in R2-2108851

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Thu, UTC 1700

    - Deadline for CR finalization: 2nd week Wed, UTC 0900

# 7	Rel-16 EUTRA Work Items

Essential corrections

## 7.1    EUTRA Rel-16 General

No documents should be submitted to 7.1. Please submit to.7.1.x

Purely editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication. If this is not done, the contribution may not be treated.

### 7.1.1	Cross WI RRC corrections

Including RRC corrections that impact multiple WIs and require discussion in the common session.

By Email [201] (1)

R2-2107774	Correction on early security reactivation upon reception of RRCConnectionReject	NEC	CR	Rel-16	36.331	16.5.0	4696	-	F	TEI16, LTE_eMTC5-Core

### 7.1.2	Feature Lists and UE capabilities

Corrections to UE capabilities should be taken up with the 36.331 and 36.306 specification editors before submitting to avoid CR duplication. If this is not done, the contribution may not be treated.

## 7.4	LTE Other WIs

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921)

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning)

(Documents relating to Rel-16 LTE but for which there is no existing RAN WI/SI, e.g. LSs from CT/SA requesting RAN2 action)

Including TEI16 corrections and issues that do not fit under any other topic.

Purely editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication. If this is not done, the contribution may not be treated.

For LTE mobility enhancements, only corrections that are LTE-specific should be submitted to this AI. Corrections that impact or are common with NR mobility enhancements should be submitted to 6.1.X instead.

By Email [201] (1)

R2-2108701	36.331 Correction on ReportConfigEUTRA for CHO/CPAC	CATT	CR	Rel-16	36.331	16.5.0	4720	-	F	LTE_feMob-Core

By Web Conf (1st week Friday) or By Email (outcome of [201])

R2-2108851	Summary of [AT115-e][201][LTE] Miscellaneous LTE CRs (Samsung)	Samsung	discussion	Rel-16	LTE-L23, LTE_eMTC4-Core, LTE_sTTIandPT, LTE_5GCN_connect-Core, TEI16, LTE_eMTC5-Core, LTE_feMob-Core

# 8	Rel-17 NR Work Items

## 8.2	MR DC/CA further enhancements

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

Email max expectation: 4 threads

No documents should be submitted to 8.2. Please submit to.8.2.x

### 8.2.1	Organizational, Requirements and Scope

Including LSs and any rapporteur inputs (which do not count against Tdoc limits).

Web Conf (Tuesday 1st week) (1)

R2-2106962	Reply LS on temporary RS for efficient SCell activation in NR CA (R4-2108364; contact: Huawei)	RAN4	LS in	Rel-17	LTE_NR_DC_enh2	To:RAN1, RAN2

No RAN2 actions, will have post-meeting email discussion, see AI 8.2.4

Noted

Web Conf (Tuesday 1st week) (1)

R2-2108688	TS 37.340 CR for CPA and inter-SN CPC	CATT	draftCR	Rel-17	37.340	16.6.0	B	LTE_NR_DC_enh2-Core

?Endorsed?

Post-meeting email discussions (running CRs + UE capabilities)

[Post115-e][227][R17 DCCA] Running NR/LTE RRCs CR for SCG deactivation (Huawei)

Scope: Create running NR and LTE RRC CRs for SCG deactivation.

Intended outcome: Running CR

Deadline:  Long

[Post115-e][228][R17 DCCA] Running MAC CR for SCG deactivation (vivo)

Scope: Create running MAC CR for SCG deactivation.

Intended outcome: Running CR

Deadline:  Long

[Post115-e][229][R17 DCCA] UE capabilities (Intel)

Scope: Discuss which (RAN2-determined) UE capabilities (for all features in this WI) are needed

Intended outcome: Report

Deadline:  Long

[Post115-e][2xx][R17 DCCA] Running Stage-2 CRs for SCG deactivation (ZTE)

Scope: Create running 37.340 CRs for SCG deactivation.

Intended outcome: Running CR

Deadline:  Long

### 8.2.2	Efficient activation / deactivation mechanism for one SCG and SCells

No documents should be submitted to 8.2.2. Please submit to.8.2.2.x

#### 8.2.2.1	Deactivation of SCG

Including outcome of [Post114-e][231][R17 DCCA] SCG activation/deactivation options (Huawei)

Including UE assistance information for SCG deactivation

Web Conf (Tuesday 1st week) (1)

Outcome of [Post114-e][231][R17 DCCA] SCG activation/deactivation options (Huawei)

R2-2108444	[Post114-e][231][R17 DCCA] SCG activation/deactivation options (Huawei)	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core	Late

Proposal 1: Discuss which option(s) to support for RACH resources for network-initiated SCG activation:

1)	common RACH resources;

2)	dedicated RACH resources indicated before SCG activation indication (when going to the SCG deactivated state or while the SCG is deactivated);

3)	dedicated RACH resources indicated in the SCG activation indication.

-	Rapporteur proposal for resolving P1 using 1) and 3):

When the SCG is deactivated, if the network sends an RRC(Connection)Reconfiguration containing an SCG RRCReconfiguration with reconfigurationWithSync , the UE initiates random access towards the PSCell. Existing specification is the baseline (e.g. for parameters and UE behaviour).

-	Apple thinks all 3) could be allowed. Ericsson agrees but disagrees with the rapporteur proposal. Nokia agrees with Ericsson: this would mean having SCG in deactivated would not work and this might always require RACH for activation. Huawei clarifies this is effectively an activation indication and not precluding anything.

-	FW thinks we should only support 1) and 3). CATT agrees and wonders if 2) provides any benefits of 2) over 3).

Show of hands

1+2+3: LGE, IDT, ZTE, vivo, Lenovo, Ericsson, QC, Convida, NEC, Samsung, Nokia, Intel, Apple (13)

1+2: - (0)

1+3: Huawei, OPPO, Futurewei, MediaTek, Sharp,. CATT, DOCOMO, Spreadtrum, Google, Interdigital, Nokia, Samsung, Intel, KDDI (14)

-	Ericsson thinks this goes against allowing network to do reconfiguration at any time. Would like to configure RACH resources before activation. Huawei thinks the question is whether we do 2) or not. LGE thinks there is no difference from MAC viewpoint.

Agreements

Support all of the following for RACH resources used in network-initiated SCG activation (at least using RRC):

1)	common RACH resources;

3)	dedicated RACH resources indicated in the SCG activation indication.

FFS if we support also 2) (proponents are requested to provide CRs next time to illustrate how this can be done)

-	Apple thinks network triggers activation with/without RACH and that should be a network decision. UE just follows. Wonders if network could allow cases where UE can "try" RACH according to some condition.

-	Huawei clarifies option 4 is similar to SDT, which is not yet finished.

-	Chair wonders if we should talk about UE or network decision instead.

Show of hands:

UE determines whether to use RACH in SCG activation: Apple, LGE, ZTE, Samsung, Lenovo, IDT, Nokia, Sharp, Qualcomm (9)

Network indicates whether UE uses RACH in SCG activation: Futurewei, vivo, Convida, NEC, CATT, ZTE, KDDI, MediaTek, OPPO, Qualcomm, DCM, Intel, Ericsson (12)

Network configures UE with RACH, UE uses configuration to determine whether to UE use RACH in SCG activation: Qualcomm, Apple, Convida, LGE, ZTE, Futurewei, Google, IDT, Huawei, vivo, MediaTek, Ericsson (12)

Show of hands:

Support RACHless SCG activation: Futurewei, LGE, Nokia, IDT, Apple, Lenovo, vivo, Ericsson, CATT, ZTE, Convida, Spreadtrum, KDDI, Sharp, Intel, Qualcomm (16)

Do not support RACHless SCG activation: DOCOMO, Samsung, OPPO, MediaTek, NEC, Google, Huawei (8)

We will support RACHless SCG activation in Rel-17

-	DCM points out we should decide on one option or not do anything. Apple wonders if NW can predict that UE has valid TA? Or which TCI UE will use? Huawei thinks network just indicates the TCI in RRC.

-	Nokia thinks option 1 is a bit odd so it's not easy to understand.

Show of hands (exclusive):

1: 	Ericsson, Lenovo, LGE, Samsung, CATT, Huawei, QC, Convida, KDDI, Sharp (10)

2:	MediaTek, DOCOMO, Spreadtrum, Apple, Intel, vivo, NEC, Futurewei, Google (9)

3: 	OPPO, IDT (2)

4: 	ZTE (1)

1+2: Futurewei, Ericsson, Lenovo, Nokia, MediaTek (2)

Do not consider options 3) and 4)

Offline [223] (Huawei) to discuss if we can combine 1) and 2). Attempt to clarify how each option works and what are their commonalities and differences. Should clarify how network knows UE has valid TA and correct TCI state.

Proposal 2: Select one solution for network-triggered SCG activation *without* RACH among the 4 listed options:

1)	the UE performs BFD and RLM based on previously activated TCI states ("implicit configuration") while the SCG is deactivated. The UE can use these beams/TCI states at SCG activation if beam/radio link failure was not detected by the UE before SCG activation (similar to operation when resuming from DRX). FFS: UE reports in case of beam/radio link failure and UE/network behaviour after reporting beam/radio link failure.

2)	the network uses information from L3 measurement reports (similar to SCell activation)

3)	the network uses L1 measurement reports (similar to switching from dormant to non-dormant BWP). FFS reporting on PUCCH (periodic)/ via MCG before activation decision or at activation decision

4)	SDT-like method: the UE performs uplink transmission using a grant associated with a good DL RS (network provides UL grants (associated with beams) together with a RSRP threshold to UE, UE selects the UL grant when associated SSB RSRP is above threshold (same method used in SDT))

Proposal 3a: Discuss the feasibility and complexity of solution 3 (SCG activation upon RACH/SR towards the SCG) without contacting the MN) so that it can be made faster than solution 1 or solution 2 for SCG activation triggered by UL data transmission.

Proposal 3b: Discuss whether to support solution 3 for MCG link recovery without RRC re-establishment.

Proposal 4 : Discuss solutions 1), 2) and 3) for UE-requested SCG deactivation.

1)	Assistance information: the UE reports that it would like the SCG to be deactivated.

2)	Deactivation request / response: the UE reports that it would like the SCG to be deactivated and the network replies to the UE whether it accepts or rejects the request.

3)	Report preference between deactivation and release: the network can configure the UE to indicate its preference between SCG deactivation and SCG release.

4)	Inactivity timer: the UE can be configured with an inactivity timer and the SCG is deactivated if the timer expires, i.e. no traffic for a certain period (note: unlike 1, 2 and 3, there is no notification to the network).

|  | 1) UE assistance information | 2) Deactivation request / response | 3) Report preference between deactivation and release | 4) Inactivity timer |
| --- | --- | --- | --- | --- |
| Benefits | - Allows the UE to reduce resources, e.g. to save power | - Allows the UE to use SCG resources for another purpose (e.g. another subscription) | - Allows the UE to indicate the best method to save power according to UE implementation or current preference | - Saves DL signalling for SCG deactivation |
| Specification impact | - Existing framework could be reused - Needs to decide the details of the indication | - Need to specify configuration, indication and response | - Need to decide whether to use the UE assistance information or a new indication, and specify the details | - Existing sCellDeactivationTimer could be reused, with little modifications (FFS whether it covers PSCell only or all SCG serving cells) |
| Drawbacks | - More uplink signalling overhead than 2) - This method is unsuitable if the UE wishes to take some action depending whether the network accepts the request now or not (e.g. to determine whether it is possible to use for another subscription hardware resources now used for the SCG) | - This method is less appropriate than 1) if the UE can wait an undetermined time for the SCG deactivation |  | - Does not allow to modify the UE configuration at SCG deactivation, unless that configuration is signalled previously and stored |

Web Conf (Tuesday 1st week), Bearer handling (1)

UP details: Bearer handling for SCG deactivation

R2-2107669	Bearer handling for SCG deactivation	Samsung	discussion	Rel-17	LTE_NR_DC_enh2-Core

Proposal 1. SRB3 is suspended upon SCG deactivation, if configured.

Proposal 2. For SRB3, the old RRC message is discarded upon SCG deactivation, if any.

Proposal 3. The SCG RLC bearer of split DRB and duplication DRB is suspended upon SCG deactivation, if configured.

Proposal 4. The normal SCG DRB is suspended upon SCG deactivation, if configured.

Proposal 5. The security key update is up to network implementation upon SCG activation from deactivation.

Proposal 6. The normal SCG DRB is resumed after RLC/PDCP re-establishment upon SCG activation, if security key is updated.

Proposal 7. The normal SCG DRB is resumed without RLC/PDCP re-establishment upon SCG activation, if security key is not updated.

Proposal 8. The transmitting PDCP entity of the normal SCG DRB discards PDCP PDUs upon SCG deactivation.

Proposal 9. The receiving PDCP entity of the normal SCG DRB stops t-Reordering if running and deliver the stored PDCP SDUs to upper layer upon SCG deactivation.

Discuss bearer handling in deactivated SCG (e.g. proposals in R2-2107669) in offline [220] (Samsung)

R2-2108445	Remaining issues on UE-requested SCG deactivation	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107018	Discussion on SCG deactivation for RRC_INACTIVE UE	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107422	Deactivation of SCG	Qualcomm Incorporated	discussion	Rel-17

R2-2107663	DC power sharing for deactivated SCG	Samsung	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107983	Deactivation of SCG	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108091	Deactivation of SCG	LG Electronics	discussion	Rel-17

R2-2108165	Details of SCG deactivation	China Telecommunications	discussion	Rel-17

R2-2108330	Comparison of SCG deactivation solutions	Convida Wireless	other	Rel-17	LTE_NR_DC_enh2-Core	R2-2106039

R2-2108388	Efficient SCG (de)activation	Ericsson	discussion	LTE_NR_DC_enh2-Core

R2-2108488	Deactivation of SCG	InterDigital	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108530	Discussions on deactivation of SCG	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108678	UE Assistance Information for SCG deactivation	SHARP Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108691	Discussion on Deactivation of SCG	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108813	Discussion on deactivation of SCG	NTT DOCOMO INC.	discussion	Rel-17	LTE_NR_DC_enh2-Core	Late

Email discussions ([220]), TBD

[AT115-e][220][R17 DCCA] Bearer handling of SCG deactivation (NN)

Scope:

    - Discuss the Bearer handling of SCG (de)activation based on online discussion

Intended outcome:

    - Discussion summary in R2-2108862 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur summary):  2nd week Mon, UTC 1000

By Email (outcome of [220])

R2-2108862	Summary of [AT115-e][220][R17 DCCA] UP aspects of SCG deactivation (NN)	NN	discussion	Rel-17	LTE_NR_DC_enh2-Core

#### 8.2.2.2	UE measurements and reporting in deactivated SCG

Including discussion on how/whether RRM/RLM/BFD measurements are done for deactivated SCG

Including discussion on TAT timer handling for deactivated SCG

Including discussion on RRM/CSI/BM measurement reporting for deactivated SCG

Web Conf (Tuesday 1st week) (1)

R2-2108389	UE measurements and reporting in deactivated SCG	Ericsson	discussion	LTE_NR_DC_enh2-Core

Proposal 1	TA timer of PSCell is kept running after SCG deactivation, if TA timer is running.

Proposal 2	UE does not need to perform any procedure, e.g. RACH, to maintain UL timing alignment with SN if TAT timer expires.

Proposal 3	Upon SCG activation, if TA timer of the PSCell has already expired or stop, UE performs RACH on the PSCell to obtain UL timing information.

Proposal 4	RAN2 confirms that when deactivated SCG PSCell is changed UE does not initiate RACH until there is need to activate SCG.

Proposal 5	RLM is performed for the deactivated SCG and legacy SCGFailureInformation message and reporting procedure can be reused after RLF is detected.

Proposal 6	BFD is performed for the deactivated SCG and SCGFailureInformation message and reporting procedure can be used when beam failure is detected.

Proposal 7	If the SCG is to be activated but BFD has been declared, random access is needed when the UE activates the PSCell.

Proposal 8	TA timer is not stopped due to BFD/RLM detection.

Proposal 9	The network can configure separate measurement configurations for activated and deactivated SCG, respectively, where the measurements for deactivated SCG may be a subset of the measurements for activated SCG.

Proposal 10	There should be at least one RRM configuration which enables the UE to maintain DL fine sync on the PSCell while the SCG is deactivated (e.g. including SFN timing and SSB selection).

Proposal 11	Maintaining DL fine sync on the deactivated SCG means that the UE is ready to transmit in next PRACH or SR occasion having processed the SCG activation command. FFS possible dependency on the measurement configuration.

Proposal 12	Assuming the UE performs BFD while the SCG is deactivated, it does not perform CSI measurement on PSCell and CSI reporting.

Proposal 13	In the SN Addition procedure during PSCell addition/change, the target SN should be able to set the SCG activation state in the response message to the MN.

Proposal 14	At PSCell addition/change, if the SCG activation target state is SCG activated, the UE performs random access in target PSCell (as in legacy).

Proposal 15	At PSCell addition/change, if the SCG activation target state is SCG deactivated, the UE does not perform random access in target PSCell.

Proposal 16	During handover preparation, source MN sends the current SCG activation state to the target MN. Whether the current SCG activation state is part of the inter-node container or in the XnAP part of the message needs also be discussed in RAN3.

Proposal 17	In the SN Addition procedure during handover preparation, the (target) SN should be able to set the SCG activation state in the response message to the target MN.

Proposal 18	At handover when SCG target state is set to deactivated, the UE does not perform a random access in the target PSCell. This applies also for the handover cases where reconfigurationWithSync for the SCG is required per legacy (e.g. at AS security key change).

R2-2107423	UE measurements and reporting in deactivated SCG	Qualcomm Incorporated	discussion	Rel-17	R2-2103893

Observation 1. Performing RLM and BFD on PSCell while in SCG deactivated can enable UE to determine upon SCG activation if it has a usable beam for performing RACH or SR, which has the potential to reduce SCG activation delay.

Observation 2. In SCG deactivated, UE performing RLM and BFD on the PSCell does not result in significant additional power consumption if RRM measurements on the PSCell are already being performed.

Observation 3. Upon receiving SCGFailureInformation, in case MN decides to release the SN or change the PSCell, MN transmits RRC reconfiguration to the UE, and if PSCell is changed, the included SN RRC reconfiguration also indicates whether the activation state of new SCG is deactivated or activated.

Observation 4. In order to assist the SN to determine the configuration of updated beams and RSs as in Proposal 6, UE should report PSCell beam measurement results in SCGFailureInformation.

Observation 5. Upon UE detecting BFD or RLM, the option in which UE reports measurements via the MCG and waits for reconfiguration (Proposals 3-8 above) seems more preferable than the option where UE waits for SCG activation and performs RACH on activation, since it is possible and more likely in the former option for UE to have a usable beam upon SCG activation.

Observation 6. In general, TA is considered valid when the TA timer is running, and this holds for the TA timer of the PSCell while UE is in SCG deactivated.

Observation 7. Transmission of CSI reports on PSCell UL impacts power savings and if TA timer of the PSCell expires, requires UE to maintain UL timing with SN while in SCG deactivated.

Observation 8. CSI-RS measurements and reporting after SCG activation do not contribute significantly to the delay for SN to begin scheduling the UE on the DL.

Observation 9. DL and UL beam management procedures involve periodic beam (L1) measurement reports on the UL or SRS transmissions, which could result in increased UE power consumption in SCG deactivated.

RLM, BFD, and radio link failure recovery in SCG deactivated

Proposal 1. UE supports performing RLM on PSCell while in SCG deactivated.

Proposal 2. UE supports performing BFD on PSCell while in SCG deactivated.

Proposal 3. UE transmits RRC SCGFailureInformation message to MN upon detecting RLF on PSCell while in SCG deactivated.

Proposal 4. Upon detecting BFD on the PSCell while in SCG deactivated, UE reports the occurrence of BFD to the network via the MCG.

Proposal 5. Upon detecting BFD on the PSCell in SCG deactivated UE reports BFD by transmitting SCGFailureInformation to the MN (a new failure type can be introduced in the message for this purpose).

Proposal 6. Upon receiving SCGFailureInformation, in case MN decides not to change the PSCell, MN and SN may optionally provide a reconfiguration to the UE in response. In particular, SN may provide in an RRC reconfiguration an updated set of beams, RLM RSs, BFD RSs, additional RSs, and TCI states for UE to measure on the PSCell while in SCG deactivated.

Proposal 7. UE should be configured to report PSCell beam measurement results in SCGFailureInformation while in SCG deactivated.

Proposal 8. Upon receiving the RRC reconfiguration message as discussed in Proposal 6, UE resumes performing RLM and BFD on the PSCell as per the provided configuration, while in SCG deactivated.

Handling Time Alignment timer (TAT) of the PSCell in SCG deactivated

Proposal 9. Upon UE entering SCG deactivated, if the TA timer of the PSCell is running, UE should keep the timer running.

Proposal 10. While in SCG deactivated, UE should not stop the TA timer of the PSCell if it is running when BFD or RLM is detected.

Proposal 11. While in SCG deactivated, if TA timer of the PSCell expires, UE does not perform any procedure, e.g., RACH on PSCell, to regain or maintain UL timing alignment with the SN.

Proposal 12. Upon SCG activation, if TA timer of the PSCell has already expired, UE performs RACH on the PSCell to obtain UL timing information.

CSI-RS measurements and reporting in SCG deactivated

Proposal 13. In SCG deactivated, UE does not perform CSI-RS measurements on the PSCell and CSI reporting based on these measurements.

Beam management in SCG deactivated

Proposal 14. In SCG deactivated, UE does not support DL or UL beam management procedures.

R2-2107746	Discussion on UE behaivour when SCG is deactivated	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

Observation 1: To support RACH-less SCG activation, at least TA should be valid, and network knows the best DL beams so that UE can be successfully scheduled.

Observation 2: In general, mobility based measurements (for both PSCell and SCG SCell) need to be performed when SCG is deactivated. Other measurements can be stopped for power saving.

Proposal 1: UE keeps TA timer running when entering SCG deactivation.

Proposal 2: UE performs BFD and RLM when enters SCG deactivation. UE stops BFD and RLM when TA timer expires, and UE stops TA timer when BFD failure or RLF is declared.

Proposal 3: When RLF is declared while SCG is deactivated, UE transmits SCGFailureInformation to SN via MCG (follow legacy behaviour).

Proposal 4: When BFD failure is declared while SCG is deactivated, UE sends indication to SN via MCG (FFS on which message is used).

Proposal 5: For RACH-less SCG activation, network provides UL grants (associated with DL beams) and a RSRP threshold in SCG activation. If RSRP of a SSB is above the threshold, UE uses corresponding UL grant to transmit data; otherwise (no SSB fulfill the threshold), RACH is performed.

Proposal 6: Optimization of SN configured RRM measurement is needed if RAN2 agrees to use lower layer signaling for SCG (de)activation.

Proposal 7: RAN2 understand UE maintains DL sync based on the RRM measurements of PSCell. The measurement period of PSCell measurement during SCG deactivation state is up to RAN4.

R2-2107603	TA Maintenance and other RRM UE actions in SCG deactivated state	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core

Observation 1: As has been used for UE mobility, the RRM measurements (with proper configuration of time-domain averaging as well as triggering thresholds) can provide a fairly reasonable estimate of whether the UE have moved towards or away from the base-station enough that the stored TA might not be valid anymore.

Observation 2: In SDT work-item, UL TA is considered to be valid based on the measured RS quality (the RSRP change < threshold), along with the SDT TA-Timer, and same logic/scenario is applicable here.

Observation 3: In small cell deployments where the SCG is typically the booster small cell, the location of the UE with respect to the small cell does not impact the TA very much due to the small cell size. In such cases, there is no need to link RRM with TA, and the UE can be configured to just re-use the TA when it moves out of SCG deactivated state.

Observation 4: With no PDCCH monitoring, RLM on the PSCell is not critical in SCG deactivated state.

Observation 5: With the ability of RRM to configure the UE to report beam RSRP measurements, additional beam management/recovery is not critical for the UE in SCG deactivated state.

Observation 6:  The delay of SCG re-activation from the overall system perspective (if we consider the SN-MN interaction needed before the re-activation command is sent to the UE) is already pretty high, and the latency added from not performing RLM/BM/BFD is not the chief contributor to the SCG re-activation latency.

Observation 7: The UE is expected to perform RRM while in SCG deactivated state. We can use this aspect to help speed up re-activation.

Observation 8: The primary purpose of the RRM by the UE in deactivated SCG state is to let the NW know about the signal conditions of at least the PSCell (which can be used for mobility and other aspects by the NW). UE measurement of SCells in SCG is absolutely necessary and the UE can benefit in power savings if the SCell RRM is relaxed.

Observation 9: The UE’s SCG RRM measurements during SCG deactivated state should be designed to also make the operation power efficient compared to the RRM activities the UE perform during SCG active state.

Proposal 1: If the UE has a valid TA during SCG deactivation, in the SCG deactivated state, the RRM measurements of the PSCell are used to determine if the UE’s stored TA is valid or not. The NW can configure the UE with the averaging/filtering parameters as well as the thresholds/bias that are used by the UE to determine the TA validity.  FFS if the new parameters are needed or existing ones can be re-used.

Proposal 2: The NW has the option to configure the UE to consider that the TA is always valid during SCG deactivation.

Proposal 3:  RLM/BFD/BM is not needed in SCG deactivated state. Beam measurements and reporting using RRM is sufficient.

Proposal 4: NW can optionally provide a separate RRM configuration to the UE to be used in SCG deactivated state. FFS if a separate explicit configuration is needed or if the UE can assume this implicitly. Proposal : RAN2 to discuss the below options for RRM activities in SCG deactivated state:

-	Reuse of  SCG C-DRX while in SCG activated state with relaxed SCG measurements, which are defined by RAN4.

-	RRM measurements in SCG deactivated state based on a MeasCyclePSCell (similar to the SCell measurement).

-	Further relaxed SCell measurement (for eg., N x MeasCycleScell)

-	Normal NCell meas for MN, while skipping RRM on NCells configured by SCG.

??Continue discussion in offline [221]

R2-2107020	UE measurements and reporting in SCG deactivation	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107328	UE behavior in deactivated SCG	NTT DOCOMO INC.	discussion	Rel-17

R2-2107753	Mobility for deactivated SCG	NTT DOCOMO INC.	discussion	Rel-17	R2-2105064

R2-2107923	UE behavior when SCG is deactivated	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2108132	Further considerations on SCG deactivation	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2105791

R2-2108166	Discussion on UE behavior in deactivated SCG 	China Telecommunications	discussion	Rel-17

R2-2108446	UE behaviour while the SCG is deactivated	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108489	Measurements and maintenance of UL synch with a deactivated SCG	InterDigital	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108649	Discussion for UE behaviour in deactivated SCG	SHARP Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2106287

R2-2108669	UE behavior when SCG is deactivated	vivo	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108692	UE Behavior in Deactivated SCG	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2105059

R2-2108721	UE Measurements in SCG Deactivation	LG Electronics	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2106107

R2-2108733	UE behavior during SCG deactivation	MediaTek Inc.	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2106336

Email discussions ([221]), TBD

[AT115-e][221][R17 DCCA] UE measurements when SCG is deactivated (NN)

Scope:

    - Discuss further details on UE measurements when SCG is deactivated (based on online discussion)

Intended outcome:

    - Discussion summary in R2-2108863 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur summary):  2nd week Mon, UTC 1000

By Email (outcome of [221])

R2-2108863	Summary of [AT115-e][221][R17 DCCA] UE measurements when SCG is deactivated (NN)	NN	discussion	Rel-17	LTE_NR_DC_enh2-Core

#### 8.2.2.3	Activation of deactivated SCG

Including outcome of [Post114-e][231][R17 DCCA] SCG activation/deactivation options (Huawei)

Including discussion on SCG activation details: For network-initiated activation, when is random access used ? Is usage of random access UE or network decision?

How can UE request SCG activation?

Web Conf (Tuesday 1st week) (1)

UP details: PHR handling when SCG is deactivated

R2-2107668	PHR issues for SCG activation	Samsung	discussion	Rel-17	LTE_NR_DC_enh2-Core

Proposal 1. PSCell is in deactivated state for deactivated SCG.

Proposal 2. For deactivated PSCell, PHR is not reported.

Proposal 3. PHR is triggered upon activation of the PSCell.

Web Conf (Tuesday 1st week) (2)

How can UE request SCG activation?

R2-2108668	Discussion on activation of a deactivated SCG	vivo	discussion	Rel-17	LTE_NR_DC_enh2-Core

Proposal 1	MAC CE based SCG activation can be supported.

Proposal 2	For MN-triggered SCG activation, if SN accepts the SCG activation, SN can indicate to the MN whether the included SN RRC container (e.g., dedicated RACH resource) needs to be sent to the UE simultaneously when activating the SCG.

Proposal 3	SN can indicate to the MN whether the MN is allowed to directly activate the SCG without SN involvement for a period of time.

Proposal 4	For SN-triggered SCG activation, when SN requests SCG activation, SN can indicate to the MN whether the included SN RRC container (e.g., dedicated RACH resource) needs to be sent to the UE simultaneously when activating the SCG).

Proposal 5	When the SCG activation is indicated to the UE via the MCG, the UE is allowed to skip RACH if the TAT is still running and reconfigurationwithsync is not provided by the SN.

Proposal 6	asks RAN4 to define SCG/PSCell activation delay for deactivated SCG in case RACH is not performed upon SCG/PSCell activation.

Proposal 7	The first active BWP of PSCell is activated upon SCG activation.

Proposal 8	Upon SCG activation, the UE shall keep the SCG SCell in deactivated except for the SCell state is reconfigured by the network in the SCG activation command.

Proposal 9	Upon SCG activation, the PHR can be triggered.

Proposal 10	For at least SN terminated SCG bearer, SN can decide accept or reject the SCG activation request received via SCG without MN involvement.

Proposal 11	When SCG is deactivated, the below options should be supported for UE-triggered SCG activation:

-	Opt1: for split bearer, the primary path is set to MCG automatically, the UE sends BSR/UL data on the MCG leg and the network decides to trigger SCG activation if needed;

-	Opt2: if UL data arrives at SCG bearers, the UE can send SCG activation request to the MCG;

-	Opt3: if UL data arrives at SCG bearers, the UE can initiate RACH/SR towards the SCG.

Proposal 12	UE-trigger SCG activation for fast MCG recovery can be supported.

Proposal 13	Network should be allowed to accept or reject the SCG activation requested by the UE.

Proposal 14	If the UE sends SCG activation request to the MCG, the final decision is sent to the UE via MCG.

Proposal 15	If the UE triggers RACH or SR towards SCG for SCG activation, the final decision is sent to the UE via SCG.

Proposal 16	UE starts monitoring the PDCCH on the SCG upon initiation of RACH or SR for requesting SCG activation.

R2-2107420	Activation of deactivated SCG	Qualcomm Incorporated	discussion	Rel-17

Observation 1. In Proposal 3, UE may send an SR on the PSCell if the following conditions hold:

-	The TA timer associated with the PSCell has not expired.

-	UE has a usable beam for transmission of the SR.

Observation 2. Even if SN seeks MN confirmation for SCG activation before allocating grant to transmit MCGFailureInformation message, the resulting procedure would be faster than RRC re-establishment, which would have to be performed otherwise upon MCG RLF.

Observation 3. In the option where UE triggers SCG activation by initiating RACH or sending SR on PSCell:

-	As discussed above, more failure handling is required compared to the option where an RRC message is transmitted via MCG to the MN, i.e., failure handling procedures are required if RACH or SR procedure fails.

-	Network needs to configure UE as discussed in Proposal 7.

Observation 4. Since UE keeps track of whether the TA timer of PSCell has expired and whether it has a usable beam, it should be decided by the UE whether to perform RACH at SCG activation.

Proposal 1. UE may initiate a request for SCG activation while in SCG deactivated in the following cases:

1) If for a DRB that uses SCG resources only, i.e., an SCG DRB, there is UL data arrival.

2) UE detects MCG RLF.

Proposal 2. While in SCG deactivated, in case of a split bearer with UL data, MN may trigger SCG activation based on received BSR from the UE.

Proposal 3. If UE detects MCG RLF while in SCG deactivated, UE triggers SCG activation either by initiating RACH or by sending an SR on the PSCell.

Proposal 4. Upon receiving the RACH preamble or the SR from the UE, SN provides an UL grant to the UE.

Proposal 5. UE uses the provided UL grant to initiate the R16 MCG Failure Information procedure by transmitting the MCGFailureInformation message to the SN.

Proposal 6. If there is UL data arrival on an SCG DRB while UE is in deactivated, UE triggers SCG activation by one of the following options:

1)	Initiating RACH or by sending an SR on the PSCell.

2)	Transmitting an SCG activation request in an RRC message via MCG to the MN. FFS whether UE Assistance Information can be used for this purpose.

Proposal 7. In Proposal 6, for the option of using RACH or SR procedure on PSCell, network needs to configure UE to use this option. Network also needs to configure UE with PUCCH and/or CFRA resources, if available, for lower SCG activation delay.

Proposal 8. If there is UL data arrival on an SCG DRB while UE is in SCG deactivated, we prefer the option where UE triggers SCG activation by transmitting an SCG activation request in an RRC message via MCG to the MN. FFS whether UE Assistance Information can be used for this purpose.

Proposal 9. UE decides whether to perform RACH at SCG activation.

Proposal 10. Upon SCG activation, UE does not need to RACH on PSCell if all of the following conditions are satisfied:

-	TA timer of PSCell is running.

-	UE has a usable beam on PSCell.

-	SCG activation message does not include a reconfigurationWithSync.

Proposal 11. MAC CE based SCG activation by the network should be supported when no UE configuration changes need to be provided during activation.

CB 2nd week: Whether to discuss in offline how to handle SCG activation [222]

R2-2107019	Open issues for activation of deactivated SCG	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107353	Discussion on UE behaviour when SCG is deactivated	Spreadtrum Communications	discussion	Rel-17

R2-2107532	Discussion on random access and UE initiation for SCG fast activation	Futurewei	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2105010

R2-2107602	Remaining aspects related to RACH-less SCG activation	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107604	UE initiation of SCG (de)activation request	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2105140

R2-2107747	Consideration on UE triggered SCG activation	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107874	UL data handling in deactivated SCG	DENSO CORPORATION	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107924	Discussion on SCG activation	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2108133	Further discussions on SCG activation	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108134	UE request for SCG activation	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108447	Lower layer signalling for SCG (de)activation	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108490	Activation of SCG	InterDigital	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108531	Discussions on activation of deactivated SCG	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108693	Considerations on Activation of Deactivated SCG	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108722	Activation of SCG	LG Electronics	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2106108

R2-2108728	Discussion on SCG activation	SHARP Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2106312

Withdrawn:

R2-2107865	UL data handling in deactivated SCG	DENSO CORPORATION	discussion	Rel-17	LTE_NR_DC_enh2-Core	Withdrawn

Email discussions ([222]), TBD

[AT115-e][222][R17 DCCA] Activation of deactivated SCG (NN)

Scope:

    - Discuss further details on deactivated SCG activation (based on online discussion)

Intended outcome:

    - Discussion summary in R2-2108864 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur summary):  2nd week Mon, UTC 1000

By Email (outcome of [222])

R2-2108864	Summary of [AT115-e][222][R17 DCCA] Activation of deactivated SCG (NN)	NN	discussion	Rel-17	LTE_NR_DC_enh2-Core

#### 8.2.2.4	Other aspects of SCG activation/deactivation

This agenda item will not be treated in this meeting .

R2-2107605	SCG bearer handling for the SCG deactivation feature	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108532	Considerations for fast MCG link recovery with deactivated SCG	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

### 8.2.3	Conditional PSCell change / addition

No documents should be submitted to 8.2.3. Please submit to.8.2.3.x

#### 8.2.3.1	CPAC procedures from network perspective

Including discussion on CPAC configuration and execution details and Stage-2 signalling flows.

Including discussion on the design of inter-node messages (to answer RAN3 LS questions).

Including discussion on whether, after T-SN provided the conditional configurations to the MN, the SN measurement configuration can be updated *before* the MN provides theses conditional configurations to the UE.

Including discussion whether the execution conditions can be updated after T-SN provided the conditional configurations to the MN.

Web Conf (Tuesday 1st week) (1)

Discussion on the design of inter-node messages (to answer RAN3 LS questions).

Discussion on whether, after T-SN provided the conditional configurations to the MN, the SN measurement configuration can be updated *before* the MN provides theses conditional configurations to the UE.

R2-2108112	Network procedures and signalling for CPAC	Ericsson	discussion	Rel-17	LTE_NR_DC_enh2-Core

Proposal 1	If solution 1 is agreed, send an LS to RAN3 and ask them to discuss the scenarios where not all candidate target cells are accepted by T-SN in option 1.

Proposal 2	If solution 1 is agreed, the specification is updated so that the UE does not have to perform measurements for measId(s) in MeasConfig that are not indicated in the condExecutionCond associated to condReconfigId.

Proposal 3	The S-SN only sends the execution conditions to the MN after it has received information about which target cells that were accepted by T-SN.

Proposal 4	Multiple PSCell candidates (and thus multiple CG-Config) should be included in a single RRC container in the S-NODE ADDITION REQUEST ACKNOWLEDGE message from target SN to MN at CPAC procedures.

Proposal 5	The existing CG-Config message is extended to include a list of the additional CG-Config(s) in order for the T-SN to provide the list of CG-Config (one per candidate PSCell) to the MN at CPAC procedures.

Proposal 6	The inter-node signalling from source SN to MN at CPAC procedures only includes a single CG-Config, even if several PSCell candidates are provided.

Proposal 7	Discuss whether the execution conditions are included in a new list or in an extension of the candidateCellInfoListSN (within MeasResultNR).

Proposal 8	The inter-node signalling from MN to candidate target SN at CPAC procedures only includes a single CG-ConfigInfo, even if several PSCell candidates are provided.

Proposal 9	RAN2 to inform RAN3 that the MN is not required to forward the execution condition(s) to the target SN (also in the SN initiated inter-SN CPC procedure) and that the MN performs the association between the execution conditions (from the source SN) and the RRC configuration of the candidate PSCell(s).

Proposal 10	A response LS should be sent to RAN3 to inform about the RAN2 decisions on inter-node RRC container design for CPAC and handling of execution conditions at SN initiated inter-SN CPC procedure. A draft LS is provided in the Annex.

R2-2108448	Source SN configuration update during CPC configuration	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108449	Reply LS on inter-node message design	Huawei, HiSilicon	LS out	Rel-17	LTE_NR_DC_enh2-Core	To:RAN3

R2-2107226	Discussion on SN initiated conditional PSCell change	NTT DOCOMO INC.	discussion

R2-2107525	On SN-initiated CPC and the working assumptions	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107421	CPAC procedures from network perspective	Qualcomm Incorporated	discussion	Rel-17

??Reply LS to RAN3 handled in post-meeting email discussion

[Post115-e][226][R17 DCCA] Reply LS to RAN3 on inter-node message design (NN)

Scope: Send LS to RAN3 on RAN2 decisions on inter-node design.

Intended outcome: Approved LS

Deadline:  Short

R2-2108135	Signaling details of SN-initiated CPC	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108162	Discussion on SN initiated inter-SN CPC	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108163	Further consideration on CPAC procedure	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107111	Considerations on SN-initiated CPC procedure	KDDI Corporation	discussion

R2-2107460	Discussion on SN initiated inter-SN CPC	China Telecommunication	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107404	Discussion on CPAC procedures from NW perspective	vivo	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107533	Remaining issues with SN initiated CPC	Futurewei	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2105012

R2-2107925	Issues related to SN initiated inter-SN CPC	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2108694	Discussion on CPAC procedures from network perspective	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108775	Further consideration on CPAC stage 2 flow, and remaining issues	Samsung Electronics	discussion	LTE_NR_DC_enh2-Core

#### 8.2.3.2	CPAC procedures from UE perspective

Including discussion on UE measurements for CPAC purposes.

Including discussion on signalling towards UE.

Including outcome of [Post114-e][233][R17 DCCA] Uu Message design for CPAC (CATT)

Web Conf (Tuesday 2nd week) (1)

Including outcome of [Post114-e][233][R17 DCCA] Uu Message design for CPAC (CATT)

R2-2108695	Summary of [Post114-e][233][eDCCA] Uu Message design for CPAC(CATT)	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core	Late

Proposal 1: Reuse the conditionalReconfiguration filed to configure CPAC (all scenarios) in Rel-17.

Proposal 2a: For NR-DC, reuse the condRRCReconfig field to contain both MCG and SCG re-configurations for each candidate PSCell configuration. I.e. the RRC message contained in the condRRCReconfig is in MN format, in which the RRC message generated by the candidate SN is encapsulated in a RRC container (e.g. mrdc-SecondaryCellGroup).

Proposal 2b: For (NG)EN-DC, reuse the condReconfigurationToApply field for (NG)EN-DC to contain both MCG and SCG re-configurations for each candidate PSCell configuration.  I.e. the RRC message contained in the condReconfigurationToApply is in MN format, in which the RRC message generated by the candidate SN is encapsulated in a RRC container (e.g. nr-SecondaryCellGroupConfig).

Proposal 3: For CPA and MN-initiated CPC, the execution conditions are configured in condExecutionCond for NR-DC, or triggerCondition for (NG)EN-DC and refer to an MCG MeasConfig.

Proposal 4: For CAP and inter-SN CPC, upon execution of CPAC, the UE includes the selected target PSCell information in the RRC Reconfiguration Complete message to the MN.

Proposal 5: For CAP and inter-SN CPC, condReconfigId/CondReconfigurationId of the selected target PSCell is included in the RRC Reconfigutation Complete message to the MN.

Proposal 6: The existing EUTRA signalling in ReportConfigInterRAT is to be modified to support B1 events for CPA and MN initiated CPC in (NG)EN-DC .

Proposal 7: The existing NR signalling in ReportConfigNR is to be modified to support A4 events for CPA and MN initiated CPC in NR-DC.

Proposal 8: RAN2 to discuss whether A3/A5 like events are applicable for MN initiated inter-SN CPC.

Proposal 9: RAN2 is requested to specify that the UE ignores measId(s) that were not indicated in the condExecutionCond/triggerCondition.

Proposal 10: The UE shall delete CPC related measConfig upon successful CPC execution.

Proposal 11: The MN does not need to comprehend the execution condition set by the source SN. The MN can associate the execution condition configuration to an RRCReconfiguration message provided by the target –SN without comprehending the execution condition set by the source SN.

Proposal 12a: A new field (e.g. condExecutionCondSN) in CondReconfigToAddMod is introduced for NR-DC to indicate that the execution condition refers to the SCG MeasConfig .

Proposal 12b: A new field (e.g. triggerConditionSN) in CondReconfigurationAddMod for (NG)EN-DC is introduced to indicate that the execution condition refers to the SCG MeasConfig .

R2-2108689	TS 38.331 CR for CPA and inter-SN CPC	CATT	draftCR	Rel-17	38.331	16.5.0	B	LTE_NR_DC_enh2-Core	Late

R2-2108690	TS 36.331 CR for CPA and inter-SN CPC	CATT	draftCR	Rel-17	36.331	16.5.0	B	LTE_NR_DC_enh2-Core	Late

R2-2108113	UE procedures and signalling for CPAC	Ericsson	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107405	Discussion on CPAC procedures from UE perspective	vivo	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107594	Discussion on CPAC open issues	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108723	Enhancements for CPAC	LG Electronics	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2103571

#### 8.2.3.3	Other CPAC aspects

This agenda item will be deprioritized in this meeting.

Including discussion on CPAC failure handling.

Including discussion on CPAC co-existence with CHO.

R2-2107524	On CPAC Procedures and Further Functionalities	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107871	Failure handling of Conditional PSCell Addition	DENSO CORPORATION	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2105444

R2-2107926	Miscellaneous issues on CPAC	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2108491	Coexistence of CHO and CPC	InterDigital, Nokia, Nokia Shanghai Bell,ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108533	Combination of CPAC and CHO	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

### 8.2.4	Temporary RS for SCell activation

This agenda item will be deprioritized in this meeting unless urgent LS from RAN1 or RAN4 is received.

R2-2107984	MAC CE for scell activation and temporary RS	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2108450	On RAN4 LS on Temporary RS for SCell activation	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2107021	Discussion on TRS activation for fast SCell activation	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

(moved from 8.2.2.2)

Topic handled in post-meeting email discussion (unless urgent LS from RAN1/4 is received)

[Post115-e][225][R17 DCCA]  (OPPO)

Scope: Discuss RAN2 impacts of TRS-based SCell activation and attempt to draft initial CRs to RRC/MAC to understand the scope.

Intended outcome: Report + draft CR to MAC/RRC

Deadline:  Long

## 8.3	Multi SIM

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-210316)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

Email max expectation: 3-4 threads

### 8.3.1	Organizational, Requirements and Scope

Including LSs and any rapporteur input.

Web Conf (Monday 1st week) (2)

R2-2106935	Reply LS on NAS-based busy indication (R3-212877; contact: ZTE)	RAN3	LS in	Rel-17	LTE_NR_MUSIM-Core	To:RAN2, SA2, CT1	Cc:SA3

Noted (discussed jointly with the SA2 reply)

R2-2106970	Reply LS on NAS-based busy indication (S2-2105150; contact: Intel)	SA2	LS in	Rel-17	LTE_NR_MUSIM-Core	To:RAN2, CT1, RAN3	Cc:SA3

-	LGE wonders how the NAS paging rejection is sent in INACTIVE? Is this sufficient? Intel thinks SA2 agreed to that.

-	QC thinks the last part is independent of busy indication. Intel agrees.

-	Nokia wonders if NAS-level busy indication means UE has to be CONNECTED?

Noted (discussed jointly with the RAN3 reply)

Will attempt to reply from this meeting, reply discussed together with contributions in 8.3.3

R2-2107300	[Draft] Reply LS on NAS-based busy indication	Intel Corporation	LS out	Rel-17	LTE_NR_MUSIM-Core	To:SA2	Cc:SA3, RAN2, CT1, RAN3

Discussed together with contributions in 8.3.3

Post-meeting email discussions (running CRs, TBD which are needed)

[Post115-e][235][MUSIM] Running NR RRC CR for MUSIM (NN)

Scope: Create running NR RRC CR for MUSIM

Intended outcome: Running CR

Deadline:  Long

[Post115-e][236][MUSIM] Running 38.304 CR for MUSIM (NN)

Scope: Create running 38.304 CR for MUSIM

Intended outcome: Running CR

Deadline:  Long

[Post115-e][237][MUSIM] Running LTE RRC CR for MUSIM (NN)

Scope: Create running LTE RRC CR for MUSIM

Intended outcome: Running CR

Deadline:  Long

[Post115-e][238][MUSIM] Running 36.304 CR for MUSIM (NN)

Scope: Create running 36.304 CR for MUSIM

Intended outcome: Running CR

Deadline:  Long

[Post115-e][239][MUSIM] Running Stage-2 CRs for MUSIM (NN)

Scope: Create running Stage-2 CRs (36.300, 38.300 and/or 37.340) for MUSIM

Intended outcome: Running CR

Deadline:  Long

### 8.3.2	Paging collision avoidance

This agenda item may be deprioritized in this meeting.

Including discussion on RAN2 aspects of paging collision avoidance

R2-2107326	Open Issues on Paging Collision Avoidance	CATT	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107388	Solutions for paging collision 	Qualcomm Incorporated	discussion

R2-2107855	Paging Collision avoidance	vivo	discussion

R2-2107974	Paging collision avoidance	Ericsson	discussion

R2-2108015	Definition and solution for paging collision, RRC Inactive, SI change	Lenovo Mobile Com. Technology	discussion	LTE_NR_MUSIM-Core

R2-2108119	Paging Collision Avoidance Open Issues	Huawei, HiSilicon	discussion	Rel-17	R2-2105917

R2-2108724	Considerations on Paging Collision	LG Electronics	discussion	Rel-17	LTE_NR_MUSIM-Core	R2-2106109

Web Conf (Monday 2nd week) (0)

CB (2nd week Monday): This AI will not be discussed in this meeting unless SA2 LS is received (RAN2 sent LS to SA2 from last meeting)

### 8.3.3	UE notification on network switching for multi-SIM

Including discussion on whether RAN2 decision on NAS-based busy indication can be retained (cv. SA2 LS S2-2105150)

Including discussion on "configured time" for AS-based solution.

Including interaction between AS-based solution and NAS-based solution for network switching

Including outcome of [Post114-e][242][MUSIM] Switching message details (vivo)

Including outcome of [Post114-e][243][MUSIM] Gap handling (ZTE)

Web Conf (Monday 1st week), SA2 LS (1)

Busy indication: AS vs. NAS, including AS/NAS interactions

R2-2107856	Open Issues on Switching Notification	vivo	discussion

Observation 1:	UE’s ongoing procedures may be impacted by switching to network B without leaving connected state in network A, such as RLM, handover, etc.

Proposal 1:	RAN2 retains the agreement on NAS-based busy indication for RRC_INACTIVE, and Reply SA2.

Proposal 2:	To minimize the interruption of ongoing service in network A, UE is allowed to stay in RRC_ CONNECTED in network A while entering the RRC_ CONNECTED state in network B for short time activities, e.g. SMS, RAU, TAU, busy indication, etc.

Proposal 3:	The range of absence time to use in the procedure for “switching without leaving RRC Connected state” is no more than 200ms.

Proposal 4:	Regarding switching with leaving RRC_CONNECTED, NAS-based solution is initiated by UE only when AS-based solution is not supported by either UE or network.

Proposal 5:	RAN2 to consider allowing UE to ignore multi-SIM gap for some critical scenarios, e.g. T310 or T312 running and suspend some timers of Scell and BWP to avoid unnecessary deactivation of Scell or BWP switching.

-	OPPO thinks NAS-based solution is aligned with previous RAN2 agreement. but SA2 didn't harmonize IDLE and INACTIVE, which is not aligned.

-	MTK thinks NAS-based busy indication in INACTIVE could be just dropped from Rel-17.

-	QC thinks SA2 agreed to CR so it's possible. Hence we should keep the previous agreement.

Show of hands

- Do not support busy indication for INACTIVE in Rel-17: 4 (MTK, Nokia, QC, Huawei)

- Support NAS-based busy indication in Rel-17 for IDLE and INACTIVE (previous decision): 15 (QC, Lenovo, Intel, huawei, DENSO, Charter, vivo, LGE, ZTE, Xiaomi, OPPO, Apple, ASUSTek, NEC)

- Agreement

- 1	RAN2 retains the agreement on NAS-based busy indication for RRC_INACTIVE, and Reply SA2.

Draft LS reply to SA2 in email discussion [230] (Intel)

R2-2107265	Analysis on AS-based solution and NAS-based solution	China Telecommunications	discussion

R2-2108076	Interaction between AS-based solution and NAS-based solution for network switching	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107301	NAS and AS procedures and their interaction for aperiodic gap request 	Intel Corporation	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107027	Interaction between AS-based and NAS-based Solution for Network Switching	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2108804	Signalling design on busy indication procedure	DENSO CORPORATION	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2108052	Discussion on AS based Leaving in MultiSIM	Sony	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2108709	Interaction between NAS and AS for network switching	ASUSTeK	discussion	Rel-17	LTE_NR_MUSIM-Core

Web Conf (Monday 1st week), gap handling email disc (1)

Outcome of [Post114-e][243][MUSIM] Gap handling (ZTE)

R2-2108077	Summary of [Post114-e][243][MUSIM] Gap handling	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core	Late

Discussion

-	OPPO has concerns on S2 and S3 due to long window. Might require very long gap length and this will impact RAN4. Could be fine with working assumption. Ericsson agrees but thinks we will see the impacts when we progress the work.

-	Huawei wonders why S4 was removed? Chair explains it had less support than others. Could be left as FFS if needed. Ericsson think it may not be in the scope.

-	Ericsson thinks we could do aperiodic gaps with periodic gaps.

-	NEC thinks RNAU is S3, not S4 since UE doesn't enter RRC_CONNECTED. Wonders if per-UE-level scheduling gap applies to both MCG and SCG? ZTE confirms this was the intent. This might require some coordination between MCG and SCG but this is out of WI scope. So we might not allow MR-DC with MUSIM.

-	Xiaomi thinks we should remove S4 since it's out of WI scope.

-	QC asks if the scenarios are going to be captured? Apple thinks we shouldn't do that.

- Agreements

- Scenarios and supported gap types

- 1	RAN2 aims to support at least the below scenarios 1/2/3 in Rel-17 for cases when the UE is allowed to switch to network B without leaving connected state at network A.

- -	Scenarios 1: Periodic switching, including SSB detection/paging reception, serving cell measurement, neighboring cell measurement including intra-frequency,inter-frequency and inter-RAT measurement;

- -	Scenarios 2:  SI receiving at network B;

- -	Scenarios 3: Aperiodic (one-shot) switching with both transmission and reception at network B but will not enter RRC-connected state in NW B (e.g. no RRC connection Resume/Setup) at network B, including On-demand SI request;

- 2	For switching without leaving connected state at network A, support gap types 2a (Normal periodic gap) and 2b (Normal aperiodic gap) in Rel-17.

- 3	Only per UE level scheduling gap is supported in Rel-17 for non-DC. FFS if we support MR-DC.

- The scenarios will only be used for deriving RRC parameters. No need to capture them in e.g. Stage-2.

-	Nokia wonders why we restrict to two periodic gaps? ZTE clarifies that too many gaps would make it difficult for implementations and RAN4.

-	OPPO wonders why P4 can be captured in specification. Fine with P5-7 but thinks only two periodical gaps are enough.

-	Huawei thinks periodical gap is better for SI reception as UE receives the SI according to SearchSpace in scheduled slot. UE may not acquire it in the first try.

-	QC thinks 2 periodic + 1 aperiodic is too restrictive. But we have other use cases like inter-frequency measurements. It's not clear if these are sufficient in practice and UE would ask for longer gap. vivo thinks these are not for RRM measurements.

-	vivo agrees with P5-7 and thinks RAN4 should just confirm the numbers. Could have working assumption for this and discuss if there are issues.

-	Ericsson thinks we could just state "at most 3 gaps can be configured".

-	Nokia thinks at least 3 periodic gaps are needed.

-	MTK thinks the absolute upper limit is important.

- Agreements

- Gap configuration and activation

- 5	The network is allowed to configure at most 3 gap patterns (for any MUSIM purpose).

- 6	Only a single aperiodic gap (for MUSIM) is supported in Rel-17. At most two periodic “gaps” (for MUSIM) and a single aperiodic gap (for MUSIM) is supported in Rel-17. FFS if signalling supports more.

- 7  	The SFN and subframe of the PCell of the network A is used in the gap configuration to calculate the gap.

-	LGE supports 8-12 but wonders what "can" means in P9? Does it mean network doesn't do it always?

-	OPPO wonders if P8 only applies to periodic or all types of gaps? Chair thinks it does.

-	OPPO thinks that P12 can be removed based on previous agreements.

-	Ericsson thinks that P16 means just UE input and does not mandate network.

-	Samsung thinks we should use "configure" in P10.

-	Xiaomi wonders if P8 applies to autonomous gaps?

-	Apple wonders if UE can request gap release in P16?

- Agreements

- Periodic/Aperiodic/autonomous Gap configuration and activation

- 8:  The switching gap configuration will explicitly provide the gap starting position (e.g. offset value or start SFN and subframe explicitly), gap length and gap repetition period.

- 10:  Switching Gaps (of any type) are configured or released by RRC signalling (e.g. RRCReconfiguration message) in Rel-17. FFS if gap can be released autonomously by UE after N repetitions.

- Gap configuration assistance information

- 16 	UE is allowed to include assistance information for setup or release of gaps for both 1) periodic gaps and 2) aperiodic gap in one UEAssistanceInformation Msg.

- 18  To report the assistance information, the UE maps the timing info of the Gap on the network B  to the network A and reports the mapped timing info to the network A.

- 20  For the gap assistance information, the Gap start time, Duration of the gap and gap repetition period (for periodic) may be included. FFS is other information is included (e.g. gap purpose).

- Do not support autonomous gaps for MUSIM in Rel-17.

Chairman's proposal for online discussion (based on the email discussion proposals)

Scenarios and supported gap types

Proposal 1: RAN2 aims to support at least the below scenarios 1/2/3 in Rel-17 for cases when the UE is allowed to switch to network B without leaving connected state at network A.

-	Scenarios 1: Periodic switching, including SSB detection/paging reception, serving cell measurement, neighboring cell measurement including intra-frequency,inter-frequency and inter-RAT measurement;

-	Scenarios 2:  SI receiving at network B;

-	Scenarios 3: Aperiodic (one-shot) switching with both transmission and reception at network B but will not enter RRC-connected state in NW B (e.g. no RRC connection Resume/Setup) at network B, including On-demand SI request;

FFS whether scenarios 4 is supported (Scenario 4: Aperiodic (one-shot) switching and enter into connected state (e.g. with RRC connection Resume/Setup) at network B, including Registration, SMS, RAU, busy Indication, etc.)

Proposal 2: For switching without leaving connected state at network A, support gap types 2a (Normal periodic gap) and 2b (Normal aperiodic gap) in Rel-17. UE does not transmit or receive during the gap duration.

Proposal 3: Only per UE level scheduling gap is supported in Rel-17.

Gap configuration and activation

Proposal 4: At least Gap Type 2b (normal aperiodical gap) will be supported for SI reception fr network B. FFS on the exact details (e.g. length, gap pattern interworking, etc.)

Proposal 5: For scenario 1, the network is allowed to configure at most 2 periodic Gap patterns (for any purpose).

Proposal 6: Only a single aperiodic gap is supported in Rel-17. At most two periodic “gaps” and a single aperiodic gap is supported in Rel-17.

Proposal 7:  The SFN and subframe of the PCell of the network A is used in the gap configuration to calculate the gap.

Periodic/Aperiodic/autonomous Gap configuration and activation

Proposal 8:  The switching gap configuration will explicitly provide the gap starting position (e.g. offset value or start SFN and subframe explicitly), gap length and gap repetition period.

Proposal 9:   The network can activate multiple periodic switching gaps at the same time.

Proposal 10:  Switching Gaps (of any type) are activated by RRC signalling (e.g. RRCReconfiguration message) in Rel-17.

Proposal 12: The network can activate multiple aperiodic Gaps at the same time only when the network can configure multiple aperiodic Gaps at the same time.

Gap configuration assistance information

Proposal 16: UE is allowed to include assistance information for both 1) multiple periodic gaps and 2) single aperiodic gap in one UEAssistanceInformation Msg.

Proposal 18:   To report the assistance information, the UE maps the timing info of the Gap on the network B  to the network A and reports the mapped timing info to the network A.

Proposal 20:  For the gap assistance information, the Gap start time, Duration of the gap and gap repetition period (for periodic) shall be included. FFS is other information is included (e.g. gap purpose).

Scenarios and supported gap types

Proposal 1: Ran 2 confirm that for the below scenario 1/2/3, the UE is allowed to switch to network B without leaving connected state at network A. For the scenario 4, it’s FFS.

-	Scenarios 1: Periodic switching, including SSB detection/paging reception, serving cell measurement, neighboring cell measurement including intra-frequency,inter-frequency and inter-RAT measurement;

-	Scenarios 2:  SI receiving at network B;

-	Scenarios 3: Aperiodic (one-shot) switching with both transmission and reception at network B but will not enter RRC-connected state in NW B (e.g. no RRC connection Resume/Setup) at network B, including On-demand SI request;

-	Scenarios 4: Aperiodic (one-shot) switching and enter into connected state (e.g. with RRC connection Resume/Setup) at network B, including Registration, SMS, RAU, busy Indication, etc.

Proposal 2: For switching without leaving connected state at network A, both Gap type 2a/2b would be considered. Gap type 3a/3b would not be considered. FFS on gap type 1a.

•	Gap Type 1a: Autonomous gap

o	Similar to the autonomous gap defined for CGI reporting; network does not know the exact time occasions (within gap duration) that UE switches to network B, as long as UE fulfills the minimum transmission requirement.

•	Gap Type 2a: Normal periodical gap

o	UE does not transmit or receive during the periodical gap duration;

•	Gap Type 2b: Normal aperiodical gap

o	UE does not transmit or receive during the aperiodical gap duration;

•	Gap Type 3a: Periodical gap with reduced capability:

o	UE can be scheduled by network A during the periodical gap duration, but with reduced capability (e.g. reduced MIMO layers, details are FFS).

•	Gap Type 3b: Aperiodical gap with reduced capability:

o	UE can be scheduled by network A during the aperiodical gap duration, but with reduced capability (e.g. reduced MIMO layers, details are FFS).

Proposal 2.1: For the periodic switching in the scenario 1, gap type 2a would be adopted; For the aperiodic switching in the scenario 3, gap type 2b would be adopted, FFS on gap type 1a;

Proposal 2.2: Which gap types shall be adopted for the scenario 2 can be further discussed in the phase 2.

Proposal 2.3: For the aperiodic switching in the scenario 4, if supported, gap type 2b would be adopted

Proposal 3: Only per UE level scheduling gap would be considered. (19/21)

Gap configuration and activation

Proposal 4: RAN2 to further confirm which Gap types shall be supported for the SI receiving:

•	Gap Type 1a: Autonomous gap (8/21)

•	Gap Type 2a: Normal periodical gap(8/21)

•	Gap Type 2b: Normal aperiodical gap (14/21)

Proposal 4a: If the aperiodical gap was supported, RAN2 further confirm whether an aperiodic gap can cover multiple SI periods and whether it supports to assign an additional periodic gap pattern in the aperiodic gap.

Proposal 5: For cases/events included the scenario 1, the network is allowed to configure at most 2 periodic Gap patterns (20/21). No need to specify or associate the gap pattern to the gap purpose. (10/21)

Proposal 5a: Even the periodic Gap pattern was adopted for the SI receiving, at most 2 periodic Gap patterns are allowed to be configured for the MUSIM. (18/21)

Proposal 6: For the RRC signaling based activation scheme, the “RRC signaling for network switching without leaving RRC_Connected state” is not allowed to configure multiple aperiodic gaps with different parameters (e.g. Durations)(13/21). FFS for the case with the L1/L2 activation mechanism. (2/21)

Proposal 6a:  “RRC signaling for network switching without leaving RRC_Connected state” is allowed to configure multiple periodic “gaps” and an aperiodic Gap (or multiple aperiodic Gaps if it was supported in the proposal 6) simultaneously.(19/21)

Proposal 7:  The SFN and subframe of the PCell of the network A is used in the gap configuration to calculate the gap. (19/21)

Periodic Gap configuration and activation

Proposal 8:  For the periodic Gap configuration, the “starting timing info (e.g. offset value or start SFN and subframe explicitly), gap length and gap repetition period shall be included. (21/21).

Proposal 9:   The network can active multiple periodic Gaps at the same time. (21/21).

Proposal 10:   Active the periodic Gaps by RRC signalling, e.g. upon receiving the RRC Reconfiguration message. (21/21).

Aperiodic Gap configuration and activation

Proposal 11:  For the aperiodic Gap configuration, the “starting timing info (e.g. offset value or start SFN and subframe explicitly) and gap length shall be included. (20/21).

Proposal 12: The network can active multiple aperiodic Gaps at the same time only when the network can configure multiple aperiodic Gaps at the same time.

Proposal 13: For the aperiodic Gap activation, take the “RRC signalling, e.g. upon receiving the RRC Reconfiguration message to active the aperiodic gap” as baseline. (21/21) FFS on the MAC CE scheme.(5/21).

Autonomous Gap configuration and activation (if supported)

Proposal 14: If autonomous Gap was supported, RAN2 to discuss which element shall be included for the autonomous gap configuration, an indication that similar to “useAutonomousGaps” for CGI reading or the autonomous gap length or both.

Proposal 15: If autonomous Gap was supported, it shall be activated by the RRC signalling, e.g. upon receiving the RRC Reconfiguration message.  (11/11)

Gap configuration assistance information

Proposal 16: UE is allowed to include multiple periodic Gaps assistance information (e.g. periodicities and durations) simultaneously e.g. in one UEAssistanceInformation Msg.

Proposal 17: If only the RRC signaling based activation scheme was supported,  UE is not allowed to include multiple aperiodic Gaps assistance information (e.g. periodicities and durations) simultaneously e.g. in one UEAssistanceInformation Msg (16/21). If MAC CE based activation scheme was supported, RAN2 to discuss whether the UE is not allowed to include multiple aperiodic Gaps assistance information.

Proposal 17a:   UE is allowed to include multiple periodic gaps and an aperiodic Gap (or multiple aperiodic gaps if allowed in the proposal 17) assistance information simultaneously, e.g. in one UEAssistanceInformation Msg.

Proposal 18:   To report the assistance information, the UE maps the timing info of the Gap on the network B  to the network A and reports the mapped timing info to the network A. (21/21)

Proposal 19:   If autonomous Gap was supported, the UE shall include the duration of the gap for the autonomous gap request in the assistance information (8/11). FFS on the autonomous gap needed or not indication (3/11).

Proposal 20:  For the periodic Gap assistance information, the Gap start time, Duration of the gap and gap repetition period shall be included (21/21). FFS on the indication of need for Gap (3/21) and the gap purpose (2/21).

Proposal 21:  For the aperiodic Gap assistance information, the Gap start time, Duration of the gap shall be included. (20/21).

Email discussions ([230])

[AT115-e][230][MUSIM] Discussion on AS vs. NAS-based busy indication (Intel)

Scope:

    - Discuss details required to reply to SA2/CT1 and draft the reply LS

Intended outcome:

    - Draft LS to SA2/CT1 in R2-2108856 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Wed, UTC 0900

    - Initial deadline (for draft LS):  1st week Wed, UTC 1700

Email discussions ([232]), TBD

[AT115-e][232][MUSIM] LS to RAN4 on gap handling (NN)

Scope:

    - Draft LS to RAN4 on RAN2 decisions for gap handling

Intended outcome:

    - LS to RAN4 in R2-2108861 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  2nd week Tue, UTC 0900

    - Initial deadline (for draft LS):  2nd week Tue, UTC 1700

By Web Conf (1st Week Friday) or By Email (outcome of [230])

R2-2108856	[Draft] Reply LS on NAS-based busy indication	Intel	LS out	Rel-17	LTE_NR_MUSIM-Core	To:SA2	Cc:SA3, CT1, RAN3

By Web Conf (2nd Week) or By Email (outcome of [232])

R2-2108861	[Draft] LS on gap handling for MUSIM	NN	LS out	Rel-17	LTE_NR_MUSIM-Core	To:RAN4	Cc: -

Web Conf (Monday 1st week), switching details email disc (1)

Outcome of [Post114-e][242][MUSIM] Switching message details (vivo)

R2-2107857	Summary of Switching message details	vivo	discussion

Discussion

P1

-	Lenovo thinks we agreed to this already. vivo clarifies this is more than gap configuration and UE can also indicate preferred RRC state. Lenovo wonders if UE wouldn't stay in CONNECTED if it reports gap configuration? vivo thinks both could be indicated.

-	Huawei wonders why UE would indicate RRC state if it indicates gap pattern? vivo clarifies that UE would just tell it wants to leave. Ericsson agrees with Huawei but thinks we don't need "after switching". Samsung agrees that UE shouldn't provide both gap configuration and RRC state-

- 	Nokia thinks these assitstance informations could be different. QC thinks this is just about leaving RRC_CONNECTED.

Signalling details

Online proposals (not agreed yet)

1	UE can indicate its preferred RRC State in assistance information for MUSIM (FFS for signalling details).

1	UE can indicate it wants to leave RRC_CONNECTED in assistance information for MUSIM (FFS for signalling details).

Proposal 1: The switching notification message carries assistance information, including Assistant information for gap configuration and the preferred RRC State after switching (FFS for the detailed state).

Proposal 3: UEAssistanceInformation message is extended for switching notification in both network switching procedures for leaving RRC_CONNECTED state and without leaving RRC_CONNECTED state.

Proposal 6: UE is configured to provide assistance info for switching notification via otherConfig of RRCReconfiguration message

Proposal 8: Introduce a new timer for the “configured time”, used for the UE to leave RRC_CONNECTED without a response.

Proposal 7: UE is not allowed to enter RRC_INACTIVE state if no NW response message is received within a certain configured time period after the network switching notification message is sent.

Proposal 9: How to handle the case, that UE performs switching without the response from network for a configured time during switching procedure without leaving RRC_CONNECTED state, is not specified.

Needs discussion or left FFS

Proposal 2: The need of NAS information in the switching notification message can be discussed based on SA2 LS, if any.

Proposal 4: FFS reuse preferredRRC-State or some changes on preferredRRC-State for Multi-SIM purpose.

Proposal 5: RAN2 to discuss whether switching notification for leaving RRC_CONNECTED state and without leaving RRC_CONNECTED state can be enabled separately.

Proposal 10: RAN2 to discuss Whether early return is allowed during switching procedure without leaving RRC_CONNECTED state

Busy indication details

R2-2107026	Further Consideration for Busy Indication	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107237	Considerations on Busy Indication Approach	Samsung	discussion

R2-2107891	Switching notification and busy indication	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2108360	Busy Indication in Multi-SIM	Qualcomm Incorporated	discussion

Busy indication in RRC_INACTIVE

R2-2108737	Busy indication in INACTIVE mode	MediaTek Inc.	discussion	Rel-17	LTE_NR_MUSIM-Core	R2-2106351

R2-2107807	Further analysis on NAS level solutions for RRC-INACTIVE	Nokia, Nokia Shanghai Bell	discussion	Rel-17

R2-2108121	On busy indication in RRC_INACTIVE	Huawei, HiSilicon	discussion

R2-2108051	Discussion on Busy Indication in Inactive State	Sony	discussion	Rel-17	LTE_NR_MUSIM-Core	R2-2105683

R2-2108075	Consideration on the busy indication at Inactive state	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

Network switching and configured time:

R2-2107791	Open Issues for MUSIM Network Switching	Charter Communications, Inc	discussion

R2-2107808	On switching notification solutions for MUSIM operation	Nokia, Nokia Shanghai Bell	discussion	Rel-17

R2-2107973	Discussion on switching procedure without leaving RRC_CONNECTED state	Ericsson	discussion

R2-2107975	Discussion on switching procedure for leaving RRC_CONNECTED state	Ericsson	discussion

R2-2108031	On coordinated switch from NW for MUSIM device	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2108182	Further consideration on the remaining issues of scheduling Gap	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107477	Network switching for Multi-USIM devices during dual connectivity	Samsung	discussion

R2-2108732	Further discussion on switching message details	Samsung Electronics Co., Ltd	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107327	Open Issues on Network Switching	CATT	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107025	Discussion on Configured Time for AS-based Solution	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107459	Network switching with leaving RRC Connected State of Multi-SIM	China Telecommunication	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107597	Signaling aspects of MUSIM Network Switching	Apple	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107598	MUSIM Band Conflict-RRC Processing Delay-Caller ID Requirements	Apple	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107781	Open issues on scheduling gap for network switching	NEC	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107789	 RAN Initiated Network switching with Leaving RRC_CONNECTED	SHARP Corporation	discussion

R2-2108361	Leaving Connected state in Multi-SIM	Qualcomm Incorporated	discussion

R2-2108387	Discussion about the usage of the autonomous gap	Xiaomi Communications	discussion

R2-2108725	Considerations on SIM Swithcing	LG Electronics	discussion	Rel-17	LTE_NR_MUSIM-Core	R2-2106110

R2-2108726	Scheduling Gap Handling	LG Electronics	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2108755	Procedures for MSIM UE notification on network switching	Futurewei Technologies	discussion	R2-2105445	Late

### 8.3.4	Paging with service indication

Including details of the paging cause value support and, if necessary, discussion on additional feedback to SA2

Web Conf (Monday 1st week), paging service indication (2)

R2-2108101	Detailed methods of the paging cause support for MUSIM	Xiaomi Communications	discussion	R2-2106401

Proposal 1: RAN2 discusses the above two methods to indicate paging cause value considering SA2’s request and agreement.

Proposal 2: Method#2 (Define a new pagingcause IE in the legacy PagingRecord IE) should be adopted to indicate the paging cause.

R2-2108122	Discussion on the paging with service indication	Huawei, HiSilicon	discussion	Rel-17	R2-2105921

Proposal: A single value of voice for paging cause indication with parallel list approach should be adopted to introduce paging cause for voice indication.

R2-2108727	Support of Paging Cause	LG Electronics	discussion	Rel-17	LTE_NR_MUSIM-Core	R2-2106111

Proposal 1. For paging cause, RAN2 adds 1-bit information to indicate voice service in the paging message.

Proposal 2. For paging cause, RAN2 adds 1-bit information to discriminate whether to support the paging cause feature in system information.

R2-2107379	Paging Prioritization	Qualcomm Incorporated	discussion

Observation 1: The paging message will include an indication that the cause of the page is for IMS voice.

Observation 2: The NW will use this indication when the UE supports such indication. Since the capability will be at NAS layer, this has no RAN2 impact.

Observation 3: The UE should be capable of differentiation between Paging from a network that does not support the Paging Cause Indication for Voice Service feature and Paging without the Voice Service Indication.

Observation 4: To fullfill the requirement in Observation 3, SA2 relies on RAN2 decision.

Proposal 1: A new cause value is added to the paging message (in PagingRecord) in both LTE and NR to indicate the cause as IMS voice.

Proposal 2: RAN2 to select between the following options to differentiate between Paging from a network that does not support the Paging Cause Indication for Voice Service feature and Paging without the Voice Service Indication:

Option A: The new cause indication in the paging message has two values of “voice” and “other”.

Option B: The support for the Paging Cause Indication for Voice Service is broadcast in SIB1.

R2-2107298	Solution analysis for supporting Multi-SIM paging cause	Intel Corporation	discussion	Rel-17	LTE_NR_MUSIM-Core

Proposal 1: RAN2 to agree that MUSIM UE discriminates whether it is paged from RAN supporting paging cause feature or not, by just receiving Uu paging message (i.e. no additional indication is needed outside Uu paging message to inform RAN’s support of paging cause feature or not).

Proposal 2: Do not extend the legacy PagingRecord IE for paging cause.

Observation: There is not a big difference between the different ASN.1 coding options for non-critical extension of Paging PDU.

Proposal 3: Introduce an explicit indication of network support for voice cause in Paging message.  Choose between the options b) and d) for indication “voice” cause.

??Discuss further details in email discussion [231] (NN)??

R2-2108549	Discussion on paging service indication for MUSIM	Futurewei Technologies	discussion	R2-2105451

R2-2107028	Paging with Service Indication	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107180	Further discussion on introduction of paging cause	China Telecommunications	discussion

R2-2107349	Discussion on the transmission of paging cause	Spreadtrum Communications	discussion	Rel-17

R2-2107350	Supporting of Paging Cause Solution detection	Spreadtrum Communications	discussion	Rel-17

R2-2107809	Service type Indication in paging for LTE/EPC	Nokia, Nokia Shanghai Bell	discussion	Rel-17

R2-2107858	Introduction of Paging Cause	vivo	discussion

R2-2107928	Discussion on support of paging cause for Multi-USIM devices	Samsung Electronics Co., Ltd	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2107976	Introduction of a Paging cause indication	Ericsson	discussion

R2-2108074	Consideration on the Service Indication	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2108738	Paging with service indication	MediaTek Inc.	discussion	Rel-17	LTE_NR_MUSIM-Core	R2-2106353

Email discussions ([231])

[AT115-e][231][MUSIM] Paging with service indication (NN), TBD

Scope:

    - Discuss remaining open issues for paging with service indication and try to have draft TPs to illustrate the necessary modifications

Intended outcome:

    - Discussion summary in R2-2108857 (by email rapporteur, including TPs)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  2nd week Mon, UTC 1000

    - Initial deadline (for rapporteur summary):  2nd week Tue, UTC 1000

By Email (outcome of [231])

R2-2108857	Summary of [AT115-e][231][MUSIM] Paging with service indication (NN)	NN	discussion	Rel-17	LTE_NR_MUSIM-Core

## 8.8	RAN slicing

(NR_Slice -Core; leading WG: RAN2; REL-17; WID: RP-211289)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

Email max expectation: 2 threads

### 8.8.1	Organizational

Rapporteur input and running CRs

Web Conf (Monday 1st week) (1+1)

R2-2106972	LS on Cell reselection with band-specific network slices (S2-2105158; contact: Nokia)	SA2	LS in	Rel-17	eNS_Ph2	To:RAN2, RAN3

(moved from 8.22)

-	Samsung wonders if there is benefit to sending configured NSSAI over "target NSSAI" (i.e. CR attached to this LS)? This would create complexity to NG-RAN? Nokia thinks this is optional for RAN node anyway. It only provides more optimized camping policies for network.

-	LGE thinks there is no RAN2 impact and this is RAN3 work only. Benefits seem to be marginal, though. Ericsson agrees and wonders why target NSSAI is not mentioned? Nokia thinks this is connected to RAN3 discussion but agrees there is no RAN2 impact.

-	Lenovo thinks the LS tries to have NSSAI assistance that can be used by network.

Will discuss reply LS after RAN slicing online session on 1st week Tuesday

Noted

Post-meeting email discussions (running CRs, TBD which are needed)

[Post115-e][245][Slicing] Running NR RRC CR for RAN slicing (NN)

Scope: Create running NR RRC CR for RAN slicing

Intended outcome: Running CR

Deadline:  Long

[Post115-e][246][Slicing] Running 38.304 CR for RAN slicing (NN)

Scope: Create running 38.304 CR for RAN slicing

Intended outcome: Running CR

Deadline:  Long

[Post115-e][247][Slicing] Running Stage-2 CRs for RAN slicing (NN)

Scope: Create running Stage-2 CRs (38.300 and/or 37.340) for MUSIM

Intended outcome: Running CR

Deadline:  Long

### 8.8.2	Cell reselection

Including discussion on whether SA2 proposal on band-specific slices in cell reselection has impacts on the RAN (cv. SA2 LS R2-2106972 / S2-2105158)

Including outcome of [Post114-e][251][Slicing] Solution direction details for slice priorities in cell reselection (Lenovo)

Including discussion on how "slice group" can be defined and indicated to UE

As 1st priority, including details of slice availability in terms of Slice grouping and frequency priority information for broadcast and RRC Release message,  usage of “intended slice” (FFS whether we use this term in specification), UE prioritisation of slice when there is more than one intended slice and how UE determines frequency priority for inter-frequency cell reselection based on these.

As 2nd priority, including details of slice based reselection for MO, different RSRP/RSRQ thresholds for inter and intra-frequency slice based cell reselection, need for Validity area in RRC Release

Web Conf (Monday 1st week) (1+1)

Including discussion on whether SA2 proposal on band-specific slices in cell reselection has impacts on the RAN (cv. SA2 LS R2-2106972 / S2-2105158))

R2-2107951	Reply proposal for LS on cell reselection with band-specific network slices (S2-2105158/ R2-2106972)	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_slice-Core

(moved from 8.8.1)

R2-2107372	Discussion on slice based cell reselection	Spreadtrum Communications	discussion	Rel-17

R2-2108554	Discussion on slice based cell reselection under network control	Huawei, HiSilicon	discussion	Rel-17	NR_slice-Core

Web Conf (Tuesday 1st week) (1+1)

Including outcome of [Post114-e][251][Slicing] Solution direction details for slice priorities in cell reselection (Lenovo)

R2-2108025	Summary of [Post114-e][251][Slicing] Solution direction	Lenovo, Motorola Mobility (Rapporteur)	discussion	NR_slice-Core

Discussion

-	Samsung thinks we need to address the FFS before agreeing to option 4 (step 6 & 7). If step 4 uses "slice", step 6 may not be needed.

-	CMCC can accept solution 4 but thinks it may lead UE to reselect to a cell that only supports 1st priority slice but not lower priority slices. Solution 5 may help in that. LGE agrees and would like to keep step 7 as FFS to allow second-priority slices to be considered. QC thinks step 7 could be removed entirely. Apple agrees but thinks CMCC point is valid so could be considered based on option 5.

-	Intel is fine with option 4 but thinks we need to discuss the steps further in details online.

-	Nokia thinks "selected slice support" step 5 may not be needed and might just delay cell reselection procedure due to SIB reading. Would like to avoid SIB reading. ZTE agrees. Apple thinks serving cell could indicate the neighbour cell slice information.

- 	Step 1: Intel thinks that we need to ask SA2/CT1 for the list.  Need to discuss what "list" means. Lenovo clarifies this is list in AS and this is sorted according to priority. Where the list comes from is different question. Priority value could be part of the list or be in order of priority. NAS provides the information to AS and AS uses it.

-	Apple thinks SA2 already discussed priority information and rejected it. UE may not be able to do it.

-	CATT thinks we need to check frequency priorities. Samsung thinks UE measuring all frequencies may not need step 6.

-	KDDI thinks current measurement rules depend on serving cell level. Low priority frequencies are only measured if serving cell level is low. Lenovo agrees.

-	BT wonders if UE would be required to measure same frequency twice for different slices? Lenovo clarifies this is up to UE implementation (same as legacy). Could depend on RAN4 requirements.

-	Step 5: Intel wonders what the frequency priority of the assigned cell is? Measurement rules are based on that. Lenovo thinks this is a valid question but was not discussed before. thinks it's as applicable for the corresponding slice, i.e. frequency priority comes from the slice. Ericsson thinks UE follows the priority of the serving cell.

-	Step 6: BT wonder if we should check slice frequencies instead of cell frequencies?

- Agreements

RAN2 needs to check with SA2/ CT1 if it is alright for AS to expect to receive slice list as well as slice priority information from NAS for cell (re)selection. Ask about both slices and slice groups.

- Agreements

2	Following is taken as the baseline for Solution Option 4:

- The “slice info” (for a single slice or slice group) agreed to be provided to the UE in the last RAN2 meeting using both broadcast and dedicated signaling are provided for the serving as well as neighboring frequencies. The following steps are used for slice based cell (re)selection in AS:

- Step 0: NAS layer at UE provides slice information to AS layer at UE, including slice priorities.

- Step 1: AS sorts slices in priority order starting with highest priority slice.

- Step 2: Select slices in priority order starting with the highest priority slice.

- Step 3: For the selected slice assign priority to frequencies received from network.

- Step 4: Starting with the highest priority frequency, perform measurements (same as legacy).

- Step 5: If the highest ranked cell is suitable (as defined in 38.304) and supports the selected slice in step 2 then camp on the cell and exit this sequence of operation; FFS: How the UE determines whether the highest ranked cell supports the selected slice.

- Step 6: If there are remaining frequencies then go back to step 4.

- Step 7: FFS: If the end of the slice list has not been reached go back to step 2.

- Step 8: Perform legacy cell reselection.

1: Solution Option 4 is selected for further work i.e., resolve the FFSs, send any required LSs and consequently start to draft specification CRs.

Proposal 2: Following is taken as the baseline for Solution Option 4:

The “slice info” agreed to be provided to the UE in the last RAN2 meeting using both broadcast and dedicated signaling are provided for the serving as well as neighboring frequencies. The following steps are used for slice based cell (re)selection:

Step 1: List slices in priority order starting with highest priority slice.

Step 2: Select slices in priority order starting with the highest priority slice.

Step 3: For the selected slice assign priority to frequencies received from network.

Step 4: Starting with the highest priority frequency, perform measurement according to the legacy procedure.

Step 5: If the highest ranked cell is suitable (as defined in 38.304) and supports the selected slice in step 2 then camp on the cell and exit this sequence of operation; FFS: How the UE determines whether the highest ranked cell supports the selected slice.

Step 6: If there are remaining cell frequencies then go back to step 4.

Step 7: FFS: If the end of the slice list has not been reached go back to step 2.

Step 8: Perform legacy cell reselection.

2 FFSs need to be resolved. In addition, based on the feedback from some companies, RAN2 needs to check with SA2/ CT1 if it is alright for AS to expect to receive slice list as well as slice priority information from NAS for cell (re)selection.

To be discussed later (2nd week)

Proposal 3: RAN2 to discuss if another Solution Option (one out of 5, 6 and 7) may also be shortlisted – If so, the final decision between Option 4 and Option ‘X’ need be taken.

R2-2108842	Resolving FFSs for Option 4	Lenovo, Motorola Mobility	discussion	Rel-17	NR_slice-Core	Late

Proposal 1: Serving cell broadcast slice support of serving and neighbor cells as part of “slice info”. Some signaling optimizations may be pursued in stage-3. FFS: If SIB3/ 4 or a new SIB should be used.

Proposal 2: RAN2 needs to discuss further online on the 2nd FFS (step 7).

R2-2107952	Proposals for slice specific cell reselection solutions	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_slice-Core

Proposal 2.1: RAN2 defines the "intended slices" as the slices that are considered during slice-based cell reselection.

Proposal 2.2: RAN2 assumes that NAS provides the slice information (slices to be considered during cell reselection, and the priorities of the slices) to AS. RAN2 should ask SA2/CT1 to confirm this assumption in an LS.

Proposal 2.3: RAN2 asks SA2/CT1 whether AS or NAS should map the individual slices to slice groups, which are used for cell reselection.

??LS to SA2/CT1 needed?

R2-2108497	Discussion on the solutions for slice based cell reselection	CMCC	discussion	Rel-17	NR_slice

R2-2107461	Discussion on slice based cell reselection	China Telecommunication, Baicells 	discussion	Rel-17	NR_slice-Core

R2-2107466	Cell reselection in RAN slicing	FGI, Asia Pacific Telecom	discussion

R2-2107505	Considerations on contents of slice related cell selection info	KDDI Corporation	discussion

R2-2107929	Discussion on slice-based cell reselection prioritization	BT plc	discussion	Rel-17

R2-2108292	Slice grouping	Ericsson	discussion	Rel-17	NR_slice-Core

R2-2107108	Further discussion on slice specific cell reselection	Qualcomm Incorporated	discussion	NR_slice

R2-2107243	Considerations on slice based cell reselection	Beijing Xiaomi Software Tech	discussion	Rel-17

R2-2107383	Discussion on Slice based Cell Reselection	CATT	discussion	NR_slice-Core

R2-2107443	Functional aspects of slice specific cell reselection	Intel Corporation	discussion	Rel-17	NR_slice-Core

R2-2107592	Slice based cell reselection under NW control	Apple	discussion	Rel-17	NR_slice-Core

R2-2107705	Discussion on slice based cell reselection	LG Electronics UK	discussion	Rel-17

R2-2107730	Discussion on slice aware cell reselection	ZTE corporation, Sanechips	discussion	Rel-17	NR_slice-Core

R2-2107739	Consideration on slice-specific cell reselection	OPPO	discussion	Rel-17	NR_slice-Core

R2-2108316	On slice priority for cell reselection	Samsung R&D Institute UK	discussion

R2-2108433	Slice information provided by RRCRelease	SHARP Corporation	discussion	Rel-17	R2-2106087

Withdrawn:

R2-2108315	Considerations on slice-based cell reselection	Lenovo, Motorola Mobility	discussion	Rel-17	NR_slice-Core	Withdrawn

Email discussions ([240])

[AT115-e][240][Slicing] Reply LS to SA2 on band-specific slices in cell reselection (Nokia)

Scope:

    - Draft reply LS to SA2 LS R2-2106972 (S2-2105158).

Intended outcome:

    - Draft LS to SA2/CT1 in R2-2108860 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 1000

    - Initial deadline (for final draft LS):  1st week Fri, UTC 0900

By Web Conf (1st week Friday) or By Email (outcome of [240])

R2-2108860	[Draft] Reply LS on Cell reselection with band-specific network slices	Nokia	LS out	Rel-17	NR_Slice-Core	To: SA2, RAN3

### 8.8.3	RACH

Including outcome of [Post114-e][252][Slicing] RACH partitioning details for slicing (CMCC)

Including discussion slice specific CBRA RACH for IDLE and INACTIVE mode. Slice-specific CBRA RACH for CONNECTED mode is deprioritized and will not be treated in this meeting.

NOTE: The common discussion on Rel-17 RACH partitioning will be discussed under AI 8.18. This AI will only consider RACH partitioning from slicing perspective.

Web Conf (Tuesday 1st week) (1+1)

Outcome of [Post114-e][252][Slicing] RACH partitioning details for slicing (CMCC)

R2-2108504	Report for [Post114-e][252][Slicing] RACH partitioning details for slicing	CMCC	discussion	Rel-17	NR_slice

Revised in R2-2108839

R2-2108839	Report for [Post114-e][252][Slicing] RACH partitioning details for slicing	CMCC	discussion	Rel-17	NR_slice

Proposed bulk agreements

2	The mapping between S-NSSAIs and slice groups should be configured to the UE through NAS signalling.

3	Network based solution is introduced to resolve the issue of prioritization parameter collision with MPS/MCS, i.e., Network indicates whether slice override MPS or MPS override slice.

5	For slice based RACH prioritization, RAN2 will stick to the current baseline parameters, i.e., scalingFactorBI and powerRampingStepHighPriority, and no additional parameters for this release.

6	For RACH type selection, UE first selects between slice-specific and common RACH, then selects between 2-step and 4-step.

7 	Reuse the legacy threshold for the selection between 2-step and 4-step slice initiated RACH

9 	The following fallback case is supported:

–	Fallback case 2: Fallback from 2-step slice specific RACH to 4-step common RACH, if 4-step slice specific RACH is not configured.

??Any dependencies on common RACH partitioning discussion?

Discuss online

1	A new slice grouping mechanism is introduced for RACH configuration.

4	If no network indication is sent in case of slice prioritization parameter collision with MPS/MCS, it will be left to UE implementation.

10	The following fallback cases are not supported in this release:

–	Fallback case 1: Fallback from 4-step slice specific RACH to 4-step common RACH

–	Fallback case 3: Fallback from 2-step slice specific RACH to 2-step common RACH, if neither 4-step slice specific RACH nor 4-step common RACH is configured.

8	It is RAN2 common understanding that 4-step common RACH needs to always be supported in initial BWP for legacy UE. And whether to configure 2-step slice specific RACH only or 4-step slice specific RACH only or both is left to network configuration.

[13/17] Proposal 1: A new slice grouping mechanism is introduced for RACH configuration.

[16/16] Proposal 2: The mapping between S-NSSAIs and slice groups should be configured to the UE through NAS signalling.

[16/18] Proposal 3: Network based solution is introduced to resolve the issue of prioritization parameter collision with MPS/MCS, i.e., Network indicates whether slice override MPS or MPS override slice.

[13/17] Proposal 4: If no network indication is sent in case of slice prioritization parameter collision with MPS/MCS, it will be left to UE implementation.

[17/17] Proposal 5: For slice based RACH prioritization, RAN2 will stick to the current baseline parameters, i.e., scalingFactorBI and powerRampingStepHighPriority, and no additional parameters for this release.

[15/18] Proposal 6: For RACH type selection, UE first selects between slice-specific and common RACH, then selects between 2-step and 4-step.

[15/17] Proposal 7: Reuse the legacy threshold for the selection between 2-step and 4-step slice initiated RACH

[11/18] Proposal 8: It is RAN2 common understanding that 4-step common RACH needs to always be supported in initial BWP for legacy UE. And whether to configure 2-step slice specific RACH only or 4-step slice specific RACH only or both is left to network configuration.

[15/17] Proposal 9: The following fallback case is supported:

–	Fallback case 2: Fallback from 2-step slice specific RACH to 4-step common RACH, if 4-step slice specific RACH is not configured.

[13/17] Proposal 10: The following fallback cases are not supported in this release:

–	Fallback case 1: Fallback from 4-step slice specific RACH to 4-step common RACH

–	Fallback case 3: Fallback from 2-step slice specific RACH to 2-step common RACH, if neither 4-step slice specific RACH nor 4-step common RACH is configured.

R2-2108498	Open issues for slice based RACH configuration	CMCC	discussion	Rel-17	NR_slice

Proposal 1: The same new slice grouping mechanism is applied for both RACH configuration and cell reselection to address security and SIB payload size issues. The solutions of broadcasting SST and access category are not pursued.

Proposal 2: The mapping between S-NSSAIs and slice groups should be configured in NAS signalling during initial registration and mobility registration/TA update procedure.

Proposal 3: For the topic of prioritization parameters collision with MPS/MCS, it can be configurable by network, and if not configured, slice specific RA prioritization parameters should override MPS/MCS specific RA prioritization parameters.

Proposal 4: Case 3/6/8 in the table are valid from network configuration perspective..

Proposal 5: The UE should first select between slice specific RA and common RA, if both are configured.

Proposal 6: It’s acceptable to introduce a new RSRP threshold or reuse the legacy threshold for the selection between 2-step and 4-step slice-initiated RACH.

Proposal 7: The parameter msgA-TransMax can be configured differently per slice group.

Proposal 8: For the cases of fallback from slice specific RACH to common RACH, only fallback from 2-step slice specific RA to 4-step common RA is supported, if 4-step slice specific RA is not configured.

Proposal 9: RAN2 agree the fallback cases in the table 2. The changes are highlighted in yellow.

|  | RACH resource configuration in one BWP | RACH type selection for slice triggered access | Fallback after MSGA or MSG1 attempt number beyond threshold |
| --- | --- | --- | --- |
| Case 1 | 2-step slice specific RACH 4-step common RACH | Always perform 2-step slice specific RACH | Fallback to 4-step common RACH |
| Case 2 | 2-step slice specific RACH 4-step slice specific RACH 4-step common RACH | RACH type selection based on RSRP threshold | Fallback to 4-step slice specific RACH No Fallback from 4-step slice specific RACH to 4-step common RACH |
| Case 3 | 4-step slice specific RACH 2-step common RACH | Always perform 4-step slice specific RACH | No fallback |
| Case 4 | 4-step slice specific RACH 4-step common RACH | Always perform 4-step slice specific RACH | No fallback |
| Case 5 | 2-step slice specific RACH 2-step common RACH 4-step slice specific RACH 4-step common RACH | RACH type selection based on RSRP threshold | Fallback to 4-step slice specific RACH No Fallback from 4-step slice specific RACH to 4-step common RACH |
| Case 6 | 2-step slice specific RACH 2-step common RACH | Always perform 2-step slice specific RACH | No Fallback |
| Case 7 | 2-step slice specific RACH 2-step common RACH 4-step common RACH | Always perform 2-step slice specific RACH | Fallback to 4-step common RACH |
| Case 8 | 4-step slice specific RACH 2-step common RACH 4-step common RACH | Always perform 4-step slice specific RACH | No Fallback from 4-step slice specific RACH to 4-step common RACH |

Proposal 10: The unified RACH configuration IEs can be added inside the current RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA of SIB1, which indicate that the specific RACH resources for different WIs.

R2-2107109	Further discussion on slice specific RACH	Qualcomm Incorporated	discussion	NR_slice

R2-2107241	Considerations on slice based RACH configuration	Beijing Xiaomi Software Tech	discussion	Rel-17

R2-2107384	Analysis on slice based RACH configuration	CATT	discussion	NR_slice-Core

R2-2107444	Further considerations of slice based RACH	Intel Corporation	discussion	Rel-17	NR_slice-Core

R2-2107506	Slice-specific RACH configurations	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_slice	R2-2105475

R2-2107593	Slice based RACH configuration	Apple	discussion	Rel-17	NR_slice-Core

R2-2107714	Slice specific RACH type selection	Samsung	discussion	Rel-17	NR_slice-Core	R2-2105345

R2-2107731	Slice specific RACH resources and RACH prioritization	ZTE corporation, Sanechips	discussion	Rel-17	NR_slice-Core

R2-2107740	Consideration on slice-specific RACH	OPPO	discussion	Rel-17	NR_slice-Core

R2-2108293	RACH for RAN slicing enhancement	Ericsson	discussion	Rel-17	NR_slice-Core

R2-2108555	Discussion on slice based RACH configuration	Huawei, HiSilicon	discussion	Rel-17	NR_slice-Core

R2-2108759	Further discussion on slice-specific RACH	LG electronics Inc.	discussion	Rel-17	NR_slice-Core

## 8.20	Extending NR operation to 71GHz

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-211584)

Time budget: 0.5

Tdoc Limitation: 2 tdocs

Note: RAN2 is to prioritize protocol support of RAN1 design and not on optimizations on items not discussed in RAN1

### 8.20.1	Organizational

Web Conf (Monday 2nd week) (2+1)

Rapporteur input, incoming LS etc.

R2-2106917	LS on how to introduce the 52.6-71GHz frequency range (R1-2106277; contact: Lenovo)	RAN1	LS in	Rel-17	NR_ext_to_71GHz-Core	To:RAN	Cc:RAN2, RAN4

Noted

R2-2106954	LS on RAN4 recommendation for the 52.6 - 71 GHz frequency range designation (R4-2107879; contact: Huawei)	RAN4	LS in	Rel-17	NR_ext_to_71GHz-Core	To:RAN	Cc:RAN1, RAN2, RAN5

Noted

R2-2108476	Workplan for Rel-17 WW Extending NR operation to 71GHz	Qualcomm Incorporated, Intel Corporation	Work Plan

Work plan

| RAN2#115e | 0.5 | - Start discussion on impacts on higher layer to support of enhancements agreed by RAN1 and RAN4 - Start discussion on UE capability structure based on the RAN plenary guidance (See Note 5 above in the WID) - With lower priority, start discussion any possible upper layer enhancement which can benefit operation above 51.6 Ghz. |
| --- | --- | --- |

### 8.20.2	General

RAN2 impact tech proposals.

Web Conf (Monday 2nd week) (1)

R2-2107551	RAN2 impact on extending NR operation to 71GHz	Intel Corporation	discussion	Rel-17	NR_ext_to_71GHz

Observation#1: RAN1 is discussing both the RO configuration and RA-RNTI/MsgB-RNTI together

Proposal#1: Wait for RAN1 to progress on the calculation of RA-RNTI/MsgB-RNTI issue

Proposal#2: An existing UE capability applicable to FR2 is also applicable to FR2-2, unless otherwise stated (i.e. in the field description of the UE capability that it is not applicable to FR2-2) in TS38.306,

Proposal#3: If a new UE capability introduced for FR2-2 is also applicable to FR2-1 and/or FR1 and the UE capability is per band, this can be expressed in the field description of the UE capability.

Proposal#4: For an existing UE capability already requires FR1-FR2 Diff and further differentiation between FR2-1 and FR2-2 is needed, the existing UE capability is replicated for FR2-2, similar to legacy capabilities also applicable to shared spectrum access operation.

Proposal#5: For a new UE feature introduced for 52.6-71GHz to be applicable also to FR2-1 or to both FR2-1 and FR1 and FRx differentiation is needed,  one easy solution is to make such feature per band, instead of per UE. For UE capability that has to be per UE, “FR1-FR2 Diff” column can be used to express the need of the FRx differentiation (via the ‘Yes/No’ and also whether it needs FR2-1 and FR2-2 differentiation).

Proposal#6: To introduce the RLC RTT vales for SCS480kHz and 960kHz in the table on RLC RTT for NR cell group per SCS. FFS on the values (wait for RAN1 progress on UE processing latency)

Further discussion on UE capabilities done in email discussion [210]

R2-2107476	RRC impact due to FR2-1 and FR2-2 distinction	Ericsson	discussion	Rel-17	NR_ext_to_71GHz-Core

R2-2107985	FR2-2 considerations	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ext_to_71GHz-Core

R2-2107255	High layer impacts of beyond 52.6GHz	OPPO	discussion

R2-2107266	Analysis of RAN2 impacts of Ext 52-71GHz	Huawei, HiSilicon	discussion	Rel-17	NR_ext_to_71GHz-Core

R2-2107267	Discussion about capability issues for Ext 52-71GHz	Huawei, HiSilicon	discussion	Rel-17	NR_ext_to_71GHz-Core

R2-2107475	Aspects of CA operation and protocol impact	Ericsson	discussion	Rel-17	NR_ext_to_71GHz-Core

R2-2108477	Upper Layer impacts of extending NR operation to 71GHz	Qualcomm Incorporated	discussion	Late

Web Conf (Monday 2nd week) (1)

UP impacts: RACH

R2-2107479	Impact of high SCS on RA-RNTI calculation	ZTE Corporation, Sanechips	discussion

Proposal 1: If 480kHz/960kHz PRACH SCS is supported,  the following enhancement schemes for RA-RNTI may be considered:

-  Option 1: Reusing Rel-15/16 RA-RNTI formula. Dividing the system frame into multiple segments and informing segment id transmitting preamble in DCI.

-   Option 2: Changing t_id range and the formula of RA-RNTI to use module according to PRACH SCS.

Proposal 2: if segment id is informed to the UE, only RARs with the same segment id may be multiplexed into one MAC PDU.

R2-2108745	Consideration on potential RACH impact	LG Electronics Inc.	discussion	Rel-17	NR_ext_to_71GHz-Core

R2-2107060	Discussion on RA(MsgB)-RNTI Design for Beyond 52.6GHz	vivo	discussion	NR_ext_to_71GHz-Core

Web Conf (Monday 2nd week) (1)

UP impacts: RLC

R2-2107963	Discussion on RLC RTT and L2 buffer size	Samsung	discussion	Rel-17

Proposal 1: RAN2 to discuss adding RLC RTTs of 13, 8, 5ms for 240, 480, 960 kHz SCS respectively.

Proposal 2: RAN2 to keep the current L2 buffer size definition using RLC RTT corresponding to the smallest SCS numerology.

Proposal 3: RAN2 to have discussion on how to relax the burden on the UE L2 buffer size while keeping the current L2 buffer definition.

R2-2107964	Impact of higher SCS on RLC operation	Samsung	discussion	Rel-17

Proposal 1: RAN2 to keep the current RLC timer values for NR operation with 480, 960 kHz SCS.

Proposal 2: RAN2 to keep the current RLC framework for NR operation over 52GHz in Rel-17.

LBT impacts:

R2-2108746	Consideration on potential LBT impact	LG Electronics Inc.	discussion	Rel-17	NR_ext_to_71GHz-Core

Observation 1. As all options in RAN1 discussion assume per beam LBT, RAN2 can expect per beam LBT would be introduced in Rel-17.

Observation 2. The current LBT failure detection and recovery procedure is designed based on omni-directional LBT.

Observation 3. To determine RAN2 impact by per beam LBT, more detailed RAN1 behaviour should be determined first.

Proposal 1. To identify clear RAN2 impacts by per beam LBT, RAN2 wait for RAN1 progress on per beam LBT or send LS to RAN1 to ask clear UE behaviour when one of multiple beams fails LBT.

R2-2107480	RAN2 impact for LBT for operation up to 71 GHz	ZTE Corporation, Sanechips	discussion

R2-2107061	Discussion on Consistent LBT Failure Detection for Beyond 52.6GHz	vivo	discussion	NR_SmallData_INACTIVE-Core

IDC impact due to WiGig:

R2-2107792	In-device coexistence for NR above 52.6GHz	Charter Communications, Inc	discussion

Observation: Extending NR operation to 52.6-71 GHz introduces new IDC issues with IEEE 802.11ad/ay.

Proposal 1: Extend IDC Assistance signalling in Rel-17 to include WiGig as a victim system type.

Email discussions ([210]), TBD

[AT115-e][210][71 GHz] Capability differentiation between FR2-1 and FR2-2 (NN)

Scope:

    - Discuss which RAN2 capabilities require differentiation between FR2-1 and FR2-2.

Intended outcome:

    - Discussion summary in R2-2108858 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  2nd week Wed, UTC 1200

    - Initial deadline (for rapporteur summary):  2nd week Thu, UTC 1200

By Email (outcome of [210])

R2-2108858	Summary of [AT115-e][210][71 GHz] Capability differentiation between FR2-1 and FR2-2 (NN)	NN	discussion	Rel-17	NR_ext_to_71GHz-Core

# 9	Rel-17 EUTRA Work Items

## 9.3	EUTRA R17 Other

Time budget: 0 TU

Tdoc Limitation:  No limitation but the AI may be entirely deprioritized depending on available time.

Email max expectation: 1 thread

TEI17 documents can be submitted under this agenda

Web Conf (Friday 1st week) (1)

R2-2106930	Reply LS to LS on User Plane Integrity Protection for eUTRA connected to EPC (R3-212812; contact: Qualcomm)	RAN3	LS in	Rel-17	To:SA3, RAN2, CT1, CT4, SA2

Noted (consistent with earlier RAN2 decision)

Web Conf (Friday 1st week) (8+1+1+1)

Event-triggered logged MDT enhancement (new but submitted earlier, postponed):

R2-2107214	Introduction of event-based trigger for LTE MDT logging	KDDI Corporation, CMCC, Telecom Italia, Samsung, Ericsson	draftCR	Rel-17	36.331	16.5.0	B	TEI17

(moved from 8.21.2)

Revised in R2-2109027

R2-2107215	Introduction of event-based trigger for LTE MDT logging	KDDI Corporation, CMCC, Telecom Italia, Samsung, Ericsson	draftCR	Rel-17	37.320	16.5.0	B	TEI17

(moved from 8.21.2)

Revised in R2-2109028

R2-2109027	Introduction of event-based trigger for LTE MDT logging	KDDI Corporation, CMCC, Telecom Italia, Samsung, Ericsson, China Unicom	draftCR	Rel-17	36.331	16.5.0	B	TEI17

R2-2109028	Introduction of event-based trigger for LTE MDT logging	KDDI Corporation, CMCC, Telecom Italia, Samsung, Ericsson, China Unicom	draftCR	Rel-17	37.320	16.5.0	B	TEI17

R2-2108556	Discussion on event triggered logged MDT for LTE	Huawei, HiSilicon	discussion	Rel-17	TEI17	R2-2106144

R2-2108557	CR to 36.306 on event triggered logged MDT for LTE	Huawei, HiSilicon	draftCR	Rel-17	36.306	16.5.0	B	TEI17

R2-2108558	CR to 36.331 on event triggered logged MDT for LTE	Huawei, HiSilicon	draftCR	Rel-17	36.331	16.5.0	B	TEI17

R2-2108559	CR to 37.320 on event triggered logged MDT for LTE	Huawei, HiSilicon	draftCR	Rel-17	37.320	16.5.0	B	TEI17

R2-2108560	CR to 36.304 on event triggered logged MDT for LTE	Huawei, HiSilicon	draftCR	Rel-17	36.304	16.4.0	B	TEI17

??Further details discussed in email discussion [203]

Positioning information enhancements to logged MDT (new)

R2-2108596	Introduction of sensor-LocationInfo for LTE MDT	KDDI Corporation	discussion

(moved from 8.21.2)

??Further details discussed in email discussion [203]

EDT enhancements (new):

R2-2107125	UE specific DRX during EDT	Qualcomm Incorporated	discussion	Rel-17	TEI17

??Further details discussed in email discussion [203]

RSSI/CO measurement capability in LTE for NR-U (basic intent agreed during RAN2#113e in NR session):

R2-2107589	Adding NR-U RSSI/CO measurement UE capability into LTE	Apple	discussion	Rel-17	DUMMY

(moved from 8.21.2)

??Further details discussed in email discussion [203]

Email discussions ([203])

[AT115-e][203][LTE] Further details of TEI17 topics (NN)

Scope:

    - Discuss details of selected TEI17 topics (as per online session agreements)

Intended outcome:

    - Discussion report in R2-2108854

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Wed, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Wed, UTC 1700

By Email (outcome of [203]), TBD

R2-2108854	Summary of [AT115-e][203][LTE] Further details of TEI17 topics (NN)	NN	discussion	Rel-17	TEI17

## 9.4	NR and EUTRA Inclusive language

Time budget: N/A

CRs were endorsed/agreed-in-principle at R2#112-e. Final approval is expected when R17 TSes are to be created and at that point CRs need to be updated towards latest TS version and submitted again. Meanwhile this AI can be used to cover missing part, if any, and for correction/modification of the endorsed/agreed-in-principle CRs e.g. for inter-group consistency, inter-group review etc. There may be a consistency review activity organized at R2#115-e, where the rapporteurs of impacted TSes are expected to participate (TBD). RAN coordinator for inclusive language is Gino Mansini (Ericsson).

Web Conf (Friday 1st week) (1+1)

R2-2106981	LS on Inclusive language for ANR (S5-213683; contact: Ericsson)	SA5	LS in	Rel-17	To:RAN3, RAN2

Noted

R2-2108297	Inclusive Language Review Status and Consistency Check		Ericsson (coordinator)	discussion	Rel-17

Discussion

-	NN thinks that...

??Reply LS drafted in email discussion [202]

Email discussions ([202])

[AT115-e][202][LTE/NR] Inclusive language (Ericsson)

Scope:

    - Draft LS (To: SA5) according to RAN2 decisions on inclusive language alignment between WGs and TSGs

Intended outcome:

    - Discussion report in R2-2108852 (if needed)

    - Agreeable LS in R2-2108853

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  2nd week Thu, UTC 0900

    - Deadline for final LS: 2nd week Thu, UTC 1200

By Email (outcome of [202]), TBD

R2-2108852	Summary of [AT115-e][202][LTE/NR] Inclusive language (Ericsson)	Ericsson	discussion	Rel-17	TEI17

R2-2108853	Reply LS on Inclusive language for ANR	RAN2	LS out	Rel-17	TEI17	To: SA5	Cc: RAN3, RAN, SA, CT

# Summary

Agreed CRs ()

LTE legacy (Rel-16 and earlier, except for LTE Rel-16 mobility) - XX CRs (X for 36.306, X for 36.331; X for Rel-15, X for Rel-16)

Rel-16 LTE mobility - XX CRs (X for 36.300, X for 36.321, X for 36.331)

Endorsed documents ()

Postponed documents ()

Approved LS out ()

Post-meeting email discussions (short) ()

Post-meeting email discussions (long) ()