3GPP TSG-RAN WG2 Meeting #115 Electronic							 R2-2109157

Electronic, 16 – 27 August 2021

Title:	LS on RMSI reception based on non-zero search space

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN1

Contact person:	Shukun Wang

wangshukun@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

- 1	Overall description

Currently, the dedicated BWP can be configured for the RRC_CONNECTED UE with optional common search space configuration. If the common search space is not set to zero for OSI/paging, RAN2 spec (38.331 section 5.2.2.3.2 for OSI and TS38.304 section 7 for paging) indicates the mapping between OSI/paging PDCCH monitoring occasions and SSBs for all RRC state UE. However, it is not clear for RMSI reception if search space for RMSI is not set to zero for RRC_CONNECTED UE.

RAN2 has discussed the issue about SIB1 reception for RRC_CONNECTED UEs. When the UE is configured with a dedicated BWP not covering the cell-defining SSB (i.e. the SSB with an RMSI associated) and CORESET#0, the common search space for SIB1 reception (i.e. searchSpaceSIB1) configured in this BWP has to be a non-zero search space. In this case, there is no mapping correlation defined in RAN2 specifications between PDCCH occasions of the non-zero search space and SSBs.

Q1: RAN2 would like to ask RAN1 in the above case when the dedicated BWP does not cover the cell-defining SSB, whether a mapping between PDCCH occasions and SSBs needs to be defined for the non-zero search space configured for reception of SIBs? Or SIB reception can be based on other means (e.g. RRC configured TCI state)?

Q2: If RAN1 confirm RRC configured TCI state is used to receive RMSI PDCCH, RAN2 wants to confirm further whether the RRC configured TCI state can also be used for OSI/paging reception?

- 2	Actions

To RAN1 group:

RAN2 respectfully asks RAN1 to consider the above questions.

- 3	Dates of next RAN2 meetings

TSG-RAN2 Meeting #116-e 	Nov. 01 – Nov. 12, 2021

TSG-RAN2 Meeting #117-e 	Feb. 21 – Feb. 25, 2022