3GPP TSG-RAN WG2 Meeting #115-e	R2-2108937

Electronic, 7th– 19th Aug, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0305 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PRS-only TP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Pos-Core |  |  |  |  |  | Date: |  |  | 2021-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The result of the TRP-ID break-out based on the following agreement in RAN2#110-e is that for almost all instances that were previously represented by the IE TPR-ID, three optional fields, i.e. nr-PhysCellID, nr-CellGlobalID, and nr-ARFCN, are used. Agreements: The TRP-ID IE is replaced by separate IEs signalled in the separate cases where previously the TRP-ID was used. The existing dl-PRS-Id field retains its range as INTEGER (0..255) and is broken out as a separate IE. The included identifiers of the NR E-CID Signal Measurement Information per cell are the NR physical cell identity, NR cell global identity (shall be provided if the device was able to determine the NCGI of the measured cell at the time of measurement) and NR ARFCN. The NR-SSB-Config IE includes NR physical cell identity and NR ARFCN but no (0..255) TRP ID. A proper UE assumption when those optional fields are not provided should be discussed given that RAN1 already agreed the CR in R1-2104065 to address the following issues. - PRS and SSB multiplexing on the same symbol if they are from the same cell - Positioning SRS power control and spatial relation configuration with respect to the PRS from the serving cell or from the non-serving cell Following the legacy LPP approach, a flag is introduced in the LPP assistance data to indicate that the PRS is from a PRS-only TP. Also, a reference has been added for TS 38.213 and the previous wrong citation has been corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ A flag for PRS-only TP is added. 2/ The clarification on absence of the fields nr-PhysCellID and nr-CellGlobalID is provided that these field shall be absent when the flag for PRS-onlyTP is present. 3/ Add field description for PCI and NCGI in NR-TimeStamp. 4/ Reference for TS 38.213 is added and the citation is corrected. 5/ Change the typo in ECID procide capabilities from “ssrsqpSup” to “ssrsrqSup” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE assumption on PRS-only TP for the purpose of PRS and SSB multiplexing for the same cell and SRS power control and spatial relation with respect to the serving cell and the non-serving cell. Impact analysis Impacted functionality NR-DL-PRS-AssistanceData, NR-SSB-Config, NR-TimeStamp, NR-DL-TDOA-SignalMeasurementInformation, NR-DL-AoD-SignalMeasurementInformation, NR-Multi-RTT-SignalMeasurementInformation Inter-operability: - If the UE is implemented according to this CR while the network is not, there is no inter-operability issue - If the network is implemented according to this CR while the UE is not, the UE may include some UE capabilities that the network cannot understand. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 6.4.3, 6.5.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | rev0 R2-2105054, submitted to RAN2#114e rev1:R2-2107332, submitted to RAN2#115e |  |  |  |  |  |  |  |  |

==================================START OF CHANGES==================================

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[3]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[4]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[5]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[6]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[7]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[8]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[9]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[10]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[11]	RTCM-SC104, RTCM Recommended Standards for Differential GNSS Service (v.2.3), August 20, 2001.

[12]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); "Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[14]	3GPP TS 44.031: "Location Services (LCS); Mobile Station (MS) - Serving Mobile Location Centre (SMLC) Radio Resource LCS Protocol (RRLP)".

[15]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[16]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[17]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer – Measurements".

[18]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	OMA-TS-LPPe-V1_0, LPP Extensions Specification, Open Mobile Alliance.

[21]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[22]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[23]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", February, 2019.

[24]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[25]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[26]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[27]	IETF RFC 6225, "Dynamic Host Configuration Protocol Options for Coordinate-Based Location Configuration Information", July 2011.

[28]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[29]	"Earth Gravitational Model 96 (EGM96)", National Geospatial-Intelligence Agency, NASA.

[30]	RTCM Standard 10403.3: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, October 7, 2016.

[31]	IGS ANTEX: "The Antenna Exchanged Format" – version 1.4, September 15, 2010.

[32]	Federal Information Processing Standards Publication 197, "Specification for the ADVANCED ENCRYPTION STANDARD (AES)", November 26, 2001.

[33]	NIST Special Publication 800-38A, "Recommendation for Block Cipher Modes of Operation Methods and Techniques", 2001.

[34]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[35]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[36]	3GPP TS 38.215: "NR; Physical layer measurements".

[37]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[38]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, Aug 2017.

[39]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017.

[40]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[41]	3GPP TS 38.211: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical channels and modulation".

[42]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[43]	IS-QZSS-L6-001, Quasi-Zenith Satellite System Interface Specification – Centimetre Level Augmentation Service, Cabinet Office, November 5, 2018.

[44]	3GPP TR 38.901: "Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz".

[45]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[46]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[47]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[xx]	3GPP TS 38.213: “NR; Physical layer procedures for control”.

===============================NEXT CHANGE=========================================

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 36.305 [2], TS 23.271 [3], 38.305 [40] and TS 23.273 [42] apply. Other definitions are provided below.

Anchor carrier: In NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Location Server: a physical or logical entity (e.g., E-SMLC, SUPL SLP, or LMF) that manages positioning for a target device by obtaining measurements and other location information from one or more positioning units and providing assistance data to positioning units to help determine this. A Location Server may also compute or verify the final location estimate.

NB-IoT: NB-IoT allows access to network services via E-UTRA with a channel bandwidth limited to 200 kHz.

Observed Time Difference Of Arrival (OTDOA): The time interval that is observed by a target device between the reception of downlink signals from two different TPs. If a signal from TP 1 is received at the moment t1, and a signal from TP 2 is received at the moment t2, the OTDOA is t2 – t1.

PRS-only TP: A TP which only transmits PRS signals for PRS-based TBS positioning or DL-PRS and is not associated with a cell.

Reference Source: a physical entity or part of a physical entity that provides signals (e.g., RF, acoustic, infra-red) that can be measured (e.g., by a Target Device) in order to obtain the location of a Target Device.

Relative Time Difference (RTD): The relative time difference between a TRP i and a TRP j, is defined as tj – ti, where ti and tj are defined as the time when TRP i and j transmit the start of one subframe respectively.

Target Device: the device that is being positioned (e.g., UE or SUPL SET).

Transmission Point (TP): A set of geographically co-located transmit antennas (e.g. antenna array (with one or more antenna elements)) for one cell, part of one cell or one PRS-only TP. Transmission Points can include base station (eNodeB) antennas, remote radio heads, a remote antenna of a base station, an antenna of a PRS-only TP, etc. One cell can be formed by one or multiple transmission points. For a homogeneous deployment, each transmission point may correspond to one cell.

Transmission-Reception Point (TRP): A set of geographically co-located antennas (e.g. antenna array (with one or more antenna elements)) supporting TP and/or RP functionality.

==========================NEXT CHANGE============================================

### 6.4.3	Common NR Positioning Information Elements

#### –	NR-DL-PRS-AssistanceData

The IE NR-DL-PRS-AssistanceData is used by the location server to provide DL-PRS assistance data.

NOTE 1:	The location server should include at least one TRP for which the SFN can be obtained by the target device, e.g. the serving TRP.

NOTE 2:	The nr-DL-PRS-ReferenceInfo defines the "assistance data reference" TRP whose DL-PRS configuration is included in nr-DL-PRS-AssistanceDataList. The nr-DL-PRS-SFN0-Offset's and nr-DL-PRS-expectedRSTD's in nr-DL-PRS-AssistanceDataList are provided relative to the "assistance data reference" TRP.

NOTE 3:	The network signals a value of zero for the nr-DL-PRS-SFN0-Offset, nr-DL-PRS-expectedRSTD, and nr-DL-PRS-expectedRSTD-uncertainty of the "assistance data reference" TRP in nr-DL-PRS-AssistanceDataList.

NOTE 4:	For NR DL-TDOA positioning (see clause 6.5.10) the nr-DL-PRS-ReferenceInfo defines also the requested "RSTD reference".

-- ASN1START

NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-ReferenceInfo-r16 			DL-PRS-ID-Info-r16,

nr-DL-PRS-AssistanceDataList-r16		SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-AssistanceDataPerFreq-r16,

nr-SSB-Config-r16					SEQUENCE (SIZE (1..nrMaxTRPs-r16)) OF

NR-SSB-Config-r16	OPTIONAL,	-- Need ON

...

}

NR-DL-PRS-AssistanceDataPerFreq-r16 ::= SEQUENCE {

nr-DL-PRS-PositioningFrequencyLayer-r16

NR-DL-PRS-PositioningFrequencyLayer-r16,

nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-AssistanceDataPerTRP-r16,

...

}

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15						OPTIONAL,	-- Need ON

nr-ARFCN-r16						ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-DL-PRS-SFN0-Offset-r16		NR-DL-PRS-SFN0-Offset-r16,

nr-DL-PRS-ExpectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-ExpectedRSTD-Uncertainty-r16

INTEGER (0..246),

nr-DL-PRS-Info-r16				NR-DL-PRS-Info-r16,

... ,

[[

prs-OnlyTP-v16xy				ENUMERATED { true }		OPTIONAL		-- Need ON

]]

}

NR-DL-PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE {

dl-PRS-SubcarrierSpacing-r16		ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...},

dl-PRS-ResourceBandwidth-r16		INTEGER (1..63),

dl-PRS-StartPRB-r16				INTEGER (0..2176),

dl-PRS-PointA-r16				ARFCN-ValueNR-r15,

dl-PRS-CombSizeN-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-CyclicPrefix-r16			ENUMERATED {normal, extended, ...},

...

}

NR-DL-PRS-SFN0-Offset-r16 ::= SEQUENCE {

sfn-Offset-r16					INTEGER (0..1023),

integerSubframeOffset-r16		INTEGER (0..9),

...}

-- ASN1STOP

NR-DL-PRS-AssistanceData field descriptions

nr-DL-PRS-ReferenceInfo This field specifies the IDs of the assistance data reference TRP.

nr-DL-PRS-AssistanceDataList This field specifies the DL-PRS resources for each frequency layer.

nr-SSB-Config This field specifies the SSB configuration of the TRPs.

nr-DL-PRS-PositioningFrequencyLayer This field specifies the Positioning Frequency Layer for the nr-DL-PRS-AssistanceDataPerFreq field.

nr-DL-PRS-AssistanceDataPerFreq This field specifies the DL-PRS Resources for the TRPs within the Positioning Frequency Layer.

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resource ID to uniquely identify a DL-PRS Resource, and is associated with a single TRP.

nr-PhysCellID This field specifies the physical cell identity of the TRP. When the field prs-OnlyTP is included, this field shall be absent.

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, as defined in TS 38.331 [35]. When the field prs-OnlyTP is included, this field shall be absent.

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. When the field prs-OnlyTP is included, this field shall be absent.

nr-DL-PRS-SFN0-Offset This field specifies the time offset of the SFN#0 slot#0 for the given TRP with respect to SFN#0 slot#0 of the assistance data reference TRP and comprises the following subfields: - sfn-Offset specifies the SFN offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the assistance data reference TRP to the beginning of the closest subsequent radio frame #0 of this neighbour TRP. - integerSubframeOffset specifies the frame boundary offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP counted in full subframes. The offset is counted from the beginning of a subframe #0 of the assistance data reference TRP to the beginning of the closest subsequent subframe #0 of this neighbour TRP, rounded down to multiples of subframes.

nr-DL-PRS-ExpectedRSTD This field indicates the RSTD value that the target device is expected to measure between this TRP and the assistance data reference TRP. The nr-DL-PRS-ExpectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of PRS positioning occasions between the two TRPs. The resolution is 4Ts, with Ts=1/(15000*2048) seconds.

nr-DL-PRS-ExpectedRSTD-Uncertainty This field indicates the uncertainty in nr-DL-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimate of the target device location. The nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device. The resolution R is - Ts if all PRS resources are in frequency range 2, - 4Ts otherwise, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the subframe for the PRS of this TRP is received within the search window of size - [-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts, where TREF is the reception time of the beginning of the subframe for the PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on - nr-DL-PRS-SFN0-Offset - dl-PRS-Periodicity-and-ResourceSetSlotOffset - dl-PRS-ResourceSlotOffset.

nr-DL-PRS-Info This field specifies the PRS configuration of the TRP.

dl-PRS-SubcarrierSpacing This field specifies the subcarrier spacing of the DL-PRS Resource. 15, 30, 60 kHz for FR1; 60, 120 kHz for FR2.

dl-PRS-ResourceBandwidth This field specifies the number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. All DL-PRS Resources of the DL-PRS Resource Set have the same bandwidth. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of DL-PRS Bandwidth and Start PRB. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-PRS-StartPRB This field specifies the start PRB index defined as offset with respect to reference DL-PRS Point A for the Positioning Frequency Layer.

dl-PRS-PointA This field specifies the absolute frequency of the reference resource block for the DL-PRS. Its lowest subcarrier is also known as DL-PRS Point A. A single DL-PRS Point A for DL-PRS Resource allocation is provided per Positioning Frequency Layer. All DL-PRS Resources belonging to the same DL-PRS Resource Set have the same DL-PRS Point A.

dl-PRS-CombSizeN This field specifies the Resource Element spacing in each symbol of the DL-PRS Resource. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of comb size N.

dl-PRS-CyclicPrefix This field specifies the Cyclic Prefix length of the DL-PRS Resource.

prs-OnlyTP This field, if present, indicates that the NR-DL-PRS-AssistanceData is provided for a PRS-only TP. Whether the field is present or absent should be the same for all the NR-DL-PRS-AssistanceData of all the PRS transmitted under the same TP. The target device shall not assume that any other signals or physical channels are present for the TRP other than DL-PRS.

==================================NEXT CHANGE==================================

#### –	NR-SSB-Config

The IE NR-SSB-Config defines SSB configuration.

-- ASN1START

NR-SSB-Config-r16 ::= SEQUENCE {

nr-PhysCellID-r16					NR-PhysCellID-r16,

nr-ARFCN-r16							ARFCN-ValueNR-r15,

ss-PBCH-BlockPower-r16				INTEGER (-60..50),

halfFrameIndex-r16					INTEGER (0..1),

ssb-periodicity-r16					ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, ...},

ssb-PositionsInBurst-r16				CHOICE {

shortBitmap-r16						BIT STRING (SIZE (4)),

mediumBitmap-r16						BIT STRING (SIZE (8)),

longBitmap-r16						BIT STRING (SIZE (64))

}																			OPTIONAL, --Need OR

ssb-SubcarrierSpacing-r16			ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, ...},

sfn-SSB-Offset-r16					INTEGER (0..15),

...

}

-- ASN1STOP

NR-SSB-Config field descriptions

nr-ARFCN This field specifies the ARFCN of the first RE of SSB's RB#10.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [xx], clause 7.

halfFrameIndex Indicates the 5 msec offset of the SSB within a 10 msec system frame.

ssb-periodicity The SSB periodicity in ms for the rate matching purpose.

ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [xx], clause 4.1. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

ssb-SubcarrierSpacing Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz (FR1), and 120 kHz or 240 kHz (FR2) are applicable.

sfn-SSB-Offset Indicates the 10 msec system frame offset of the SSB within the SSB periodicity. Value 0 indicates that the SSB is transmitted in the first system frame; 1 indicates that the SSB is transmitted in the second system frame and so on. This field shall be configured according to the field ssb-Periodicity and the indicated system frame shall not exceed the configured SSB periodicity.

====================================NEXT CHANGE====================================

#### –	NR-TimeStamp

The IE NR-TimeStamp defines the UE measurement associated time stamp.

-- ASN1START

NR-TimeStamp-r16 ::= SEQUENCE {

dl-PRS-ID-r16				INTEGER (0..255),

nr-PhysCellID-r16			NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16			NCGI-r15						OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-SFN-r16					INTEGER (0..1023),

nr-Slot-r16 					CHOICE {

scs15-r16				INTEGER (0..9),

scs30-r16				INTEGER (0..19),

scs60-r16				INTEGER (0..39),

scs120-r16				INTEGER (0..79)

},

...

}

-- ASN1STOP

NR-TimeStamp field descriptions

dl-PRS-ID This field specifies the DL-PRS ID of the TRP for which the nr-SFN is applicable.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID associated with the dl-PRS-ID.

nr-SFN This field specifies the NR system frame number for the time stamp.

nr-Slot This field specifies the NR slot number within the NR system frame number indicated by nr-SFN for the time stamp.

====================================NEXT CHANGE====================================

#### 6.5.9.4	NR E-CID Capability Information

#### –	NR-ECID-ProvideCapabilities

The IE NR-ECID-ProvideCapabilities is used by the target device to indicate its capability to support NR E-CID and to provide its NR E-CID positioning capabilities to the location server.

-- ASN1START

NR-ECID-ProvideCapabilities-r16 ::= SEQUENCE {

nr-ECID-MeasSupported-r16		BIT STRING {	ssrsrpSup		(0),

ssrsrqSup		(1),

csirsrpSup		(2),

csirsrqSup		(3)} (SIZE(1..8)),

periodicalReporting-r16			ENUMERATED { supported }							OPTIONAL,

triggeredReporting-r16			ENUMERATED { supported }							OPTIONAL,

...

}

-- ASN1STOP

NR-ECID-ProvideCapabilities field descriptions

nr-ECID-MeasSupported: Indicates the supported NR ECID measurements: - ssrsrpSup indicates the UE supports SSB based cell/beam specific RSRP measurement; - ssrsrqSup indicates the UE supports SSB based cell/beam specific RSRQ measurement; - csirsrpSup indicates the UE supports CSI-RS based cell/beam specific RSRP measurement; - csirsrqSup indicates the UE supports CSI-RS based cell/beam specific RSRQ measurement.

==================================END OF CHANGES==================================