3GPP TSG-RAN WG2 Meeting #115-e	R2-2109586

eLocation,  eLocation, 9 – 27th August 2021	rev R2-2108829

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | NA | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support Non-Terrestrial Networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-core |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of Non-Terrestrial Networks support |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of specific NTN vocabulary, architecture and protocols aspects |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN is not supported in NR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4.x and 16.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: Stage 2 runningCR creation based on email discussion in R2-2010781 (RAN2#112-e), RAN2#113-e outcomes (chairman’s notes) and RAN3#111- outcomes (R2-2102057). RAN2#113bis-e & 114-e outcomes (R2-2106539). RAN2#115-e outcomes (R2-2109586) This stg2 Running CR will completed with the RAN3 stg2 BL CR when completed. For information the current endorsed RAN3 stg2 BL CR can be found in R3-214514 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Changes Begin >>>>>>>>>>>>>>>>>>>>

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

GFBR	Guaranteed Flow Bit Rate

GSO	Geo Synchronous Orbit

HRNN	Human-Readable Network Name

HAPS	High Altitude Platform Station

IAB	Integrated Access and Backhaul

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

LDPC	Low Density Parity Check

LEO	Low Earth Orbit

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MT	Mobile Termination

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

NB-IoT	Narrow Band Internet of Things

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NGSO	Non-Geo Synchronous Orbit

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

NTN	Non-Terrestrial Network

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

UCI	Uplink Control Information

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

BH RLC channel: an RLC channel between two nodes, which is used to transport backhaul packets.

CAG Cell: a PLMN cell broadcasting at least one Closed Access Group identity.

CAG Member Cell: for a UE, a CAG cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN, and for that PLMN, a CAG identifier belonging to the Allowed CAG list of the UE for that PLMN.

CAG-only cell: a CAG cell that is only available for normal service for CAG UEs.

Cell-Defining SSB: an SSB with an RMSI associated.

Child node: IAB-DU's and IAB-donor-DU's next hop neighbour node; the child node is also an IAB-node.

Conditional Handover (CHO): a handover procedure that is executed only when execution condition(s) are met.

CORESET#0: the control resource set for at least SIB1 scheduling, can be configured either via MIB or via dedicated RRC signalling.

DAPS Handover: a handover procedure that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB.

Downstream: Direction toward child node or UE in IAB-topology.

Early Data Forwarding: data forwarding that is initiated before the UE executes the handover.

Feeder link: [to be provided by the RAN3 stg2 BL CR].

Geo Synchronous Orbit: Earth-centered orbit at 35786 kilometres above Earth's equator and synchronised with Earth's rotation.

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

High Altitude Platform Station: airborne vehicle embarking the NTN payload placed at an altitude between 8 and 50 km.

IAB-donor: gNB that provides network access to UEs via a network of backhaul and access links.

IAB-donor-CU: as defined in TS 38.401 [4].

IAB-donor-DU: as defined in TS 38.401 [4].

IAB-DU: gNB-DU functionality supported by the IAB-node to terminate the NR access interface to UEs and next-hop IAB-nodes, and to terminate the F1 protocol to the gNB-CU functionality, as defined in TS 38.401 [4], on the IAB-donor.

IAB-MT: IAB-node function that terminates the Uu interface to the parent node using the procedures and behaviours specified for UEs unless stated otherwise. IAB-MT function used in 38-series of 3GPP Specifications corresponds to IAB-UE function defined in TS 23.501 [3].

IAB-node: RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes. The IAB-node does not support backhauling via LTE.

Intra-system Handover: Handover that does not involve a CN change (EPC or 5GC).

Inter-system Handover: Handover that involves a CN change (EPC or 5GC).

Late Data Forwarding: data forwarding that is initiated after the source NG-RAN node knows that the UE has successfully accessed a target NG-RAN node.

MSG1: preamble transmission of the random access procedure for 4-step random access (RA) type.

MSG3: first scheduled transmission of the random access procedure.

MSGA: preamble and payload transmissions of the random access procedure for 2-step RA type.

MSGB: response to MSGA in the 2-step random access procedure. MSGB may consist of response(s) for contention resolution, fallback indication(s), and backoff indication.

Multi-hop backhauling: Using a chain of NR backhaul links between an IAB-node and an IAB-donor.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Non-CAG Cell: a PLMN cell which does not broadcast any Closed Access Group identity.

Non-Geo Synchronous orbit: Earth-centered orbit with an orbital period that does not match Earth's rotation on its axis. This includes Low and Medium Earth Orbit (LEO and MEO).

Non-terrestrial networks: [to be provided by the RAN3 stg2 BL CR].

NR backhaul link: NR link used for backhauling between an IAB-node and an IAB-donor, and between IAB-nodes in case of a multi-hop backhauling.

NR sidelink communication: AS functionality enabling at least V2X communication as defined in TS 23.287 [40], between two or more nearby UEs, using NR technology but not traversing any network node.

NTN-gateway: [to be provided by the RAN3 stg2 BL CR].

NTN payload: [to be provided by the RAN3 stg2 BL CR].

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

Parent node: IAB-MT's next hop neighbour node; the parent node can be IAB-node or IAB-donor-DU

PLMN Cell: a cell of the PLMN.

Satellite: a space-borne vehicle orbiting the Earth embarking the NTN payload.

Service link: Wireless link between the NTN payload and UE.

SNPN Access Mode: mode of operation whereby a UE only accesses SNPNs.

SNPN-only cell: a cell that is only available for normal service for SNPN subscribers.

SNPN Identity: the identity of Stand-alone NPN defined by the pair (PLMN ID, NID).

Upstream: Direction toward parent node in IAB-topology.

V2X sidelink communication: AS functionality enabling V2X communication as defined in TS 23.285 [41], between nearby UEs, using E-UTRA technology but not traversing any network node.

Xn: network interface between NG-RAN nodes.

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

## 4.x	Non-Terrestrial Networks overview

[to be provided by the RAN3 stg2 BL CR]

Three types of service links are supported.

  - Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites)

  - Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams)

  - Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

Non-Geo Synchronous orbit (NGSO) includes Low Earth Orbit at altitude approximately between 300 km and 1500 km and Medium Earth Orbit at altitude approximately between 1500 km and 35786 km.

With NGSO satellites, the gNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while gNB operating with GSO satellite can provide Earth fixed cell coverage.

In this release, the UE supporting NTN is GNSS-capable.

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

## 16.x	Non-Terrestrial Networks

### 16.x.1 Overview

Support for Non-Terrestrial Networks (NTNs) is facilitated by the mechanisms described in the following clauses.

### 16.x.2 User Plane aspects

Editor’s note: Impact on timing aspects to be added once the RAN1 has drafted a text on Koffset and Kmac principles (a figure illustrating the timing relationship between UL and DL would be beneficial)

To accommodate the long propagation delays, some adaptions are done for physical layer and higher layers. For example UL HARQ retransmission can be enable/disable per HARQ process”.

Editor’s note: A description of the UE autonomous TA pre-compensation and frequency pre-compensation will be added once RAN1 have drafted a text.

The UEs may be configured to report information about the UE specific Timing Advance pre-compensation. This can be used by the gNB to estimating the Timing Advance used by the UE and adapt scheduling timing in order to minimize the uplink delay.

During Random Access procedures, if configured by the network (system information), the UE reports information about the UE specific Timing Advance pre-compensation.

The UE may be configured to reports information about UE specific TA pre compensation in connected mode.  If Event-triggering is used, the reporting can be triggered based based on the change of the TA value.

Editor’s note: UE may also reports its UE location

For downlink, HARQ feedback can be enabled or disabled.

For dynamic grants, RRC can configure semi-statically  an [UL HARQ retransmission state] per LCH and per HARQ process, based on whether UL scheduling based on UL transmission decoding results is expected (considering delay and reliability characteristics of on going services).

Based on configured [UL HARQ retransmission state], UE determines timing behaviour per HARQ process.

Editor’s note: FFS whether “UL HARQ retransmission state” is the most appropriate name. FFS on mapping method.

### 16.x.3 System information

In this clause, only NTN specific system information handling is defined, else the clause in 7.3 applies.

Editor’s note: The discussion on whether to introduce a new SIB for NTN has been postponed until more progress is made on the content of NTN specific system information.

The network may broadcast more than one Tracking Area Code (TAC) per PLMN in a cell.

Editor’s note: UE determination of the tracking area using other information than the broadcast TAC(s) is not excluded.

TACs in NTN are fixed to geographical location on Earth.

Editor’s note: UE’s location information can be used for TAI selection..

The network provides a validity time for the TAC broadcasted

A TAC change in SI is under network control, i.e. it may not exactly sync up with real-time illumination on ground.

Editor’s note: FFS whether broadcasting TAC update time can also be considered.

In the quasi-Earth fixed cell scenario, timing and location information are provided via system information (e.g. reference location of serving or neighboring cells and when a given cell is going to stop serving a geographical area.

### 16.x.4 Mobility and State transition

#### 16.x.4.1 Mobility in RRC_IDLE

The same principles as described in 9.2.1 apply to cell selection and reselection in NTN except for what is described below:

-	An NTN UE may perform cell selection and reselection based on the NTN (satellite/HAPS) ephemeris provisioned.

Editor’s note: FFS on the definition of satellite/HAPS ephemeris. FFS under what circumstance UE will perform cell selection/reselection based on satellite/HAPS ephemeris.

Editor’s note: For quasi Earth fixed, location assisted cell reselection can be triggered based on the distance between UE and the reference location of the cell (serving cell and/or neighbor cell) if UE is aware of its location.

The UE is made aware that a cell is provided by NTN.

Editor’s note: FFS on whether to make the UE aware of NTN in an implicit or explicit way.

For cell reselection, based on configuration criteria, an NTN UE may prioritise a network type (e.g., TN or NTN).

Editor’s note: Configuration details are FFS. FFS the exact solution for TN priorization over NTN for idle mode

The NTN ephemeris is divided into serving cell’s ephemeris and neighbour’s ephemeris.

Editor’s note: The NTN ephemeris can be provided through pre-configuration in uSIM, NAS, SIB and RRC signaling.

At least in the quasi-earth fixed cell scenario, the timing information associated to a cell is used to assist cell reselection in NTN to decide when to perform measurement on neighbor cells (i.e. when to search for neighbor cells).

UE is not expected to do TAU if one of the currently broadcasted TACs belongs to UE’s registration area.

#### 16.x.4.2 Mobility in RRC_CONNECTED

##### 16.x.4.2.1 Overview

In the case of NGSO, service link switch refers to a change of serving satellite.

##### 16.x.4.2.2 Handover

The same principle as described in 9.2.3.2 applies to NTN handover except for what is described below:

During mobility between NTN and Terrestrial Network, a UE is not required to connect to both NTN and Terrestrial Network at the same time.

##### 16.x.4.2.3 Conditional Handover

The same principle as described in 9.2.3.4 applies to intra-NTN conditional handover except for what is described below:

Measurement event A4 is supported for CHO.

Editor’s note: Any deviation on existing CHO operation (e.g. wrt Joint configuration of triggers is FFS.

Additional CHO events are defined for NTN are:

- ’Time-based event’, where time window is configured, defining when the UE is allowed to execute CHO

- ’Location-based event’, where a distance threshold is configured, defining where the UE should execute CHO. The distance threshold corresponds to a distance between the UE and a reference location of the cell (i.e. serving/source cell and/or target cell), where this reference location is defined as coordinates provided by the network.

NOTE: These time-based or location-based events must be configured jointly with A3, A4 or A5 measurement events.

The same CHO trigger conditions and RRM events can be used for intra-NTN or NTN-Terrestrial Network mobility provided these are supported by the UE. NTN-Terrestrial Network mobility refers to both directions, i.e. from NTN to Terrestrial Network (hand-in) and from Terrestrial Network to NTN (hand-out).

Editor’s note: FFS for enhancements of CHO trigger conditions and/or RRM events.

#### 16.x.4.3 Measurements

The same principle as described in 9.2.4 applies to measurements in NTN.

Editor’s note: For NTN, the location-based measurement event, in combination with the existing measurement event in NR, is supported. FFS on how to configure the location based measurement event.

Editor’s note: location report is piggybacked with measurement report when the location based event triggers it.

The UE can be configured with multiple SMTCs per carrier.

The UE shall report upon network configuration or request, assistance information for the network to calculate the offset for SMTC/GAP configurations.

SMTC configuration and UE measurement gap can be configured per channel and for a given set of cells (e.g., per satellite or any other suitable set per gNB determination) to accommodate the larger propagation delays due to altitude/ minimum elevation.  The configuration of one or multiple offsets (i.e. to support multiple SMTC configuration) and its updates is under network control.

Editor’s note:

- FFS new UE assistance for network to properly (re)configure the SMTC and/or measurement gap.

- FFS the following open questions:

(a) can the UE be configured with multiple SMTCs per carrier and use them all in parallel?

(b) How the NW knows which SMTC (incl. offsets/periodicity, etc.) is relevant for a particular UE?

(c) Is there any validity: in time or for certain location only, foreseen in such multiple SMTC configuration?

(d) What is the potential impact on the signalling, assuming this delay is a dynamic value?

(e) What about the feeder link delay? Is it considered anywhere?

### 16.x.5	Switch over

#### 16.x.5.1 Definitions [to be provided by the RAN3 stg2 BL CR]

#### 16.x.5.2 Assumptions [to be provided by the RAN3 stg2 BL CR]

#### 16.x.5.3 Operations [FFS] [to be provided by the RAN3 stg2 BL CR]

#### 16.x.5.4 Procedures [FFS] [to be provided by the RAN3 stg2 BL CR]

### 16.x.6	Signalling [to be provided by the RAN3 stg2 BL CR]

### 16.x.7	O&M Requirements [to be provided by the RAN3 stg2 BL CR]

### 16.x.8	UE location aspects

Editor’s note: The network is able to ensure that the CGI constructed by NG-RAN corresponds to a fixed geographical area comparable with a TN cell with a radius of ~2km or more.

Editor’s note: During initial access, the UE can report its location in Msg5. This location information corresponds to a coarse GNSS coordinates of ~2 km accuracy

After AS security is established, gNB can obtain a GNSS-based location information from the UE using existing signalling method,

Periodic location reporting can be configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

# Annex B.x	Example implementation of Non-Terrestrial Networks (informative) [to be provided by the RAN3 stg2 BL CR]

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

The following appendices shall be removed from final CR

# A	Appendix: RAN2 agreements for WI NR-NTN-solutions

## A.1 Scope, requirements, scenarios, architecture

RAN2#111-e Agreements

RAN2 to stick to WI scenarios: Any restriction, e.g. on the LEO altitude (if needed) could come from other groups.

From RAN2 perspective, the table 4.2-2 of [TR 38.821] is used as a baseline for the normative work, with the removal of the regenerative payload option

(as the WI is restricted to transparent payload) we assume that the feeder link will use NR (how the satellite is controlled is out of the scope of the WI)

RAN2 confirms the assumptions on the UE ground speed in the handheld and VSAT cases

| User equipment characteristics | Handheld | VSAT (Note 1) |
| --- | --- | --- |
| Motion on the earth | Up to 500 km/h (e.g. on board a high speed train) | Up to 1200 km/h (e.g. aircraft mounted) |

In Rel-17, only UEs with GNSS capabilities are supported

Both Earth fixed and earth moving beam scenarios are considered with NGSO constellation.

Both soft and hard feeder link switchover (e.g. for Non GSO) are supported.

Note: This requires satellite to be connected to one NTN GW at a time (hard switch) or at least two NTN GWs simultaneously (soft switch).

RAN2 to start discussing enhancements for soft feeder link switchover and then solutions for hard feeder link switchover.

As part of the NR-NTN WI, the following stepped approach is proposed:

- Step 1: Assessment of the Rel-16 LCS framework/application protocols (3GPP TS 23.273, TS 29.572, TS 38.455, TS 38.305, in particular but not excluding other TS) and its applicability to NTN

- Step 2: Assess whether changes to the existing network-based location methods are needed and define them if needed

The NTN network based positioning of UE should provide an accuracy comparable with the network based UE location accuracy of terrestrial networks.

For TN/NTN mobility, the UE is not required to connect to both TN and NTN at the same time.

RAN2 to discuss about trigger(s) of TN / NTN mobility, once the Intra NTN mobility has sufficiently progressed. Intra NTN mobility refers to idle and connected mode mobility between NTN cells (e.g. intra or inter satellite).

Transparent HAPS is assumed with the IMT BS on the ground and the HAPS is a relay.

The RAN2 work plan described in R2-2007565 should be considered as a basis for work

The work plan should be based on the following prioritization principles:

- 1st priority: user plane, control plane (idle and connected)

- 2nd priority: NTN-TN service continuity, network based UE location

Comeback to the discussion on capability of UE to support TN / NTN mobility in the future

## A.2 User plane – RACH & MAC aspects

RAN2#115-e Agreements

1.	UE specific TA reporting during RACH procedure is enabled/disabled by SI (FFS for RACH in connected mode)

1. In the MAC specification section 5.1.5, delay the start of ra-ContentionResolutionTimer by the UE-gNB RTT (i.e. sum of UE's TA and K_mac)

1.	The content of UE specific TA pre-compensation reported in RA procedure using MAC CE is UE specific TA (this can be revisited after receiving RAN1 response).

2.	Reporting on the information about UE specific TA in connected mode is supported, FFS via RRC signalling or MAC CE

3.	Event-triggers for reporting on the information about UE specific TA in connected mode is supported. FFS on the details. Confirmation by RAN1 is also needed

4.	If configured, the UE shall report information of the UE specific TA pre-compensation to the target cell during the random access. FFS if a new indication in RRC reconfiguration with sync is needed or not (besides the SIB indication carried in HO command on whether TA report is enabled/disabled in the target cell).

5.	Information about UE specific TA pre-compensation is not reported in RA procedures triggered due to “Request for Other SI”

1.	The event-triggers for reporting information about UE specific TA are based on TA values (confirmation from RAN1 is needed)

2.	A TA offset threshold can be used for event-triggered reporting, at least the offset threshold can be between current information about UE specific TA and the last successfully reported information about UE specific TA

3.	The event-triggers for reporting information about UE specific TA based on time threshold is not supported in NTN.

4.	No new indication in RRC reconfiguration with sync is needed to configure the UE to report information about UE specific TA in handover procedure (besides the SIB indication carried in HO command on whether TA report is enabled/disabled in the target cell).

1.	Under the work assumption "the UE location information cannot be reported in connected mode", the content of UE specific TA reported in connected mode is UE specific TA pre-compensation(for the details of the TA value, confirmation from RAN1 is needed).

2.	If the reported content of information about UE specific TA is UE location information in connected mode, RRC signalling is used to report.

1.	Under the work assumption "the UE location information can be reported in connected mode", for TA reporting purposes in connected mode, the network can configure the UE to send either the UE specific TA pre-compensation (for the details of the TA value, confirmation from RAN1 is needed) or the UE location information

Working Assumption: If the reported content of information about UE specific TA is TA pre-compensation value in connected mode, MAC CE is used to report

1.	Confirm the RAN2 working assumption that offset to drx-HARQ-RTT-TimerUL length is equal to UE-gNB RTT (i.e. sum on UE's TA and K_mac).

2.	Confirm the RAN2 working assumption that for HARQ processes with DL HARQ feedback enabled, the drx-HARQ-RTT-TimerDL length is increased by an offset equal to UE-gNB RTT (i.e. sum on UE's TA and K_mac).

3.	No new LCP restrictions are introduced for exisiting UL MAC CEs (if new MAC CEs will be introduced we can revisit this)

4.	For dynamic grants, each LCH can optionally be semi statically configured (by RRC) to be mapped to one or more HARQ processes (FFS if it's possible to map to more than one HARQ process/ process type. FFS on mapping method). If there is no RRC configuration for this, this mapping has no effect (legacy behaviour applies).

1a.	For at least dynamic grants, the network may optionally configure an UL HARQ retransmission state per HARQ process. Two UL HARQ retransmission states are defined in NTN: HARQ state A and HARQ state B (FFS whether "HARQ state A" and "HARQ state B" should be renamed)

1b.	HARQ state A/B are defined as follows:

-	HARQ state A: length of drx-HARQ-RTT-TimerUL is extended by UE-gNB RTT (i.e. UE PDCCH monitoring is optimized to support UL retransmission grant based on UL decoding result).

-	HARQ state B:  drx-HARQ-RTT-TimerUL is not started.

2.	Configuration of UL HARQ retransmission state is semi-static, signalled via RRC, and the decision and criteria to configure UL HARQ retransmission state is under network control.

3.	For dynamic grants, each LCH can be optionally mapped to an UL HARQ retransmission state via semi-static RRC configuration. If there is no configuration, the mapping has no effect (legacy behaviour applies).

4.	If HARQ process has not been configured with an UL HARQ retransmission state, new LCH mapping rule has no effect (i.e. UE applies legacy behaviour).

5.	The following behaviours are supported for drx-HARQ-RTT-TimerUL in NTN per HARQ process: 1) Timer length is extended by offset; 2) Timer disabled (i.e. not started)

6.	UE determines drx-HARQ-RTT-TimerUL behaviour per HARQ process based on configured UL HARQ retransmission state.

7.	For HARQ process(es) not configured with an UL HARQ retransmission state, drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL behave as per legacy.

1.	An UL HARQ retransmission state is configured per HARQ process to support new LCH mapping restriction and proper configuration of drx-HARQ-RTT-TimerUL behaviour.

2.	The network may consider delay and reliability characteristics of ongoing services when choosing to configure an UL HARQ retransmission state.

3.	Alternative naming for HARQ state A/B can be further considered during stage 3, however UE behaviour in each state should be defined in specification.

4.	RAN2 understanding is that UE behaviour in HARQ state A (i.e. extending the drx-HARQ-RTT-TimerUL by UE-gNB RTT) best supports reception of UL retransmission grant based on UL decoding result. (No RAN2 specification impact)

5.	RAN2 understanding is that UE behaviour in HARQ state B (i.e. not starting drx-HARQ-RTT-TimerUL) best supports no UL retransmission and/or blind UL retransmission. (No RAN2 specification impact)

1.	For HARQ state B, FFS to run drx-RetransmissionTimerUL for blind UL retransmission

2.	UE configured with an UL HARQ retransmission state (i.e. A or B) will always act as indicated in a grant/assignment provided during a valid occasion (i.e. subject to legacy restrictions in e.g. MAC and RAN1 specifications). (No RAN2 specification impact)

RAN2#114-e Agreements

If enabled by the network, the UE reports information about UE specific TA pre-compensation at the random access procedure (MSGA/MSG3 or MSG5) using a MAC CE. Actual content is FFS and also depends on further RAN1 input (we can revise this whole agreement if RAN1 come to a different conclusion in terms of what needs to be conveyed to the NW)

RAN2#113-bis-e Agreements

Legacy mechanism for RA type selection based on RSRP threshold is the baseline for NTN. Optimizations can still be suggested, showing the gain (in any case, any method needs to be combined with RSRP based approach)

Reuse legacy RA type switching mechanism

Extend the timer length of sr-ProhibitTimer (FFS on the details).

RAN2 wait for RAN1’s feedback on UE obtaining UE-gNB RTT

RAN2 wait for RAN1’s progress and postpone the discussion on how to broadcast parameters, if any, for TA pre-compensation.

RAN2 send an LS to RAN1, focusing on below aspects:

-	Ask RAN1 to prioritize the TA pre-compensation work on whether and/or what parameters to broadcast for TA pre-compensation, and when broadcasted, how often the broadcasted parameters are expected to change over time;

-	RAN2 has agreed to use UE-gNB RTT as the offset to start some UP timers (e.g. drx-HARQ-RTT-TimerDL). Ask RAN1 to provide inputs on (i) how UE acquires UE-gNB RTT and (ii) what additional information needs to be broadcasted other than that for TA pre-compensation, if any.

At least for uplink scheduling adaptations, the UE may report information about the UE specific TA pre-compensation. The exact information and frequency of reports depend on RAN1 outcome. FFS on when/how to report.

The UE reports the UE specific TA pre-compensation during RACH procedure using MAC CE (FFS if this needs to be configured). Actual content is FFS and also depends on further RAN1 input.

It is NW scheduling strategy to avoid NTN UE in HARQ stalling state. From RAN2 perspective, the NW can continuously schedule the UE using one or a combination of scheduling strategies, such as without HARQ retransmissions, or with blind retransmissions, or with HARQ retransmissions based on DL HARQ feedback (or UL decoding result).

RAN2 confirms that in NTN if the UE is in DRX Active Time for any reason, the UE should monitor the PDCCH regardless of whether drx-HARQ-RTT-TimerUL or drx-HARQ-RTT-TimerDL is running or not. No specification change is needed.

RAN2 confirms that in NTN using the value= “zero” for drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL is possible. No specification change is needed.

In NTN, The drx-HARQ-RTT-TimerUL is configured per UE DRX group and the behaviour can be configured per HARQ process. FFS the different behaviours and how to indicate the behaviour to the UE and the number of behaviours (e.g., two or more behaviours).

LCP restrictions should be further considered for an UL HARQ process in NTN. FFS if no further LCP restrictions are needed, or if (R16) existing LCP restrictions can be re-used or if new LCP restriction shall be defined for this purpose.

RAN2#113-e Agreements

Both Type 1 and Type 2 configured grant are feasible in NTN.

From RAN2’s perspective, no need to modify parameter periodicity of IE ConfiguredGrantConfig to support NTN.

No need to modify maxNrofConfiguredGrantConfig-r16 and maxNrofConfiguredGrantConfigMAC-r16 to support NTN.

UE in NTN can have both 2-step RACH and configured grant configurations at the same time.

For HARQ processes with DL HARQ feedback disabled, drx-HARQ-RTT-TimerDL is not started.

FFS: method(s) to support blind retransmission for HARQ processes with HARQ feedback disabled.

From RAN2 perspective, for HARQ processes where gNB can sends UL grant without waiting for decoding result of previous PUSCH transmission, no new network scheduling restrictions are introduced to schedule subsequent grants (i.e. up to network implementation. (Can come back if we don't find an agreement on p8)

For HARQ processes with DL HARQ feedback enabled, drx-HARQ-RTT-TimerDL length is increased by offset (i.e. existing values within value range increased by offset). RAN2 working assumption: offset is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it)

RAN2#112-e Agreements

RAN2 working assumption (for RRC idle. FFS for Inactive/Connected): Rel-17 UE with pre-compensation capability obtains UE specific UE-gNB RTT based on its GNSS in LEO/GEO. FFS how this is calculated and what/if anything needs to be broadcasted for the different pre-compensation methods (e.g. common TA) to help the UE to obtain the full UE-gNB RTT.

If the UE-gNB RTT is pre-compensated, preamble ambiguity is not an issue in Rel-17 NTN (i.e. no enhancements are necessary). FFS how and by whom the possibly multiple components of UE-gNB RTT are pre-compensated

From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to gNB implementation to ensure sufficient time on UE side for the Msg3 transmission.

For UE with pre-compensation capability (at least for the HARQ-feedback enabled case. FFS for HARQ-feedback disabled, if supported), drx-HARQ-RTT-TimerDL is offset by UE-specific RTT (UE-gNB delay) in LEO/GEO. FFS if offset is applied to: 1) the start of the timers or 2) the timer value range (i.e. existing values within value range increased by offset)

From RAN2 perspective, for dynamic grant, one possibility for "enabling"/"disabling" HARQ uplink retransmission at UE transmitter is without introducing an additional mechanism (i.e. gNB can send grant with NDI not toggled/toggled without waiting for decoding result of previous PUSCH transmission). FFS on the handling of RTT timers. Other solutions for enabling/disabling HARQ UL reTX are not precluded

If the start of the ra-ResponseWindow and msgB-ResponseWindow is accurately compensated by UE-gNB RTT, ra-ResponseWindow and msgB-ResponseWindow are not extended in LEO/GEO.

At least the following are FFS in Rel-17 NTN:

•	Report UE-calculated TA in e.g. msg3/msg5/msgA

•	Enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH

•	LCP impact caused by disabling HARQ UL retransmission

RAN2 decision on starting ra-ContentionResolutionTimer, ra-ResponseWindow and msgB-ResponseWindow is postponed until further progress in RAN1 regarding UE pre-compensation method and TA estimation accuracy.

RAN2#111-e Agreements

From RAN2 perspective, an offset is applied to the start of ra-ResponseWindow in NTN for both LEO and GEO scenarios.

An offset to the start of the ra-ContentionResolutionTimer is introduced for both LEO and GEO scenarios.

Modification of drx-LongCycleStartOffset, drx-StartOffset, drx-ShortCycle, drx-ShortCycleTimer, drx-onDurationTimer, drx-SlotOffset and drx-InactivityTimer is not needed in Rel-17 NTN.

From a RAN2 perspective, for DL, HARQ feedback can be enabled/disabled in Rel-17 NTN, but HARQ processes remain configured. The criteria and decision to enable/disable HARQ feedback is under network control and is signalled to the UE via RRC in a semi-static manner. FFS for UL

At least the following methods to enhance UL scheduling are further studied in NTN: configured grant and BSR over 2-step RACH. (other solutions to enhance UL scheduling are not precluded)

Both 2-step and 4-step RACH are supported in Rel-17 NTN. FFS enhancements to RACH to accommodate the NTN environment.

## A.3 User plane – Other aspects (RLC & PDCP)

RAN2#115-e Agreements

1.	Introduce a new t-ReassemblyExt-r17 IE, which is optional present for NTN network scenario.

2.	Introduce a new discardTimerExt-r17 IE with a new value ms2000 and several spare bits for future extension.

3.	RAN2 consider not to extend PDCP t-Reordering timer or use several spare bits in legacy IE to add several greater values up to 4400ms.

RAN2#114-e Agreements

The following options are supported for drx-HARQ-RTT-TimerUL in NTN per HARQ process: 1) Timer length is extended by offset; 2) Timer set to zero and/or 3) Timer disabled (i.e. not started). FFS if this is based on explicit configuration or not. We can also come back to see whether both 2 and 3 are needed

RAN2 working assumption: Offset for drx-HARQ-RTT-TimerUL is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it).

drx-RetransmissionTimerDL timer length is not extended in NTN.

The drx-HARQ-RTT-TimerUL behaviour applied for each HARQ process is up to the network (e.g. to support NW scheduling strategy to avoid HARQ stalling).

RAN2 Working Assumption: No new CG-specific LCP restriction is introduced for NTN. If a new LCP restriction is agreed for dynamic grant, the proposal does not preclude future discussion on whether it may also apply to configured grant

Repetition transmission based HARQ retransmission is always allowed and is explicitly indicated per HARQ process via DCI (as in legacy).

At least the following options for LCP in NTN are further studied: 1) allowedPHY-PriorityIndex is re-used; and 2) A new LCP restriction is introduced to map LCH to one or more HARQ process(es). FFS if HARQ processes can be classified as having retransmission “enabled” or “disabled” in this case.

RAN2#113-bis-e Agreements

1.	The UE utilizes the t-Reassembly timer value that does not depend on the time-varying UE-gNB delay.

2.	The value range of t-Reassembly shall be extended. The following set of values are possibly added for t-Reassembly timer: {ms210, ms220, ms340, ms350, ms550, ms1100, ms1650, ms2200}. Any other values are FFS.

3.	The network can configure the values of PDCP discardTimer and PDCP t-Reordering timer greater than the RLC t-Reassembly timer.

4.	Extend the range of the PDCP discardTimer and the PDCP t-reordering timer. One option is to enlarge the set of allowed values for the PDCP discardTimer and the PDCP t-reordering timer. The exact values FFS

RAN2#112-e Agreements

RLC t-Reassembly timer needs to be extended in NR-NTN.

There is no need to extend t-PollRetransmit Timer in NR-NTN.

There is no need to extend t-statusProhibit Timer in NR-NTN.

There is no need to extend RLC SN length in NR-NTN

There is no need to extend PDCP SN length in NR-NTN

## A.4 Control plane - Earth fixed moving beams related issues

RAN2#115-e Agreements

1.	RAN2 confirms AS indicates to NAS layer all received TACs per PLMN.

2.	RAN2 responds to CT1 and SA2 with the confirmation that AS indicates to NAS layer all received TACs per PLMN. In addition it is stated that TACs in NTN are fixed to geographical location on Earth and UE’s location information can be used for TAI selection. Final decision on which criteria to apply (e.g. UE location information or other) is anyway up to CT1 and SA2 judgement

RAN2#114-e Agreements

Change in TAC in SIB1 triggers SI update notification procedure as legacy behaviour. It is FFS whether broadcasting TAC update time can also be considered

In rel-17, other enhancements like virtual tracking area concept might be considered with low priority at the end of the WI.

RAN2#113-bis-e Agreements

When the network stops broadcasting a TAC, the UE needs to know it (FFS on further details)

RAN2 assume UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area.

RAN2 confirm that in NTN when TAC change in SI happens is up to network implementation, i.e. it may not exactly sync up with real-time illumination on ground.

Send a LS to CT1 and SA2, with Cc RAN3. The content is: currently RAN2 has two options on table, and the preference is “AS indicates all received TACs to NAS layer when more than one TAC per PLMN is broadcasted in NTN cell”, compared to “AS still reports only one TAC to NAS layer”, and ask for CT1’s feedback. Also include justification for RAN2 preference

RAN2#113-e Agreements

In NTN, the UE determines the TA based on the broadcast information (the use of other information is not excluded). In any case RAN2 will not go in a different direction than other groups

In NTN, the network may broadcast more than one TACs per PLMN in a cell, which is to up to network implementation.

RAN2#112-e Agreements

RAN2 to consider the case where gNB is co-located at the GW with higher priority.

RAN2 will continue working with the assumption that service link switch implies L3 mobility (meaning that at least in case the SSBs are on the same sync raster point the PCIs need to be different). Check if an LS to RAN1 asking for feasibility of having same PCI as well can be agreed

Existing cell reselection principles are considered as baseline and that information about when a cell is going to stop serving the area and information about new upcoming cell can be further considered. In which form and how this is exactly implemented in the cell reselection principles is FFS.

## A.5 Control plane – Idle/Inactive mode aspects

RAN2#115-e Agreements

1.	Broadcast of cell stop time in SIB is only applicable to quasi earth fixed cell (not to moving cell). No further work in this release to address any moving cell specific details on using the cell stop time to assist measurements or cell reselection

2.	For quasi-earth fixed cell, the reference location of the cell (serving cell or the neighbor cells) is broadcast in system information

1.	For quasi-earth fixed cell, UE should start measurements on neighbour cells before the serving cell stops covering the current area.

2.	For quasi-earth fixed cell, the broadcast “timing information on when a cell is going to stop serving the area” refers to the time when a cell stops covering the current area.

3.	For quasi-earth fixed cell, specify that UE should start measurements on neighbour cells before the broadcast stop time of the serving cell, i.e. the time when the serving cell stops covering the current area, and the exact time to start measurements is up to UE implementation.

Working Assumption: Location assisted cell reselection, with the distance between UE and the reference location of the cell (serving cell and/or neighbor cell) taken into account, is supported for quasi-earth fixed cell, if UE has valid location information, which means location acquisition will not be triggered at UE side only for location assisted cell reselection. FFS on the details.

RAN2#114-e Agreements

At least in the quasi-earth fixed case (FFS for moving case), the timing information on when a cell is going to stop serving the area is needed to assist cell reselection in NTN for earth fixed scenario.

At least in the quasi-earth fixed case (FFS for moving case), the timing information on when a cell is going to stop serving the area is used to decide when to perform measurement on neighbor cells.

At least in the quasi-earth fixed case (FFS for moving case), the timing information on when a cell is going to stop serving the area for earth fixed scenario is broadcast to UE via system information.

RAN2#113-e Agreements

The NTN ephemeris is divided into serving cell’s ephemeris and neighbour’s ephemeris. FFS how would they differ regarding e.g. the required accuracy or signalling impact.

Consider pre-configuration in uSIM, NAS, SIB and RRC signalling for providing the NTN ephemeris. Further discussion depends on the agreed ephemeris contents.

RAN2 thinks that a UE needs to know whether the network is a TN or NTN no later than SIB1 reception

The information on when a cell is going to stop serving the area and/or the timing information (e.g. timer or absolute time) about new upcoming cell is supported at least in Earth-fixed NTN scenario. FFS if both types of information are needed. FFS if this is known from system information and/or the ephemeris.

RAN2#111-e Agreements

Cell selection / reselection in NR is the baseline in NTN idle mode procedure.

Satellite/HAPS ephemeris based cell selection and reselection should be defined for NTN (FFS what the term satellite/HAPS ephemeris actually means). FFS when this ephemeris based cell selection / reselection can be used. FFS whether UE location (and/or other information) based cell selection and reselection should be introduced for NTN

The satellite ephemeris should be provided to UE, at least for Satellite/HAPS ephemeris based cell selection and reselection (FFS what the term satellite/HAPS ephemeris actually means).

The network type (i.e. TN or NTN) should be known to UE. FFS whether to achieve this in an implicit or explicit way.

The existing cell reselection priority configuration can be taken as a baseline in NTN. FFS on any further enhancement.

Postpone the discussion on whether to introduce a new SIB until we have more progress on the content of NTN specific system information.

## A.6 Control plane – Connected mode aspects

RAN2#115-e Agreements

Working Assumptions: Combination of serving and target cell reference location is supported for location report trigger event and for CHO location trigger

Working Assumptions: Specify that measurement reports can be configured to be piggybacked with location report when location based event triggers it

1.	The following event is supported: condEvent L4: Distance between UE and the PCell’s reference location becomes larger than absolute threshold1 AND the distance between UE and the Conditional reconfiguration candidate becomes shorter than absolute threshold2.

FFS other options

2.	Specify hysteresis and time to trigger for the location event for RRM and CHO

3.	Timing information from RRCReconfiguration message in RRC running CR is removed

4.	UE is allowed to perform HO only during T1 to T2

5.	Agree to limit to A or B and continue discussion between options A and B

Option A: UTC time + duration/timer, e.g. 00:00:01 + 40s

Option B: Two UTC time to indicate the start (T1) and end time (T2) of the candidate cell, e.g. 00:00:01 + 00:00:41

1.	RAN2 adopts Option 1: UTC time + duration/timer, e.g. 00:00:01 + 40s for representing T1 and T2 for CHO time event.

2.	RAN2 adopts options C: location and RRM and D: time and RRM to be configuration options for CHO

3.	RAN2 down priorities further enhacnements for connected mode for Rel-17 for TN-NTN mobility

4.	RAN2 continue discussing the exact solution for TN priorization over NTN for idle mode

1.	The specific maximum number of SMTC configuration in one measurement object with the same ssbFrequency can be 4. And a LS will be sent to RAN4 to confirm the conclusion.

2.	In NTN, NW-based solution is supported, i.e. the final SMTC/measurement gap configuration is generated and provided by NW in NTN to a given UE (based on the propagation delay difference between at least one target cell and the serving cell of a given UE). FFS whether UE-based solution is supported or not.

3.	In NTN, it is necessary of the UE to report assistant information to the NW (which can be configured by NW or upon NW’s request) to assist NW calculating the offset for SMTC/GAP configurations. FFS the detailed information.

1.	The UE can be configured with multiple SMTCs per carrier. FFS if the UE can use only a partial set or all of them in parallel, and in case FFS whether based on network configuration or UE implementation

RAN2#114-e Agreements

Support CHO location trigger as the distance between UE and a reference location which may be configured as the serving cell reference location or the candidate target cell reference location. FFS if combination can be allowed.

The reference location for the event description is defined as cell center.

For CHO, joint configuration of location and RSRP as well as time and RSRP triggers are supported.

For idle mode reselection, based on configuration NTN UE can prioritise TN over NTN. Configuration details FFS.

CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell.

Same CHO trigger conditions and RRM events can be used within NTN and NTN-TN mobility provided these are supported by the UE. NTN-TN means both “from NTN to TN (hand-in)” and “from NTN to TN (hand-in) and from TN to NTN (hand-out)". FFS for enhancements.

RAN2#113-bis-e Agreements

For Rel-17 NTN, Rel-17 NR operation is enhanced (e.g. the SMTC configuration and UE measurement gap configuration) aiming to address the issues associated with the different/larger propagation delays, and the satellites (considering e.g. their deployment, mobility, height, minimum elevation and prioritizing typical NTN scenarios).

Rel-17 NTN will not rely only on network implementation to address the issue explained in agreement 1.

Enhancements of the SMTC configuration is supported for Rel-17 NTN.

Optional new UE assistance is defined in Rel-17 NTN for network to properly (re)configure the SMTC and/or measurement gap

For Rel-17 NTN, one or more SMTC configuration(s) associated to one frequency can be configured. FFS solution details.

-	The SMTC configuration can be associated with a set of cells (e.g., per satellite or any other suitable set per gNB determination).

-	The multiple SMTC configurations are enabled by introducing different new offsets in addition to the legacy SMTC configuration. FFS how the offsets will be managed/signalled.

FFS the following open questions:

(a) can the UE be configured with multiple SMTCs per carrier and use them all in parallel?

(b) How the NW knows which SMTC (incl. offsets/periodicity, etc.) is relevant for a particular UE?

(c) Is there any validity: in time or for certain location only, foreseen in such multiple SMTC configuration?

(d) What is the potential impact on the signalling, assuming this delay is a dynamic value?

(e) What about the feeder link delay? Is it considered anywhere?

The configuration of one or multiple offsets is left up to the network implementation.

It is up to network to update the SMTC configuration of the UE to accommodate the different propagation delays.

Measurement gaps enhancements should be supported. FFS on the details

Timing information in CHO execution triggering for NTN describes the time after which the UE is allowed to execute CHO to the candidate target cell.

Working assumption: the timing information for CHO execution triggering in NTN is defined in the form of a timer/timers. This can be revised and a solution based on UTC/system frame number can be considered if problems are found (e.g. if the timer lacks accuracy due to RTT in NTN).

The location in location-based CHO execution triggering for NTN describes the distance between the UE and the reference location of the cell (serving cell or the target cell). FFS what the reference location of the cell is (e.g. cell center or other) and how this is provided to the UE

RAN2#113-e Agreements

Support A4 event for NTN CHO. FFS whether other triggers needs to be combined with this.

RAN2#112-e AgreementsReconfiguration with sync is the baseline for connected mode mobility in NTN (the use of legacy RLF and re-establishment mechanism are not excluded)

The CHO can be used in NTN for both moving cell and fixed cell scenarios, and the CHO procedure and execution condition defined in Rel-16 is the baseline for NTN CHO.

NTN specific CHO execution condition can be further discussed.

The existing measurement framework (e.g. measurement configuration, execution and reporting) is the baseline, and all the existing measurement criteria and event can be used in NTN. Support for new measurement is not excluded.

Legacy SSB periods (as in TN) shall be supported in NTN

Time or timer based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario.  FFS on how to configure the time or timer based CHO triggering event. Also FFS how to consider the feeder/service link switch timing.

DAPS HO for NTN is de-prioritized in this release.

Location based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario. FFS on how to configure the location based CHO triggering event. FFS if location based CHO triggering event only (not in combination with other events) can also be considered.

The Location-based measurement event, in combination with the existing measurement event in NR, should be supported in NTN for both moving cell and fixed cell scenarios. FFS on how to configure the location based measurement event.

RAN2 understanding that UE shall not be forced to detect the SSB burst outside the corresponding configured SMTC window in NTN, just like the principle in TN.

SMTC and gap configuration in NTN are configured based on the timing of PCell

RAN2 can first identify the scenarios and discuss how serious the impact is before addressing any enhancement for SMTC configuration in NTN.

RAN2 can’t assume that the network will always have UE accurate location info for SMTC window configuration in NTN

UE along with the network in NTN should also have the same understanding of the timing, including the timing for measurement gap, to avoid any un-synchronized scheduling between UE and the network, just like the way we have in TN

## A.7 Control plane – LCS aspects

RAN2#115-e Agreements

1.	If SA3 replies with concern on reporting UE location with any granularity during initial access, RAN2 will revisit agreement/solution for reporting UE location during initial access.

2.	UE coarse location information refers to coarse GNSS coordinates (FFS on the details, e.g. X MSB bits out of 24 bits of longitude/latitude or GNSS coordinates with ~2km accuracy). FFS if any enhancements to validate the UE’s coarse location information is needed. FFS whether this is only used in initial access or also in connected

1.	If SA3 has no concern reporting coarse location during initial access, the coarse location information is reported in Msg5, i.e., via RRCSetupComplete/RRCResumeComplete message.

2.	For coarse UE location reporting during initial access, the location granularity is not indicated to UE via SIB

3.	Enhancements to validate the UE ’s coarse location information is not needed from RAN2 perspective. Whether this is needed by the network is up to other WGs.

4.	After AS security is established, gNB can obtain a GNSS-based location information from the UE using existing signalling method, i.e., by configuring includeCommonLocationInfo in the corresponding reportConfig. It is up to SA3 to decide whether User Consent is required before NW acquires location information from the UE in NTN. RAN2 discuss whether to send LS to SA3

5.	Aperiodic location reporting (e.g., via DCI) is not supported.

Working assumption: Event triggered-based UE location reporting are configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED

1.	Send new LS to SA3 for the need of NTN specific user consent for obtaining UE location by gNB."

1.	If accepted by SA3, if the gNB has user consent to obtain UE location in NTN, reporting of finer location information/full GNSS coordinates in RRC_CONNECTED can be supported after AS security is enabled

2.	Periodic location reporting can also be configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED. RAN2 discuss whether it is part of existing periodic measurement report configuration or a new configuration for periodic reporting of UE location.

RAN2#114-e Agreements

RAN2 will work on a solution to ensure that the CGI constructed by NG-RAN corresponds to a fixed geographical area with a size comparable with a cell for TN including connected mode and initial access.

RAN2 Working Assumption: RAN2 doesn’t need to do anything to ensure that final UE location information at the core network is trustable so far (it's other WGs business to define solutions to verify the UE location).

RAN2 will work on a solution to ensure that the CGI constructed by NG-RAN can correspond to a fixed geographical area comparable with a TN cell with a radius of ~2km or more.

Send an LS to RAN3, SA2, SA3 and SA3-LI to inform them of RAN2 decision and check whether it's consistent with their requirements.

# B	Appendix: Who to contact about their comments

| Name | Company | Email |  |
| --- | --- | --- | --- |
| Nicolas Chuberre | Thales | Nicolas.chuberre@thalesaleniaspace.com |  |

END