3GPP TSG RAN WG2 Meeting #116-e                                           R2-2109673

# Electronic meeting, 1st -12th November 2021

Agenda item:			8.11.1

Source:			Intel Corporation

Title:		Report of email discussion [Post115-e][609][POS] RAT-dependent stage 2 CR (Intel)

Document for:	 	Discussion and decision

# Introduction

This is the email discussion report for following email discussion:

[Post115-e][609][POS] RAT-dependent stage 2 CR (Intel)

Scope: Progress the CR to 38.305 for RAT-dependent positioning to reflect decisions up to this meeting.

Intended outcome: Endorsable CR

Deadline:  Long

Rapporteur would like to split the discussion in two phases:

Phase 1: To check the proposals from Rapporteur and the draft TP; The deadline for this 1st phase of email discussion is Wednesday Oct 13 , 0900 UTC.

Phase 2: To finalize the draft running CR; The deadline for this 2nd  phase of email discussion is Wednesday Oct 20 , 0900 UTC.

# Annex: companies’ point of contact

| Company | Point of contact | Email address |
| --- | --- | --- |
| Intel Corporation | Yi Guo | Yi.guo@intel.com |
| Huawei, HiSilicon | Yinghao Guo | yinghaoguo@huawei.com |
| Qualcomm | Sven Fischer | sfischer@qti.qualcomm.com |
| ZTE | Yu Pan | pan.yu24@zte.com.cn |
| CATT | Jianxiang Li | lijianxiang@datangmobile.cn |
| vivo | Xiang Pan | panxiang@vivo.com |
| Xiaomi | Xiaolong Li | lixiaolong1@xiaomi.com |
| Ericsson | Ritesh Shreevastav | Ritesh.shreevastav@ericsson.com |
| InterDigital | Jaya Rao, Fumihiro Hasegawa | jaya.rao@interdigital.com, fumihiro.hasegawa@interdigital.com |
| Nokia | Mani Thyagarajan | mani.thyagarajan@nokia.com |

# Phase 1the draft on how to capture meeting agreements

The agreements on latency reduction, Positioning in RRC_INACTIVE, On-demand PRS request and PRU are captured in section 6. The scope of this email discussion is to discuss how to capture agreements of these items into stage 2, TS38.305.

## How to capture the agreements on Latency reduction

For latency reduction, so far scheduled location time, storing capability in AMF are on the table. RAN2 concluded that support pre- configuration of assistance data, however the impact of stage 2 will be discussed in [Post115-e][605][POS]. In addition, it is still open on whether scheduled location time is transparent to UE/RAN or not.

For storing capability in AMF, it is unclear what RAN impact should be. RAN2 has sent LS to SA2 on variability of UE positioning capability. We have to wait for the further inputs from SA2.

In general, we should copy every details from TS23.273 into TS38.305. As discussed in [1], section 7.3 service layer support using combined LPP and NRPPa procedures could be a good place to capture changes due to scheduled location time and storing capability in AMF;

Rapporteur provides following TP based on the information/agreements we had so far:

### TS38.305 TP on latency reduction (scheduled location time and storing capability in AMF)

- 7.3.2	NI-LR and MT-LR Service Support

Figure 7.3.2-1 shows the sequence of operations for an NI-LR or MT-LR location service, starting at the point where the AMF initiates the service in the LMF.

Figure 7.3.2-1: UE Positioning Operations to support an MT-LR or NI-LR

1.	The AMF sends a location request to the LMF for a target UE and may include associated QoS, the scheduled location time and the UE positioning capabilities.

2.	The LMF may obtain location related information from the UE and/or from the serving NG-RAN Node. In the former case, the LMF instigates one or more LPP procedures to transfer UE positioning capabilities, provide assistance data to the UE and/or obtain location information from the UE. The UE may also instigate one or more LPP procedures after the first LPP message is received from the LMF (e.g., to request assistance data from the LMF). If scheduled location time is provided in step 1, the LMF may provide pre-configuration of assistance data to the UE far before when the location is needed, and indicate the UE to utilize the pre-configured assistance data via the LPP Request Location Information message occur at or near to the scheduled location time. The LPP procedures to transfer UE positioning capabilities may be skipped if the LMF already obtained the UE positioning capabilities from the AMF in step 1.

3.	If the LMF needs location related information for the UE from the NG-RAN, the LMF instigates one or more NRPPa procedures. Step 3 is not necessarily serialised with step 2; if the LMF and NG-RAN Node have the information to determine what procedures need to take place for the location service, step 3 could precede or overlap with step 2. If scheduled location time is provided in step 1, the LMF may schedule location measurements by the NG-RAN to occur at or near to the scheduled location time.

4.	The LMF returns a location response to the AMF with any location estimate obtained as a result of steps 2 and 3.

Editor's Note:	The scheduled location time and storing UE positioning capabilities in AMF may be updated based on further inputs from SA2 and further discussion in RAN, e.g. when/whether LMF forwards UE positioning capabilities to AMF, transparent of scheduled location time, criterion of preconfigured AD ,etc.

- 7.3.3	MO-LR Service Support

Figure 7.3.3-1 shows the sequence of operations for an MO-LR service, starting at the point where an LCS Client in the UE or the user has requested some location service (e.g., retrieval of the UE's location or transfer of the UE's location to a third party).

Figure 7.3.3-1: UE Positioning Operations to support an MO-LR

1.	The UE sends an MO-LR location service request message included in a UL NAS TRANSPORT message as specified in TS 24.501 [29] to the AMF. The MO-LR location service request message may carry an LPP PDU to instigate one or more LPP procedures to transfer capabilities, request assistance data, and/or transfer location information and a scheduled location time.

2.	The AMF invokes the Nlmf Determine Location Request service operation towards the LMF as specified in TS 29.572 [33] and includes any LPP PDU and scheduled location time received in step 1.

3.	The LMF may obtain location related information from the UE and/or from the serving NG-RAN node. In the former case or if an immediate response is needed to any LPP procedure instigated by the UE in step 1 (e.g., a request for assistance data), the LMF instigates one or more LPP procedures to transfer UE positioning capabilities, provide assistance data to the UE and/or obtain location information from the UE. The UE may also instigate further LPP procedures after the first LPP message is received from the LMF (e.g., to request assistance data or to request further assistance data). If scheduled location time is provided in step 2, the LMF may provide pre-configuration of assistance data to the UE far before when the location is needed, and indicate the UE to utilize the pre-configured assistance data via the LPP Request Location Information message occur at or near to the scheduled location time. The LPP procedures to transfer UE positioning capabilities may be skipped if the LMF already obtained the UE positioning capabilities from the AMF in step 2.

4.	If the LMF needs location related information for the UE from the NG-RAN, the LMF instigates one or more NRPPa procedures. Step 4 may also precede step 3 or occur in parallel with it. If scheduled location time is provided in step 2, the LMF may schedule location measurements by the NG-RAN to occur at or near to the scheduled location time.

5.	The LMF invokes the Nlmf Determine Location Response service operation towards the AMF as specified in TS 29.572 [33] which includes any location estimate obtained as a result of steps 3 and 4.

6.	If the UE requested location transfer to a third party the AMF transfers the location received from the LMF in step 5 to the third party as defined in TS 23.273 [35].

7.	The AMF sends an MO-LR location service response message included in a DL NAS TRANSPORT message as specified in TS 24.501 [29].

Editor's Note:	The scheduled location time and storing UE positioning capabilities in AMF may be updated based on further inputs from SA2 and further discussion in RAN, e.g. when/whether LMF forwards UE positioning capabilities to AMF, transparent of scheduled location time, criterion of preconfigured AD ,etc.

- 7.3.4	Deferred MT-LR Event Reporting Support

Figure 7.3.4-1 shows the sequence of operations for an Deferred MT-LR Event Reporting starting at the point where the UE reports an event to the LMF.

Figure 7.3.4-1: UE Positioning Operations to support a Deferred MT-LR

1.	The UE sends a supplementary services event report message to the LMF as described in TS 24.571 [41] which is transferred via the serving AMF and is delivered to the LMF using an Namf_Communication_N1MessageNotify service operation. The event report may indicate the type of event being reported and may include an embedded positioning message which includes any location measurements or location estimate.

2.	If LMF determines no positioning procedure is needed, steps 3 and 4 are skipped.

3.	The LMF may utilize any location information received in step 1. The LMF may also retrieve location related information from the UE and/or from the serving NG-RAN Node. In the former case, the LMF instigates one or more LPP procedures to provide assistance data to the UE and/or obtain location information from the UE. The UE may also instigate one or more LPP procedures after the first LPP message is received from the LMF (e.g., to request assistance data from the LMF).

4.	If the LMF needs location related information for the UE from the NG-RAN, the LMF instigates one or more NRPPa procedures. Step 3 is not necessarily serialised with step 2; if the LMF and NG-RAN Node have the information to determine what procedures need to take place for the location service, step 3 could precede or overlap with step 2.

5.	The LMF invokes an Nlmf_Location_EventNotify service operation towards the GMLC with an indication of the type of event being reported and any location estimate obtained as a result of steps 2 and 3.

Editor's Note:	The scheduled location time and storing UE positioning capabilities in AMF may be updated based on further inputs from SA2 and further discussion in RAN, e.g. when/whether LMF forwards UE positioning capabilities to AMF, transparent of scheduled location time, criterion of preconfigured AD ,etc.

Companies are invited to provide your view on the TP shown as above. Also please indicate if anything is missing. E.g. different sections, etc.

| Company’s name | Agree or not | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Agree with comments | - For “scheduled location time” there is no definition given in the current TP. We think it would be sufficient if we just give a reference to this i.e., add the SA2 spec TS 23.273 where people can find the definition for it. - For the following part of spec, we think it might not be directly related to “pre-configuration of assistance data”, which we may think is not needed for the description here. If scheduled location time is provided in step 1, the LMF may provide pre-configuration of assistance data to the UE far before when the location is needed, and indicate the UE to utilize the pre-configured assistance data via the LPP Request Location Information message occur at or near to the scheduled location time. The LPP procedures to transfer UE positioning capabilities may be skipped if the LMF already obtained the UE positioning capabilities from the AMF in step 1.. |
| Qualcomm | Agree with comments | 7.3.2: Step 1: Suggest to add "…when available" at the end of the new sentence, since storage of capabilities happens at Step 4. Step 2: "Pre-configured" assistance data seem not correct in this context. Step 2 and 3 is one single LPP and NRPPa session, respectively, as usual. "far before when the location is needed" would need to be quantified. This should be deleted and the same wording as for step 3 can be used here as well: "If a scheduled location time is provided in step 1, the LMF may schedule location measurements by the UE to occur at or near to the scheduled location time." Step 4: This should also include the positioning capabilities, and should mention that an AMF may store them, with reference to 23.273. Editor's Note: "pre-configured assistance data" should not be in this section, since this is a normal/single NI-LR and MT-LR description. 7.3.3: Step 3: Same comment as for step 2 above. Editor's Note: Same comment as above. |
| Apple | Agree with comments | Step 3 – suggest to modify the added text as follows: “If a scheduled location time is provided in step 2, the LMF may provide assistance data to the UE ahead of time and indicate the UE to utilize the provided assistance data via the LPP Request Location Information message at or near to the scheduled location time. The LPP procedures to transfer UE positioning capabilities may be skipped if the LMF already obtained the UE positioning capabilities from the AMF in step 2.” |
| ZTE |  | We think scheduled location time is not used to restrict “pre-configured assistance data”, but to restrict all assistance data. So in 7.3.2 step 2 and 7.3.3 step 3, the ‘pre-configuration of ‘ and ‘pre-configured ’part should be deleted |
| CATT | Agree with comments | Prefer not to capture the pre-configured AD aspect so far before we make the definition clear. Comments on 7.3.2: If scheduled location time is provided in step 2, the LMF may send LPP Request Location Information message at or near to the scheduled location time. The LPP procedures to transfer UE positioning capabilities may be skipped if the LMF already obtained the UE positioning capabilities from the AMF in step 2. Editor's Note: The scheduled location time and storing UE positioning capabilities in AMF may be updated based on further inputs from SA2 and further discussion in RAN, e.g. when/whether LMF forwards UE positioning capabilities to AMF, transparent of scheduled location time, ,etc. Comments on 7.3.3: If scheduled location time is provided in step 2, the LMF may send LPP Request Location Information message at or near to the scheduled location time. The LPP procedures to transfer UE positioning capabilities may be skipped if the LMF already obtained the UE positioning capabilities from the AMF in step 2. Editor's Note: The scheduled location time and storing UE positioning capabilities in AMF may be updated based on further inputs from SA2 and further discussion in RAN, e.g. when/whether LMF forwards UE positioning capabilities to AMF, transparent of scheduled location time, ,etc. |
| vivo | Agree with comments | We agree with rapporteur that the pre-configuration can be used in both scheduled location and deferred MT-LR. We can revisit this issue after the definition and use case of pre-configuration are clarified. For other section, we suggest the following: 5.4.4 Function description of LMF Introduce the new function of LMF, i.e., support the provision of UE positioning capability to AMF. |
| Xiaomi | Agree with comments | We think the pre-configured assistance data should not be captured since the pre-configured assistance data may not be configured for a single positioning session based on another email discussion. |
| Ericsson | Agree with comments | We are fine with suggestion from Apple. Also agree that it is not pre-configured AD here. |
| InterDigital | Agree with comments | Share same views with others that the assistance data need not be limited to “pre-configured” and “pre-configuration”. We are generally ok with the modification to TP for Step 2 of 7.3.2 suggested by Apple as a starting point. |
| Nokia | See comments | - In general, we are concerned to see that we are adding more service layer support details in a RAN specification which may lead to more maintenance effort for RAN2. We should have a high-level feature description for a) scheduled location and b) stored UE positioning capabilities features under separate respective sections under 7.3 and these sections should point to SA2 and NAS spec for details. - In the NI-LR, MT-LR and MO-LR call flows, I am not sure why we are tying up pre-configuration of assistance data details with scheduled location aspects. We don’t think RAN2 agreed that LMF receiving scheduled location time T is a trigger for pre-configuration of assistance data. - The Editor’s note text about “transparent of scheduled location time” is not clear. Change that text to “whether scheduled location time is signaled to UE/NG-RAN” - In the MO-LR call flow, the text in step 1 and step 2 is ambiguous regarding whether scheduled location time is carried by the LCS message or the LPP PDU inside the LCS message. This needs clarification but our preference is to avoid details in call flows and just add a high-level feature description as mentioned under the first bullet above. In the MO-LR call flow, the usage of LPP Request Location Information message in step 3 is not very clear. What exactly is this message “indicating”? Current text is ambiguous and seems to mention both pre-configuration of assistance data and indicating something about “doing something near the scheduled location time” |

Phase 1-Summary: 10 companies provided inputs;

Based on companies’ comments, 8 companies in general ok to add the description on scheduled location time and storing UE capability in stage 2. However preconfigured AD should be removed and TS 23.273  should be referred;

Qualcomm, Apple, CATT also provided detailed comments; Apple’s changes got support from Ericsson and interdigital;

In addition, vivo suggested to introduce the new function of LMF in section 5.4.4, i.e., support the provision of UE positioning capability to AMF.

Nokia suggested to have high level description under 7.3 and points to SA2 and NAS spec for details, and commented that the wording “transparent of scheduled location time” in EN is not clear and should be changed to “whether scheduled location time is signaled to UE/NG-RAN”.

Rapporteur would like to update the TP with following changes:

- Remove preconfigured AD

- Add new function of LMF in section 5.4.4

- Updated EN wording to “whether scheduled location time is signaled to UE/NG-RAN”.

Proposed TP is shown as below (also in draft running CR):

### TS38.305 TP on latency reduction (scheduled location time and storing capability in AMF)

### 5.4.4	Location Management Function (LMF)

The LMF manages the support of different location services for target UEs, including positioning of UEs and delivery of assistance data to UEs. The LMF may interact with the serving gNB or serving ng-eNB for a target UE in order to obtain position measurements for the UE, including uplink measurements made by an NG-RAN and downlink measurements made by the UE that were provided to an NG-RAN as part of other functions such as for support of handover.

The LMF may interact with a target UE in order to deliver assistance data if requested for a particular location service, or to obtain a location estimate if that was requested.

The LMF may interact with multiple NG-RAN nodes to provide assistance data information for broadcasting. The assistance data information for broadcast may optionally be segmented and/or ciphered by the LMF. The LMF may also interact with AMFs to provide ciphering key data information to the AMF as described in greater detail in TS 23.273 [35].

For positioning of a target UE, the LMF decides on the position methods to be used, based on factors that may include the LCS Client type, the required QoS, UE positioning capabilities, gNB positioning capabilities and ng-eNB positioning capabilities. The LMF then invokes these positioning methods in the UE, serving gNB and/or serving ng-eNB. The positioning methods may yield a location estimate for UE-based position methods and/or positioning measurements for UE-assisted and network-based position methods. The LMF may combine all the received results and determine a single location estimate for the target UE (hybrid positioning). Additional information like accuracy of the location estimate and velocity may also be determined.

The LMF may interact with the AMF to support the provision of UE positioning capability to AMF.

### 7.3.2	NI-LR and MT-LR Service Support

Figure 7.3.2-1 shows the sequence of operations for an NI-LR or MT-LR location service, starting at the point where the AMF initiates the service in the LMF.

Figure 7.3.2-1: UE Positioning Operations to support an MT-LR or NI-LR

1.	The AMF sends a location request to the LMF for a target UE and may include associated QoS, the scheduled location time and the UE positioning capabilities when available, as described TS 23.273 [35].

2.	The LMF may obtain location related information from the UE and/or from the serving NG-RAN Node. In the former case, the LMF instigates one or more LPP procedures to transfer UE positioning capabilities, provide assistance data to the UE and/or obtain location information from the UE. The UE may also instigate one or more LPP procedures after the first LPP message is received from the LMF (e.g., to request assistance data from the LMF). If a scheduled location time is provided in step 1, the LMF may provide assistance data to the UE ahead of time and schedule location measurements by the UE to occur at or near to the scheduled location time. The LPP procedures to transfer UE positioning capabilities may be skipped if the LMF already obtained the UE positioning capabilities from the AMF in step 1.

3.	If the LMF needs location related information for the UE from the NG-RAN, the LMF instigates one or more NRPPa procedures. Step 3 is not necessarily serialised with step 2; if the LMF and NG-RAN Node have the information to determine what procedures need to take place for the location service, step 3 could precede or overlap with step 2. If scheduled location time is provided in step 1, the LMF may schedule location measurements by the NG-RAN to occur at or near to the scheduled location time.

4.	The LMF returns a location response to the AMF with any location estimate obtained as a result of steps 2 and 3. The LMF may also return the LPP UE capabilities as described in TS 23.273 [35].

Editor's Note:	The scheduled location time and storing UE positioning capabilities in AMF may be updated based on further inputs from SA2 and further discussion in RAN, e.g. when/whether LMF forwards UE positioning capabilities to AMF, whether scheduled location time is signaled to UE/NG-RAN ,etc.

### 7.3.3	MO-LR Service Support

Figure 7.3.3-1 shows the sequence of operations for an MO-LR service, starting at the point where an LCS Client in the UE or the user has requested some location service (e.g., retrieval of the UE's location or transfer of the UE's location to a third party).

Figure 7.3.3-1: UE Positioning Operations to support an MO-LR

1.	The UE sends an MO-LR location service request message included in a UL NAS TRANSPORT message as specified in TS 24.501 [29] to the AMF. The MO-LR location service request message may carry an LPP PDU to instigate one or more LPP procedures to transfer capabilities, request assistance data, and/or transfer location information and a scheduled location time , as described TS 23.273 [35].

2.	The AMF invokes the Nlmf Determine Location Request service operation towards the LMF as specified in TS 29.572 [33] and includes any LPP PDU and scheduled location time received in step 1.

3.	The LMF may obtain location related information from the UE and/or from the serving NG-RAN node. In the former case or if an immediate response is needed to any LPP procedure instigated by the UE in step 1 (e.g., a request for assistance data), the LMF instigates one or more LPP procedures to transfer UE positioning capabilities, provide assistance data to the UE and/or obtain location information from the UE. The UE may also instigate further LPP procedures after the first LPP message is received from the LMF (e.g., to request assistance data or to request further assistance data). If a scheduled location time is provided in step 2, the LMF may provide assistance data to the UE ahead of time and schedule location measurements by the UE to occur at or near to the scheduled location time. The LPP procedures to transfer UE positioning capabilities may be skipped if the LMF already obtained the UE positioning capabilities from the AMF in step 2.

4.	If the LMF needs location related information for the UE from the NG-RAN, the LMF instigates one or more NRPPa procedures. Step 4 may also precede step 3 or occur in parallel with it. If scheduled location time is provided in step 1, the LMF may schedule location measurements by the NG-RAN to occur at or near to the scheduled location time.

5.	The LMF invokes the Nlmf Determine Location Response service operation towards the AMF as specified in TS 29.572 [33] which includes any location estimate obtained as a result of steps 3 and 4. The LMF may also return the LPP UE capabilities as described in TS 23.273 [35].

6.	If the UE requested location transfer to a third party the AMF transfers the location received from the LMF in step 5 to the third party as defined in TS 23.273 [35].

7.	The AMF sends an MO-LR location service response message included in a DL NAS TRANSPORT message as specified in TS 24.501 [29].

Editor's Note:	The scheduled location time and storing UE positioning capabilities in AMF may be updated based on further inputs from SA2 and further discussion in RAN, e.g. when/whether LMF forwards UE positioning capabilities to AMF, whether scheduled location time is signaled to UE/NG-RAN ,etc.

### 7.3.4	Deferred MT-LR Event Reporting Support

Figure 7.3.4-1 shows the sequence of operations for an Deferred MT-LR Event Reporting starting at the point where the UE reports an event to the LMF.

Figure 7.3.4-1: UE Positioning Operations to support a Deferred MT-LR

1.	The UE sends a supplementary services event report message to the LMF as described in TS 24.571 [41] which is transferred via the serving AMF and is delivered to the LMF using an Namf_Communication_N1MessageNotify service operation. The event report may indicate the type of event being reported and may include an embedded positioning message which includes any location measurements or location estimate.

2.	If LMF determines no positioning procedure is needed, steps 3 and 4 are skipped.

3.	The LMF may utilize any location information received in step 1. The LMF may also retrieve location related information from the UE and/or from the serving NG-RAN Node. In the former case, the LMF instigates one or more LPP procedures to provide assistance data to the UE and/or obtain location information from the UE. The UE may also instigate one or more LPP procedures after the first LPP message is received from the LMF (e.g., to request assistance data from the LMF).

4.	If the LMF needs location related information for the UE from the NG-RAN, the LMF instigates one or more NRPPa procedures. Step 3 is not necessarily serialised with step 2; if the LMF and NG-RAN Node have the information to determine what procedures need to take place for the location service, step 3 could precede or overlap with step 2.

5.	The LMF invokes an Nlmf_Location_EventNotify service operation towards the GMLC with an indication of the type of event being reported and any location estimate obtained as a result of steps 2 and 3.

Editor's Note:	The scheduled location time and storing UE positioning capabilities in AMF may be updated based on further inputs from SA2 and further discussion in RAN, e.g. when/whether LMF forwards UE positioning capabilities to AMF, whether scheduled location time is signaled to UE/NG-RAN ,etc.

## How to capture the agreements on positioning in RRC_INACTIVE

In last meeting, based on email discussion [3] RAN2 discussed how to capture positioning in RRC_INACTIVE into stage 2, and concluded that

LPP PDU and LCS message transfer:

Proposal 1:	The LPP PDU Transfer Procedure in Annex A is used as baseline for further work.

NOTE 1:	Some details may depend on further progress of the SDT work item.

NOTE 2:	Whether such a procedure needs to be captured in Stage 2 specification or not can be decided later when the procedure has been fully developed/agreed. That is, the procedure can be considered as "running baseline".

Proposal 2:	The LCS Message Transfer Procedure in Annex B is used as baseline for further work.

NOTE 1:	Some details may depend on further progress of the SDT work item.

NOTE 2:	Whether such a procedure needs to be captured in Stage 2 specification or not can be decided later when the procedure has been fully developed/agreed. That is, the procedure can be considered as "running baseline".

DL and RAT-independent positioning:

Proposal 4:	The Deferred 5GC-MT-LR Procedure with SDT for DL-only and RAT-independent positioning in Annex C is used as baseline for further work.

NOTE 1:	Some details may depend on further progress of SDT work item.

NOTE 2:	Whether such a procedure needs to be captured in Stage 2 specification or not can be decided later when the procedure has been fully developed/agreed. That is, the procedure can be considered as "running baseline".

However there is no consensus on whether the procedure shown in Annex A, Annex B and Annex C in [3] should be captured into stage 2 specification or not since quite some companies have concern to capture SDT details into stage 2.

As mentioned in [1],

There are two options to capture positioning in RRC_INACTIVE into stage 2. Option 1 (as shown in email discussion by email discussion Rapporteur), update all relevant procedures to address message transmission via SDT in RRC_INACTIVE, e.g. - Introduce new section 6.4.3 for LPP PDU transmission using SDT; - Introduce new section 6.4.4 for LCS transmission using SDT; - Introduce new section for Low Power Periodic and Triggered 5GC-MT-LR Procedure with SDT on DL/RAT independent positioning, UL positioning and UL+DL positioning; Option 2: introduce new section 5.2.1 Positioning in RRC_INACTIVE under 5.2 UE Positioning Operations to capture general statement on positioning in RRC_INACTIVE, e.g. ### 5.2.1 Positioning in RRC_INACTIVE Positioning may be performed when a UE is in RRC_INACTIVE. LPP PDU, LCS message can be transferred between the UE and the LMF when the UE is in RRC_INACTIVE state and supports Small Data Transmission (SDT). As agreed in RAN2#114, RRC state is transparent to LPP and LMF. The LPP and LMF do not need to be aware whether SDT is used or not. If we follow option 1, i.e. copy every SDT details which is under the discussion in SDT WI, that means we have to update positioning stage 2 whenever the additional changes are made for SDT. Option 2 is the simplest way to clarify SDT framework is used instead of copying SDT details in LPP procedure. We should avoid duplicated work and should decouple the discussion between two WIs as much as possible. Proposal 3: introduce new section 5.2.1 Positioning in RRC_INACTIVE to capture general statement on positioning in RRC_INACTIVE instead of copying every SDT details in all relevant procedures.

Rapporteur would suggest to go with option 2 first, and decide later whether detailed procedure is needed.

### TS38.305 TP on positioning in RRC_INACTIVE

### 5.2.x	Positioning in RRC_INACTIVE

Positioning may be performed when a UE is in RRC_INACTIVE. LPP PDU, LCS message may be transferred between the UE and the LMF when the UE is in RRC_INACTIVE state and Small Data Transmission (SDT) is enabled.

Companies are invited to provide your view on the TP shown as above. Also please indicate if anything is missing.

| Company’s name | Agree or not | Comments, if any |
| --- | --- | --- |
| Qualcomm | Disagree | This is under a wrong main clause, since positioning in RRC_INACTIVE requires SDT, which in this Release is instigated by the UE. However, the UE positioning operations description in clause 5.2 is common for MT-LR, NI-LR, and MO-LR. Since Stage 2 does currently not support location in RRC inactive state with SDT at least the following sub-clauses should be added: - 6.4.3 LPP PDU Transfer in RRC_INACTIVE State using Small Data Transmission - 6.4.4 LCS Message Transfer in RRC_INACTIVE State using Small Data Transmission An Editor's Note can be added for now until the NOTE's in the agreements above are resolved. |
| Apple | See comments | Agree with QC that 5.2 is not the right place, but 6.4 may not be the right place either. How about clause 4.1? |
| ZTE | Disagree | There is no need to introduce a new session for RRC_INACITVE (There is no new session for RRC_INLE, either). RRC state only impact how the signal and data are transferred. Also, exposing detailed SDT procedure everywhere is not accepted by lots of companies. Therefore, we think adding a note in section 5.2, 6.4.2, 6.5.2, 6.5.3 is enough. The note can be like this: Note: the above procedures between a UE and a NG-RAN Node can be proceeded in UE RRC_CONNECTED state or by SDT in RRC_INACTIVE state as defined in TS 38.331. In different sessions the wording of the Note can be different. We can agree on the ‘add note’ solution first, and further determine the wording details in each session. |
| CATT | See comments | We had agreed that exposure of the RRC state of the UE to the LPP layer of the UE for RRC_INACTIVE UL and DL positioning will not be specified. And 5.2 is a general sequence of events applicable to the UE, NG-RAN and LMF for any location service and there is no sub-clause of 5.2 to describe positioning in other RRC state(s). Thus, we prefer to capture the agreements in 5.2 directly, not a sub-clause of 5.2. |
| vivo | See comments | Currently, there is no subsection in 5.2, therefore, we prefer not to introduce a new section 5.2.1 in 5.2. To make the stage 2 spec clear, we are ok to introduce new sections 6.4.3 and 6.4.4 and check the description when the SDT in R17 is stable. |
| Xiaomi | Disagree | We prefer to add a note in the existing procedures. |
| Ericsson | Disagree | Agree with ZTE and Xiaomi. There is no need to introduce separate section. If needed a note can be added. |
| InterDigital | Disagree, with comments | Share similar view with QC that capturing the TP on positioning in INACTIVE in a clause under 5.2 may not be the right place. For aligning with the agreements, we think adding editor’s notes under new sub-clauses 6.4.3, 6.4.4 indicated in Option 1 of [1] should be ok. |
| Nokia | See comments | We support an incremental approach to first have a generic description of positioning in RRC_INACTIVE in some section in the running CR and investigating how further details (including call flows) could be included later as more details become available on SDT. |

Phase 1-Summary: 9 companies provided inputs;

In general, companies do not agree to introduce subsection under section 5.2.

ZTE (got support from Xiaomi, Ericsson) mentioned no need to introduce a new section for RRC_INACTIVE since we do not have it for RRC_IDLE. If anything is needed, a simple Note could be added in 5.2, 6.4.2, 6.5.2 and 6.5.3, e.g.

Note: the above procedures between a UE and a NG-RAN Node can be proceeded in UE RRC_CONNECTED state or by SDT in RRC_INACTIVE state as defined in TS 38.331.

Vivo, Qualcomm, InterDigital preferred to add new section 6.4.3 and 6.4.4 dedicated for RRC_INACTIVE.

Rapporteur would like to update the TP with following changes:

- Add a general Note under 5.2 as

NOTE:	The positioning procedures between a UE and network for UEs in RRC_CONNECTED state are also applied for UEs in RRC_INACTIVE state using SDT.

Proposed TP is shown as below (also in draft running CR):

## 5.2	UE Positioning Operations

To support positioning of a target UE and delivery of location assistance data to a UE with NG-RAN access in 5GS, location related functions are distributed as shown in the architecture in Figure 5.1-1 and as clarified in greater detail in TS 23.501 [2] and TS 23.273 [35]. The overall sequence of events applicable to the UE, NG-RAN and LMF for any location service is shown in Figure 5.2-1.

Note that when the AMF receives a Location Service Request in case of the UE is in CM-IDLE state, the AMF performs a network triggered service request as defined in TS 23.502 [26] and TS 23.273 [35] in order to establish a signalling connection with the UE and assign a specific serving gNB or ng-eNB. The UE is assumed to be in connected mode before the beginning of the flow shown in the Figure 5.2-1; that is, any signalling that might be required to bring the UE to connected mode prior to step 1a is not shown. The signalling connection may, however, be later released (e.g. by the NG-RAN node as a result of signalling and data inactivity) while positioning is still ongoing.

NOTE:	The positioning procedures between a UE and network for UEs in RRC_CONNECTED state are also applied for UEs in RRC_INACTIVE state using SDT.

Figure 5.2-1: Location Service Support by NG-RAN

1a.	Either: some entity in the 5GC (e.g. GMLC) requests some location service (e.g. positioning) for a target UE to the serving AMF.

1b.	Or: the serving AMF for a target UE determines the need for some location service (e.g. to locate the UE for an emergency call).

1c.	Or: the UE requests some location service (e.g. positioning or delivery of assistance data) to the serving AMF at the NAS level.

2.	The AMF transfers the location service request to an LMF.

3a.	The LMF instigates location procedures with the serving and possibly neighbouring ng-eNB or gNB in the NG-RAN – e.g. to obtain positioning measurements or assistance data.

3b.	In addition to step 3a or instead of step 3a, the LMF instigates location procedures with the UE – e.g. to obtain a location estimate or positioning measurements or to transfer location assistance data to the UE.

4.	The LMF provides a location service response to the AMF and includes any needed results – e.g. success or failure indication and, if requested and obtained, a location estimate for the UE.

5a.	If step 1a was performed, the AMF returns a location service response to the 5GC entity in step 1a and includes any needed results – e.g. a location estimate for the UE.

5b.	If step 1b occurred, the AMF uses the location service response received in step 4 to assist the service that triggered this in step 1b (e.g. may provide a location estimate associated with an emergency call to a GMLC).

5c.	If step 1c was performed, the AMF returns a location service response to the UE and includes any needed results – e.g. a location estimate for the UE.

Location procedures applicable to NG-RAN occur in steps 3a and 3b in Figure 5.2-1 and are defined in greater detail in this specification. Other steps in Figure 5.2-1 are applicable only to the 5GC and are described in greater detail and in TS 23.502 [26] and TS 23.273 [35].

Steps 3a and 3b can involve the use of different position methods to obtain location related measurements for a target UE and from these compute a location estimate and possibly additional information like velocity. Positioning methods supported in this release are summarized in clause 4.3 and described in detail in clause 8.

The case that the NG-RAN node functions as an LCS client is not supported in this version of the specification.

## How to capture the agreements on On-Demand DL-PRS transmission

In last meeting, based on email discussion [7] RAN2 discussed how to capture positioning in On-Demand DL-PRS transmission into stage 2, and concluded that

Agreements:

Before providing available DL-PRS configuration to the UE, the LMF may obtain configuration information on what DL-PRS can be supported from one or more TRPs via NRPPa.

Capture the steps provided above as a baseline, along with a note indicating it remains FFS if the UE can send the MO-LR to request on-demand PRS.

FFS if we indicate to SA2 that MO-LR can be used to trigger on-demand PRS procedure.

It is up to Network (LMF) implementation on the steps to follow (accept/reject/ignore) on receiving request from UE for changing the DL-PRS configurations.

Therefore Rapporteur would suggest to use the TP from [7] as baseline for further discussion.

Note: Rapporteur did small editorial changes, and added additional Editor note to mention open issues.

### TS38.305 TP on On-Demand DL-PRS transmission

## 7.x Procedures for On-Demand DL-PRS transmission

### 7.x.1	General

On-Demand DL-PRS transmission is to allow a UE or LMF to request the transmission or the change of DL-PRS resources for positioning measurements. Both UE and LMF can initiate the request of On-Demand DL-PRS transmission.

### 7.x.2	On-Demand DL-PRS transmission procedures

Figure 7.x.2-1 shows the general positioning procedure for On-Demand DL-PRS transmission.

Figure 7.x.2-1: Procedures to support On-Demand DL-PRS transmission.

- The LMF may provide available DL-PRS configuration via LPP Provide Assistance Data message or via posSI.

- The UE sends an on-Demand DL-PRS request to the LMF via LPP Request Assistance Data message.

- The LMF determines the need of a new On-Demand DL-PRS configuration.

- The LMF requests the serving and non-serving gNBs/TRPs for a new On-Demand DL-PRS configuration via NRPPa PRS CONFIGURATION REQUEST message.

- The gNBs/TRPs provide the DL-PRS transmission update in the NRPPa PRS CONFIGURATION RESPONSE message accordingly.

- LMF provides the On-Demand DL-PRS configuration via LPP Provide Assistance Data message or posSI to the UE.

NOTE 1:	LMF may use existing positioning methods to obtain (ECID) SSB/CSI-RS RSRP measurements or (DL-AoD) DL-PRS RSRP measurements in order to assist step 3.

NOTE 2:	It is up to Network (LMF) implementation on the steps to follow (accept/reject/ignore) on receiving request from UE for changing the DL-PRS configurations.

Editor's Note:	Depending upon RAN3 input, the above description may need to be updated especially for NRPPa procedure, e.g. the name of the message, exchange between RAN and LMF on allowed PRS configuration, etc.

Editor's Note:	FFS if the UE can send the MO-LR to request On-Demand PRS.

Editor's Note:	FFS on the condition when UE can trigger the On-Demand PRS request.

Companies are invited to provide your view on the TP shown as above. Also please indicate if anything is missing.

| Company’s name | Agree or not | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Agree | We have the following comments regarding the description above: - The Figure needs to be cleaned up to align with the format of the other figures in the stage2. Vertical lines need to be added for serving and non-serving gNBs. |
| Qualcomm | see comments | - Step 1a/6b need a corresponding NRPPa procedure. - Before Step 3, there should be a box with possible LPP procedure(s) to support NOTE 1. This box can then also include Step 1. - Step 6a seems not correct. I.e., at this step DL-PRS assistance data should be provided (not on-demand configuration). - The description for the individual steps should provide more details. |
| Apple | Agree with comments | 7.x.1: “On-Demand DL-PRS transmission procedure allows a UE or LMF to request the transmission of or the change of DL-PRS resources for positioning measurements. Both UE and LMF can initiate the procedure.” 7.x.2: Step 2: “The UE sends an on-Demand DL-PRS request to the LMF via LPP Request Assistance Data message, including the id of the PRS configuration.” |
| ZTE | Agree with comments | In step 2, the detailed description of on-demand PRS request should be provided(index of PRS configuration or explicit PRS parameters). |
| CATT | See comments | - Before step1, the LMF should acquire necessary information from the TRP with NRPPa message (e.g., TRP INFORMATION EXCHANGE). - Step 1 and 2 are optional or add UE-initiated in the names of steps. - Step 6a the name should be corrected. (On-Demand DL-PRS configuration) should be deleted because step 6a has the same name of step1. - Step 6b the name will be corrected because it has the same name as step1a. Suggested name: posSI (updated DL-PRS). |
| vivo | See comments | - Similar with Step1, the Step 2 is optional and should be dotted for LMF initialed on-demand PRS. - The phrase ‘on-demand PRS configuration’ is ambiguous in step 1, 3 and 6 in the figure and can be rephrased to ‘allowed on-demand PRS configuration’, ‘new PRS configuration’ and ‘updated PRS configuration’. |
| Xiaomi | Agree with comments | Before step 1, the NRPPa procedures are needed to acquire on-demand PRS configuration. |
| Ericsson | See comments | Step 0 as below is needed. Step 0. The LMF may receive information on the possible on-demand DL-PRS configurations that the gNB can support during the TRP Information Exchange procedure. Depending upon this “FFS if the UE can send the MO-LR to request On-Demand PRS” The step 1 and 2 may have to be redrawn from dashed line (optional) to solid line (pre-requisite) AMF can be removed from the diagram as it is not involved. |
| InterDigital | See comments | We share same view with QC that i) a box can be included to cover Note 1 and ii) Step 6a should be modified. Also agree with Apple and ZTE that Step 2 can include further description on the info carried in on-demand PRS request. |
| Nokia | Agree with comments | - Avoid using the prefix DL- for PRS. If concerned about scope of support for the procedure, you can make the heading NR specific by use of “NR procedures” or “NR On-Demand PRS transmission” or something like that. - Section 7.x.1: On-Demand PRS transmission is to allow a UE or LMF to request the transmission or the change to PRS transmission characteristics for positioning measurements. Either UE or LMF can initiate the On-Demand PRS transmission request. - Step 1: It is not clear what is meant by available PRS configuration. Change the description as follows: “1. In case of UE-initiated on-demand PRS, the LMF may configure the UE with pre-defined PRS configurations via LPP Provide Assistance Data message or via posSI.” - Step 2: Suggested text: “2. In case of UE-initiated on-demand PRS, the UE sends an on-Demand DL-PRS request to the LMF via LPP Request Assistance Data message. The on-Demand PRS request may be a request for PRS transmission or change to the PRS transmission characteristics for positioning measurements.” - Step 3: Suggested text: “3. The LMF determines the need for PRS transmission or change to PRS transmission characteristics. In case of LMF-initiated on-demand PRS, the LMF may obtain UE measurements prior to step 3.” - Step 4: Suggested text: “4. The LMF requests the serving and non-serving gNBs/TRPs for new PRS transmission or PRS transmission with changes to the PRS configuration via NRPPa PRS CONFIGURATION REQUEST message.” - Step 6: Suggested text: “6. LMF provides the updated PRS configuration used for PRS transmission via LPP Provide Assistance Data message or posSI to the UE.” |

Phase 1-Summary: 10 companies provided inputs;

Companies provided detailed comments on the TP.

Rapporteur would like to update the TP with following changes:

- Update the figure format based on Huawei’s comments;

- Update the descriptions based on Nokia, Qualcomm and Apple’s comments;

- For ZTE, InterDigital’s comments to provide details for on demand PRS request. RAN1 and RAN2 did not agree the content, and therefore Rapporteur only add EN for it.

- Ericsson’s comments to add step 0 and remove AMF;

- Change step 2 to  dot line;

- Correct name of 6a/6b;

- Remove DL from PRS, i.e. DL-PRS to PRS;

Proposed TP is shown as below (also in draft running CR):

### TS38.305 TP on On-Demand DL-PRS transmission

## 7.x Procedures for On-Demand PRS transmission

### 7.x.1	General

On-Demand PRS transmission procedure allows a UE or LMF to request the PRS transmission or the change to PRS transmission characteristics for positioning measurements. Either UE or LMF can initiate the On-Demand PRS transmission request.

### 7.x.2	On-Demand PRS transmission procedures

Figure 7.x.2-1 shows the general positioning procedure for On-Demand PRS transmission.

Figure 7.x.2-1: Procedures to support On-Demand PRS transmission.

0.	The LMF may receive information on the possible On-Demand PRS configurations that the gNB can support during the TRP Configuration Information Exchange procedure.

1.	In case of UE-initiated On-demand PRS, the LMF may configure the UE with pre-defined PRS configurations via LPP Provide Assistance Data message or via posSI.

2.	In case of UE-initiated on-demand PRS, the UE sends an On-Demand PRS request to the LMF via LPP Request Assistance Data message. The On-Demand PRS request may be a request for PRS transmission or change to the PRS transmission characteristics for positioning measurements.

3.	The LMF determines the need for PRS transmission or change to PRS transmission characteristics. In case of LMF-initiated On-demand PRS, the LMF may obtain UE measurements prior to step 3.

4.	The LMF requests the serving and non-serving gNBs/TRPs for new PRS transmission or PRS transmission with changes to the PRS configuration via NRPPa PRS CONFIGURATION REQUEST message.

5.	The gNBs/TRPs provide the PRS transmission update in the NRPPa PRS CONFIGURATION RESPONSE message accordingly.

6.	LMF provides the updated PRS configuration used for PRS transmission via LPP Provide Assistance Data message or posSI to the UE.

NOTE 1:	LMF may use existing positioning methods to obtain (ECID) SSB/CSI-RS RSRP measurements or (DL-AoD) DL-PRS RSRP measurements in order to assist step 3.

NOTE 2:	It is up to Network (LMF) implementation on the steps to follow (accept/reject/ignore) on receiving request from UE for changing the DL-PRS configurations.

Editor's Note:	Depending upon RAN3 input, the above description may need to be updated especially for NRPPa procedure, e.g. the name of the message, exchange between RAN and LMF on allowed PRS configuration, etc.

Editor's Note:	FFS if the UE can send the MO-LR to request On-Demand PRS.

Editor's Note:	FFS on the condition when UE can trigger the On-Demand PRS request.

Editor's Note:	FFS on the content of  On-Demand PRS request.

## How to capture the agreements on PRU

Based on [10], RAN2 discussed PRU and concluded that

Agreements:

Proposal 1 (modified): For purposes of RAN2 discussion, the PRU functionality as described in the RAN1 LS can be considered as UE with known location (to some degree of accuracy) at least (16/17).

PRU modelled as a gNB can be discussed in RAN3 (no RAN2 action).

Agreement:

RAN2 confirm that the PRU considered as a UE supports the normal LPP procedures for assistance data transfer and location information transfer.

Rapporteur provides following TP based on the information/agreements we had so far:

### TS38.305 TP on PRU

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

A-AoA	Azimuth-Angle of Arrival

ADR	Accumulated Delta Range

AoA	Angle of Arrival

AP	Access Point

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

BSSID	Basic Service Set Identifier

CID	Cell-ID (positioning method)

CLAS	Centimetre Level Augmentation Service

DL-AoD	Downlink Angle-of-Departure

DL-PRS	Downlink Positioning Reference Signal

DL-TDOA	Downlink Time Difference Of Arrival

E-SMLC	Enhanced Serving Mobile Location Centre

E-CID	Enhanced Cell-ID (positioning method)

ECEF	Earth-Centered, Earth-Fixed

ECI	Earth-Centered-Inertial

EGNOS	European Geostationary Navigation Overlay Service

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDMA	Frequency Division Multiple Access

FKP	Flächenkorrekturparameter (Engl: Area Correction Parameters)

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GMLC	Gateway Mobile Location Centre

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

GRS80	Geodetic Reference System 1980

HESSID	Homogeneous Extended Service Set Identifier

LCS	LoCation Services

LMF	Location Management Function

LPP	LTE Positioning Protocol

MAC	Master Auxiliary Concept

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

Multi-RTT	Multi-Round Trip Time

NG-C	NG Control plane

NG-AP	NG Application Protocol

NI-LR	Network Induced Location Request

N-RTK	Network – Real-Time Kinematic

NRPPa	NR Positioning Protocol A

OTDOA	Observed Time Difference Of Arrival

PDU	Protocol Data Unit

posSI	Positioning System Information

posSIB	Positioning SIB

PPP	Precise Point Positioning

PPP-RTK	Precise Point Positioning – Real-Time Kinematic

PRS	Positioning Reference Signal (for E-UTRA)

PRU	Positioning reference unit

QZSS	Quasi-Zenith Satellite System

RP	Reception Point

RRM	Radio Resource Management

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTK	Real-Time Kinematic

SBAS	Space Based Augmentation System

SET	SUPL Enabled Terminal

SIB	System Information Block

SLP	SUPL Location Platform

SP	Semi-Persistent

SRS	Sounding Reference Signal

SSID	Service Set Identifier

SSR	State Space Representation

STEC	Slant TEC

SUPL	Secure User Plane Location

TADV	Timing Advance

TBS	Terrestrial Beacon System

TEC	Total Electron Content

TP	Transmission Point

TRP	Transmission-Reception Point

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL-TDOA	Uplink Time Difference of Arrival

URA	User Range Accuracy

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

Z-AoA	Zenith Angles of Arrival

### 5.4.1	User Equipment (UE)

The UE may make measurements of downlink signals from NG-RAN and other sources such as E-UTRAN, different GNSS and TBS systems, WLAN access points, Bluetooth beacons, UE barometric pressure and motion sensors. The measurements to be made will be determined by the chosen positioning method.

The UE may also contain LCS applications, or access an LCS application either through communication with a network accessed by the UE or through another application residing in the UE. This LCS application may include the needed measurement and calculation functions to determine the UE's position with or without network assistance. This is outside of the scope of this specification.

The UE may also, for example, contain an independent positioning function (e.g., GPS) and thus be able to report its position, independent of the NG-RAN transmissions. The UE with an independent positioning function may also make use of assistance information obtained from the network.

The UE may also act as positioning reference unit (PRU), and provide its own known location coordinate information to the LMF based on the request from the LMF.

Companies are invited to provide your view on the TP shown as above. Also please indicate if anything is missing.

| Company’s name | Agree or not | Comments, if any |
| --- | --- | --- |
| Qualcomm | disagree | This should not be under section 5.4.1; a new subsection should be introduced instead. The PRU operation can be performed by a "UE", but this is not "UE positioning" (since PRU location is already known). The description should be the other way around (as in the agreement text and should describe the possible PRU functions. I.e., the PRU can be considered as a UE with known location). Suggest the following general introduction text: "A Positioning Reference Unit (PRU) at a known location can perform positioning measurements (e.g., RSTD, RSRP, UE Rx-Tx Time Difference measurements, etc.) and report these measurements to a location server. In addition, the PRU can transmit SRS to enable TRPs to measure and report UL positioning measurements from PRUs at known location (e.g., RTOA, UL-AoA, gNB Rx-Tx Time Difference, etc.). The PRU measurements can be compared by a location server with the measurements expected at the known PRU location to determine correction terms for other nearby target devices. The DL- and/or UL location measurements for other target devices can then be corrected based on the previously determined correction terms. From a location server perspective, the PRU functionality can be realized as a UE with known location." |
| Apple | Disagree, also see comments | OK to include the “PRU” acronym, just change all the words to start with capital letters. We did not agree that PRU provides its own known location, so this “and provide its own known location coordinate information to the LMF based on the request from the LMF” should be removed. “also” needs to be removed as well. A PRU is a special UE, a regular UE may not act as a PRU. In summary, the text should be “A UE may act as PRU”. |
| ZTE | Disagree | Agree with QC that a new session for PRU is needed. |
| CATT | Disagree | Don’t agree with the context in 5.4.1 User Equipment (UE) because I’m afraid it will bring confusion to commercial devices. The expected new sections for PRU are: 5.X PRU Positioning Operations describe the operations of PRU 5.4.Y Positioning Reference Unit (PRU) Describe the functionalities of PRU. The general introduction text from Qualcomm can be put here. |
| vivo | Disagree | Agree with above that the PRU should be captured in a separate section. Suggest to revise the description proposed by QC: From a location server perspective, the PRU functionality can be realized treated as a UE with known location. However, the commercial UE is not expected to has the PRU functionality. |
| Xiaomi | Disagree | The PRU should be captured in the new section and we suggest to use ‘the PRU can be considered as UE with known location’ instead of ‘the UE may also act as PRU’. |
| Ericsson | Disagree | The term “known location to some degree” is very convoluted term. A simplified way to illustrate PRU is as below. PRU is a UE with known or configured location. It supports the normal LPP procedures for assistance data transfer and location information transfer, providing positioning measurements and location estimation based upon LMF’s request. FFS: A separate capability to support PRU functionality in the device. We also disagree with Apple’s comment about PRU: a PRU is a subset of UE functionality, so a UE may indeed act as a PRU. And according to the RAN1 LS, a PRU is configured with its own coordinates and may provide them to the LMF based on LMF’s request, so the current discussion seems to hold. Given that the PRU is a subset of a UE, there seems to be no need for a specific section on PRU positioning operations. |
| InterDigital | Disagree | Share similar views with QC and CATT that the description on PRU should be under a different clause, not under 5.4.1. Also, the description should be “PRU can be considered as a UE with known location”. To align the TP with the agreements made in RAN2, we think the functionalities as described in RAN1 LS (R1-2106326) can be considered. |
| Nokia | See comments | - Agree to capture PRU under Abbreviations section. - Agree also to capture “The UE may also act as positioning reference unit (PRU)” but since the exact definition and functions of PRU are still FFS, we suggest the following text: “The UE may also act as positioning reference unit (PRU) with known location for positioning of a target UE” FFS: The exact positioning functionalities supported, and the assistance data/location information transfers supported by PRU. The text proposed by Qualcomm is also fine but the reason we did not suggest this level of detail is because all these details were not agreed yet in RAN2. Also, the LS R2-2106920 from RAN1 listed some measurements explicitly which all seemed to imply use of PRU for DL timing-based methods only. For UL, the RAN1 LS did not list any specific measurements or applicability to timing or angle based or both methods. Which is why we suggested to add a FFS in our text proposal. |

Phase 1-Summary: 9 companies provided inputs;

In general, companies would like to introduce new subsection for PRU. Qualcomm also provided the draft TP on this.

APPLE commented that for PRU acronym, all words should start with capital letters;

In addition, Apple, CATT commented that a PRU is a special UE instead of the regular/commercial UE. However Ericsson had different understanding on this.

Nokia suggested to provide simple description for now considering the details were not agreed in RAN2.

Rapporteur would like to update the TP with following changes:

- Change PRU abbreviation, all words start with capital letters;

- Adapt QC’s text proposal as starting point, and add EN as suggested by Nokia;

Proposed TP is shown as below (also in draft running CR):

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

A-AoA	Azimuth-Angle of Arrival

ADR	Accumulated Delta Range

AoA	Angle of Arrival

AP	Access Point

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

BSSID	Basic Service Set Identifier

CID	Cell-ID (positioning method)

CLAS	Centimetre Level Augmentation Service

DL-AoD	Downlink Angle-of-Departure

DL-PRS	Downlink Positioning Reference Signal

DL-TDOA	Downlink Time Difference Of Arrival

E-SMLC	Enhanced Serving Mobile Location Centre

E-CID	Enhanced Cell-ID (positioning method)

ECEF	Earth-Centered, Earth-Fixed

ECI	Earth-Centered-Inertial

EGNOS	European Geostationary Navigation Overlay Service

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDMA	Frequency Division Multiple Access

FKP	Flächenkorrekturparameter (Engl: Area Correction Parameters)

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GMLC	Gateway Mobile Location Centre

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

GRS80	Geodetic Reference System 1980

HESSID	Homogeneous Extended Service Set Identifier

LCS	LoCation Services

LMF	Location Management Function

LPP	LTE Positioning Protocol

MAC	Master Auxiliary Concept

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

Multi-RTT	Multi-Round Trip Time

NG-C	NG Control plane

NG-AP	NG Application Protocol

NI-LR	Network Induced Location Request

N-RTK	Network – Real-Time Kinematic

NRPPa	NR Positioning Protocol A

OTDOA	Observed Time Difference Of Arrival

PDU	Protocol Data Unit

posSI	Positioning System Information

posSIB	Positioning SIB

PPP	Precise Point Positioning

PPP-RTK	Precise Point Positioning – Real-Time Kinematic

PRS	Positioning Reference Signal (for E-UTRA)

PRU	Positioning Reference Unit

QZSS	Quasi-Zenith Satellite System

RP	Reception Point

RRM	Radio Resource Management

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTK	Real-Time Kinematic

SBAS	Space Based Augmentation System

SET	SUPL Enabled Terminal

SIB	System Information Block

SLP	SUPL Location Platform

SP	Semi-Persistent

SRS	Sounding Reference Signal

SSID	Service Set Identifier

SSR	State Space Representation

STEC	Slant TEC

SUPL	Secure User Plane Location

TADV	Timing Advance

TBS	Terrestrial Beacon System

TEC	Total Electron Content

TP	Transmission Point

TRP	Transmission-Reception Point

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL-TDOA	Uplink Time Difference of Arrival

URA	User Range Accuracy

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

Z-AoA	Zenith Angles of Arrival

### 5.4.x	Positioning Reference Unit (PRU)

A Positioning Reference Unit (PRU) at a known location can perform positioning measurements (e.g., RSTD, RSRP, UE Rx-Tx Time Difference measurements, etc.) and report these measurements to a location server. In addition, the PRU can transmit SRS to enable TRPs to measure and report UL positioning measurements from PRUs at known location (e.g., RTOA, UL-AoA, gNB Rx-Tx Time Difference, etc.). The PRU measurements can be compared by a location server with the measurements expected at the known PRU location to determine correction terms for other nearby target devices. The DL- and/or UL location measurements for other target devices can then be corrected based on the previously determined correction terms.

From a location server perspective, the PRU functionality can be realized as a UE with known location.

Editor's Note:	FFS: The exact positioning functionalities supported, and the assistance data/location information transfers supported by PRU.

# Phase 2

Companies are invited to provide your view on summary, proposed (updated) TPs from phase 1. Also please indicate if anything is missing.

| Company’s name | Concerned TP, summary | Comments, if any |
| --- | --- | --- |
| Ericsson | 5.4.x | One question that appears is who will trigger the LPP session towards PRU, It is done mainly from a trigger from an LCS client (UE, GMLC or AMF for NI-LR). However, for PRU it is unknown. SA2 input would be needed. As SA2 is going to study this in rel-18; we prefer this topic is postponed to Rel-18. |

Summary:

Ericsson commented that whether PRU should be postponed to Rel-18. Rapporteur assumes it is unrelated to this email discussion, and therefore no proposal here.

# Summary report and proposals

There is not proposal in this summary since it is just to capture the discussion progress. We can discuss the draft CR directly in the meeting.

# Reference

- R2-2107674	Consideration on stage 2 structure on RAT dependent positioning	Intel Corporation

- R2-2107680	"Summary of agenda 8.11.2	Latency enhancements"	Intel Corporation

- R2-2108383	Summary of [Post114-e][602][POS] Stage 2 procedure for deferred MT-LR in RRC_INACTIVE	Qualcomm Incorporated

- R2-2108826	Summary of AI 8.11.3 for RRC INACTIVE positioning	ZTE

- R2-2108605	Way-forward for INACTIVE positioning	Huawei, China Unicom, China Telecom, Futurewei, HiSilicon, Intel Corporation, Interdigital, Spreadtrum Communications, VIVO, Xiaomi, ZTE Corporation

- R2-2108946	[AT115-e][615][POS] UL and UL DL positioning in RRC_INACTIVE (Huawei)	Huawei

- R2-2108400	Report on [Post114-e][603][POS] Procedures and signalling for on-demand PRS (Ericsson)	Ericsson

- R2-2108827	Summary of Agenda Item 8.11.4 On-demand PRS	CATT

- Draft TS 38.305 g60-v3

- R2-2108940	[AT115-e][610][POS] PRUs (CATT)	CATT

# Annex-Agreements on RAT dependent positioning methods

## Latency reduction

### 3GPP TSG-RAN WG2 Meeting #114-e	R2-21xxxxx Online, 19-27 May 2021

Agreements:

Support pre-configuration of assistance data to the UE at least in an LPP session.  Details of how to enable this are FFS (e.g. what additional functionality beyond deferred location procedure might be needed).

The LPP Request Location Information message can serve as an indication to the UE to utilize the pre-configured AD.  FFS additional conditions/validity criteria for using the pre-configured AD.

### 3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108835

Agreement:

Proposal 3:	Regarding the validity conditions/criteria associated with pre-configured assistance data, consider at least the following options:

	Option A: Based on a validity area (e.g. a list of cells)

	Option B: Based on a (configured) validity timer or a numerical limit on number of times it is utilized

	Option C: Based on explicit modification or release from the LMF/NG-RAN

	Option D: Based on the UE’s current location and/or the time

Agreement:

Proposal 6 (modified):	In response to the question asked by SA2 regarding UE positioning capability, it is proposed to capture that the positioning related UE capabilities can be variable.

NOTE: P6 was edited after agreement for clarity (deletion marked with strikeout).  Checked in email discussion [AT115-e][600].

## RRC_INACTIVE

### 3GPP TSG-RAN WG2 Meeting #113b-e	R2-21xxxxx Online, 12-20 April 2021

Agreements:

WA: Any uplink LCS or LPP message can be transported in RRC_INACTIVE from RAN2 perspective, subject to the data volume supported by AS layers.  I.e. RAN2 do not specify a restriction on message type.

FFS if LPP needs to select transport, i.e. if the message is just submitted to lower layers which decide how to deliver it (SDT, change state, etc.).

FFS if RRC state is exposed to LPP.

### 3GPP TSG-RAN WG2 Meeting #114-e	R2-21xxxxx Online, 19-27 May 2021

Agreements:

Any uplink LCS or LPP message can be transported in RRC_INACTIVE from RAN2 perspective.

Follow Rel-17 SDT framework for INACTIVE UL and DL positioning:

	If the UE initiated data transmission using UL SDT, the network can send DL LCS, LPP message and RRC message (e.g. to configure SRS (TBD on what message is used), if UL positioning supported) to the UE.

	Otherwise, if UE did not initiate UL SDT, rely on legacy operation, i.e. the network shall transition the UE to RRC_CONNECTED, e.g. based on RAN paging.

Agreements:

Exposure of the RRC state of the UE to the LPP layer of the UE for RRC_INACTIVE UL and DL positioning will not be specified.  This does not exclude cross-layer behaviour in implementations.

The RRC state of the UE is not exposed to the LMF for INACTIVE UL and DL positioning.

### 3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108835

Agreements:

LPP PDU and LCS message transfer:

Proposal 1:	The LPP PDU Transfer Procedure in Annex A is used as baseline for further work.

NOTE 1:	Some details may depend on further progress of the SDT work item.

NOTE 2:	Whether such a procedure needs to be captured in Stage 2 specification or not can be decided later when the procedure has been fully developed/agreed. That is, the procedure can be considered as "running baseline".

Proposal 2:	The LCS Message Transfer Procedure in Annex B is used as baseline for further work.

NOTE 1:	Some details may depend on further progress of the SDT work item.

NOTE 2:	Whether such a procedure needs to be captured in Stage 2 specification or not can be decided later when the procedure has been fully developed/agreed. That is, the procedure can be considered as "running baseline".

Proposal 3:	UL LPP message segmentation can also be used by the UE in RRC_INACTIVE state; i.e., a LPP message body can be sent in several shorter LPP messages instead of one long LPP message by using the SDT "Subsequent Data Transmission" phase.  FFS spec impact.

DL and RAT-independent positioning:

Proposal 4:	The Deferred 5GC-MT-LR Procedure with SDT for DL-only and RAT-independent positioning in Annex C is used as baseline for further work.

NOTE 1:	Some details may depend on further progress of SDT work item.

NOTE 2:	Whether such a procedure needs to be captured in Stage 2 specification or not can be decided later when the procedure has been fully developed/agreed. That is, the procedure can be considered as "running baseline".

NOTE 3:	Once the procedure is stable from RAN2 perspective, send an LS to SA2 including the baseline procedure.

Agreement:

(High priority)Proposal 1: Support all the RAT independent positioning methods in RRC_INACTIVE state.

Agreement:

gNB can configure the UE with periodic SRS (assuming periodic SRS is supported in RRC_INACTIVE) by RRCRelease with suspendConfig at least when periodic event is configured for deferred MT-LR.  Other cases can be further discussed.

## On demand PRS

### 3GPP TSG-RAN WG2 Meeting #113b-e	R2-21xxxxx Online, 12-20 April 2021

Agreements:

UE-initiated on-demand PRS request is enabled by enhancing LPP RequestAssistanceData.  FFS how much control the network has over the UE request.

The UE-initiated mechanism is enabled by the UE request triggering a request from the LMF, and the C.

Put the stage 2 description for UE-initiated and LMF-initiated PRS request under the same framework.

### 3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108835

Agreements:

Before providing available DL-PRS configuration to the UE, the LMF may obtain configuration information on what DL-PRS can be supported from one or more TRPs via NRPPa.

Capture the steps provided above as a baseline, along with a note indicating it remains FFS if the UE can send the MO-LR to request on-demand PRS.

FFS if we indicate to SA2 that MO-LR can be used to trigger on-demand PRS procedure.

It is up to Network (LMF) implementation on the steps to follow (accept/reject/ignore) on receiving request from UE for changing the DL-PRS configurations.

## PRU

### 3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108835

Agreements:

Proposal 1 (modified): For purposes of RAN2 discussion, the PRU functionality as described in the RAN1 LS can be considered as UE with known location (to some degree of accuracy) at least (16/17).

PRU modelled as a gNB can be discussed in RAN3 (no RAN2 action).

Agreement:

RAN2 confirm that the PRU considered as a UE supports the normal LPP procedures for assistance data transfer and location information transfer.