3GPP TSG-RAN WG2 Meeting #116-e                            R2-2110150

Online, November 01 – November 12, 2021

Agenda item:	8.3.5

Source:	Samsung

Title:	Discussion on UE capability for MUSIM

Document for:	Discussion & Decision

# Introduction

RAN2 is discussing the support of MUSIM devices. From [1] the objectives of this WI are as below.

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2] - RAT Concurrency: Network A can be NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx. - Specify mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose) [RAN2]: - RAT Concurrency: Network A is NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx - Unless SA2 find an alternative solution or decides otherwise , specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR.[ RAN2] - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR. - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

In this contribution we discuss the UE capabilities needed for supporting the above objectives.

# Discussion

Since paging collision avoidance and paging with service indication happens mainly in RRC_IDLE/RRC_INACTIVE mode and are handled mainly by NAS layer, there is no need for AS level UE capability for these items. RAN2 needs to discuss the UE capabilities for switching objective.

## UE capability for RRC based switching procedures

We would like to first discuss whether two separate UE capability bits are needed for RRC based switching procedures i.e. one for leaving RRC_CONNECTED state and the other without leaving RRC_CONNECTED state. It needs to be noted that that implementation of “switching without leaving” can be more complex, may require more tests to introduce and may have more complex performance requirements than “switching with leaving”. For instance, switching with leaving has many similarities to R16 power saving scenarios while switching without leaving involves supporting multiple gaps, allocation of gaps as requested by the UE etc. which are not similar to any R16 scenarios. On the other hand, for some UEs it may be preferred to implement “switching without leaving” only as UEs can use NAS based leaving instead of RRC based “switching with leaving”

Proposal 1: Separate UE capability bits are used for RRC based switching procedures for leaving RRC_CONNECTED state and without leaving RRC_CONNECTED state.

It is agreed to introduce only per-UE gaps for MUSIM switching purpose. Per-FR gaps or gaps with other granularity are not supported in R17.Hence it is obvious that the capabilities for MUSIM switching are per-UE capabilities. Capabilities also need to be optional and there is no need for FDD-TDD DIFF or FR1-FR2 DIFF.

Proposal 2: Capabilities for MUSIM switching are per-UE, optional and there is no FDD-TDD DIFF or FR1-FR2 DIFF.

If UE capability separation is needed, then it is obvious that NW should enable to configure the UE for two RRC based switching procedures independently. Even if not, we still believe that it should be allowed for NW to configure either one or both from a specification point of view. For example, some networks may want to implement switching with leaving first as it is relatively simpler than switching without leaving.

Proposal 3: NW can configure the UE to report MUSIM UE assistance information for leaving RRC_CONNECTED state and without leaving RRC_CONNECTED state independently.

# Conclusion

Based on the discussion in the previous sections, RAN2 is requested to discuss and if possible agree on the following proposals:

Proposal 1: Separate UE capability bits are used for RRC based switching procedures for leaving RRC_CONNECTED state and without leaving RRC_CONNECTED state.

Proposal 2: Capabilities for MUSIM switching are per-UE, optional and there is no FDD-TDD DIFF or FR1-FR2 DIFF.

Proposal 3: NW can configure the UE to report MUSIM UE assistance information for leaving RRC_CONNECTED state and without leaving RRC_CONNECTED state independently.

# References

- RP-201309 Revision on WID Support for Multi-SIM devices.

- R2-2109301, RAN2#115-e Meeting Report.

# Annexure-1 UE Capability Definition

TS 38.306 definition with two separate capabilities are given below.

4.2.2 General parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| accessStratumRelease Indicates the access stratum release the UE supports as specified in TS 38.331 [9]. | UE | Yes | No | No |
| delayBudgetReporting Indicates whether the UE supports delay budget reporting as specified in TS 38.331 [9]. | UE | No | No | No |
| dl-DedicatedMessageSegmentation-r16 Indicates whether the UE supports reception of segmented DL RRC messages. | UE | No | No | No |
| drx-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on DRX parameters for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| inactiveState Indicates whether the UE supports RRC_INACTIVE as specified in TS 38.331 [9]. | UE | Yes | No | No |
| inDeviceCoexInd-r16 Indicates whether the UE supports IDC (In-Device Coexistence) assistance information as specified in TS 38.331 [9]. | UE | No | No | No |
| maxBW-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on the maximum aggregated bandwidth for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | Yes |
| maxCC-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on the maximum number of secondary component carriers for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| maxMIMO-LayerPreference-r16 Indicates whether the UE supports providing its preference of a cell group on the maximum number of MIMO layers for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | Yes |
| mcgRLF-RecoveryViaSCG-r16 Indicates whether the UE supports recovery from MCG RLF via split SRB1 (if supported) and via SRB3 (if supported) as specified in TS 38.331[9]. | UE | No | No | No |
| minSchedulingOffsetPreference-r16 Indicates whether the UE supports providing its preference on the minimum scheduling offset for cross-slot scheduling of the cell group for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| mpsPriorityIndication-r16 Indicates whether the UE supports mpsPriorityIndication on RRC release with redirect as defined in TS 38.331 [9]. | UE | No | No | No |
| onDemandSIB-Connected-r16 Indicates whether the UE supports the on-demand request procedure of SIB(s) or posSIB(s) while in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| overheatingInd Indicates whether the UE supports overheating assistance information. | UE | No | No | No |
| partialFR2-FallbackRX-Req Indicates whether the UE meets only a partial set of the UE minimum receiver requirements for the eligible FR2 fallback band combinations as defined in Clause 4.2 of TS 38.101-2 [3] and Clause 4.2 of TS 38.101-3 [4]. If not indicated, the UE shall meet all the UE minimum receiver requirements for all the FR2 fallback combinations in TS 38.101-2 [3] and TS 38.101-3 [4]. The UE shall support configuration of any of the FR2 fallback band combinations regardless of the presence or the absence of this field. | UE | No | No | No |
| redirectAtResumeByNAS-r16 Indicates whether the UE supports reception of redirectedCarrierInfo in an RRCRelease message in response to an RRCResumeRequest or RRCResumeRequest1 which is triggered by the NAS layer, as specified in TS 38.331 [9]. | UE | No | No | No |
| reducedCP-Latency Indicates whether the UE supports reduced control plane latency as defined in TS 38.331 [9] | UE | No | No | No |
| referenceTimeProvision-r16 Indicates whether the UE supports provision of referenceTimeInfo in DLInformationTransfer message and in SIB9 and reference time information preference indication via assistance information, as specified in TS 38.331 [9]. | UE | No | No | No |
| releasePreference-r16 Indicates whether the UE supports providing its preference assistance information to transition out of RRC_CONNECTED for power saving, as specified in TS 38.331 [9]. | UE | No | No | No |
| resumeWithStoredMCG-SCells-r16 Indicates whether the UE supports not deleting the stored MCG SCell configuration when initiating the resume procedure. | UE | No | No | No |
| resumeWithStoredSCG-r16 Indicates whether the UE supports not deleting the stored SCG configuration when initiating resume. The UE which indicates support for resumeWithStoredSCG-r16 shall also indicate support for resumeWithSCG-Config-r16. | UE | No | No | No |
| resumeWithSCG-Config-r16 Indicates whether the UE supports (re-)configuration of an SCG during the resume procedure. | UE | No | No | No |
| splitSRB-WithOneUL-Path Indicates whether the UE supports UL transmission via MCG path and DL reception via either MCG path or SCG path, as specified for the split SRB in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode). | UE | No | No | No |
| splitDRB-withUL-Both-MCG-SCG Indicates whether the UE supports UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode). | UE | Yes | No | No |
| srb3 Indicates whether the UE supports direct SRB between the SN and the UE as specified in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode). This field is not applied to NE-DC. | UE | Yes | No | No |
| musim-PreferredRRC-State-r17 Indicates whether the UE supports providing its MUSIM assistance information to transit out of RRC_CONNECTED as specified in TS 38.331 [9]. | UE | NO | NO | NO |
| musim-GapRequestList-r17 Indicates whether the UE supports providing its preference of MUSIM gap(s) as specified in TS 38.331 [9]. | UE | NO | NO | NO |

# Annexure-2-OtherConfig

The below section captures TP for changes in handling Other configuration with respect to running NR RRC CR.

#### 5.3.5.9	Other configuration

The UE shall:

1>	if the received otherConfig includes the delayBudgetReportingConfig:

2>	if delayBudgetReportingConfig is set to setup:

3>	consider itself to be configured to send delay budget reports in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to send delay budget reports and stop timer T342, if running.

1>	if the received otherConfig includes the overheatingAssistanceConfig:

2>	if overheatingAssistanceConfig is set to setup:

3>	consider itself to be configured to provide overheating assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide overheating assistance information and stop timer T345, if running;

1>	if the received otherConfig includes the idc-AssistanceConfig:

2>	if idc-AssistanceConfig is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

1>	if the received otherConfig includes the drx-PreferenceConfig:

2>	if drx-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on DRX parameters for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on DRX parameters for power saving for the cell group and stop timer T346a associated with the cell group, if running;

1>	if the received otherConfig includes the maxBW-PreferenceConfig:

2>	if maxBW-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group and stop timer T346b associated with the cell group, if running;

1>	if the received otherConfig includes the maxCC-PreferenceConfig:

2>	if maxCC-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group and stop timer T346c associated with the cell group, if running;

1>	if the received otherConfig includes the maxMIMO-LayerPreferenceConfig:

2>	if maxMIMO-LayerPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group and stop timer T346d associated with the cell group, if running;

1>	if the received otherConfig includes the minSchedulingOffsetPreferenceConfig:

2>	if minSchedulingOffsetPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group and stop timer T346e associated with the cell group, if running;

1>	if the received otherConfig includes the releasePreferenceConfig:

2>	if releasePreferenceConfig is set to setup:

3>	consider itself to be configured to provide assistance information to transition out of RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide assistance information to transition out of RRC_CONNECTED and stop timer T346f, if running.

1>	if the received otherConfig includes the obtainCommonLocation:

2>	include available detailed location information for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

NOTE 1:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

1>	if the received otherConfig includes the btNameList:

2>	if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the wlanNameList:

2>	if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the sensorNameList:

2>	if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

1>	if the received otherConfig includes the sl-AssistanceConfigNR:

2>	consider itself to be configured to provide configured grant assistance information for NR sidelink communication in accordance with 5.7.4;

1>	if the received otherConfig includes the referenceTimePreferenceReporting:

2>	consider itself to be configured to provide UE reference time assistance information in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide UE reference time assistance information;

1>	if the received otherConfig includes the musim-AssistanceConfigForLeave:

2>	if musim-AssistanceConfigForLeave is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for switching with leaving RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for switching with leaving RRC_CONNECTED.

1>	if the received otherConfig includes the musim-AssistanceConfigWithoutLeave:

2>	if musim-AssistanceConfigWithoutLeave is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for switching without leaving RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for switching without leaving RRC_CONNECTED