3GPP TSG-RAN WG2 Meeting #116-e     			          	                                 R2-2110259

Electronic Meeting, November 1 – 12, 2021

Agenda item:	8.8.4

Source:	CMCC

Title:	Discussion on UE capability for RAN slicing enhancement

WID/SID:	NR_slice

Document for:	Discussion and Decision

Introduction

This contribution will focus on the UE capability for RAN slicing enhancement.

Discussion

For slice-based cell reselection, the frequency priority can be provided in both system information and dedicated signalling [1]. And in RAN2#113bis-e [2], RAN2 agreed that the UE is only configured with either the exiting dedicated priority configuration or the slice info in dedicated signalling.

- RAN2#113bis-e: UE is only configured with either the existing dedicated priority configuration or the slice info in RRC Release. - RAN2#114-e: Frequency priority mapping for each of the slice (slice -> frequency(ies) -> absolute priority of each of the frequency) is part of the “slice info” agreed to be provided to the UE using both broadcast and dedicated signaling.

Thus, the UE should report to network whether it supports slice-based cell reselection, then the network decides to provide exiting dedicated frequency priority configuration or the slice info in RRCRelease message based on this UE capability.

Proposal 1: Introduce a new UE capability parameter (e.g. sliceSpecifcReselection-r17) to indicate whether the UE supports slice-based cell reselection.

For slice-based RACH configuration, RAN2 has discussed some issues about RACH isolation resources and RACH prioritization parameters. As specified in TS 38.331 [2], dedicated RACH resources and RACH prioritization parameters for CBRA and CFRA can be configured in dedicated RRC signalling (e.g. RRCReconfiguration message) for connected mode UE. Although according to the agreements in RAN2#113bis-e, whether the RACH prioritization parameters can be configured in dedicated RRC signalling is FFS, and connected mode is down prioritized in this WI, from the perspective of network operator, we think that the RACH configuration could be configured in dedicated signalling following legacy principles.

- RAN2#113bis-e: scalingFactorBI and powerRampingStepHighPriority can be configured at least in SIB (FFS for dedicated RRC signalling). RAN2 will prioritize the discussion for slice specific RACH for IDLE and INACTIVE mode. And CONNECTED mode is down prioritized and can be considered if time allows. Slice specific RACH (including RACH isolation and RACH prioritization) is only applied for CBRA but not for CFRA.

Therefore, the UE should also report to the network whether it supports slice-based RACH configuration, then the network decides whether to provide dedicated RACH resources and RACH prioritization parameters in dedicated signalling. Theoretically, it is allowed for a UE to only support slice-based cell reselection or only support slice-based RACH configuration or support both of them, so it’s better and flexible to introduce a separate IE to indicate whether the UE supports slice-based RACH configuration (including RACH isolation and RACH prioritization).

Proposal 2: Introduce a new UE capability parameter (e.g. sliceSpecificRACH-r17) to indicate whether the UE supports slice-based RACH configuration.

In addition, the new indications for slice feature should be both optional, and the UE shall not set the FDD/TDD specific fields (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode) and FR1/FR2 specific fields for this capability.

Based on P1 and P2, we have prepared the draft TP for TS 38.306 in Annex, and we suggest to use the TP as the start point for discussion on the UE capability for RAN slicing enhancement.

Proposal 3: Draft CR for TS 38.306 in the Annex is captured in the 38.306 Running CR.

Conclusion

Here are the proposals for UE capability for RAN slicing enhancement.

Proposal 1: Introduce a new UE capability parameter (e.g. sliceReselection-r17) to indicate whether the UE supports slice-based cell reselection.

Proposal 2: Introduce a new UE capability parameter (e.g. sliceRACH-r17) to indicate whether the UE supports slice-based RACH configuration.

Proposal 3: Draft CR for TS 38.306 in the Annex is captured in the 38.306 Running CR.

References

- Report of 3GPP TSG RAN WG2 meeting #114-e, Online, 19 - 27 May, 2021

- Report of 3GPP TSG RAN WG2 meeting #113bis-e, Online, 12 - 20 April, 2021

- 3GPP TS 38.331, NR; Radio Resource Control (RRC) protocol specification (Release 16)

Annex: TP for TS 38.306

- 4.2.2	General parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| accessStratumRelease Indicates the access stratum release the UE supports as specified in TS 38.331 [9]. | UE | Yes | No | No |
| delayBudgetReporting Indicates whether the UE supports delay budget reporting as specified in TS 38.331 [9]. | UE | No | No | No |
| dl-DedicatedMessageSegmentation-r16 Indicates whether the UE supports reception of segmented DL RRC messages. | UE | No | No | No |
| drx-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on DRX parameters for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| inactiveState Indicates whether the UE supports RRC_INACTIVE as specified in TS 38.331 [9]. | UE | Yes | No | No |
| inDeviceCoexInd-r16 Indicates whether the UE supports IDC (In-Device Coexistence) assistance information as specified in TS 38.331 [9]. | UE | No | No | No |
| maxBW-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on the maximum aggregated bandwidth for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | Yes |
| maxCC-Preference-r16 Indicates whether the UE supports providing its preference of a cell group on the maximum number of secondary component carriers for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| maxMIMO-LayerPreference-r16 Indicates whether the UE supports providing its preference of a cell group on the maximum number of MIMO layers for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | Yes |
| mcgRLF-RecoveryViaSCG-r16 Indicates whether the UE supports recovery from MCG RLF via split SRB1 (if supported) and via SRB3 (if supported) as specified in TS 38.331[9]. | UE | No | No | No |
| minSchedulingOffsetPreference-r16 Indicates whether the UE supports providing its preference on the minimum scheduling offset for cross-slot scheduling of the cell group for power saving in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| mpsPriorityIndication-r16 Indicates whether the UE supports mpsPriorityIndication on RRC release with redirect as defined in TS 38.331 [9]. | UE | No | No | No |
| onDemandSIB-Connected-r16 Indicates whether the UE supports the on-demand request procedure of SIB(s) or posSIB(s) while in RRC_CONNECTED, as specified in TS 38.331 [9]. | UE | No | No | No |
| overheatingInd Indicates whether the UE supports overheating assistance information. | UE | No | No | No |
| partialFR2-FallbackRX-Req Indicates whether the UE meets only a partial set of the UE minimum receiver requirements for the eligible FR2 fallback band combinations as defined in Clause 4.2 of TS 38.101-2 [3] and Clause 4.2 of TS 38.101-3 [4]. If not indicated, the UE shall meet all the UE minimum receiver requirements for all the FR2 fallback combinations in TS 38.101-2 [3] and TS 38.101-3 [4]. The UE shall support configuration of any of the FR2 fallback band combinations regardless of the presence or the absence of this field. | UE | No | No | No |
| redirectAtResumeByNAS-r16 Indicates whether the UE supports reception of redirectedCarrierInfo in an RRCRelease message in response to an RRCResumeRequest or RRCResumeRequest1 which is triggered by the NAS layer, as specified in TS 38.331 [9]. | UE | No | No | No |
| reducedCP-Latency Indicates whether the UE supports reduced control plane latency as defined in TS 38.331 [9] | UE | No | No | No |
| referenceTimeProvision-r16 Indicates whether the UE supports provision of referenceTimeInfo in DLInformationTransfer message and in SIB9 and reference time information preference indication via assistance information, as specified in TS 38.331 [9]. | UE | No | No | No |
| releasePreference-r16 Indicates whether the UE supports providing its preference assistance information to transition out of RRC_CONNECTED for power saving, as specified in TS 38.331 [9]. | UE | No | No | No |
| resumeWithStoredMCG-SCells-r16 Indicates whether the UE supports not deleting the stored MCG SCell configuration when initiating the resume procedure. | UE | No | No | No |
| resumeWithStoredSCG-r16 Indicates whether the UE supports not deleting the stored SCG configuration when initiating resume. The UE which indicates support for resumeWithStoredSCG-r16 shall also indicate support for resumeWithSCG-Config-r16. | UE | No | No | No |
| resumeWithSCG-Config-r16 Indicates whether the UE supports (re-)configuration of an SCG during the resume procedure. | UE | No | No | No |
| sliceSpecificReselection-r17 Indicates whether the UE supports slice-based cell reselection. | UE | No | No | No |
| sliceSpecificRACH-r17 Indicates whether the UE supports the slice-based RACH configuration (including RACH isolation resources and RACH prioritization parameters). | UE | No | No | No |
| splitSRB-WithOneUL-Path Indicates whether the UE supports UL transmission via MCG path and DL reception via either MCG path or SCG path, as specified for the split SRB in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode). | UE | No | No | No |
| splitDRB-withUL-Both-MCG-SCG Indicates whether the UE supports UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode). | UE | Yes | No | No |
| srb3 Indicates whether the UE supports direct SRB between the SN and the UE as specified in TS 37.340 [7]. The UE shall not set the FDD/TDD specific fields for this capability (i.e. it shall not include this field in UE-MRDC-CapabilityAddXDD-Mode). This field is not applied to NE-DC. | UE | Yes | No | No |