3GPP TSG RAN WG2 meeting #116 electronic                                      R2-2110333

Online, 1st Nov. to 12th Nov. 2021                                                   Revision of R2-2107906

Agenda Item:	8.17.2

Source:	Lenovo, Motorola Mobility

Title:	Discussion on support of inter-cell multi-TRP operation

Document for:	Discussion

Introduction

The RAN1 agreements for inter-cell multi-TRP operations were achieved as follows.

Agreement For QCL /TCI related enhancement for enhanced inter-cell multi-TRP operations, support RRC configuration of non-serving cell information - Non-serving cell information can be associated with the TCI state and/or QCL -info at least when “neighbor cell SSB” is used as “QCL referenceSignal ” - FFS : Whether beam indication enhancement is needed in addition to QCL -info enhancement - FFS : Whether the association is explicit or implicit Agreement The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information Agreement Non-serving cell information at least includes non-serving cell PCI to support inter-cell multi-DCI multi-TRP operation - FFS: Whether the indication of PCI is implicit or explicit Conclusion Reuse Rel-15/16 QCL rule between the source and target RS/channel for non-serving cell RS/channel. Agreement At least following non-serving cell SSB information are needed in inter-cell MTRP operation - SSB time domain position - SSB transmission periodicity - SSB transmission power FFS: Other non-serving cell information FFS: Whether indication of these information is implicit or explicit Agreement - For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC - The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC. - Applicable at least for non-cross carrier QCL indication - FFS: Cross carrier scheduling QCL indication - RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP - Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e Agreement For intercell MTRP operation, downselect one or more of the following alternatives in RAN1#105-e - Alt1: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with only one CORESETPoolIndex - Alt2: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with more than one CORESETPoolIndex - Alt3: one PCI associated with TCI states for [PDSCH]/PDCCH via QCL relationship without association with CORESETPoolIndex Note: This agreement is not related to the down-selection of one of the 5 options from RAN1#104-e Note: Above should be specified by reusing Rel-15/Rel-16 QCL rules as concluded in RAN1#104-e

This contribution discusses how to support the inter-cell multi-TRP operation from RAN2 perspective.

Discussion

For the inter-cell multi-TRP scenario provided in Figure 1, the UE is located in the area covered by Cell A(associated with PCID-1) and Cell B(associated with PCID-2), where Cell A is the serving cell and Cell B is a non-serving cell. The UE is served by TRP1 and TRP2 in multi-DCI based multi-TRP mode. CORESET pool 0 is configured for TRP1 and CORESET pool 1 is configured for TRP2. The UE could receive dedicated PDCCH/PDSCH from or transmit dedicated PUCCH/PUSCH to TRP2 associated with non-serving PCID-2, while still receiving system information from the serving cell A.

Figure 1 Inter-cell multi-TRP scenario

Necessary enhancements on the QCL/TCI indication for dedicated PDCCH/PDSCH reception from non-serving cell B are required. It was concluded to reuse Rel-15/16 QCL rule between the source and target RS/channel for non-serving cell RS/channel. RAN1 also agreed that SSB from a non-serving cell can be configured as the source QCL RS for the PDCCH and PDSCH reception from a non-serving cell in multi-DCI based multi-TRP mode. Figure 2 provides the example of QCL chain configuration for DMRS of PDCCH and PDSCH reception from TRP1 and TRP2. For the PDSCH/PDCCH transmitted from TRP1, UE expects the NW configuring SSB from the serving cell A as the source QCL-TypeC and QCL-TypeD RS for the TRS used for corresponding DMRS reception. For PDSCH/PDCCH transmitted from TRP2, the UE expects the NW configuring SSB from non-serving cell as the source QCL-TypeC and QCL-TypeD RS for the TRS used for corresponding DMRS reception.

Figure 2 QCL-chain for inter-cell multi-TRP operation

The key issue is how to configure the SSB from a non-serving cell as the source QCL RS in the TCI state for TRS. At least the following parameters should be configured for the non-serving cell SSB

- PCI

- SSB time domain position

- SSB transmission periodicity

- SSB transmission power

For a UE supporting positioning, SSB index from a non-serving cell can be configured as the spatial relation info for the SRS resources used for positioning by introducing RRC signaling SSB-Configuration-r16 and ssb-InfoNcell-r16. The same mechanism can be introduced for TCI state. A example RRC configuration of TCI state is provided in Table 1.

Table 1 RRC configuration for enhanced TCI state

TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ... } QCL-Info ::= SEQUENCE { cell ServCellIndex OPTIONAL, -- Need R bwp-Id BWP-Id OPTIONAL, -- Cond CSI-RS-Indicated referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceId, ssb-IndexServing SSB-Index ssb-Ncell SSB-InfoNcell-r16 }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD}, ... }

The higher layer parameter ssb-IndexServing indicates a SSB index from the serving cell, while ssb-Ncell indicates a SSB from a non-serving cell. The other information of the non-serving cell are configured by SSB-InfoNcell-r17 and SSB-Configuration-r17 by RRC signaling provided in Table 2.

Table 2 RRC configuration for non-serving cell’s information

SSB-InfoNcell-r17 ::= SEQUENCE { physicalCellId-r17 PhysCellId, ssb-IndexNcell-r17 SSB-Index OPTIONAL, -- Need S ssb-Configuration-r17 SSB-Configuration-r17 OPTIONAL -- Need S } SSB-Configuration-r17 ::= SEQUENCE { ssb-Periodicity-r16 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2,spare1 } OPTIONAL, -- Need S sfn0-Offset-r16 SEQUENCE { sfn-Offset-r16 INTEGER (0..1023), integerSubframeOffset-r16 INTEGER (0..9) OPTIONAL -- Need R } OPTIONAL, -- Need R Sfn-SSB-Offset-r16 INTERGE (0..15) ss-PBCH-BlockPower-r16 INTEGER (-60..50) OPTIONAL -- Cond Pathloss }

Proposal 1: SSB index from a non-serving cell is configured in QCL-info, and the corresponding information for the SSB is configured by SSB-InfoNcell-r17 and SSB-Configuration-r17 provided in Table 2.

The another issue which needs to be addressed is the relationship between CORESETPoonIndex and the additional PCI. The WID [1] clearly states that inter-cell multi-TRP operation will be specified by assuming multi-DCI based multi-PDSCH reception. Therefore, two CORESET pools are expected to be configured for TRP1 and TRP2 as illustrated in Figure 1. Different CORESETPoolIndex values are configured for each CORESET pool. In mDCI mTRP, system information is transmitted from the CORESET within CORESET pool 0. It is reasonable that CORESET pool 0 is configured for the serving cell and CORESET pool 1 is configured for the non-serving cell for inter-cell multi-TRP operation. In other word, CORESET pool 0 or CORESETPoolIndex with value 0 is associated with the serving cell, while CORESET pool 1 or CORESETPoolIndex with value 1 is associated with additional cell different from the serving cell. By this association, Rel-16 TRP-specific configuration, e.g. activated TCI states and RateMatchPatternLTE-CRS, and separate HARQ-ACK feedback can work well with inter-cell multi-TRP scenario.

Proposal 2: In inter-cell multi-TRP operation, the CORESETPoolIndex with value 0 is associated with the serving cell, while CORESETPoolIndex with value 1 is associated with the non-serving cell.

Conclusion

Proposal 1: SSB index from a non-serving cell is configured in QCL-info, and the corresponding information for the SSB is configured by SSB-InfoNcell-r17 and SSB-Configuration-r17 provided in Table 2.

Proposal 2: In inter-cell multi-TRP operation, the CORESETPoolIndex with value 0 is associated with the serving cell, while CORESETPoolIndex with value 1 is associated with the non-serving cell.

References

- RP-202024  Revised WID proposal FeMIMO.

- 3GPP RAN1 #103-e Chairman Notes

- 3GPP RAN1 #104-e Chairman Notes

- 3GPP RAN1 #104bis-e Chairman Notes

- 3GPP RAN1 #105-e Chairman Notes