3GPP TSG RAN WG2 Meeting# 116 electronic   	                            R2-2110346

Online, November 1 – 12, 2021

Agenda Item:	8.1.3.1

Source: 	Sony

Title:	MBS Interest indication details

Document for:	Discussion

# Introduction

We discuss that MBMS interest indication should include if UE can simultaneously receive MBS and unicast on a particular frequency of interest.

# Discussion

UE capability, BWP and MBS Interest Indication (MII):

TS 38.300 running CR:

The UE in RRC_CONNECTED state may send MBS Interest Indication to the gNB for broadcast session. Editor’s note: It still needs to be discussed what the UE indicates in MBS Interest Indication, e.g. UE interested frequency, service etc.

UE will anyway indicate its UE capability to support MBS due to support of MRB and other L1/L2 features for MBS. We think UE needs to inform the network if it can receive MBS and unicast simultaneously on a particular frequency. This may be due to interference in reception. In addition, whether it can be part of MII frequency list and the priority of MBS and unicast is sufficient needs to be discussed. In our understanding, the frequency list reported in MII should be where the UE can receive MBS either along with unicast or with/without unicast and then indicate priority of MBS and unicast.

This question was discussed in the email discussion #091 but we assume that there was a confusion on whether simultaneous reception is linked to dual connectivity or HO or unicast and multicast.

Proposal 1: RAN2 to agree that the frequency list reported in MII should be where the UE can receive MBS either along with unicast or with/without unicast i.e. based on supported BC.

MII contents discussed during the email discussion are:

- mbms-FreqList-r11 (a list of frequencies): MBS frequency(ies) of interest - mbms-Priority-r11 (1bit per UE): Reception priority between MBS reception and unicast reception - mbms-Services-r13 (a list of TMGI(s)): MBS service(es) of interest

The combination of above parameters should indicate the frequency of interest to the UE for receiving MBS service. It should also indicate if UE can perform simultaneous reception of MBS and unicast on a particular frequency.

In summary, we think the discussion should be if MBS priority should be per UE and instead should be per frequency.

Proposal 2: MII should additionally indicate if UE can perform simultaneous reception of MBS and unicast on a particular frequency i.e. whether the priority field is per frequency.

On MII RRC message, the email discussion concluded:

“whether MII is reported via UEAssistanceInformation or a new RRC message is dependent on the reply to the LS RAN2 sent to SA3, hence is not discussed at the moment”

We think RAN2 can still make a decision if we go ahead with a new RRC message and further details can be worked out later. We think a new RRC message is a clean solution.

Proposal 3: Define new RRC message for MBS Interest Indication.

# Conclusion

We propose RAN2 to discuss and agree on following proposals:

Proposal 1: RAN2 to agree that the frequency list reported in MII should be where the UE can receive MBS either along with unicast or with/without unicast i.e. based on supported BC.

Proposal 2: MII should additionally indicate if UE can perform simultaneous reception of MBS and unicast on a particular frequency i.e. whether the priority field is per frequency.

Proposal 3: Define new RRC message for MBS Interest Indication.

# References

[1]	RP-201038: New WID: NR Multicast and Broadcast Services, Huawei

[2] 	RAN1 chair notes

[3]	R2-21xxxx: [Post115-e][091][MBS] Email disc CP aspects