3GPP TSG-RAN2 #116-e	R2-2110585

Elbonia, November 01 – 12, 2021

Agenda item:		8.12.2

Source:	LG Electronics

Title: 	Discussion on SI indication for camping restrictions for RedCap UEs

Document for:	Discussion and Decision

1.	Introduction

In this contribution, we would like to discuss on SI indication for camping restrictions for RedCap UEs.

2.	Discussion

In RAN2#114-e meeting, RAN2 agreed as follows, and RAN2 is working on the draft CR.

- SIB1 (not MIB) indicates cell barring for 1 Rx branch and 2 Rx branches separately for RedCap UEs. Further details of the solution are FFS

Some companies want to introduce “cellBarredRedCap2Rx” and “cellBarredRedCap1Rx” while some companies prefer to use “cellBarredRedCap” rather than “cellBarredRedCap2Rx”. We suggest to specify ““cellBarredRedCap” to indicate whether the cell is barred or not to RedCap UEs and whether the cell is barred or not to both 1Rx and 2Rx RedCap UEs.

The presentation in TS 38.331 is suggested as below.

- cellBarredRedcap      		ENUMERATED {true}

- cellBarredRedCap1Rx		ENUMERATED {true}

Depending on the set

- “00” means the cell is barred for RedCap UEs with 1Rx branch and 2 Rx branches

- “10” means the cell is not barred for RedCap UEs with 2Rx branches, and the cell is barred for RedCap UEs with 1Rx branch, and

- “11” means the cell is not barred for all RedCap UEs.

cellBarredRedCap1Rx is specified only if cellBarredRedCap is set to “true”. If cellBarredRedCap1Rx is not present, it means RedCap UEs with 2Rx branches is allowed to access the cell while RedCap UEs with 1Rx branch is not. If cellBarredRedCap1Rx is set to “true”, it means all RedCap UEs with 1Rx branch and 2Rx branches are allowed to access the cell.

Proposal. Specify “cellBarredRedCap” and “cellBarredRedCap1Rx” to indicate camping restrictions for RedCap UEs with 1Rx branch and RedCap UEs with 2Rx branches in SIB1.

3.	Conclusions

In this contribution, we have following proposal:

Proposal. Specify “cellBarredRedCap” and “cellBarredRedCap1Rx” to indicate camping restrictions for RedCap UEs with 1Rx branch and RedCap UEs with 2Rx branches in SIB1.