3GPP RAN WG2 Meeting #116e	R2-2110863

eMeeting November 1st – 12th, 2021

Agenda Item:	8.10.1

Source:	InterDigital

Title:	MAC open issues in NTN

Document for:	Discussion, Decision

# Introduction

In the Rel-17 NTN WID the following objectives have been listed for MAC, where text highlighted in green indicates concluded topics, and those in yellow indicating open aspects requiring agreement in RAN2:

- MAC

  - Random access:

    - Definition of an offset for the start of the ra-ResponseWindow for NTN.

    - Introduction of an offset for the start of the ra-ContentionResolutionTimer to resolve Random access contention

    - Solutions for resolving preamble ambiguity and extension of RAR window.

    - Adaptation for Msg-3 scheduling

      - Only for the case with pre-compensation of timing and frequency offset at UE side)

  - Enhancement on UL scheduling to reduce scheduling latency.

  - DRX:

    - If HARQ feedback is enabled, introduction of offset for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL.

    - If HARQ is turned off per HARQ process, adaptions in HARQ procedure

  - Scheduling Request: Extension of the value range of sr-ProhibitTimer

This contribution lists remaining open issues identified by RAN2 and the NTN MAC CR rapporteur.

# Open Issues

## TA reporting

Open Issue 1: Content of UE specific TA pre-compensation report

In RAN2#114e it was agreed that if enabled by the network, the UE reports information about UE specific TA pre-compensation at the random access procedure (MSGA/MSG3 or MSG5) using a MAC CE. However, the exact information and frequency of reports depend on RAN1 outcome. An LS was approved in [10] asking RAN1 to provide input on the exact content and frequency of UE reporting of information about the UE specific TA pre-compensation, however RAN1 response LS has not provided an answer.

In RAN2#115e, RAN2 agreed to the following:

- The content of UE specific TA pre-compensation reported in RA procedure using MAC CE is UE specific TA (this can be revisited after receiving RAN1 response).

In RAN1#106bise, RAN1 has reached a number of agreements regarding TA content and granularity, and is preparing a response LS to RAN2. RAN2 may wait for RAN1 LS reponse before conclusion on at least content and frequency of TA reporting for UE-specific TA precompensation.

Open Issue 2: UE location information for TA reporting pruposes:

In RAN2#115e, the following agreements were reached:

- Under the work assumption "the UE location information cannot be reported in connected mode", the content of UE specific TA reported in connected mode is UE specific TA pre-compensation(for the details of the TA value, confirmation from RAN1 is needed).

- Under the work assumption "the UE location information can be reported in connected mode", for TA reporting purposes in connected mode, the network can configure the UE to send either the UE specific TA pre-compensation (for the details of the TA value, confirmation from RAN1 is needed) or the UE location information

An LS was sent to SA3 about the need for NTN specific user consent for obtaining UE location for gNB. RAN2 to wait until SA3 response prior to confirmation UE location can be used for TA reporting purposes and associated specification implementation.

Open Issue 3: Enable/disable TA reporting during RACH in connected mde:

In RAN2#115e it was agreed that UE specific TA reporting during RACH is enabled/disabled by SI, however it is currently FFS whether this also applies for RACH in connected mode.

RAN2 to confirm whether enable/disable indication in SI also applies for RACH in connected mode.

Open Issue 4: Signalling of UE specific TA in connected mode

RAN2 has previously agreed that MAC CE is used to report UE specific TA in RACH procedure, however it is currently a working assumption that MAC CE is also used for TA reporting in connected mode (if content of TA report is TA value, if UE location information RRC signalling is used).

RAN2 to either: 1) confirm working assumption and revisit if RAN1 decides that required TA information in IDLE vs CONN is different, or 2) wait until RAN1 response LS on UE specific TA report content and frequency.

Open Issue 5: Avoidance of transmiting UE-specific TA MAC CE in multiple messages

The SI parameter in draft RRC CR to enable/disable UE-specific TA reporting in RACH currently includes only value "enabled", however RAN2 agreement states that UE-specific TA report can be sent in "MsgA/Msg3 or Msg5".

RAN2 to discuss possible options (e.g. additional indication in SI, restriction of UL grant size) to ensure that UE-specific TA is not sent in multiple messages e.g. Msg3 and Msg5.

Open Issue 6: Priority of UE-specific TA MAC CE

RAN2 to discuss priority of UE-specific MAC CE for MAC section 5.4.3.1.3, ideally minimizing impact to exiting prioritization.

## SR Prohibit Timer

Open Issue 7: Details of SR-Prohibit Timer extension.

In RAN2#113bis-e [8] it was agreed that the sr-ProhibitTimer length is to be extended to accommodate additional propagation delay in NTN.

RAN2 to confirm details of how to extend SR prohibit timer.

## Other aspects

Open Issue 8: Support for blind retransmission with disabled HARQ

The following agreements regarding support of blind retransmission have been captured as FFS in RAN2#113e and RAN2#115e respectively:

- FFS: method(s) to support blind retransmission for HARQ processes with HARQ feedback disabled

- For HARQ state B, FFS to run drx-RetransmissionTimerUL for blind UL retransmission

RAN2 to further discuss possible enhancements to support blind retransmission in NTN.

Open Issue 9: Identification of non-terrestrial network cells

In current version of MAC CR, there are several references to “non-terrestrial network” in order to trigger specific UE behaviour (e.g. timer offset). The following agreements have been made regarding indication of network type:

Agreement RAN2#111e

- The network type (i.e. TN or NTN) should be known to UE. FFS whether to achieve this in an implicit or explicit way.

Agreement RAN2#113e

- RAN2 thinks that a UE needs to know whether the network is a TN or NTN no later than SIB1 reception

RAN2 to confirm how UE detects cell originates from a non-terrestrial network.

Open Issue 10: Extension of ra-ReponseWindow and msgB-ResponseWindow

In RAN2#112-e, it was agreed that whether to extend the ra-ReponseWindow and msgB-ResponseWindow relies on the accuracy of UE-gNB RTT estimation, specifically:

- If the start of the ra-ResponseWindow and msgB-ResponseWindow is accurately compensated by UE-gNB RTT, ra-ResponseWindow and msgB-ResponseWindow are not extended in LEO/GEO.

In RAN2#106e a number of different options were evaluated to avoid UE over-precompesating it’s TA during RACH procedure, including definition of a TA margin. In RAN1#106bise, it was concluded that a TA margin is unnecessary, and a number of additional paremetrers including Ccommon TA drift rate and Common TA drift rate variation were defined. Additionally, RAN1 has agreed to a validity timer for Common TA and satellite ephemeris to avoid outdated information.

Considering RAN1 has concluded a TA margin is not necessary, RAN2 to confirm that UE estimate of UE-gNB RTT is sufficiently accurate to not require ra-ResponseWindow and msgB-ReponseWindow extension.

# Conclusions

Based on the above open issues, MAC CR rapporteur proposes the following way forward on open issues:

Proposal 1	RAN2 to agree on details regarding sr-ProhibitTimer extension in RAN2#116e.

Proposal 2	RAN2 to confirm an extension to ra-ReponseWindow and msgB-ResponseWindow is not necessary.

Proposal 3	RAN2 to agree on details regarding how UE determines a cell originates from a non-terrestrial network.

Proposal 4	RAN2 to prioritize conclusion of remaining UE-specific TA reporting open aspects.

# References

R2-2100001 Report of 3GPP TSG RAN2#112-e meeting – ETSI MCC

R2-2008701 Report of 3GPP TSG RAN2#111-e meeting – ETSI MCC

Final_Minutes_report_RAN1#103-e_v100

Final_Minutes_report_RAN1#102-e_v100

R2-2102601Report of 3GPP TSG RAN2#113-e meeting – ETSI MCC

[Post113-e][106][NTN] MAC aspects (Huawei)

[Post113-e][105][NTN] MAC Running CR (InterDigital)

Draft_RAN2_113bis-e_Meeting_Report_v1

Draft_Minutes_report_RAN1#104bis-e_v010

R2-2104376 LS on TA precompensation – RAN2

Final_Minutes_report_RAN1#104-e_v100

Draft_Minutes_report_RAN1#105-e_v010

R2-2106924 Reply LS on TA precompensation – RAN1

Draft_RAN2_114-e_Meeting_Report_v1

Draft_R2-115e_Meeting_Report_v1

Final_Minutes_report_RAN1#106-e_v300

Chair’s notes RAN1#106bis-e 8.4 v06

# MAC Agreements

The following RAN2 user plane agreements have been capture in NTN so far [1] [2] [5] [8] [14] [15]:

## RAN2#115e Agreements

UE specific TA reporting during RACH procedure is enabled/disabled by SI (FFS for RACH in connected mode) In the MAC specification section 5.1.5, delay the start of ra-ContentionResolutionTimer by the UE-gNB RTT (i.e. sum of UE's TA and K_mac) The content of UE specific TA pre-compensation reported in RA procedure using MAC CE is UE specific TA (this can be revisited after receiving RAN1 response). Reporting on the information about UE specific TA in connected mode is supported, FFS via RRC signalling or MAC CE Event-triggers for reporting on the information about UE specific TA in connected mode is supported. FFS on the details. Confirmation by RAN1 is also needed If configured, the UE shall report information of the UE specific TA pre-compensation to the target cell during the random access. FFS if a new indication in RRC reconfiguration with sync is needed or not (besides the SIB indication carried in HO command on whether TA report is enabled/disabled in the target cell). Information about UE specific TA pre-compensation is not reported in RA procedures triggered due to “Request for Other SI” The event-triggers for reporting information about UE specific TA are based on TA values (confirmation from RAN1 is needed) A TA offset threshold can be used for event-triggered reporting, at least the offset threshold can be between current information about UE specific TA and the last successfully reported information about UE specific TA The event-triggers for reporting information about UE specific TA based on time threshold is not supported in NTN. No new indication in RRC reconfiguration with sync is needed to configure the UE to report information about UE specific TA in handover procedure (besides the SIB indication carried in HO command on whether TA report is enabled/disabled in the target cell). Under the work assumption "the UE location information cannot be reported in connected mode", the content of UE specific TA reported in connected mode is UE specific TA pre-compensation(for the details of the TA value, confirmation from RAN1 is needed). If the reported content of information about UE specific TA is UE location information in connected mode, RRC signalling is used to report. Under the work assumption "the UE location information can be reported in connected mode", for TA reporting purposes in connected mode, the network can configure the UE to send either the UE specific TA pre-compensation (for the details of the TA value, confirmation from RAN1 is needed) or the UE location information Working Assumption: If the reported content of information about UE specific TA is TA pre-compensation value in connected mode, MAC CE is used to report Confirm the RAN2 working assumption that offset to drx-HARQ-RTT-TimerUL length is equal to UE-gNB RTT (i.e. sum on UE's TA and K_mac). Confirm the RAN2 working assumption that for HARQ processes with DL HARQ feedback enabled, the drx-HARQ-RTT-TimerDL length is increased by an offset equal to UE-gNB RTT (i.e. sum on UE's TA and K_mac). No new LCP restrictions are introduced for exisiting UL MAC CEs (if new MAC CEs will be introduced we can revisit this) For dynamic grants, each LCH can optionally be semi statically configured (by RRC) to be mapped to one or more HARQ processes (FFS if it's possible to map to more than one HARQ process/ process type. FFS on mapping method). If there is no RRC configuration for this, this mapping has no effect (legacy behaviour applies). For at least dynamic grants, the network may optionally configure an UL HARQ retransmission state per HARQ process. Two UL HARQ retransmission states are defined in NTN: HARQ state A and HARQ state B (FFS whether "HARQ state A" and "HARQ state B" should be renamed) HARQ state A/B are defined as follows: - HARQ state A: length of drx-HARQ-RTT-TimerUL is extended by UE-gNB RTT (i.e. UE PDCCH monitoring is optimized to support UL retransmission grant based on UL decoding result). - HARQ state B: drx-HARQ-RTT-TimerUL is not started. Configuration of UL HARQ retransmission state is semi-static, signalled via RRC, and the decision and criteria to configure UL HARQ retransmission state is under network control. For dynamic grants, each LCH can be optionally mapped to an UL HARQ retransmission state via semi-static RRC configuration. If there is no configuration, the mapping has no effect (legacy behaviour applies). If HARQ process has not been configured with an UL HARQ retransmission state, new LCH mapping rule has no effect (i.e. UE applies legacy behaviour). The following behaviours are supported for drx-HARQ-RTT-TimerUL in NTN per HARQ process: 1) Timer length is extended by offset; 2) Timer disabled (i.e. not started) UE determines drx-HARQ-RTT-TimerUL behaviour per HARQ process based on configured UL HARQ retransmission state. For HARQ process(es) not configured with an UL HARQ retransmission state, drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL behave as per legacy. An UL HARQ retransmission state is configured per HARQ process to support new LCH mapping restriction and proper configuration of drx-HARQ-RTT-TimerUL behaviour. The network may consider delay and reliability characteristics of ongoing services when choosing to configure an UL HARQ retransmission state. Alternative naming for HARQ state A/B can be further considered during stage 3, however UE behaviour in each state should be defined in specification. RAN2 understanding is that UE behaviour in HARQ state A (i.e. extending the drx-HARQ-RTT-TimerUL by UE-gNB RTT) best supports reception of UL retransmission grant based on UL decoding result. (No RAN2 specification impact) RAN2 understanding is that UE behaviour in HARQ state B (i.e. not starting drx-HARQ-RTT-TimerUL) best supports no UL retransmission and/or blind UL retransmission. (No RAN2 specification impact) For HARQ state B, FFS to run drx-RetransmissionTimerUL for blind UL retransmission UE configured with an UL HARQ retransmission state (i.e. A or B) will always act as indicated in a grant/assignment provided during a valid occasion (i.e. subject to legacy restrictions in e.g. MAC and RAN1 specifications). (No RAN2 specification impact)

## RAN2#114e Agreements

If enabled by the network, the UE reports information about UE specific TA pre-compensation at the random access procedure (MSGA/MSG3 or MSG5) using a MAC CE. Actual content is FFS and also depends on further RAN1 input (we can revise this whole agreement if RAN1 come to a different conclusion in terms of what needs to be conveyed to the NW) The following options are supported for drx-HARQ-RTT-TimerUL in NTN per HARQ process: 1) Timer length is extended by offset; 2) Timer set to zero and/or 3) Timer disabled (i.e. not started). FFS if this is based on explicit configuration or not. We can also come back to see whether both 2 and 3 are needed. RAN2 working assumption: Offset for drx-HARQ-RTT-TimerUL is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it). drx-RetransmissionTimerDL timer length is not extended in NTN The drx-HARQ-RTT-TimerUL behaviour applied for each HARQ process is up to the network (e.g. to support NW scheduling strategy to avoid HARQ stalling). RAN2 Working Assumption: No new CG-specific LCP restriction is introduced for NTN. If a new LCP restriction is agreed for dynamic grant, the proposal does not preclude future discussion on whether it may also apply to configured grant Repetition transmission based HARQ retransmission is always allowed and is explicitly indicated per HARQ process via DCI (as in legacy). At least the following options for LCP in NTN are further studied: 1) allowedPHY-PriorityIndex is re-used; and 2) A new LCP restriction is introduced to map LCH to one or more HARQ process(es). FFS if HARQ processes can be classified as having retransmission “enabled” or “disabled” in this case.

## RAN2#113bis-e Agreements

Legacy mechanism for RA type selection based on RSRP threshold is the baseline for NTN. Optimizations can still be suggested, showing the gain (in any case, any method needs to be combined with RSRP based approach) Reuse legacy RA type switching mechanism Extend the timer length of sr-ProhibitTimer (FFS on the details) RAN2 wait for RAN1’s feedback on UE obtaining UE-gNB RTT. RAN2 wait for RAN1’s progress and postpone the discussion on how to broadcast parameters, if any, for TA pre-compensation. RAN2 send an LS to RAN1, focusing on below aspects: - Ask RAN1 to prioritize the TA pre-compensation work on whether and/or what parameters to broadcast for TA pre-compensation, and when broadcasted, how often the broadcasted parameters are expected to change over time; - RAN2 has agreed to use UE-gNB RTT as the offset to start some UP timers (e.g. drx-HARQ-RTT-TimerDL). Ask RAN1 to provide inputs on (i) how UE acquires UE-gNB RTT and (ii) what additional information needs to be broadcasted other than that for TA pre-compensation, if any. At least for uplink scheduling adaptations, the UE may report information about the UE specific TA pre-compensation. The exact information and frequency of reports depend on RAN1 outcome. FFS on when/how to report. It is FFS whether the UE reports the UE specific TA pre-compensation at the RACH procedure (MSG3 or MSG5) using a MAC CE. Actual content is FFS and also depends on further RAN1 input. Configurability is FFS It is NW scheduling strategy to avoid NTN UE in HARQ stalling state. From RAN2 perspective, the NW can continuously schedule the UE using one or a combination of scheduling strategies, such as without HARQ retransmissions, or with blind retransmissions, or with HARQ retransmissions based on DL HARQ feedback (or UL decoding result). RAN2 confirms that in NTN if the UE is in DRX Active Time for any reason, the UE should monitor the PDCCH regardless of whether drx-HARQ-RTT-TimerUL or drx-HARQ-RTT-TimerDL is running or not. No specification change is needed. RAN2 confirms that in NTN using the value= “zero” for drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL is possible. No specification change is needed. In NTN, The drx-HARQ-RTT-TimerUL is configured per UE DRX group and the behaviour can be configured per HARQ process. FFS the different behaviours and how to indicate the behaviour to the UE and the number of behaviours (e.g., two or more behaviours). LCP restrictions should be further considered for an UL HARQ process in NTN. FFS if no further LCP restrictions are needed, or if (R16) existing LCP restrictions can be re-used or if new LCP restriction shall be defined for this purpose.

## RAN2#113-e Agreements

Both Type 1 and Type 2 configured grant are feasible in NTN. From RAN2’s perspective, no need to modify parameter periodicity of IE ConfiguredGrantConfig to support NTN. No need to modify maxNrofConfiguredGrantConfig-r16 and maxNrofConfiguredGrantConfigMAC-r16 to support NTN. UE in NTN can have both 2-step RACH and configured grant configurations at the same time. For HARQ processes with DL HARQ feedback disabled, drx-HARQ-RTT-TimerDL is not started. FFS: method(s) to support blind retransmission for HARQ processes with HARQ feedback disabled. From RAN2 perspective, for HARQ processes where gNB can sends UL grant without waiting for decoding result of previous PUSCH transmission, no new network scheduling restrictions are introduced to schedule subsequent grants (i.e. up to network implementation. (Can come back if we don't find an agreement on p8) For HARQ processes with DL HARQ feedback enabled, drx-HARQ-RTT-TimerDL length is increased by offset (i.e. existing values within value range increased by offset). RAN2 working assumption: offset is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it)

## RAN2#112-e Agreements

RAN2 working assumption (for RRC idle. FFS for Inactive/Connected): Rel-17 UE with pre-compensation capability obtains UE specific UE-gNB RTT based on its GNSS in LEO/GEO. FFS how this is calculated and what/if anything needs to be broadcasted for the different pre-compensation methods (e.g. common TA) to help the UE to obtain the full UE-gNB RTT. If the UE-gNB RTT is pre-compensated, preamble ambiguity is not an issue in Rel-17 NTN (i.e. no enhancements are necessary). FFS how and by whom the possibly multiple components of UE-gNB RTT are pre-compensated From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to gNB implementation to ensure sufficient time on UE side for the Msg3 transmission. For UE with pre-compensation capability (at least for the HARQ-feedback enabled case. FFS for HARQ-feedback disabled, if supported), drx-HARQ-RTT-TimerDL is offset by UE-specific RTT (UE-gNB delay) in LEO/GEO. FFS if offset is applied to: 1) the start of the timers or 2) the timer value range (i.e. existing values within value range increased by offset) From RAN2 perspective, for dynamic grant, one possibility for "enabling"/"disabling" HARQ uplink retransmission at UE transmitter is without introducing an additional mechanism (i.e. gNB can send grant with NDI not toggled/toggled without waiting for decoding result of previous PUSCH transmission). FFS on the handling of RTT timers. Other solutions for enabling/disabling HARQ UL reTX are not precluded If the start of the ra-ResponseWindow and msgB-ResponseWindow is accurately compensated by UE-gNB RTT, ra-ResponseWindow and msgB-ResponseWindow are not extended in LEO/GEO. At least the following are FFS in Rel-17 NTN: - Report UE-calculated TA in e.g. msg3/msg5/msgA - Enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH - LCP impact caused by disabling HARQ UL retransmission RAN2 decision on starting ra-ContentionResolutionTimer, ra-ResponseWindow and msgB-ResponseWindow is postponed until further progress in RAN1 regarding UE pre-compensation method and TA estimation accuracy.

## RAN2#111-e Agreements

From RAN2 perspective, an offset is applied to the start of ra-ResponseWindow in NTN for both LEO and GEO scenarios. An offset to the start of the ra-ContentionResolutionTimer is introduced for both LEO and GEO scenarios. Modification of drx-LongCycleStartOffset, drx-StartOffset, drx-ShortCycle, drx-ShortCycleTimer, drx-onDurationTimer, drx-SlotOffset and drx-InactivityTimer is not needed in Rel-17 NTN. From a RAN2 perspective, for DL, HARQ feedback can be enabled/disabled in Rel-17 NTN, but HARQ processes remain configured. The criteria and decision to enable/disable HARQ feedback is under network control and is signalled to the UE via RRC in a semi-static manner. FFS for UL At least the following methods to enhance UL scheduling are further studied in NTN: configured grant and BSR over 2-step RACH. (other solutions to enhance UL scheduling are not precluded) Both 2-step and 4-step RACH are supported in Rel-17 NTN. FFS enhancements to RACH to accommodate the NTN environment.