3GPP TSG-RAN WG2 Meeting #116 Electronic	R2-2110954

# Elbonia, 01 – 12 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | Num | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of L2 architecture figure for multicast session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current running CR to 38.300 for NR MBS, the labels for ingress points depicted in the Figure 16.x.3-1: Downlink Layer 2 Architecture for Multicast Session, under Section 16.x.3, are incorrect for the following reasons: - The term “Multicast PDU session” is not a valid term. TS 23.247 defines only Multicast MBS session and Unicast PDU session. According to TS 23.247 the Multicast MBS session is over N3mb reference point. - Saying “Unicast PDU session for Multicast service” in the same figure that shows a multicast service over MRB is not correct because TS 23.247, Section 4.1 says: “For MBS multicast communication, if the NG-RAN node supports 5G MBS, the network shall use the 5GC Shared MBS traffic delivery method for MBS data transmission”. Unicast PDU session for multicast service is possible only in a gNB not supporting MBS service. Also, it is not possible for the same gNB to multiplex both a MRB for multicast service and a DRB for multicast service for the same UE. A gNB can only multiplex a MRB for multicast service and a DRB for non-multicast service for a UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following are the changes: - Changed “Multicast PDU session for Multicast service” to “Shared N3mb” - Changed “Unicast PDU session for Multicast service” to “Unicast N3” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect terminologies and misalignment with TS 23.247 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.x.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a draftCR and is based on the running CR in R2-2108978. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 16.x.3	Protocol Architecture

<…>

Figure 16.x.3-1: Downlink Layer 2 Architecture for Multicast Session

End of Changes