3GPP TSG-RAN WG2 Meeting#116-e		       					       										R2-2111010

Electronic, 1st - 12th November 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Further consideration on slice specific cell reselection

Agenda item:		8.8.2

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some understanding on slice info in system information and RRCRelease message, the candidate solutions and the slice availability in system information and RRCRelease message.

- Discussion

  - The ASN.1 structure for the slice info

In the existing SIB2, SIB4 and RRCRelease, the reselection priority is provided in a per frequency manner [1]. When introducing the slice info, we understand the same structure applies and the slice info, i.e. the supported slice and the corresponding slice specific reselection priority, should also be provided in a per frequency manner as shown in the following table.

| Slice info |  |  |  |
| --- | --- | --- | --- |
| For the serving frequency |  |  |  |
|  | Slice id-1/Slice Group Id-1 (Optional) | Slice specific Freq-priority (Optional) | PCI list (FFS) |
|  | Slice id-2/Slice Group Id-2 (Optional) | Slice specific Freq-priority (Optional) | PCI list (FFS) |
|  | .... | … | … |
| For inter-frequency |  |  |  |
| Frequency 1 | Slice id-1/Slice Group Id-1(Optional) | Slice specific Freq-priority (Optional) | PCI list (FFS) |
|  | Slice id-2/Slice Group Id-2 (Optional) | Slice specific Freq-priority (Optional) | PCI list (FFS) |
|  | ... | ... | … |
| Frequency 2 | Slice id-1/Slice Group Id-1(Optional) | Slice specific Freq-priority (Optional) | PCI list (FFS) |
|  | Slice id-2/Slice Group Id-2 (Optional) | Slice specific Freq-priority (Optional) | PCI list (FFS) |
|  | ... | ... | … |
| ... |  |  |  |

Proposal 1: The slice info, i.e. the supported slice and the corresponding slice specific reselection priority, should be provided in a per-frequency manner following the existing structure in SIB2, SIB4 and RRCRelease message.

In addition, we understand it should be allowed that NW provides the supported slice of the serving and neighbor frequencies but not willing to configure slice specific reselection priority value.

Proposal 2: NW should be allowed to provide the supported slice/slice group of the serving and neighbor frequencies without configuring specific reselection priority value for a slice/slice group supported in the serving or neighbor frequencies in SIB2, SIB4 and the RRCRelease message.

| Slice info |  |  |
| --- | --- | --- |
| For the serving frequency F0 |  |  |
|  | Slice id-1 | Slice specific priority: 6 |
|  | Slice id-2 | / |
| For inter-frequency |  |  |
| Frequency 1 | Slice id-1 | / |
|  | Slice id-2 | Slice specific priority: 6 |
| Frequency 2 | Slice id-1 | Slice specific priority: 4 |
|  | Slice id-2 | Slice specific priority: 2 |
| ... |  |  |

For the case when network provide the supported slice or slice group for a frequency without providing the slice specific frequency priority but slice specific reselection priority has been configured for other frequencies supporting this slice or slice group, this frequency without slice specific priority value should be treated as the lowest priority frequency for this slice or slice group. In the example given above, the priority for all these frequencies would be:

For slice id -1: F0 (6) > F2 (4) > F1 (no slice specific priority value configured)

For slice id -2: F1 (6) > F2 (2) > F0 (no slice specific priority value configured)

Proposal 3: For the case when network provide the supported slice or slice group for a frequency without providing the slice specific frequency priority but slice specific reselection priority has been configured for other frequencies supporting this slice or slice group, this frequency without slice specific priority value should be treated as the lowest priority frequency for this slice or slice group.

In RRCRelease message, we have the following note to allow NW configure frequencies not configured by system information.

NOTE 3: The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

For the slice info in RRCRelease message, we understand the same principle applies and NW is allowed to configure slice info for frequencies not configured by system information.

Proposal 4: In RRCRelease message, the network may provide slice info for frequencies not configured by system information.

  - Other solutions useful in slice specific reselection

The following candidate solutions have been raised during email discussion [Post114-e] [251] [Slicing] Solution direction details for slice priorities in cell reselection (Lenovo) [2]:

- Option 4: First select the highest priority slice, and use the corresponding frequency priority to this slice to perform cell reselection.Repeat the procedure with all slices if the highest ranked cell does not support the selected slice.

- Option 5: Consider the frequency which provides the most number slices as the highest priority.

- Option 6: Difference compared to Option 4: Adjust frequency priority based on actually supported slice(s) in best ranked cell, to avoid wrongly setting a too high frequency priority corresponding to unsupported slice.

- Option 7: For each frequency, UE assigns a frequency priority equal to the highest frequency priority among those identified slices.

The above options mainly focus on the determination of the frequency priority and can be categorized into three directions:

- Direction 1: Determine the frequency priority based on the selected slice and the corresponding slice specific priority => Option 4 and Option 6

- Direction 2: Determine the frequency priority based on the number of supported allowed S-NSSAIs => Option 5

- Direction 3: Determine the frequency priority based on the highest slice specific priority value configured for a frequency => Option 7

With option 4 already agreed for further work, focusing on how to determine the frequency priority based on the configured slice specific value, there seems to be no need to have option 6 and option 7, which also pays much attention to decide the the frequency priority based on the configured slice specific value.

However, option 5 would be useful in a special case where NW only provides the supported slice/slice group of the serving frequency and neighbor frequencies but not willing to configure slice specific reselection priority values for these frequencies.

Thus, we understand solution 5 is useful for the case when the supported slice/slice group for each frequency is provided without slice specific reselection priority for any slice or any frequency.

Proposal 5: Solution 5 is useful for the case when the supported slice/slice group for each frequency is provided without slice specific reselection priority for any slice or any frequency and should also be selected for further work:

Solution 5:

- Step 1: The UE will consider the frequency priority in cell reselection based on the number of supported slices among UE’s intended slices (i.e. allowed S-NSSAIs). That is, the frequency that supports the maximum number of slices among UE’s intended slices has the highest priority in cell reselection. The frequency that supports the second most slices among UE’s intended slices has the second highest priority in cell reselection, and so on.

- Step 2: If more than one frequency supports the same number of slices among UE’s intended slices, the UE can treat them with equal priority, or further consider the existing absolute cell reselection frequency priority if provided.

- Step 3: The UE performs the legacy cell reselection (specified in TS 38.304) following the priority assigned based on the above rules.

- Conclusion and proposals

With the above analysis, we have the following conclusions and proposals:

Proposal 1: The slice info, i.e. the supported slice and the corresponding slice specific reselection priority, should be provided in a per-frequency manner following the existing structure in SIB2, SIB4 and RRCRelease message.

In addition, we understand it should be allowed that NW provides the supported slice of the serving and neighbor frequencies but not willing to configure slice specific reselection priority value.

Proposal 2: NW should be allowed to provide the supported slice/slice group of the serving and neighbor frequencies without configuring specific reselection priority value for a slice/slice group supported in the serving or neighbor frequencies in SIB2, SIB4 and the RRCRelease message.

Proposal 3: For the case when network provide the supported slice or slice group for a frequency without providing the slice specific frequency priority but slice specific reselection priority has been configured for other frequencies supporting this slice or slice group, this frequency without slice specific priority value should be treated as the lowest priority frequency for this slice or slice group.

Proposal 4: In RRCRelease message, the network may provide slice info for frequencies not configured by system information.

Proposal 5: Solution 5 is useful for the case when the supported slice/slice group for each frequency is provided without slice specific reselection priority for any slice or any frequency and should also be selected for further work:

Solution 5:

- Step 1: The UE will consider the frequency priority in cell reselection based on the number of supported slices among UE’s intended slices (i.e. allowed S-NSSAIs). That is, the frequency that supports the maximum number of slices among UE’s intended slices has the highest priority in cell reselection. The frequency that supports the second most slices among UE’s intended slices has the second highest priority in cell reselection, and so on.

- Step 2: If more than one frequency supports the same number of slices among UE’s intended slices, the UE can treat them with equal priority, or further consider the existing absolute cell reselection frequency priority if provided.

- Step 3: The UE performs the legacy cell reselection (specified in TS 38.304) following the priority assigned based on the above rules.

- References

- TS 38.331 V16.6.0

- Report of [Post114-e] [251] [Slicing] Solution direction details for slice priorities in cell reselection (Lenovo)

- Text proposal

#### –	SliceInfo

The SliceInfo indicates the slice information to assist cell reselection.

SliceInfo-r17::=         SEQUENCE {

sliceIdentityList-r17       SEQUENCE (SIZE (1.. maxSlicesConfigured)) OF SliceIdentity,

cellReselectionPriority     CellReselectionPriority          OPTIONAL,  -- Need R

cellReselectionSubPriority  CellReselectionSubPriority        OPTIONAL  -- Need R

sliceCellListNR-r17 			SliceCellListNR-r17					OPTIONAL,  -- Need R

}

SliceCellListNR-r17 ::=          SEQUENCE (SIZE (1..maxCellSlice)) OF PhysCellId

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=                            SEQUENCE {

cellReselectionInfoCommon           SEQUENCE {

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)          OPTIONAL,       -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                     OPTIONAL,       -- Need S

rangeToBestCell                     RangeToBestCell                                 OPTIONAL,       -- Need R

q-Hyst                              ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars           SEQUENCE {

mobilityStateParameters             MobilityStateParameters,

q-HystSF                        SEQUENCE {

sf-Medium                       ENUMERATED {dB-6, dB-4, dB-2, dB0},

sf-High                         ENUMERATED {dB-6, dB-4, dB-2, dB0}

}

}                                                                                   OPTIONAL,       -- Need R

...

},

cellReselectionServingFreqInfo      SEQUENCE {

s-NonIntraSearchP                   ReselectionThreshold                            OPTIONAL,       -- Need S

s-NonIntraSearchQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need S

threshServingLowP                   ReselectionThreshold,

threshServingLowQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need R

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                      OPTIONAL,       -- Need R

...,

[[

sliceInfo-r17						SliceInfoList-r17										OPTIONAL       -- Need R

]]

},

intraFreqCellReselectionInfo        SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                      OPTIONAL,       -- Need R

q-QualMin                           Q-QualMin                                       OPTIONAL,       -- Need S

s-IntraSearchP                      ReselectionThreshold,

s-IntraSearchQ                      ReselectionThresholdQ                           OPTIONAL,       -- Need S

t-ReselectionNR                     T-Reselection,

frequencyBandList                   MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need S

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need R

p-Max                               P-Max                                           OPTIONAL,       -- Need S

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S

ss-RSSI-Measurement                 SS-RSSI-Measurement                             OPTIONAL,       -- Need R

ssb-ToMeasure                       SSB-ToMeasure                                   OPTIONAL,       -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

...,

[[

t-ReselectionNR-SF                  SpeedStateScaleFactors                          OPTIONAL        -- Need N

]],

[[

smtc2-LP-r16                        SSB-MTC2-LP-r16                                 OPTIONAL,        -- Need R

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                    OPTIONAL         -- Cond SharedSpectrum

]]

},

...,

[[

relaxedMeasurement-r16              SEQUENCE {

lowMobilityEvaluation-r16           SEQUENCE {

s-SearchDeltaP-r16                  ENUMERATED {

dB3, dB6, dB9, dB12, dB15,

spare3, spare2, spare1},

t-SearchDeltaP-r16                  ENUMERATED {

s5, s10, s20, s30, s60, s120, s180,

s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                   OPTIONAL,       -- Need R

cellEdgeEvaluation-r16              SEQUENCE {

s-SearchThresholdP-r16              ReselectionThreshold,

s-SearchThresholdQ-r16              ReselectionThresholdQ                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition-r16     ENUMERATED {true}                               OPTIONAL,       -- Need R

highPriorityMeasRelax-r16           ENUMERATED {true}                               OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]]

}

SliceInfoList-r17 ::=         SEQUENCE (SIZE (1..maxSlicesConfigured)) OF SliceInfo-r17

RangeToBestCell    ::= Q-OffsetRange

-- TAG-SIB2-STOP

-- ASN1STOP

#### –	SIB4

SIB4 contains information relevant for inter-frequency cell re-selection (i.e. information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection), which can also be used for NR idle/inactive measurements. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SIB4 information element

-- ASN1START

-- TAG-SIB4-START

SIB4 ::=                            SEQUENCE {

interFreqCarrierFreqList            InterFreqCarrierFreqList,

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

interFreqCarrierFreqList-v1610      InterFreqCarrierFreqList-v1610              OPTIONAL   -- Need R

]]

}

InterFreqCarrierFreqList ::=        SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

InterFreqCarrierFreqList-v1610 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1610

InterFreqCarrierFreqInfo ::=        SEQUENCE {

dl-CarrierFreq                      ARFCN-ValueNR,

frequencyBandList                   MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Cond Mandatory

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                      OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                                 OPTIONAL,   -- Need S

smtc                                SSB-MTC                                                     OPTIONAL,   -- Need S

ssbSubcarrierSpacing                SubcarrierSpacing,

ssb-ToMeasure                       SSB-ToMeasure                                               OPTIONAL,   -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                         OPTIONAL,

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                   OPTIONAL,   -- Need S

p-Max                               P-Max                                                       OPTIONAL,   -- Need S

t-ReselectionNR                     T-Reselection,

t-ReselectionNR-SF                  SpeedStateScaleFactors                                      OPTIONAL,   -- Need S

threshX-HighP                       ReselectionThreshold,

threshX-LowP                        ReselectionThreshold,

threshX-Q                           SEQUENCE {

threshX-HighQ                       ReselectionThresholdQ,

threshX-LowQ                        ReselectionThresholdQ

}                                                                                               OPTIONAL,   -- Cond RSRQ

cellReselectionPriority             CellReselectionPriority                                     OPTIONAL,   -- Need R

cellReselectionSubPriority          CellReselectionSubPriority                                  OPTIONAL,   -- Need R

q-OffsetFreq                        Q-OffsetRange                                               DEFAULT dB0,

interFreqNeighCellList              InterFreqNeighCellList                                      OPTIONAL,   -- Need R

interFreqBlackCellList              InterFreqBlackCellList                                      OPTIONAL,   -- Need R

...,

[[

sliceInfo-r17						SliceInfoList-r17													OPTIONAL       -- Need R

]]

}

InterFreqCarrierFreqInfo-v1610 ::=  SEQUENCE {

interFreqNeighCellList-v1610        InterFreqNeighCellList-v1610                                OPTIONAL,    -- Need R

smtc2-LP-r16                        SSB-MTC2-LP-r16                                             OPTIONAL,    -- Need R

interFreqWhiteCellList-r16          InterFreqWhiteCellList-r16                                  OPTIONAL,    -- Cond SharedSpectrum2

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                                OPTIONAL,    -- Cond SharedSpectrum

interFreqCAG-CellList-r16           SEQUENCE (SIZE (1..maxPLMN)) OF InterFreqCAG-CellListPerPLMN-r16   OPTIONAL     -- Need R

}

InterFreqNeighCellList ::=          SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo

InterFreqNeighCellList-v1610 ::=    SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-v1610

InterFreqNeighCellInfo ::=          SEQUENCE {

physCellId                          PhysCellId,

q-OffsetCell                        Q-OffsetRange,

q-RxLevMinOffsetCell                INTEGER (1..8)                                              OPTIONAL,   -- Need R

q-RxLevMinOffsetCellSUL             INTEGER (1..8)                                              OPTIONAL,   -- Need R

q-QualMinOffsetCell                 INTEGER (1..8)                                              OPTIONAL,   -- Need R

...

}

InterFreqNeighCellInfo-v1610 ::=    SEQUENCE {

ssb-PositionQCL-r16                 SSB-PositionQCL-Relation-r16                                OPTIONAL    -- Cond SharedSpectrum2

}

InterFreqBlackCellList ::=          SEQUENCE (SIZE (1..maxCellBlack)) OF PCI-Range

InterFreqWhiteCellList-r16 ::=      SEQUENCE (SIZE (1..maxCellWhite)) OF PCI-Range

InterFreqCAG-CellListPerPLMN-r16 ::= SEQUENCE {

plmn-IdentityIndex-r16              INTEGER (1..maxPLMN),

cag-CellList-r16                    SEQUENCE (SIZE (1..maxCAG-Cell-r16)) OF PCI-Range

}

-- TAG-SIB4-STOP

-- ASN1STOP

#### –	RRCRelease

The RRCRelease message is used to command the release of an RRC connection or the suspension of the RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=                      SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcRelease                          RRCRelease-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCRelease-IEs ::=                  SEQUENCE {

redirectedCarrierInfo               RedirectedCarrierInfo                                                       OPTIONAL,   -- Need N

cellReselectionPriorities           CellReselectionPriorities                                                   OPTIONAL,   -- Need R

suspendConfig                       SuspendConfig                                                               OPTIONAL,   -- Need R

deprioritisationReq                 SEQUENCE {

deprioritisationType                ENUMERATED {frequency, nr},

deprioritisationTimer               ENUMERATED {min5, min10, min15, min30}

}                                                                                                               OPTIONAL,   -- Need N

lateNonCriticalExtension                OCTET STRING                                                        OPTIONAL,

nonCriticalExtension                    RRCRelease-v1540-IEs                                                OPTIONAL

}

RRCRelease-v1540-IEs ::=            SEQUENCE {

waitTime                           RejectWaitTime                OPTIONAL, -- Need N

nonCriticalExtension               RRCRelease-v1610-IEs          OPTIONAL

}

RRCRelease-v1610-IEs ::=            SEQUENCE {

voiceFallbackIndication-r16        ENUMERATED {true}                             OPTIONAL, -- Need N

measIdleConfig-r16                 SetupRelease {MeasIdleConfigDedicated-r16}    OPTIONAL, -- Need M

nonCriticalExtension               RRCRelease-v1650-IEs                          OPTIONAL

}

RRCRelease-v1650-IEs ::=            SEQUENCE {

mpsPriorityIndication-r16          ENUMERATED {true}                             OPTIONAL, -- Cond Redirection2

nonCriticalExtension               RRCRelease-v17xy-IEs                                   OPTIONAL

}

RRCRelease-v17xy-IEs ::=            SEQUENCE {

sliceCellReselectionPriorities-r17      SliceFreqPriorityListNR-r17      		OPTIONAL   -- Need R

nonCriticalExtension               SEQUENCE {}                                    OPTIONAL

}

SliceFreqPriorityListNR-r17 ::=              SEQUENCE (SIZE (1..maxFreq)) OF SliceFreqPriorityNR-r17

SliceFreqPriorityNR-r17 ::=                  SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

sliceInfoList-r17					SliceInfoList-r17  								OPTIONAL -- Need R

}

RedirectedCarrierInfo ::=           CHOICE {

nr                                  CarrierInfoNR,

eutra                               RedirectedCarrierInfo-EUTRA,

...

}

RedirectedCarrierInfo-EUTRA ::=     SEQUENCE {

eutraFrequency                      ARFCN-ValueEUTRA,

cnType                              ENUMERATED {epc,fiveGC}                                             OPTIONAL    -- Need N

}

CarrierInfoNR ::=                   SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

ssbSubcarrierSpacing                SubcarrierSpacing,

smtc                                SSB-MTC                                                             OPTIONAL,      -- Need S

...

}

SuspendConfig ::=                   SEQUENCE {

fullI-RNTI                          I-RNTI-Value,

shortI-RNTI                         ShortI-RNTI-Value,

ran-PagingCycle                     PagingCycle,

ran-NotificationAreaInfo            RAN-NotificationAreaInfo                                            OPTIONAL,   -- Need M

t380                                PeriodicRNAU-TimerValue                                             OPTIONAL,   -- Need R

nextHopChainingCount                NextHopChainingCount,

...

}

PeriodicRNAU-TimerValue ::=         ENUMERATED { min5, min10, min20, min30, min60, min120, min360, min720}

CellReselectionPriorities ::=       SEQUENCE {

freqPriorityListEUTRA               FreqPriorityListEUTRA                                               OPTIONAL,       -- Need M

freqPriorityListNR                  FreqPriorityListNR                                                  OPTIONAL,       -- Need M

t320                                ENUMERATED {min5, min10, min20, min30, min60, min120, min180, spare1} OPTIONAL,     -- Need R

...

}

PagingCycle ::=                     ENUMERATED {rf32, rf64, rf128, rf256}

FreqPriorityListEUTRA ::=           SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA

FreqPriorityListNR ::=              SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR

FreqPriorityEUTRA ::=               SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                          OPTIONAL        -- Need R

}

FreqPriorityNR ::=                  SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                          OPTIONAL        -- Need R

}

RAN-NotificationAreaInfo ::=        CHOICE {

cellList                            PLMN-RAN-AreaCellList,

ran-AreaConfigList                  PLMN-RAN-AreaConfigList,

...

}

PLMN-RAN-AreaCellList ::=           SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell

PLMN-RAN-AreaCell ::=               SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-AreaCells                       SEQUENCE (SIZE (1..32)) OF  CellIdentity

}

PLMN-RAN-AreaConfigList ::=         SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig

PLMN-RAN-AreaConfig ::=             SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-Area                            SEQUENCE (SIZE (1..16)) OF  RAN-AreaConfig

}

RAN-AreaConfig ::=                  SEQUENCE {

trackingAreaCode                    TrackingAreaCode,

ran-AreaCodeList                    SEQUENCE (SIZE (1..32)) OF  RAN-AreaCode                            OPTIONAL    -- Need R

}

-- TAG-RRCRELEASE-STOP

-- ASN1STOP