3GPP TSG-RAN WG2 Meeting #116 electronic	                     R2-2111104

Online, 1st – 12th November, 2021

Agenda item:	8.5.3

Source:	III

Title:	Remaining issues of IIoT in UCE

Document for:	Discussion

- Introduction

In 3GPP Rel-17 NR IIoT/URLLC, “the uplink enhancements for URLLC in unlicensed controlled environments (UCE)” has been accepted to be the working item. The objects focus on [1]:

- Specify support for UE-initiated COT for FBE with minimum specification effort

- Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

## To harmonize UL configured-grant enhancements for NR-U and IIoT/URLLC in unlicensed controlled environments, there are some issues need to be clarified.

## In the last RAN2 115-e meeting, there are still have some remaining issues requiring further discussion. In this contribution, we share our views on these issues.

- Discussion

  - Deprioritised UL grant when autoTx is not configured and CGRT is configured

## After the RAN2 115-e meeting [2], the following proposal is still requiring further discussion:

Proposal 7 (9/21): If cg-RetransmissionTimer is configured and autonomousTx is not configured, the cg-RetransmissionTimer is not stopped when the associated CG is deprioritized.

## From our perspective, if cg-retransmission timer is configured and autonomousTx is not configured, the following issue should be discussed:

- When both cg-retransmission timer and lch-basedPrioritization are configured, the importance of procedure “retransmit the deprioritized MAC PDU”.

## Proposal#1: RAN2 should first discuss, when both cg-retransmission timer and lch-basedPrioritization are configured, how important is the procedure “retransmit the deprioritized MAC PDU”.

## According to the different importance of procedure “retransmit the deprioritized MAC PDU”, we have the following two proposals:

- If procedure “retransmit the deprioritized MAC PDU” is necessary, when both cg-retransmission timer and lch-basedPrioritization are configured, autonomousTx is always configured,

- If procedure “retransmit the deprioritized MAC PDU” is unnecessary, autonomousTx can configure or not configure, if autonomousTx is not configured, confirm the earlier agreement that a deprioritized MAC PDU is not retransmitted autonomously.

## Proposal#2: If procedure “retransmit the deprioritized MAC PDU” is necessary, when both cg-retransmission timer and lch-basedPrioritization are configured, autonomousTx is always configured.

## Proposal#3: If procedure “retransmit the deprioritized MAC PDU” is unnecessary, autonomousTx can configure or not configure, if autonomousTx is not configured, confirm the earlier agreement that a deprioritized MAC PDU is not retransmitted autonomously.

- Conclusion

For URLLC/IIoT in unlicensed controlled environments, there are some discussed in this contribution with the following observation made and proposal proposed:

## Proposal#1: RAN2 should first discuss when both cg-retransmission timer and lch-basedPrioritization are configured, how important is the procedure “retransmit the deprioritized MAC PDU”.

## Proposal#2: If procedure “retransmit the deprioritized MAC PDU” is necessary, when both cg-retransmission timer and lch-basedPrioritization are configured, autonomousTx is always configured.

## Proposal#3: If procedure “retransmit the deprioritized MAC PDU” is unnecessary, autonomousTx can configure or not configure, if autonomousTx is not configured, confirm the earlier agreement that a deprioritized MAC PDU is not retransmitted autonomously.

References

[1]	RP-201310, Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NRWID, 3GPP TSG RAN Meeting #88e, Electronic meeting, June 29 – July 3, 2020;

[2]	R2-2108834, Report for Rel-17 Small data and URLLC/IIoT, 3GPP TSG RAN WG2 Meeting #115-e electronic Online, August, 2021.