3GPP TSG-RAN WG2 Meeting #116 electronic											R2-2111230

Online, 1st – 12th Nov, 2021

Agenda item:       6.2.2

Source: 	Huawei, HiSilicon

Title: 	Review report for CP contributions

Document for:	Discussion and Decision

# Introduction

This document summarizes the contributions submitted to AI 6.2.2. Rapporteur’s recommendations on how to treat them are provided.

# Correction CRs to be discussed together with Rapporteur’s Misc CR

The following correction CRs could be discussed together. The agreed changes could be merged into Rapporteur’s miscellaneous correction CR.

Table 1: Correction CRs to be discussed together offline

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2109596 | Miscelleneous CR on 38.331 | Huawei, HiSilicon | - “In the LS R1-2108393 from RAN1, RAN2 is requested to capture following RAN1 agreement on sidelink resource allocation mode 2 that A UE is expected to be (pre-)configured with a set sl-ResourceReservePeriod containing value of 0 ms Accordingly, in field description of sl-ResourceReservePeriodList in IE SL-ResourcePool, it should be clarified that in case Mode 2 is configured, at least value 0ms should be included in the list. “ sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool. In case Mode 2 is configured, at least value 0ms should be included in the list. - Fix the editorial errors. | For the first change of Rapp’s Misc CR and the other four CRs in this group, all are based on RAN1 LS R1-2108393. The discussion would be to find the optimal wording for a straightforward clarification. |
| R2-2109630 | CR to 38.331 on ResourceReservationPeriodList | Qualcomm Finland RFFE Oy | sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool. The value ms0 is always configured. |  |
| R2-2110611 | Corrections on RRC parameter sl-ResourceReservePeriodList | CATT | sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool, value ms0 shall be included. |  |
| R2-2110795 | Inclusion of 0 ms resource reservation period in sl-ResourceReservePeriodList | MediaTek Inc. | sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool. Network includes a list entry with the value ms0 for each resource pool. |  |
| R2-2110831 | Correction on TS 38.331 from the latest RAN1 decision | ZTE Corporation, Sanechips | sl-ResourceReservePeriodList Set of possible resource reservation period allowed in the resource pool in the unit of ms. Up to 16 values can be configured per resource pool. A UE is expected to be (pre-)configured with a set sl-ResourceReservePeriod containing value of 0 ms. |  |

Proposal 1: Discuss the CRs listed in Table 1 together, the agreed changes could be merged into Rapporteur’s miscellaneous correction CR.

# Correction CRs to be discussed separately

The following correction CRs could be discussed separately.

Table 2: Correction CRs could be discussed separately

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2109806 | Correction of IE sl-HARQ-FeedbackEnabled | Nokia, Nokia Shanghai Bell | sl-HARQ-FeedbackEnabled This field is mandatory present. It indicates the HARQ feedback enabled/disabled restriction in LCP for this sidelink logical channel. If set to enabled, the sidelink logical channel will be multiplexed only with a logical channel which enabling the HARQ feedback. If set to disabled, the sidelink logical channel cannot be multiplexed with a logical channel which enabling the HARQ feedback. Corresponds to 'sl-HARQ-FeedbackEnabled' in TS 38.321 [3]. If this field of at least one sidelink logical channel for the UE is set to enabled, sl-PSFCH-Config should be mandatory present in at least one of the SL-ResourcePool. | The change itself as one NBC approach is sensible/agreeable, once RAN2 acknowledges that there is indeed discrepancy between RRC and MAC spec regarding the configuration of “sl-HARQ-FeedbackEnabled”. |
| R2-2110269 | Correction on SL RLC parameter configuration | vivo | SL-RLC-Config field descriptions. sl-MaxRetxThreshold Parameter value of maxRetxThreshold for RLC AM for NR sidelink communications, see TS 38.322 [4]. Value t1 corresponds to 1 retransmission, value t2 corresponds to 2 retransmissions and so on.. sl-PollByte Parameter value of pollByte for RLC AM for NR sidelink communications, see TS 38.322 [4]. Value kB25 corresponds to 25 kBytes, value kB50 corresponds to 50 kBytes and so on. infinity corresponds to an infinite amount of kBytes.. sl-PollPDU Parameter value of pollPDU for RLC AM for NR sidelink communications, seeTS 38.322 [4]. Value p4 corresponds to 4 PDUs, value p8 corresponds to 8 PDUs and so on. infinity corresponds to an infinite number of PDUs.. sl-SN-FieldLength This field indicates the RLC SN field size for NR sidelink communication, see TS 38.322 [4]. For groupcast and broadcast, only value size6 (6 bits) is configured for the field sl-SN-FieldLengthUM.. sl-T-PollRetransmit Timer value of t-PollRetransmit for RLC AM for NR sidelink communications, see TS 38.322 [4], in milliseconds. Value ms5 means 5 ms, value ms10 means 10 ms and so on.. #### 9.1.1.4 SCCH configuration Parameters that are specified for unicast of NR sidelink communication, which is used for the sidelink signalling radio bearer of PC5-RRC message. The SL-SRB using this SCCH configuration is named as SL-SRB3. Name; Value; Semantics description; Ver. PDCP configuration. >t-Reordering; Undefined; Selected by the receiving UE, up to UE implementation. >pdcp-SN-Size; 12. RLC configuration; AM RLC. >sn-FieldLength; 12. >t-Reassembly; Undefined; Selected by the receiving UE, up to UE implementation. >t-PollRetransmit; Undefined; Selected by the transmitting UE, up to UE implementation. >pollPDU; Undefined; Selected by the transmitting UE, up to UE implementation. >pollByte; Undefined; Selected by the transmitting UE, up to UE implementation. >maxRetxThreshold; Undefined; Selected by the transmitting UE, up to UE implementation. >t-StatusProhibit; Undefined; Selected by the receiving UE, up to UE implementation. >logicalChannelIdentity; 3. MAC configuration. >priority; 1. >prioritisedBitRate; infinity. >logicalChannelGroup; 0. >schedulingRequestId; 0; The scheduling request configuration with this value is applicable for this SCCH if configured by the network.. | The change on the description of “SL-RLC-Config”, following the approach for the similar issue with "PDCP spec", is reasonable, also it is straightforward to add the missing parameters, once RAN2 confirms the changes are needed. |

Proposal 2: CR in R2-2109806 and CR in R2-2110269 are to be discussed separately.

# Correction CR to be deprioritized

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2110830 | Correction on power control parameter | ZTE Corporation, Sanechips | SL-PowerControl-r16 ::= SEQUENCE { dummy INTEGER (-30..33), sl-Alpha-PSSCH-PSCCH-r16 ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need M dl-Alpha-PSSCH-PSCCH-r16 ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need S sl-P0-PSSCH-PSCCH-r16 INTEGER (-16..15) OPTIONAL, -- Need S dl-P0-PSSCH-PSCCH-r16 INTEGER (-16..15) OPTIONAL, -- Need M dl-Alpha-PSFCH-r16 ENUMERATED {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} OPTIONAL, -- Need S dl-P0-PSFCH-r16 INTEGER (-16..15) OPTIONAL, -- Need M ... } | To Rapp’s knowledge, "sl-MaxTransPower " may still be useful for at least RAN4 spec one way or another, so cannot be voided now. The proponent is encouraged to check with RAN4 (which may discuss this issue during their next meeting). The correction by RAN2 can be postponed. |

Proposal 3: CR in R2-2110830 can be postponed.

# Conclusions

The contributions submitted to AI 6.2.2 are summarized, Rapporteur’s recommendations are as follows.

Proposal 1: Discuss the CRs listed in Table 1 together, the agreed changes could be merged into Rapporteur’s miscellaneous correction CR.

Proposal 2: CR in R2-2109806 and CR in R2-2110269 are to be discussed separately.

Proposal 3: CR in R2-2110830 can be postponed.