3GPP TSG-RAN WG2 #116-e	R2-2111350

Electronic meeting, 1st – 12th November 2021

Agenda Item:	8.12.3.1

Source:	Apple Inc.

Title:	[Draft] Summary of [AT116-e][105][RedCap] eDRX cycles aspects (Apple) – Phase 2

Document for:	Discussion and Decision

# Introduction

The document summarizes the following offline discussion:

[116-e][105][RedCap] eDRX cycle aspects (Apple)

Updated scope: continue the discussion based on the proposals in R2-2111335

Updated intended outcome: Summary of the offline discussion with e.g.:

  List of proposals for agreement (if any)

  List of proposals that require online discussions

  List of proposals that should not be pursued (if any)

Updated deadline (for companies' feedback): Friday 2021-11-05 1000 UTC

Updated deadline (for rapporteur's summary in R2-2111350): Friday 2021-11-05 1800 UTC

Proposals marked "for agreement" in R2-2111350  not challenged until Monday 2021-11-08 1200 UTC will be declared as agreed via email by the session chair (for the rest the discussion will further continue offline until the CB session in Week2).

Which follows the first phase of the discussion (details below) [16]:

[116-e][105][RedCap] eDRX cycle aspects (Apple)

Initial scope: Discuss proposals in AI 8.12.3.1 (skipping those on INACTIVE eDRX >10.24sec and on pure ASN.1 aspects)

Initial intended outcome: Summary of the offline discussion with e.g.:

§  List of proposals for agreement (if any)

§  List of proposals that require online discussions

§  List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Tuesday 2021-11-02 2000 UTC

Initial deadline (for rapporteur's summary in R2-2111335): Wednesday 2021-11-03 00:00 UTC

Proposals marked "for agreement" in R2-2111335 not challenged until Wednesday 2021-11-03 1100 UTC will be declared as agreed via email by the session chair (for the rest the discussion will continue online during the CB session).

# Contact information

| Company | Name and email address |
| --- | --- |
| MediaTek | Pradeep Jose (pradeep dot jose at mediatek dot com) |
| DENSO | Haruhiko.sogabe.j4r@jp.denso.com |
| Xiaomi | Liyanhua1@xiaomi.com |
| OPPO | Haitao Li (lihaitao@oppo.com) |
| LGE | HyunJung (stella.choe@lge.com) |
| Huawei, HiSilicon | Odile Rollinger (odile.rollinger@huawei.com) |
| Apple | Naveen Palle (naveen.palle@apple.com) |
| Sequans | Noam Cayron (noam.cayron@sequans.com) |
| Vivo | Chenli5g@vivo.com |
| CATT | Pierre Bertrand ; pierrebertrand@catt.cn |
| ZTE | LiuJing (liu.jing30@zte.com.cn) |
| Qualcomm | Linhai He (linhaihe@qti.qualcomm.com) |
| Intel | Marta Martinez Tarradell (marta.m.tarradell@intel.com) |
| Sharp | LIU Lei (lei.liu@cn.sharp-world.com) |

# Discussion

## Following the discussion from [1], the below agreements were taken in the online session and also the below topics were agreed to be discussed in this offline session:

Agreements via email - from offline 105: - The max eDRX cycle length for RRC Inactive is 10.24s in Rel-17 - PO determination for non-overlapping CN/RN case is applicable to eDRX - When IDLE eDRX and INACTIVE eDRX are configured and both cycles are no longer than 10.24s, PO is determined by IDLE eDRX. - When IDLE eDRX is configured and is no longer than 10.24s, INACITVE eDRX cycle is not configured, PO is determined by IDLE eDRX. - During CN PTW when IDLE eDRX is configured and longer than 10.24s, and INACTIVE eDRX is configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer. - During CN PTW when IDLE eDRX is configure and is longer than 10.24s, INACTIVE eDRX cycle is not configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer. - eDRX supporting UEs are assumed to also support the UE capability on PO determination for non overlapping CN/RN case (Further discuss on the reporting of eDRX capability) - The below working agreement is now changed to an agreement. When IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline, as below. FFS whether CN PTW_start position could be configurable by network and in case which node decides the N value. Note: this formula would be revisited if INACTIVE eDRX cycle can be above 10.24s PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation: SFN = 1024/N* ieDRX, where ieDRX = floor(UE_ID_H /TeDRX,H) mod N FFS N = 4 or 8, FFS if N can take other values - The same LTE hashed UE_ID calculation is used for UE_ID_H for NR. Items to discuss: Proposal 9 [13/13]: eDRX supporting UEs are assumed to also support the UE capability on PO determination for non overlapping CN/RN case. - Ericsson would like to further discuss this - Apple thinks Ericsson views is aligned to others - Huawei suggests to remove the last part. Ericsson agrees. - vivo supports the original proposal - eDRX supporting UEs are assumed to also support the UE capability on PO determination for non overlapping CN/RN case - Further discuss on the reporting of eDRX capability To discuss (continue the discussion based on the proposals): Proposal 3 : It is FFS if an explicit table capturing the determination of ‘T’ for different DRX/eDRX configurations, is needed in TS38.304. If the table is agreed to be added, this table can also include the restrictions from P2. Proposal 10 [To discuss]:: RAN2 to discuss if a eDRX capability is also needed to be reported to gNB. Proposal 12 [8/13]: The value of N is 8 and it is not configurable.. Proposal 14 [11/13]: For eDRX in NR, the UE_ID is given by 5G-S-TMSI mod 4096 Proposal 15 [to discuss]: DRX cycle the UE uses for comparing with the modification period to decide if the eDRX acquisition period is to be used: - [6/13] CN_eDRX for both RRC_IDLE and RRC_INACTIVE (same as LTE) - [4/13] separate CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (use CN eDRX if RAN eDRX is not configured) Proposal 16 [10/2]: eDRX acquisition period set to the maximum configurable value of the eDRX cycle. Proposal 17 [7/2]: eDRX acquisition period is the same for RRC_IDLE and RRC_INACTIVE (as in LTE). Proposal 18 [11/2/]: eDRX specific on-demand SI enhancements are not considered for Rel-17. Proposal 19 [9/4]: Option 2 is agreed for the below two cases: For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle. For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, T is determined by RAN paging cycle.

The phase-2 discussion is planned by inserting the discussion into the relevant sections (as many of the proposals were not discussed at all in the online session). The irrelevant parts of the text are greyed out for ease of use.

## Topics that are open for discussion

#### Max eDRX cycle length for INACTIVE

As noted in [1][6][8][12], SA2 confirmed that eDRX cycle length of >10.24sec is not considered for Rel-17.

In SA2’s LS to RAN2 (S2-2106978), SA2 provided their agreements on eDRX for RRC Inactive:

- For RRC Inactive, SA2 agreed to support eDRX of up to 10.24s. - Regarding eDRX extension beyond 10.24s for RRC Inactive, SA2 did not agree to support it in Rel-17 and instead will study potential solutions in Rel-18.

- Do companies agree that max eDRX cycle length for RRC Inactive is 10.24s in Rel-17?

| Company | Yes / No | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |
| DENSO | Yes | Agree to respect SA2 decision. |
| Xiaomi | Yes | Respect SA2 decision |
| OPPO | Yes |  |
| LGE | Yes |  |
| Huawei, Hisilicon | Yes |  |
| Apple | Yes |  |
| Sequans | Yes |  |
| Vivo | Yes |  |
| CATT | Yes |  |
| ZTE | Yes |  |
| Qualcomm | Yes |  |
| Intel | Yes |  |

#### Summary

13 companies responded and all the responded companies agree to capturing in RAN2 that RRC inactive max eDRX cycle length is 10.24s.

Moderator proposes that this gets agreed:

Proposal 1 [13/13] :  The max eDRX cycle length for RRC Inactive is 10.24s in Rel-17

#### Relation between INACTIVE and IDLE eDRX configurations

RAN2 has reached agreement the following two configurations are not supported. According to [9] it is better to explicitly capture the restriction (which was FFS from last meeting).

Agreements via email - from offline 105 third round

- RAN2 considers the configuration as an invalid case, where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured. FFS whether to capture this restriction in RAN2 spec.

- RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle. FFS whether to capture this restriction in RAN2 spec.

In [6], one company proposes that TS38.304 can include a table depicting the interpretation of the DRX cycle in different configurations, along with adding a NOTE to the configurations that are not allowed, including the above.

| UE in | eDRX configured by upper layer | eDRX configured by RRC layer | UE specific (e)DRX configured by upper layer | UE specific (e)DRX configured by RRC layer | Default DRX value |
| --- | --- | --- | --- | --- | --- |
| RRC_IDLE | None | None | Applies if it is the shortest of the configured ones (legacy operation). |  |  |
|  | Less or equal to 1024 rf | None, or any value | Applies | NA | NA |
|  | More than 1024 rf | None, or any value | Applies during PTW if it is the shortest of the configured ones. | NA | Applies during PTW if it is the shortest of the configured ones. |
| RRC_INACIVE | None | None | Applies if it is the shortest of the configured ones (legacy operation). |  |  |
|  | Less or equal to 1024 rf | None | FFS-1 | FFS-1 | FFS-1 |
|  | Less or equal to 1024 rf | Less or equal to 1024 rf | Applies if it is the shortest of the configured ones. |  | NA |
|  | Less or equal to 1024 rf | More than 1024 rf | NA (note-1) | NA (note-1) | NA (note-1) |
|  | More than 1024 rf | None | - During CN configured PTW, applies if it is the shortest of the configured ones. - Outside of CN configured PTW, NA | - During CN configured PTW, applies if it is the shortest of the configured ones. - Outside of CN configured PTW, FFS-2 | - During CN configured PTW, applies if it is the shortest of the configured ones. - Outside of CN configured PTW, FFS-2 |
|  | More than 1024 rf | Less or equal to 1024 rf | - During CN configured PTW, applies if it is the shortest of the configured ones. - Outside of CN configured PTW, NA | - During CN configured PTW, applies if it is the shortest of the configured ones. - Outside of CN configured PTW, applies eDRX one | - During CN configured PTW, applies if it is the shortest of the configured ones. - Outside of CN configured PTW, NA |
|  | More than 1024 rf | More than 1024 rf | NA (note-2) | NA (note-2) | NA (note-2) |

- Companies are requested to provide views on the below:

2.1 Add to TS 38.304 the above table summarizing how T is determined for a UE in RRC_IDLE and RRC_INACTIVE with different DRX cycle configurations.

2.2 Agree to capture the configuration restrictions from the below into the specification?

Agreements via email - from offline 105 third round

RAN2 considers the configuration as an invalid case, where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured.

RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle.

2.3 If yes to 2.2, agree to include this in the table from 2.1?

| Company’s name | Do companies agree to |  |  | Comments, suggestions if any, for each of the items |
| --- | --- | --- | --- | --- |
|  | 2.1 | 2.2 | 2.3 |  |
| MediaTek | No | Yes | No | We are concerned that the table could be confusing and difficult to maintain. Agreements can be captured in a note. |
| DENSO | Y | Y | Y | Support to capture the table, e.g. in the Annex section to see the whole picture of possible eDRX configurations. |
| Xiaomi | No | Yes | No | We do not think a table is needed as long as we put it clearly in the context of the spec. |
| OPPO | No | Yes | No | We think it would be sufficient to capture t he configuration restrictions as notes in the spec. |
| LGE | No | Yes | No | We think simple description to reflect the agreements is enough. |
| Huawei, Hisilicon | FFS | Yes | FFS | We think a table would be helpful to describe the T used by the UE in the different eDRX combination. However, we think the table could be simplified e.g. the three columns could be merged in one, giving the definition of T within and outside the PTW |
| Apple | Yes | Yes | Yes | Some form of table is better. |
| Sequans | Y | Y | Y | While the table can be improved somewhat to be made even clearer, we think it does a very good job of clarifying a complex issue. We don’t see why these agreements should be captured as notes, it makes them sound like a recommendation when they are explicit restrictions. |
| vivo | No | Yes | No | We think it is not a good idea to capture and maintain such table in the specification. We could agree a tale to summarize all possible cases and corresponding behaviours. Regarding the table itself, we tend to describe it in a way like LTE regarding the determination of T, for example as below, an example in case when eDRX configured by upper layer is more than 1024 rf and RRC layer doesn’t configure the eDRX: UE_in; eDRX configured by upper layer; eDRX configured by RRC layer; T. RRC_INACTIVE; More than 1024 rf; None; - During CN configured PTW, T is the shortest of UE specific DRX cycle, if configured by upper layer, UE specific (e)DRX configured by RRC layer, default DRX value. -outside PTW, FFS. Regarding the configuration restriction, we think it could be captured as a note in the specification. |
| CATT | FFS | Yes | FFS | Normative text should explicitly capture the RAN2 agreement. In addition a table could help summarizing all cases, although this is not typical usage in specifications. However we find the above table unclear as it mixes DRX and eDRX configurations in 4th and 5th columns. |
| ZTE | No | Yes | No | The configuration restrictions can be captured either in 38.300 or in field description in 38.331. Regarding the table, we think it is not needed and may be hard to maintain. We prefer to describe different cases in normal text in 38.304. |
| Qualcomm | No | Yes | No | We think that for 38.304, it is better to capture different combinations of configurations in text, which typically can be made more precise than entries in a table. Maybe such a table can be considered for 38.300? We prefer to capture the configuration restrictions as notes in 38.304. |
| Intel | Yes | See comment | See comment | We understand that it would be helpful if TS 38.304 includes a table summarizing how T is determine for a UE in RRC_IDLE and RRC_INACTIVE depending on whether eDRX cycle is configured and whether UE is monitoring paging inside or outside the CN configured PTW (e.g. as shown in above Table or other kind of summary table). In addition, this table could describe the combination of eDRX configurations not expected by the UE (i.e. those not allowed or not supported for Rel-17). However we do not think that corresponding restriction needs to be defined in TS 38.331. |

#### Summary

13 companies responded to the views on three questions.

Regarding, capturing explicitly in the spec on the restrictions of INACTIVE eDRX and it’s relation to IDLE eDRX, all the companies agree to this.

Regarding adding a table in 38.304 on determination of T : 7 companies do NOT want to add a table to 38.304, while 4 want it, and 2 companies have their view as ‘FFS’.

Only the same set of companies that want to capture the table, want to capture the restrictions of INACTIVE eDRX and it’s relation to IDLE eDRX in that table, whle the other companies do not prefer.

Based on all of the above, the moderator proposes that we progress by capturing in the spec explicitly, the restrictions of INACTIVE eDRX and it’s relation to IDLE eDRX,.

Do to low support and a couple of companies proposing as FFS, whether to capture this in a table, whether a table is really needed can be FFS.

Proposal 2 [13/13] :  Capture in the specification explicitly the below restrictions:

- RAN2 considers the configuration as an invalid case, where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured.

- RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle.

Proposal 3 : It is FFS if an explicit table capturing the determination of ‘T’ for different DRX/eDRX configurations, is needed in TS38.304. If the table is agreed to be added, this table can also include the restrictions from P2.

Based on the online discussion on Proposal 2 (pasted below), it can be inferred that Proposal -3 does not need a discussion at this time, and is better suited to be addressed during the actual DRs drafting time.

Proposal 2 [13/13] : Capture in the specification explicitly the below restrictions: - RAN2 considers the configuration as an invalid case, where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured. - RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle. - Nokia would like to further discuss this, as they don't think we should have restrictions - Intel thinks we could cover this as a restriction on the UE side. - Ericsson thinks we already agreed on this and we can discuss later how to capture this. Intel agrees - Discuss later when working on the actual CRs

So the moderator proposes to skip this one for phase-2 discussion.

#### RAN and CN paging PO non-overlap topic

In [9] one company brings out the CN/RAN PO non-overlap issue from the below formula:

The PF and PO for paging are determined by the following formula:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

According to definition of N (the number of total paging frames in T), different T results in different N. Thus, the PO determined for RRC_IDLE and RRC_INACTIVE may be different if Ns is not 1.

In NR control plane discussion in last RAN2 meeting, following agreement is reached to address this problem, with the solution being that the UE in RRC_INACTIVE should use the same i_s to determine PO as for RRC_IDLE:

- We introduce a solution, from R17, where the following is the baseline: - R2-2109077 Solution 2 (i.e. UE in RRC _INACTIVE should use the same i_s to determine PO as for RRC _IDLE) is supported to address the RAN and CN paging PO non-overlap problem. - UE capability should be introduced to indicate support for using the same i_s in PO determination in RRC _INACTIVE state as in RRC _IDLE state.

In the same paper from [9], the company proposes that for the below 4 cases, the same solution from NR control plane discussion should be applied for eDRX as well.

- Companies are requested to provide views on the below sub-proposals:

3.0 : Do companies think that the PO determination for non-overlapping CN/RN case is also valid and should be applied for eDRX?

If yes, do companies agree with the below:

3.1 : When IDLE eDRX and INACTIVE eDRX are configured and both cycles are no longer than 10.24s, PO is determined by IDLE eDRX.

3.2 : When IDLE eDRX is configured and is no longer than 10.24s, INACITVE eDRX cycle is not configured, PO is determined by IDLE eDRX.

3.3: During CN PTW when IDLE eDRX is configured and longer than 10.24s, and INACTIVE eDRX is configured, PO is determined by the shortest value of default paging cycle and UE specific paging cycle if configured by upper layer.

3.4 : During CN PTW when IDLE eDRX is configure and is longer than 10.24s, INACTIVE eDRX cycle is not configured, PO is determined by the shortest value of default paging cycle and UE specific paging cycle if configured by upper layer.

| Company’s name | Do companies agree to |  |  |  |  | Comments, if any |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3.0 | 3.1 | 3.2 | 3.3 | 3.4 |  |
| MediaTek | Yes | Yes | Yes | Yes | Yes | It seems that PO mismatch issue can also exist in some eDRX cases, which can be resolved similar to the legacy I-DRX solution: do not consider RAN paging for PO determination. |
| DENSO | Y | Y | Y | Y | Y |  |
| Xiaomi | Y | Y | Y | Y | Y | We agree with this intention. |
| OPPO | Y | Y | Y | Y | Y | RAN2 has reached agreement to address this problem for DRX, we think this is also a value issue for eDRX. |
| LGE | Y | Y | Y | Y | Y |  |
| Huawei, Hisilicon | Yes | Yes | Yes | yes | yes | We agree the UE in RRC_INACTIVE uses the same i_s to determine PO as for RRC_IDLE in case of RRC state mismatch. To be consistent with agreements in last meeting, the term “UE specific paging cycle” in 3.3 and 3.4 are suggested to be updated with “UE specific DRX cycle” |
| Apple | Y | Y | Y | Y | Y |  |
| Sequans | Y | Y | Y | Y | Y |  |
| vivo | Y | Y | Y | Y | Y | We agree the PO determination for non-overlapping CN/RN case is also valid. |
| CATT | Yes | Yes | Yes | Yes | Yes | The issue indeed exists and can be addressed by the proposed solutions |
| ZTE | Y | Y | Y | Y | Y | The CN and RAN paging PO non-overlapping issue also exists in the listed eDRX configurations. We think the same solution for non-eDRX configuration should be reused for eDRX cases. |
| Qualcomm | Yes | Yes | Yes | Yes | Yes |  |
| Intel | Y | Y | Y | Y | Y |  |

#### Summary

13 companies responded to the views on the five questions, much to the relief of the moderator, all the companies agree to all of these!

Proposal 4 [13/13]: PO determination for non-overlapping CN/RN case is applicable to eDRX

Proposal 5 [13/13]:: When IDLE eDRX and INACTIVE eDRX are configured and both cycles are no longer than 10.24s, PO is determined by IDLE eDRX.

Proposal 6 [13/13]:: When IDLE eDRX is configured and is no longer than 10.24s, INACITVE eDRX cycle is not configured, PO is determined by IDLE eDRX.

Proposal 7 [13/13]:: During CN PTW when IDLE eDRX is configured and longer than 10.24s, and INACTIVE eDRX is configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer.

Proposal 8 [13/13]:: During CN PTW when IDLE eDRX is configure and is longer than 10.24s, INACTIVE eDRX cycle is not configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer.

In the same paper from [9], the company also proposes a UE capability for both eDRX and non eDRX supporting UEs. While the moderator thinks that the support for non-eDRX supporting UEs might be out of scope of the RedCap session, it is wise to get views from companies:

- Assuming that the PO determination for non-overlapping CN/RN case is also valid and should be applied for eDRX, companies are requested to provide views on the below:

4.1 : Since a new UE capability is already agreed to be supported for non-eDRX UEs, the UE capability on PO determination for non overlapping CN/RN case for eDRX supporting UEs, is needed

4.2 : (assuming a ‘yes’ for 4.1) the support of eDRX by the UE also indicates the UE capability on PO determination for non overlapping CN/RN case and no new capability is needed. Pls note: there is no explicit capability (so far) for eDRX support in RAN.

| Company’s name | Do companies agree to |  | Comments, suggestions if any, for each of the items |
| --- | --- | --- | --- |
|  | 4.1 | 4.2 |  |
| MediaTek | Yes | Yes | We assumed that eDRX capability would anyway be introduced in Rel-17. As the PO mismatch problem will not be present for eDRX from the beginning, and all UEs supporting eDRX would apply the solution in DP 3, a separate capability is not needed. |
| DENSO | Y | Y | The new UE capability for non-eDRX UE together with the eDRX capability via NAS can cover the eDRX non-overlapping CN/RN case. |
| Xiaomi | Y | Y | UEs supporting e-DRX in R17 would naturally apply the solution to avoid mismatching. |
| OPPO | Yes | Yes | We share the same view as MediaTek. |
| LGE | Y | Y |  |
| Huawei, Hisilicon | FFS | FFS | We agree with the intention that eDRX UEs support the new PO determination. Now, since eDRX is a new feature in R17, we prefer that the new PO determination is a fundamental part of eDRX supporting UEs. In other words, there is no need for a UE capability on PO determination for non overlapping CN/RAN case for eDRX supporting UEs. It is implicit by the support of eDRX. |
| Apple | Y | Y |  |
| Sequans | FFS | FFS | Agree with HW |
| vivo | N | N | In our view, the UE supports eDRX cycle and the capability on PO determination for non overlapping CN/RN case are two separate aspects for a UE, i.e., if a UE supports PO adjustment for non overlapping CN/RN case, then no matter it works in DRX mode or eDRX mode, it can deal with the non overlapping issue. Besides, we also think there is no need to introduce explicit capability signalling on whether UE supports eDRX as in LTE. When UE and the CN negotiate the eDRX cycle with NAS signalling, the UE includes an extended idle mode DRX parameters information element in the Registration Request message to implicitly inform CN it supports eDRX. |
| CATT | Yes | Yes |  |
| ZTE | Y | Y | For eDRX, there is no mismatching issue before R17. So we can assume a UE supporting eDRX also supports the new PO determination in RRC_INACTIVE state, and separate capability is not needed. |
| Qualcomm | Yes | Yes |  |
| Intel | See comment | See comment | Our understanding is that UE would require signaling its capability support for eDRX in RRC_IDLE and/or RRC_INACTIVE. |

13 companies responded and all the companies agree that from Rel-17 (where eDRX would be introduced), the PO determinations can be part of eDRX support without explicit capability.

Proposal 9 [13/13]: eDRX supporting UEs are assumed to also support the UE capability on PO determination for non overlapping CN/RN case, and no additional capability is needed.

However, it is not yet agreed/discussed that a eDRX capability would be reported in RAN (different from the RedCap capability).

Phase -2 discussion on P9:

P9 was discussed online (notes below) and it is agreed that eDRX supporting UEs implicitly also support he PO determination for non-overlapping CN/RN case. What is left to discuss is whether we would need a capability for the (RedCap) UE report the support of eDRX.

Proposal 9 [13/13]: eDRX supporting UEs are assumed to also support the UE capability on PO determination for non overlapping CN/RN case. - Ericsson would like to further discuss this - Apple thinks Ericsson views is aligned to others - Huawei suggests to remove the last part. Ericsson agrees. - vivo supports the original proposal - eDRX supporting UEs are assumed to also support the UE capability on PO determination for non overlapping CN/RN case - Further discuss on the reporting of eDRX capability

Proposal 10 [To discuss]:: RAN2 to discuss if a eDRX capability is also needed to be reported to gNB.

While one can argue that RedCap UEs might all need to support eDRX (eDRX is one of the key features, and the main feature for power saving), it was also agreed in RAN2 that eDRX feature can also be supported without dependency of RedCap feature. In other words, there can be non-RedCap UEs which also support eDRX.

While the UE does report the eDRX capability to the CN (using NAS signaling), it was also commented in phase 1 discussion by Intel that RAN node might need to know if the UE supports eDRX, esp for configuring the eDRX for INACTIVE cases.

With these arguments, the moderator proposes the below for company views.

Discussion point PH2.1) Do companies agree that RAN2 define a new UE capability for eDRX support and allow also non-RedCap UEs report this capability? (Based on the online agreement, this capability implies that the UE also supports PO offset determination in non-overlapping cases)

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | No | A UE with eDRX capability would negotiate an eDRX configuration with CN, so CN is certainly aware of UE’s eDRX capability. CN would inform RAN about the UE’s eDRX configuration, so that RAN may configure the UE with RAN eDRX when it decides to release the UE in RRC INACTIVE. Note that for a UE in RRC INACTIVE, RAN could configure the UE with RAN eDRX only if the UE is configured with CN eDRX, which is also the reference for RAN eDRX configuration. In our view, since RAN could be aware of UE’s eDRX capability via the UE’s eDRX configuration from CN, we see no need to introduce a new UE capability for eDRX to report to RAN. We agree that eDRX function may also apply to non-RedCap UEs. |
| MediaTek | Yes (FFS to report it with UE capability signaling) | It seems that in LTE eDRX capability was not introduced, it is an optional feature without a UE radio access capability parameter signaled by the UE. If the CN can provide the information of whether a UE supports eDRX or not to RAN, then there may be no need to introduce a separate eDRX UE capability signaled in NR. If the capability is signaled, it should also apply to the non-RedCap UEs. |
| Apple | Yes (a new RAN UE capability for eDRX support, as part of UE capabilities) | And this is not just to RedCap UEs, but other Rel-17 UEs can report this as well. PO offset determination is assumed to be supported by this UE. |
| Qualcomm | Yes | UE reports its support for eDRX to both CN (NAS capability) and RAN (RAN capability via UE Radio Paging Information). |
| Sharp | Yes | Agree with MediaTek |
| Intel | Yes (but see comment) | We understand that gNB needs to know whether UE supports eDRX in INACTIVE or not. This can be done by different means: - Same NAS capability of supporting eDRX in IDLE is extended to also indicate its support for eDRX in INACTIVE. - A new UE’s capability is defined for UE to indicate its support of eDRX in INACTIVE. If so, two options are possible: - New NAS capability is defined and AMF shares this with gNB - New radio capability is defined. In our understanding, all options are feasible. However, whether to configure or not eDRX in INACTIVE is under gNB control, therefore having a UE radio capability defined seems sufficient (option 2.b)). However, we understand that RAN2 agreed that if UE is in eDRX INACTIVE, UE is also in eDRX IDLE (where eDRX IDLE >= eDRX INACTIVE). Therefore, other 2 options, i.e. 1) and 2.a), are also reasonable. In addition, we want to clarify the point explained in the question that eDRX capability should be independent on whether the UE is or not RedCap. |
| Huawei, HiSilicon | No | We prefer to reuse the method in LTE, i.e., there is no additional capability to be defined. In LTE, support of eDRX in RRC_INACTIVE is linked to support of RRC_INACTIVE (optional for eMTC) and IDLE eDRX being configured. |
| CATT | No | The NW is made aware of the UE supporting eDRX when the UE requests to be configured with eDRX. We agree with OPPO that it can be via NAS and NAS informs RAN |
| ZTE | Yes (FFS to report it with UE capability signaling) | Similar view as MediaTek. This capability is also applicable for non-RedCap UE. |
| Sequans | No | Agree with HW |
| Samsung | No | Agree with OPPO |
| DENSO | Yes | If the new UE capability can be used to notify gNB directly from the UE, there is a possibility that redundancy will be eliminated compared to the conventional means (via CN). Therefore, we support the definition of a new UE capability and the availability of this feature for non-RedCap UEs. |
| Xiaomi | Yes | We also think UE reports its capability for CN e-DRX to CN (NAS capability) as well as RAN eDRX to RAN (RAN capability via UE Radio Paging Information). |
| Ericsson | No | There is no need to introduce UE capability to inform RAN that the UE supports eDRX explicitly. It is already possible for the CN to provide such information. This applies regardless of whether the UE is a RedCap UE. We do not think there is a need for a capability bit to indicate support for eDRX in RRC_ACTIVE either. Similar to LTE this should be linked to whether RRC_INACTIVE is supported and eDRX is configured in RRC_IDLE. |

Summary for PH2.1:

14 companies responded to this question, and 6 did not accept to a RAN2 capability (and suggest to re-use the NAS signaling) while 5 companies prefer a RAN capability, and 3 companies are ok with both options.

Since there is no consensus on RAN2 specific capability, to progress forward, the moderator proposes the below:

Proposal 2.1 : eDRX feature can be supported by non RedCap UEs.

Proposal 2.2 : eDRX feature request by the UE is using the NAS signalling. FFS if additional capability signalling in RAN in the UE capability message, is also needed

While we are on this topic, the moderator thinks it’s better to get clarified of the below item related to this capability.

Discussion point PH2.2) Should the RedCap UE mandatorily report the support the eDRX capability? (assuming that RAN2 agrees to support this capability)

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | No | Firstly, we see no need to introduce an eDRX capability for UE to report to RAN (see our comments above). Secondly, we don’t think RedCap UEs should mandatorily support eDRX, this function should be optionally supported, which would be more flexible from UE’s perspective. |
| MediaTek | No | Not all RedCap use cases require eDRX support. |
| Apple | No | Not all RedCap use cases require eDRX support. |
| Qualcomm | No | Agree with MediaTek and Apple |
| Sharp | No |  |
| Intel | No | As explained in previous PH2.1) we understand that the support of eDRX should be treated as an independent feature. |
| Huawei, HiSilicon | No | As agreed in last meeting, eDRX feature is optional for any UE (including RedCap and non-RedCap UEs). |
| CATT | No | We already discussed in SI phase RedCap usecases not requiring eDRX. |
| ZTE | No | It should be optional for both RedCap and non-RedCap UE. |
| Sequans | No | Agree with MediaTek |
| Samsung |  | Even if RAN2 agrees to support this capability, eDRX feature is optional. |
| DENSO | No | Not all RedCap use cases require eDRX support. |
| Xiaomi | No |  |
| Ericsson | No | Note that it has already been agreed that there is no need to tie these features together |

Summary for PH2.2:

14 companies responded to this question, and all agree that eDRX support is optional for the RedCap UE.

Proposal 2.3 : eDRX support is optional for the RedCap UE.

#### PTW_Start calculation

In the last meeting, progress in terms of agreements were done on PH and PTW_end calculation, and the PTW length and step length are also agreed.

But for PTW_start the below working assumption with the FFS was reached:

### Working Assumption: - When IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline, as below. FFS whether CN PTW_start position could be configurable by network and in case which node decides the N value. Note: this formula would be revisited if INACTIVE eDRX cycle can be above 10.24s ### PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation: ### SFN = 1024/N* ieDRX, where ### ieDRX = floor(UE_ID_H /TeDRX,H) mod N ### FFS N = 4 or 8, FFS if N can take other values

Regarding the value of N, the companies views are divergent. [4][6][14] prefer the LTE operation with N=4, while [8][9][13] prefer a fixed value of N=8, while some other companies are ok with a configurable value, and some also propose that this value is given to the CN by RAN.

The moderator would like to first check if there are any objections to the working assumption and then proceed to get company views on the value of N.

- companies are requested to provide views on the below:

5.1 : Do companies object to the working assumption?

If no to 5.1:

5.2 : N=4?

5.3 : N=8?

5.4 : N= 4 or 8, configurable by the RAN

5.5 : N can be other values, configurable by the NW (pls add info in comments).

5.6 : RAN informs CN about N (if N is variable)

5.7: N can be configurable by CN

| Company’s name |  | Do companies agree to |  |  |  |  |  |  |  |  |  |  |  |  |  | Comments, if any |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5.1 (object to WA)? |  | 5.2 N=4 |  | 5.3 N=8 |  | 5.4 N=4 or 8 configurable by RAN |  | 5.5 N can have other values configurable by RAN |  | 5.6 RAN informs CN about N (if N is variable) |  | 5.7 N can be configurable by CN |  |  |  |
| MediaTek |  | No |  |  |  | Yes |  |  |  |  |  |  |  |  |  | In Rel-17, for simplicity, we can go with N=8, that resolves unused time gaps during PTW, and has minimal impact on signalling/CT1. |  |
| DENSO |  | N |  | N |  | N |  | Y |  | Open |  | N |  |  |  | On 5.5, fine to add more values, albeit there are not any other values than 4 and 8 proposed from our side. On 5.6, CN should decide the value of N, instead of RAN, since the other eDRX parameters (eDRX cycle, PTW length) are determined by CN in case of LTE eDRX. |  |
| Xiaomi |  | No |  | Yes |  |  |  |  |  |  |  |  |  |  |  | We think currently using the LTE way is sufficient. If we want to change this, we need to confirm with CT1/SA2 first. |  |
| OPPO |  | No |  |  |  |  |  |  |  |  |  |  |  | Yes |  | It seems our proposal from [2] is missing. - In our view, making N value configurable is more flexible compared to using a fixed N value. - For a RedCap UE configured with an eDRX cycle longer than 10.24s, CN should be aware of UE’s PTW start so that CN could send paging just before the occurrence of the start of PTW or during PTW to avoid storing paging messages in gNB. If N is determined by gNB, we might have different N values to be used in different cells, meaning a UE configured with eDRX may have different PTW_start in different cells. This would lead to CN implementation complexity or gNB implementation complexity when CN needs to page a UE within multiple cells. - So we think N value in the PTW_start calculation formula is configured by CN. |  |
| LGE |  | N |  | Y |  |  |  |  |  |  |  |  |  |  |  | We think legacy LTE calculation is fine, but if majority want to use N=8, it’s acceptable. |  |
| Huawei, Hisilicon |  | No |  | Yes |  | FFS |  | No |  | No |  | No |  |  |  | We think that only a small portion of UEs will be configured with a PTW equal to 1.28s, so we do not see this a problem. And we prefer N =4 We could accept N=8 if this is the majority view We cannot agree with a configurable value as this will impact all parts of the system, CN, gNB and UEs for no obvious benefit |  |
| Apple |  | No |  |  |  | Y |  |  |  |  |  |  |  |  |  | It is the simpler option. |  |
| Sequans |  | No |  | Yes |  | Yes |  | No |  | No |  | No |  |  |  | Prefer fixed N=8, it has only benefits. Configurable N, has many complications and little actual benefit, even if configured by CN (which would probably be the lesser evil in that case). |  |
| vivo |  | N |  | N |  | Y |  | N |  | N |  | N |  |  |  | We don’t see the need to make N configurable by RAN or CN. In our understanding, the easiest way is to make N is a static number, since the minimum PTW length is 1.28s, N=8 is reasonable. |  |
| CATT |  | N |  | N |  | N |  | Y |  | N |  | N |  | N |  | Having it configurable provides RAN flexibility to either fill all time resources or to stick to legacy (LTE) configuration |  |
| ZTE |  | No |  | See comments |  | Yes |  | No |  | No |  | No |  | No |  | In LTE, the minimum PTW length is 2.56s, so N=4; But in NR, we have agreed the minimum PTW length is 1.28s, that is why N=8 should be considered. N=4 will cause gaps between PTWs, but if majority companies agree with N=4, we are fine. However, we did not see benefits in solutions (5.4, 5.5, 5.6). Making it configurable requires coordination between CN and RAN in order to have the same PTW start among gNBs, which brings more complexity. |  |
| Qualcomm |  | No |  |  |  | Yes |  |  |  |  |  |  |  |  |  | We share the same view as ZTE |  |
| Intel |  |  |  |  |  | y |  | y |  |  |  | See comment |  |  |  | Q5.1 is a little confusing. We are ok with WA although we have slightly preference towards option 5.3 or 5.4 If 5.4. is agreed, RAN2 could inform SA2/CT1 about its agreement and ask whether this information needs to be provided from RAN to CN. |  |
| Samsung |  | N |  | N |  | N |  | N |  | N |  | N |  | Yes |  | We support configurable N by "CN". |  |
| Ericsson |  | No |  | Yes |  | No |  | No |  | No |  | No |  | No |  | Similar view with HW; it does not seem to be beneficial or the benefits do not seem to be significant if N is configurable especially considering the impact on the system in overall. One should also consider, for example the IODT testing aspects. |  |

#### Summary

13 companies responded to the views on the seven questions and the answers are very diverse. This needs further discussion!

All the companies agree to converting the working agreement to an explicit agreement.

Proposal 11 [13/13]: The below working agreement is now changed to an agreement.

### When IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline, as below. FFS whether CN PTW_start position could be configurable by network and in case which node decides the N value. Note: this formula would be revisited if INACTIVE eDRX cycle can be above 10.24s

### PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation:

### SFN = 1024/N* ieDRX, where

### ieDRX = floor(UE_ID_H /TeDRX,H) mod N

FFS N = 4 or 8, FFS if N can take other values

We had an online discussion of the calculation of PTW_Start and agreed to the below formula, but the value of N is still FFS.

Proposal 11 [13/13]: The below working agreement is now changed to an agreement.

When IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline, as below. FFS whether CN PTW_start position could be configurable by network and in case which node decides the N value. Note: this formula would be revisited if INACTIVE eDRX cycle can be above 10.24s

PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation:

SFN = 1024/N* ieDRX, where

ieDRX = floor(UE_ID_H /TeDRX,H) mod N

FFS N = 4 or 8, FFS if N can take other values

- Agreed

Proposal 13 [13/13]: the same LTE hashed UE_ID calculation is used for UE_ID_H for NR.

- Agreed

From Ph1 discussion, the moderator noted the below inferences and Proposal 12

Regarding the value of N, 5 companies prefer N=4 (LTE style), while 3 did not agree, and the rest did not comment. Regarding the value of N=8, 7 companies agree, 2 disagree, while 1 company is willing to go if this the majority option. Regarding the value of N to being 4 or 8 (configurable by the gNB). 3 companies agree, while 4 oppose and the rest did not provide their view. Regaring the value of N taking other values along with 4,8, five companies oppose this, while 1 company thinks this can be explored. 6 companies provided views on the option of RAN informing CN about the N, and all of them do not agree to this. 2 companies proposed CN configuring N and 2 companies oppose this. Considering that the views are diverse, in attempt to make progress, the moderator intends to take the path of least resistance, and propose the below: Proposal 12 [8/13]: The value of N is 8 and it is not configurable..

To summerize: 8 companies are willing to N=8, 5 companies for N=4, 3 companies to N= 8 or 4.

Since looks like 11 companies are ok in one form of other to N=8 and 3 support of configurability while 4 oppose, it might be better to go with N=8 without any configurability.

Discussion point PH2.3) Companies are requested to see if they can come to a consensus on N=8 without configurability. Pls provide in comments section if companies think they have a different view.

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | No | In our view, making N value configurable (4 or 8) is more flexible compared to using a fixed N value. Similar to LTE, in NR, CN should be aware of UE’s PTW start so that CN could send paging just before the occurrence of the start of PTW or during PTW to avoid storing paging messages in gNB. If N is determined by gNB, we might have different N values to be used in different cells, meaning a UE configured with eDRX may have different PTW_start in different cells. This would lead to CN implementation complexity or gNB implementation complexity when CN needs to page a UE within multiple cells. So we think CN should be responsible for N configuration. |
| MediaTek | Yes | Fixed N=8 value is acceptable for us. |
| Apple | Yes | We are ok with N=8 with no option for extra configuration. Avoid extra signaling and keep this simple. |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| Intel | Yes |  |
| Huawei, HiSilicon | Yes | N = 8 without configurability. A flexible N value brings limited benefits but potential system impact and implementation complexity |
| CATT | Yes | We are OK to compromise on this, to move forward. |
| ZTE | Yes | A fixed number 8 is enough. We do not see the benefit of configurable solutions. |
| Sequans | Yes | N=8 covers all cases without gaps. Flexibility does not bring real benefit and mostly complications, as commented by many companies in phase 1. |
| Samsung | Yes |  |
| DENSO | No | We believe that supporting N = 4 and 8 can address multiple use cases. For example, when the existing IoT service supported by LTE eDRX is replaced with RedCap UE capable of eDRX, N = 4 is applied to offer the same configuration as in the legacy. For new services (e.g. factory IoT, smart city) offered by RedCap UEs capable of eDRX, either 4 or 8 can be used, according to the number of RedCap UEs and traffic characteristics. |
| Xiaomi | Yes | We are OK to compromise |
| Ericsson | No | There seems to be no significant benefit compared to when N is equal to 4 (as in LTE) if N is equal to 8 or it is configurable. |

Summary for PH2.3:

14 companies responded to this question, and 11 agree to keeping N=8, while 3 companies do not agree to the consensus.

Proposal 2.4 [to discuss, 11 for, 3 against]: For the eDRX PTW start calculation, agree to N=8. No signalling needed to CN.

#### UE_ID topic

Assuming the calculation from discussion point 5 is changed from working assumption to an agreement, there are some open items on the UE_ID calculation (UE_ID_H as well as UE_ID itself).

In [8][13][14], companies proposed that UE_ID_H follows the same hashed UE ID calculation from the UE_ID formula from LTE. While in [3], one company proposes to change the UE_ID used for eDRX claiming that UEs can be mapped only to 1024 POs and other POs will remain unused in eDRX with N=1024.

- companies are requested to provide views on the below:

6.1 : Do companies agree to the using the same LTE hashed UE_ID calculation for UE_ID_H for NR?

6.2 : Separate from the PTW/PH (i.e., irrespective of whether PTW/PH is used or not) do companies agree to the proposal that for extended DRX, the UE_ID is given by 5G-S-TMSI mod 4096

| Company’s name | Do companies agree to |  |  | Comments, suggestions if any, for each of the items |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 6.1 | 6.2 |  |  |  |  |
| MediaTek | Yes | Yes |  |  |  |  |
| DENSO | Y | Y |  | Although 6.2 is O.K, the similar issue is supposed to be discussed for the paging sub-grouping under the ePower saving WI. The details have to be discussed for further. |  |  |
| Xiaomi | Yes | Yes |  |  |  |  |
| OPPO | Yes | Yes |  |  |  |  |
| LGE | Y | Y |  |  |  |  |
| Huawei, Hisilicon | Yes | Yes |  | The use of 4096 is to distribute UEs into all the POs available. However, this should only be used when eDRX is configured and enable in the cell. |  |  |
| Apple | Y | Y |  |  |  |  |
| Sequans | Yes | Yes |  |  |  |  |
| vivo | Y | N |  | We agree to use the same LTE hashed UE_ID calculation for UE_ID_H for NR. Regarding the proposal, “for extended DRX, the UE_ID is given by 5G-S-TMSI mod 4096”, we agree there is a problem that all UEs configured 1024 rf as eDRX cycle are aggregated in the first PO of PF. However, we don’t think the issue is severe, as the number of UEs configured with 1024 rf eDRX cycle won’t be huge and they could be equally distributed among different PFs. Besides, the same problem will occur in LTE, i.e, when both eDRX cycle configured by CN and RRC are 10.24s, then UE in RRC_INACTIVE will monitor PO outside the CN PTW with T equal to 10.24s. |  |  |
| CATT | Yes | FFS |  | We think the issue exists, especially when eDRX=10.24s. And this option can solve the problem. But for the case when CN eDRX>10.24s, and RAN eDRX<10.24s or not configured, it doesn’t need to use the UE ID by 5G-S-TMSI mod 4096. |  |  |
| ZTE | Yes | Yes |  |  |  |  |
| Qualcomm | Yes | Yes |  |  |  |  |
| Intel | Yes | Maybe |  | We do not have strong view on this but considering that RAN2 agreed that the maximum PTW length is 40.96s when IDLE eDRX cycle is longer than 10.24s (like in NB-IoT), it seems reasonable to do MOD 4096 instead to better spread UE’s. |  |  |
| Ericsson | Yes | Yes |  |  |  |
| Samsung | Yes |  | Yes |  |  |  |

#### Summary

13 companies responded to the two questions on UE_ID, UE_ID_H.

All the companies agree to the using the same LTE hashed UE_ID calculation for UE_ID_H for NR.

Proposal 13 [13/13]: the same LTE hashed UE_ID calculation is used for UE_ID_H for NR.

Regarding the UE_ID for extended DRX, the proposal stating that the UE_ID is given by 5G-S-TMSI mod 4096, 10 companies agree, 1 does not, and 1 thought it is FFS while another company has a strong view (but tend to agree). Due to the majorty of support, moderator proposes the below:

Proposal 14 [11/13]: For eDRX in NR, the UE_ID is given by 5G-S-TMSI mod 4096

Proposal 14 [11/13]: For eDRX in NR, the UE_ID is given by 5G-S-TMSI mod 4096

Regarding the UE_ID for extended DRX, the proposal stating that the UE_ID is given by 5G-S-TMSI mod 4096, 12 companies agree, 1 does not, and 1 thought it is FFS while another company has a strong view (but tend to agree). Due to the majority of support, moderator proposes the below: Proposal 14 [12/14]: For eDRX in NR, the UE_ID is given by 5G-S-TMSI mod 4096

Discussion point PH2.4) Companies are requested to see if they can come to a consensus on agreeing to the UE_ID as defined by 5G-S-TMSI mod 4096. Pls provide in comments section if companies think they have a different view.

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Apple | We are ok to accept. |  |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| Intel | Yes |  |
| Huawei, HiSilicon | Yes |  |
| CATT | Yes |  |
| ZTE | Yes |  |
| Sequans | Yes |  |
| Samsung | Yes |  |
| DENSO | Yes | However, support of paging sub-grouping discussed under the ePower Saving WI might affect the calculation of UE_ID. |
| Xiaomi | Yes |  |
| Ericsson | Yes |  |

Summary for PH2.4:

14 companies responded to this question, and all agree to defining the UE_ID as 5G-S-TMSI mod 4096.

Proposal 2.5: the UE_ID for eDRX is defined by 5G-S-TMSI mod 4096.

## System Information Related

#### Applicable DRX cycle for SI modification period

In RAN2#113bis-e, RAN2 made the following agreements on eDRX acquisition period and about taking the LTE as the baseline.

- RAN2 confirms that SI modification mechanism from LTE is used as a baseline for SI change (other than ETWS and CMAS), i.e. by using an eDRX acquisition period and a flag to indicate SI modification for eDRX in Short Message (e.g. systemInfoModification-eDRX)

There are several open items related to the eDRX acquisition period and if there would be any diff between eDRX in IDLE and INACTIVE, esp with different possible configurations. Also, the decisions made here affect the DRX cycle calculation (T) for the two open items (which would be discussed in the later sections).

We start with the topic of which DRX cycle the UE should compare with the modification period to decide if the eDRX acquisition period is to be used. [7] proposes to reuse the LTE method and the same theme is maintained in [13] as well. In [4] a different method is proposed.

- companies are requested to provide views on the below:

Which DRX cycle the UE uses for comparing with the modification period to decide if the eDRX acquisition period is to be used:

Op7.1 : LTE style : CN_eDRX for both RRC_IDLE and RRC_INACTIVE

Op7.2 : CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (CN_eDRX in RAN eDRX is not configured)

| Company’s name | Do companies agree to |  |  |  |  |  |  | Comments, suggestions if any, for each of the items |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Op 7.1 |  | Op 7.2 |  |  | Others (pls provide more info in comments) |  |  |
| MediaTek |  |  | Yes |  |  |  |  | Because the difference between the maximum eDRX cycles for IDLE and INACTIVE is large, we think different values can be used unless there are any critical issues. |
| DENSO | Y |  | N |  |  |  |  | To simply the UE behaviour. Not sure if the SI update is critical for RedCap UEs. |
| Xiaomi | Y |  |  |  |  |  |  | Since currently the RAN e-drx cycle is below 10.24s, we can reuse LTE. We can further study if RAN e-DRX cycle beyond 10.24s is introduced. |
| OPPO | N |  | N |  |  | CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (RAN DRX if RAN eDRX is not configured) |  | For UE in RRC INACTIVE, since UE would always monitor for RAN paging based on RAN eDRX if configured for RRC_INACTIVE or RAN DRX if RAN eDRX is not configured, we think it would be better to use RAN eDRX or RAN DRX to compare with the modification period. |
| LGE | Yes |  |  |  |  |  |  |  |
| Huawei, Hisilicon |  |  | Yes |  |  |  |  | In the case of IDLE eDRX being longer than MP and INACTIVE eDRX cycle being no longer than MP, the RRC_INACTIVE UE will monitor PO with INACTIVE eDRX cycle outside PTW and would not miss legacy SI change indicator systemInfoModification. So for RRC INACTIVE UE, the RAN eDRX if configured shall be used to compare with MP. |
| Apple | Yes |  | N |  |  |  |  | Prefer LTE style |
| Sequans |  |  | Yes |  |  |  |  | Agree with HW |
| vivo | N |  | N |  |  | CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE |  | We assume there is a typo in Op7.2 ”CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (CN_eDRX if RAN eDRX is not configured)” In our view, for RRC_INACTIVE, if RAN eDRX cycle is not configured, the UE won’t miss the SI change notification: - when CN eDRX is not configured, UE in RRC_INACTIVE monitors PO based on T= minimum {UE specific DRX cycle if configured, RAN paging cycle, default paging cycle} - when CN eDRX is no longer than 10.24s, UE in RRC_INACTIVE monitors PO based on T= minimum {CN eDRX cycle, RAN paging cycle, default paging cycle} - when CN eDRX is longer than 10.24s, UE in RRC_INACTIVE monitors PO outside PTW based on T= minimum {RAN paging cycle, default paging cycle(ffs)} T is obviously no longer than the modification period in above cases. Hence there is no need using eDRX acquisition period. And if RAN eDRX cycle is configured and no longer than modification period： - when CN eDRX is no longer than 10.24s, UE in RRC_INACTIVE monitors PO based on T= minimum {CN eDRX cycle, RAN eDRX cycle} - when CN eDRX is longer than 10.24s, UE in RRC_INACTIVE monitors PO outside PTW based on T= RAN eDRX cycle. In the two cases, we think only when RAN eDRX cycle is longer than 10.24s, the eDRX acquisition period will be used. In summary, we support that: CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (CN_eDRX in RAN eDRX is not configured) |
| CATT | Y |  | N |  |  |  |  | SI modification is applicable to both inactive UE and idle UE, so one common eDRX acquisition period should be used for idle UE and inactive UE. Taking this into consideration, for inactive UE configured with CN eDRX with/without RAN eDRX, one common DRX cycle used to compare SI modification period should be adopted. i.e. CN eDRX. |
| ZTE |  |  | See comments |  |  |  |  | Considering the smallest RAN eDRX cycle is 2.56s, it is possible CN eDRX cycle is longer than SI modification period but RAN eDRX cycle is not. In this case, it is reasonable to apply legacy SI update mechanism. In addition, if RAN eDRX is not configured, but CN eDRX cycle and RAN paging cycle are longer than SI modification period, the UE may miss SI change notification too. In this case, the UE should also apply eDRX acquisition period mechanism. So, we think Op7.2 should be revised to: Op7.2 : CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (CN_eDRX RAN paging cycle if RAN eDRX is not configured) |
| Qualcomm |  |  | Yes |  |  |  |  |  |
| Intel |  |  |  |  |  | See comment |  | eDRX SI indication and modification period feature is critical when eDRX cycle is longer than BCCH modification period. Therefore option 7.2 seems preferable. However, it was agreed that “RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle”. Therefore it seems sufficient with option 7.1 assuming that UEs configured with INACTIVE eDRX can tolerate similar SI notification delays as UEs configured with IDLE eDRX. |
| Ericsson |  |  | Yes |  |  |  |  |  |  |
| Samsung |  | Y |  |  |  |  | Prefer simpler design. |  |  |

#### Summary

13 companies responded to the two questions on, how which DRX cycle should be used for determination of eDRX acquisition cycle and for the LTE style (using the IDLE CN eDRX) 5 companies agree, while 2 do not.. One company also seem to agree, but did not answer in the column (but in the comments section).

For the companies that oppose the LTE style, On using a separate CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE, 4 companies agree.

There are also different proposals, where 1 company proposes ‘RAN paging cycle’ in case RAN eDRX is not configured for INACTIVE. 1 company proposes not using eDRX acquisition period if CN eDRX is not configured.

There is no convergence! And to proceed further, since almost half of the responded companies (~6) are ok with LTE style, the moderator proposed the below options.

Proposal 15 [to discuss]: DRX cycle the UE uses for comparing with the modification period to decide if the eDRX acquisition period is to be used:

- [6/13] CN_eDRX for both RRC_IDLE and RRC_INACTIVE (same as LTE)

- [4/13] separate CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (use CN eDRX if RAN eDRX is not configured)

15 companies responded to the two questions on, how which DRX cycle should be used for determination of eDRX acquisition cycle and for the LTE style (using the IDLE CN eDRX) 6 companies agree, while 2 do not.. One company also seem to agree, but did not answer in the column (but in the comments section). For the companies that oppose the LTE style, On using a separate CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE, 5 companies agree. There are also different proposals, where 1 company proposes ‘RAN paging cycle’ in case RAN eDRX is not configured for INACTIVE. 1 company proposes not using eDRX acquisition period if CN eDRX is not configured. There is no convergence! And to proceed further, since almost half of the responded companies (~6) are ok with LTE style, the moderator proposed the below options. Proposal 15 [to discuss]: DRX cycle the UE uses for comparing with the modification period to decide if the eDRX acquisition period is to be used: - [6/13] CN_eDRX for both RRC_IDLE and RRC_INACTIVE (same as LTE) - [4/13] separate CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (use CN eDRX if RAN eDRX is not configured)

Since is no clear convergence, the intent of the moderator is to atleast narrow down the choices RAN2 needs to discuss. While some companies proposed using RAN paging cycle or not using eDRX acquisition period at all, in a broad sense the discussion is divided between using LTE style or separate DRX cycle for IDLE/INACTIVE.

The moderator proposes to collect views one more time to see if there would be a consensus between the two options:

Discussion point PH2.5) Companies are requested to provide which one they support among the below. Pls provide in comments section if companies think they have a different view.

| Company’s name | For comparing with the modification period to decide if the eDRX acquisition period is to be used do companies agree to |  | Comments, (esp if a diff DRX cycle is to be used in RAN INACTIVE) |
| --- | --- | --- | --- |
|  | Use CN_eDRX for both RRC_IDLE and RRC_INACTIVE | Use separate CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (Otherwise CN eDRX would be used) |  |
| OPPO | No | No | We agree that for UEs in RRC IDLE, CN eDRX cycle should be used to compare with SI MP, but we have different opinion for the case of UEs in RRC INACTIVE. For a UE in RRC INACTIVE: If a RAN eDRX cycle is configured, since the UE would always monitor for RAN paging based on RAN eDRX cycle, we think it is more reasonable to use RAN eDRX cycle instead of CN eDRX cycle to compare with SI MP. If RAN eDRX cycle is not configured, how does UE acquire updated SI would depend on UE behavior on paging monitoring (which is still FFS). - If default paging cycle is used to determine T, since default paging cycle is always shorter than SI MP, UE should follow legacy systemInfoModification to acquire updated SI. - If default paging cycle is not used to determine T, since the UE would always monitor for RAN paging based on RAN DRX cycle, RAN DRX cycle rather than CN eDRX cycle should be used to compare with the SI MP. |
| MediaTek | Yes | No | We are willing to compromise on this issue and go with the LTE mechanism (relying on CN eDRX cycle only) for the sake of progress of WI. |
| Apple | Yes | No | Keep it as LTE, no need to go with complex design, not needed for eDRX UEs to acquire SI in a time critical manner. |
| Qualcomm | No | Yes | It is not uncommon that UE may be configured with long eDRX cycle for RRC Idle and a (relatively) short eDRX cycle for RRC Inactive. If network has UE in RRC Inactive, it means network does expect UE is in a state where data arrives with short periodicity. If we can agree with this assumption, then timely acquisition of SI update is relevant and important for a smooth operation in RRC Inactive, e.g. otherwise it make take UE a long time to acquire updated SI if it is configured with very long eDRX cycle for RRC Idle. |
| Sharp | No | Yes | Considering RAN eDRX cycle is no longer than CN eDRX cycle, RAN eDRX seems more appropriate to be used to compare with MP. |
| Intel | Yes | Not essential | As we explained, RAN2 agreed that “RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle”. eDRX SI indication and modification period feature is critical when eDRX cycle is longer than BCCH modification period. Therefore assuming that UEs configured with INACTIVE eDRX can tolerate similar SI notification delays as UEs configured with IDLE eDRX, the use of CN_eDRX for both RRC_IDLE and RRC_INACTIVE seems sufficient. |
| Huawei, HiSilicon | No | FFS | Agree with OPPO. This is related to the answer of Discussion point PH2.9) |
| CATT | Yes | No | Same view as Apple/Intel |
| ZTE | Yes | No | We are fine to follow LTE mechanism now. For UE in RRC_INACTIVE, anyway, the UE will check the validity of stored SI before initiates RRC resume. Thus it is not critical to have in time SI update notification mechanism. We prefer to keep the mechanism simple. |
| Sequans | Yes | Yes | Prefer separate values, but OK to compromise |
| Samsung | See comment | See comment | The separate design (i.e. Use separate CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE) aims to reduce latency of SI update "only for UE in RRC_INACTIVE". However, we would like to note that this latency issue also occurs in UEs in IDLE. Therefore, we would like address this latency issue for both RRC_IDLE and RRC_INACTVE, by reusing existing on-demand SI request (to be discussed in Discussion point PH2.8). If RAN2 adopts to reuse on-demand SI request, RAN2 can go for simper option (i.e., Use CN_eDRX for both RRC_IDLE and RRC_INACTIVE) in this discussion point. Otherwise, we prefer to go 2nd option (i.e., Use separate CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE) |
| DENSO | Yes | No | Given that it was previously agreed that the SI change mechanism from LTE is used as the baseline for SI changes in RAN2#113bis meeting, we think that based on that LTE scheme would be sufficient coverage. |
| Xiaomi | Yes | No |  |
| Ericsson | No | FFS | This depends on whether the eDRX cycle in RRC-INACTIVE, if configured, is larger than the modification period and the outcome of the discussion PH2.9 |

Summary for PH2.5:

14 companies responded to this question, and 8 are ok with using the CN_eDRX as the DRX for both RRC_IDLE and RRC_INACTIVE to compare with modification period for eDRX acquisition period.

Among the 6 companies that are not ok, 3 would like a different DRX cycle for IDLE and INACTIVE to check with the modification period.

The moderator acknowledges that more than one option is possible, if we do not agree to using CN_eDRX for both IDLE/INACTIVE. And that the topic from PH2.9 is also linked to this a bit.

It would be better to discuss this online along with proposal P2.10.

Proposal 2.6 [to discuss]: DRX cycle the UE uses for comparing with the modification period to decide if the eDRX acquisition period is to be used:

- CN_eDRX for both RRC_IDLE and RRC_INACTIVE (same as LTE)

- separate CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (use CN eDRX if RAN eDRX is not configured)

#### eDRX acquisition period value

In [13] one company proposes that the eDRX acquisition period should be set to the maximum configurable value of the eDRX cycle, while in [2] an different view point is presented (which deviates from the LTE logic) in that the eDRX acquisition period is different for INACTIVE and IDLE (stating that UE monitors the RAN/INACTIVE paging cycle in INACTIVE, and that this can be used for SI change reception).

- companies are requested to provide views on the below:

Op 8.1 : eDRX acquisition period should be set to the maximum configurable value of the eDRX cycle

Op 8.2 : Introduce separate eDRX acquisition periods and SI modification indications for RRC idle UEs and RRC inactive UEs.

| Company’s name | Do companies agree to |  |  | Comments, suggestions if any, for each of the items |  |
| --- | --- | --- | --- | --- | --- |
|  | Op 8.1 | Op 8.2 | Others (pls provide more info in comments) |  |  |
| MediaTek | Yes | No |  | Inactive eDRX cycle can be compared against the existing SI modification period so that the UE will only use the longer value (e.g. eDRX acquisition period) if the Inactive eDRX cycle is longer than the existing SI modification period (unlikely as the Inactive eDRX values are quite short). |  |
| DENSO | Y | N |  | Same reason as the discussion point 7. |  |
| Xiaomi | Yes |  |  | The same as in LTE. |  |
| OPPO | No | Yes |  | For UEs in RRC inactive, if we follow the SI modification mechanism in LTE, a UE in RRC inactive configured with a CN eDRX longer than the modification period will acquire the update system information with a period much longer than the period the UE uses to monitor paging, which ends up with slow SI update. This may be acceptable to NB-IoT UEs who do not care much about the latency. However, we think this should be enhanced for NR UEs. We think a better solution would be to enable separate eDRX acquisition periods and SI modification indications for RRC idle UEs and RRC inactive UEs, and the eDRX acquisition periods for RRC idle UEs and RRC inactive UEs are 10485.76s and 10.24s, respectively. |  |
| LGE | Yes |  |  |  |  |
| Huawei, Hisilicon | Yes | No |  | We prefer reuse the LTE principle. About Op 8.2, if different indications are applied, we wonder what changes would apply only to RRC INACTIVE UEs but not RRC_IDLE UEs |  |
| Apple | Y | N |  | Same as LTE |  |
| Sequans | Yes | FFS |  | We are fine with LTE as baseline, but are OK to consider an enhancement if companies see the need |  |
| vivo | N | Y |  | Considering the upper bound of eDRX for RRC_IDLE and RRC_INACTIVE are 10485.76s and 10.24s, we think for RRC_IDLE, the boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 =0, and for RRC_INACTIVE, the boundaries of the eDRX acquisition period are determined by SFN values for which SFN mod 1024 =0. |  |
| CATT | Y | N |  | We don’t think the additional complexity of 8.2 is justified. There is not much difference SI-wise between Idle and Inactive states, so the system works fine with 8.1. |  |
| ZTE | Yes | No |  | For 8.2, separate eDRX acquisition periods imply the system information update timing will be different for UEs in RRC_IDLE and RRC_INACTVIE, this brings more complexity to network side. |  |
| Qualcomm | Yes |  |  |  |  |
| Intel | Yes | No |  | We support option 8.1 understanding that this follows legacy LTE eDRX definition for this. |  |
| Samsung | Yes | No |  | Pursue simplicity |  |
| Ericsson | Yes | No |  |  |

It is the view of the moderator, that depending on the direction discussion topic 8 goes, further details on this in terms of applicability in various cases (if needed) would be discussed later.

#### Summary

13 companies responded to the two questions on, how the length of the eDRX acquisition period and if separate periods are needed for IDLE and INACTIVE.

For the proposal that eDRX acquisition period should be set to the maximum configurable value of the eDRX cycle, only 2 companies oppose to this while 12 agree.

Proposal 16 [12/2]: eDRX acquisition period set to the maximum configurable value of the eDRX cycle.

Regarding if a separate eDRX acquisition period/SI modification indications is needed for IDLE and INACTIVE, 9 oppose, 2 agree and 1 company is FFS. To progress, moderator proposes the below:

Proposal 17 [9/2]: eDRX acquisition period is the same for RRC_IDLE and RRC_INACTIVE (as in LTE).

13 companies responded to the two questions on, how the length of the eDRX acquisition period and if separate periods are needed for IDLE and INACTIVE. For the proposal that eDRX acquisition period should be set to the maximum configurable value of the eDRX cycle, only 2 companies oppose to this while 12 agree. Proposal 16 [12/2]: eDRX acquisition period set to the maximum configurable value of the eDRX cycle. Regarding if a separate eDRX acquisition period/SI modification indications is needed for IDLE and INACTIVE, 9 oppose, 2 agree and 1 company is FFS. To progress, moderator proposes the below: Proposal 17 [9/2]: eDRX acquisition period is the same for RRC_IDLE and RRC_INACTIVE (as in LTE).

There seem to be a large majority for the definition of eDRX acquisition period. Moderator hopes that the remaining companies agree to reacha consensus on this.

Discussion point PH2.6) Companies are requested to see if they can come to a consensus on defining the  eDRX acquisition period to the maximum configurable value of the eDRX cycle. Pls provide in comments section if companies think they have a different view.

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Apple | Yes |  |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| Intel | Yes |  |
| Huawei, HiSilicon | Yes |  |
| CATT | Yes |  |
| ZTE | Yes |  |
| Sequans | Yes |  |
| Samsung | Yes |  |
| DENSO | Yes |  |
| Xiaomi | YEs |  |
| Ericsson | Yes |  |

Summary for PH2.6:

14 companies responded to this question, and all agree to defining the eDRX acquisition period to the maximum configurable value of the eDRX cycle.

Proposal 2.7: the eDRX acquisition period is the maximum configurable value of the eDRX cycle

Discussion point PH2.7) Companies are requested to see if they can come to a consensus on agreeing to the eDRX acquisition period being the same for RRC_IDLE and RRC_INACTIVE (as in LTE). Pls provide in comments section if companies think they have a different view.

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | No | For UEs in RRC inactive, if we follow the SI modification mechanism in LTE, a UE in RRC inactive configured with a CN eDRX longer than the modification period will acquire the update system information with a period much longer than the period the UE uses to monitor paging, which ends up with slow SI update. This may be acceptable to NB-IoT UEs who do not care much about the latency. However, we think this should be enhanced for NR RedCap UEs. Since the maximum RAN eDRX cycle is 10.24s, we think a better solution would be to enable separate eDRX acquisition periods and SI modification indications for RRC idle UEs and RRC inactive UEs, and the eDRX acquisition periods for RRC idle UEs and RRC inactive UEs are 10485.76s and 10.24s, respectively. |
| MediaTek | Yes | We are willing to compromise on this issue and go with the LTE mechanism for the sake of progress of WI. |
| Apple | Yes | Same as LTE |
| Qualcomm | No | We share the same view as OPPO |
| Sharp | Yes | Same as LTE |
| Intel | Yes | As explained in previous PH2.5) |
| Huawei, HiSilicon | Yes |  |
| CATT | Yes | As stated in ph1, we don’t think the additional complexity of having two eDRX acquisition periods is justified. There is not much difference SI-wise between Idle and Inactive states, so the system works fine with the LTE approach. |
| ZTE | Yes | Separate eDRX acquisition periods imply the system information update timing will be different for UEs in RRC_IDLE and RRC_INACTVIE, this brings more complexity to network side. |
| Sequans | Yes | We don’t see the real benefit in UEs in Idle and Inactive using different SI, this is unnecessary complexity for the NW |
| Samsung |  | Please see our response in PH2.5. Similarly, if RAN2 adopts to reuse on-demand SI request, we can support to use the same eDRX acquisition period for both RRC_IDLE and RRC_INACITVE. Otherwise, eDRX acquisition period for RRC_INACTIVE should be much shorter than eDRX acquisition period for RRC_IDLE, in order to reduce latency of SI update as mentioned by OPPO. |
| DENSO | Yes | Same as LTE |
| Xiaomi | Yes |  |
| Ericsson | Yes |  |

Summary for PH2.7:

14 companies responded to this question, and 11 agree to the eDRX acquisition period being the same for RRC_IDLE and RRC_INACTIVE (as in LTE). 2 companies do not agree to this, while 1 companies do not agree to this if the SI on-demand is not agreed.

Proposal 2.8 [ to discuss, for 11, no 2] : the eDRX acquisition period is the same for IDLE and INACTIVE.

#### On-Demand SI

In [3] one company proposes that the on-demand for SI acquisition for UEs with eDRX configuration. The moderator wants to check the companies view to see if this is worth pursuing.

- Do companies agree that for SI update of eDRX UEs configured to use DRX cycle longer than modification period, on-demand SI request is reused, with details and further enhancements as FFS.

| Company | Yes / No | Comments |  |
| --- | --- | --- | --- |
| MediaTek | Maybe | Whether on-demand SI request can also be used needs further discussion, and signaling overhead may need to be considered. However, LTE baseline (eDRX acquisition period) should be the baseline. |  |
| DENSO | No | For UEs configured with the long eDRX cycle, it can be assumed that the service is delay tolerant. Therefore, the delay of acquiring SIB is not so urgent. |  |
| Xiaomi | - | Not sure. LTE baseline e-DRX acquisition should be the baseline. |  |
| OPPO | No | Share the same view as DENSO |  |
| LGE | No | We think SI acquisition wouldn’t be so critical for RedCap UEs. |  |
| Huawei, Hisilicon | Yes |  |  |
| Apple | No | Not very crucial |  |
| Sequans | No | Agree with DENSO |  |
| vivo | N | We don’t think the SI change notification is urgent, to guarantee all UEs receive the notification, the latency of the new SI transmission inevitably will be longer. Introducing the enhancement will bring other problems, e.g. the network overhead. Hence, we think the enhancement is not needed. |  |
| CATT | No | Assuming the LTE mechanism that UE should acquire SI always before triggering the RRC establishment is reused (which is the most critical issue in our view), then there seems no need for using on-demand SI. |  |
| ZTE | No | Not all SIs are on-demand SI, the solution requires the UE to update some SI according to eDRX acquisition period, while to update other SI upon receiving the short message with systemInfoModification-eDRX (as proposed in [3]), this causes more complexity. |  |
| Qualcomm | See comment | LTE’s behavior can be used as baseline. Whether to use on-demand SI can be left to UE implementation. |  |
| Intel | Same as legacy | We would like that proponent companies explain why eDRX scenario would require different operation/specification than for a legacy UE in RRC_IDLE or RRC_INACTIVE on this regard. We believe that the features could be supported together unless specified otherwise |  |
| Samsung | Yes | In LTE, given length of e-DRX acquisition period (eAP) is 2621.44s (>43 min), the max. latency from reception of paging message to SI update is also >43 min. In NR, if RAN2 introduces a similar value of eAP with LTE, reusing on-demand SI request can be considered as a reasonable option. |  |
| Ericsson |  | Yes |  |

#### Summary

15 companies responded to the on-demand SI aspect for eDRX, 11 companies oppose any additional enhancements, 3 company agree and other company has this as FFS, while the rest think the legacy on-demand operation as is should be enough.

Due to majority on keeping it the same, the moderator proposes the below:

Proposal 18 [11/2/]: eDRX specific on-demand SI enhancements are not considered for Rel-17.

15 companies responded to the on-demand SI aspect for eDRX, 11 companies oppose any additional enhancements, 3 company agree and other company has this as FFS, while the rest think the legacy on-demand operation as is should be enough. Due to majority on keeping it the same, the moderator proposes the below: Proposal 18 [11/2/]: eDRX specific on-demand SI enhancements are not considered for Rel-17.

For this topic as well, the majority prefer not to enhance, but rather use the current SI on demand construct.

Discussion point PH2.8) Companies are requested to see if agree to not considering any eDRX specific on-demand SI enhancements are for Rel-17). Pls provide in comments section if companies think they have a different view.

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes | Actually, we don’t think there is any relation between eDRX and on-demand SI. |
| MediaTek | Yes |  |
| Apple | Yes |  |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| Intel | Yes | We support not to do anything special/new in specification to support/restrict its usage. |
| Huawei, HiSilicon | Yes |  |
| CATT | Yes |  |
| ZTE | Yes |  |
| Sequans | Yes | Agree with OPPO. |
| Samsung | No | Our input was missing in previous offline discussion: In LTE, given length of e-DRX acquisition period (eAP) is 2621.44s (>43 min), the max. latency from reception of paging message to SI update is also >43 min. In NR, if RAN2 introduces a similar value of eAP with LTE, reusing on-demand SI request can be considered as a reasonable option. For example, upon receiving a short message including the systemInfoModification-eDRX, UE triggers a request to acquire the SI immediately, rather than waiting for the next eAP. It can drastically improve latency of SI update. Besides, reusing on-demand SI request can address issues raised in PH2.5 and 2.7. |
| DENSO | Yes |  |
| Xiaomi | YEs | In my understanding, UE will check the SI validity before initiating a RRC connection establishment/resume procedure. Does that easy Samsung’s concern? |
| Ericsson | Yes |  |

Summary for PH2.8:

14 companies responded to this question, and all except for 1 company agree to not considering any eDRX specific on-demand SI enhancements are for Rel-17

Proposal 2.9: No eDRX specific on-demand SI enhancements are considered for Rel-17

## DRX (T) cycle determination for the FFS case

And finally the open item from the last meeting on the UE determination of T for the below FFS case. The arguments for/against both option-1/2 were already discussed in the last meeting.

Agreements online:

- For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, FFS which option below is adopted for paging monitoring:

Option 1: T is determined by the shortest of RAN paging cycle, IDLE eDRX cycle, and default paging cycle.

Option 2: T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, FFS which option below is adopted for paging monitoring:

Option 1: T is determined by the shortest of RAN paging cycle and default paging cycle.

Option 2: T is determined by RAN paging cycle.

While many companies (that have submitted papers on this) preferred op2 for both scenarios [13][12][9][8][7][5][2] (with main reason being that it’s aligned with LTE), in [1][4][6][14] op1 is preferred.

However, there are (at least in the moderator’s view) some items that might need consideration when discussing this topic:

- The eDRX support is optional for the gNB (meaning there could be gNBs that support and configure to RedCap UEs, but do not support eDRX – so would NOT configure eDRX for INACTIVE)

- We can have non RedCap UE supporting eDRX (this is not precluded as of now) and these UEs could be handle by gNBs that do not support RedCap (as well as gNBs that do).

- Forcing the UE to include ‘default paging cycle’ into the determination of T implies that SI change update is important for the UE (redcap or non-redcap with eDRX support) and important enough to have the UE also monitor the default paging cycle outside of PTW: In other words, if the NW allowed the UE to operate in CN-eDRX mode, is the SI update critical for the UE during RAN inactive (while it isn’t during RAN eDRX – if configured by the gB).

- Also the discussion on the SI update from 3.2 might be a factor in this discussion.

The moderator requests the companies to provide their (initial) view on which of Op1/Op2 is needed, also noting that the same option for scenarios is preferred (as also voiced in [15]).

- Companies are requested to input which option for both the FFS scenarios:

| Company’s name | Which option do companies prefer? (pls justify if the same option is not selected for both scenarios). |  | Comments |  |
| --- | --- | --- | --- | --- |
|  | Option for RRC_INACTIVE without eDRX, while eDRX for IDLE is 10.24 or lower Op1: T is determined by the shortest of RAN paging cycle, IDLE eDRX cycle, and default paging cycle. Op2: T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle. | Option for RRC_INACTIVE without eDRX, while eDRX for IDLE is more than 10.24 Op1: T is determined by the shortest of RAN paging cycle, and default paging cycle. Op2: T is determined by the RAN paging cycle |  |  |
| MediaTek | Op2 | Op2 | Any possibility of missed SI change notification can be resolved by introducing an eDRX acquisition period. Op2 aligns with LTE. Op1 does not give the network any chance to configure the UE with T > default paging cycle, which reduces the benefit of eDRX in Inactive. |  |
| DENSO | Op2 | Op2 | If the service is delay tolerant for RedCap UEs, SI update may not be critical. |  |
| Xiaomi | Opt2 | Op2 | Op2 aligns with LTE. We do not see strong reason to deviate from LTE way. |  |
| OPPO | Opt2 | Op2 | We prefer to reuse LTE mechanism. |  |
| LGE | Op2 | Op2 |  |  |
| Huawei, Hisilicon | Option 1 | Option 1 | To avoid missing SI change indication( and having to check the VT before every access) |  |
| Apple | Op2 | Op2 | Same as LTE, no need to over-optimize. |  |
| Sequans | Opt 1 | Opt 1 | Agree with HW |  |
| vivo | Op2 | Op2 | The difference between the two options only occurs when RAN paging cycle is longer than default paging cycle. In this case, option 1 means T=default paging cycle and option 2 means T= RAN paging cycle. It is true that if the UE monitors PO as in option2, it may miss some PO when the SI change occurs. However, it is not a big issue as the SI change notification will repeat in modification period. Thus, we think reusing LTE principle is straightforward. |  |
| CATT | Opt2 | Opt2 | SI update mechanism is primarily governed by the SI modification and eDRX acquisition periods, rather than default paging cycle. |  |
| ZTE | Op2 | Op2 | Same view as MediaTek |  |
| Qualcomm | Op1 | Op1 | If UE is in RRC Inactive and it is not configured with eDRX for RAN paging, it means network wants UE to monitor SI update per default paging cycle. Otherwise, it would have configured eDRX for RRC Inactive. |  |
| Intel | Option 1 | Option 1 | We understand that during this time a UE behaves same as legacy UE in RRC_INACTIVE in which case UE monitors the shortest of all the configured/applicable DRX cycles (including default paging cycle) |  |
| Ericsson | Option 1 | Option 1 |  |
| Samsung |  |  | No strong view, but prefer to keep consistency (i.e., option 1 in both scenarios, or option 2 in both) |  |

#### Summary

15 companies responded to the ‘T’ determination for the 2 FFS cases (with a selection between Op1/Op2).

9 companies support Op2, and 5 support Op1.1 company is ok with either.

Moderator proposes that companies reach a consensus here.

Proposal 19 [9/5]: Option 2 is agreed for the below two cases:

For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.

For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, T is determined by RAN paging cycle.

15 companies responded to the ‘T’ determination for the 2 FFS cases (with a selection between Op1/Op2). 9 companies support Op2, and 5 support Op1.1 company is ok with either. Moderator proposes that companies reach a consensus here. Proposal 19 [9/5]: Option 2 is agreed for the below two cases: For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle. For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, T is determined by RAN paging cycle.

While the moderator predicts that an online discussion is needed, it is worth trying one more time if companies can reach a consensus.

Since Op2 seems to have a bit of majority,

Discussion point PH2.9) Companies are requested to see they are willing to reach a consensus on using Option 2 for both scenarios.  Pls provide in comments section if companies think they have a different view.

For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.

For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, T is determined by RAN paging cycle.

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Apple | Yes | It is not critical for RedCap UEs to monitor with default paging cycle to be up-todate with the SI (esp outside the PTW). This requirement can be relaxed, esp when CN has already allowed the eDRX operation. |
| Qualcomm | Yes |  |
| Sharp | Yes |  |
| Intel | Yes | This is not our preference but at this point, we are willing to go with majority view on this. |
| Huawei, HiSilicon | No | To avoid missing SI change indication and having to check the VT before every access Note this is also related to discussion PH2.5 |
| CATT | Yes |  |
| ZTE | Yes | Same view as Apple. |
| Sequans | No, but can compromise | Prefer other option, agree with HW |
| Samsung | Yes |  |
| DENSO | Yes |  |
| Xiaomi | Yes |  |
| Ericsson | Yes, but | Further details need to be sorted out with respect to the discussion on SI change indication |

Summary for PH2.9:

14 companies responded to this question, and 12 companies are willing to compromise to op2, and 1 company is also ok with option2, but would like assess this with PH2.5 topic.

Only 1 company does not accept on the option-2.

Proposal 2.10: (13 ok, 1 against)

- For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, T is determined by RAN paging cycle.

## Others

- Any other relevant issues need to be discussed?

| Company | Issue description |
| --- | --- |
| Xiaomi | We want to confirm with people whether we need to differentiate gNB’s capability for idle mode e-DRX or for inactive mode e-DRX. An example is UE with an idle mode e-DRX with a CN PTW moves to a gNB not supporting idle mode e-DRX will trigger CN PTW disabled while may be not impacted under a gNB not supporting inactive mode e-DRX. [Moderator requests the company to bring this up in the online discussion] |

# Summary

- Phase – 1:

- Easy agreements:

Proposal 1 [13/13] :  The max eDRX cycle length for RRC Inactive is 10.24s in Rel-17

Proposal 2 [13/13] :  Capture in the specification explicitly the below restrictions:

- RAN2 considers the configuration as an invalid case, where INACTIVE eDRX cycle is configured but IDLE eDRX cycle is not configured.

- RAN2 considers the configuration as invalid case, where INACTIVE eDRX cycle is longer than IDLE eDRX cycle.

Proposal 4 [13/13]: PO determination for non-overlapping CN/RN case is applicable to eDRX

Proposal 5 [13/13]:: When IDLE eDRX and INACTIVE eDRX are configured and both cycles are no longer than 10.24s, PO is determined by IDLE eDRX.

Proposal 6 [13/13]:: When IDLE eDRX is configured and is no longer than 10.24s, INACITVE eDRX cycle is not configured, PO is determined by IDLE eDRX.

Proposal 7 [13/13]:: During CN PTW when IDLE eDRX is configured and longer than 10.24s, and INACTIVE eDRX is configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer.

Proposal 8 [13/13]:: During CN PTW when IDLE eDRX is configure and is longer than 10.24s, INACTIVE eDRX cycle is not configured, PO is determined by the shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer.

Proposal 9 [13/13]: eDRX supporting UEs are assumed to also support the UE capability on PO determination for non overlapping CN/RN case, and no additional capability is needed.

Proposal 11 [13/13]: The below working agreement is now changed to an agreement.

When IDLE eDRX cycle is longer than 10.24s, CN PTW_start calculation formula defined in LTE is re-used as the baseline, as below. FFS whether CN PTW_start position could be configurable by network and in case which node decides the N value. Note: this formula would be revisited if INACTIVE eDRX cycle can be above 10.24s

PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation:

SFN = 1024/N* ieDRX, where

ieDRX = floor(UE_ID_H /TeDRX,H) mod N

FFS N = 4 or 8, FFS if N can take other values

Proposal 13 [13/13]: the same LTE hashed UE_ID calculation is used for UE_ID_H for NR.

- To discuss:

Proposal 3 : It is FFS if an explicit table capturing the determination of ‘T’ for different DRX/eDRX configurations, is needed in TS38.304. If the table is agreed to be added, this table can also include the restrictions from P2.

Proposal 10 [To discuss]:: RAN2 to discuss if a eDRX capability is also needed to be reported to gNB.

Proposal 12 [8/13]: The value of N is 8 and it is not configurable..

Proposal 14 [11/13]: For eDRX in NR, the UE_ID is given by 5G-S-TMSI mod 4096

Proposal 15 [to discuss]: DRX cycle the UE uses for comparing with the modification period to decide if the eDRX acquisition period is to be used:

- [6/13] CN_eDRX for both RRC_IDLE and RRC_INACTIVE (same as LTE)

- [4/13] separate CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (use CN eDRX if RAN eDRX is not configured)

Proposal 16 [10/2]: eDRX acquisition period set to the maximum configurable value of the eDRX cycle.

Proposal 17 [7/2]: eDRX acquisition period is the same for RRC_IDLE and RRC_INACTIVE (as in LTE).

Proposal 18 [11/2/]: eDRX specific on-demand SI enhancements are not considered for Rel-17.

Proposal 19 [9/4]: Option 2 is agreed for the below two cases:

For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.

For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, T is determined by RAN paging cycle.

Phase 2 Summary:

(For agreement):

Proposal 2.1 : eDRX feature can be supported by non RedCap UEs.

Proposal 2.2 : eDRX feature request by the UE is using the NAS signalling. FFS if additional capability signalling in RAN in the UE capability message, is also needed

Proposal 2.3 : eDRX support is optional for the RedCap UE.

Proposal 2.5: the UE_ID for eDRX is defined by 5G-S-TMSI mod 4096.

Proposal 2.7: the eDRX acquisition period is the maximum configurable value of the eDRX cycle

Proposal 2.9: No eDRX specific on-demand SI enhancements are considered for Rel-17

(Needs discussion for agreements):

Proposal 2.4 [to discuss, 11 for, 3 against]: For the eDRX PTW start calculation, agree to N=8. No signalling needed to CN.

Below proposals deal with SI update for eDRX devices:

Proposal 2.6 [to discuss]: DRX cycle the UE uses for comparing with the modification period to decide if the eDRX acquisition period is to be used:

- CN_eDRX for both RRC_IDLE and RRC_INACTIVE (same as LTE)

- separate CN eDRX for RRC_IDLE, and RAN eDRX if configured for RRC_INACTIVE (use CN eDRX if RAN eDRX is not configured)

Proposal 2.8 [ to discuss, for 11, no 2] : the eDRX acquisition period is the same for IDLE and INACTIVE.

Proposal 2.10: (13 ok, 1 against)

- For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle.

- For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, T is determined by RAN paging cycle.

# References

R2-2109449	Remaining issues on eDRX	Qualcomm Incorporated	discussion	Rel-17	FS_NR_redcap

R2-2109495	Discussion on eDRX for RedCap Ues	OPPO	discussion	Rel-17	NR_redcap-Core

R2-2109537	UE_ID for extended DRX cycle and SI update aspects	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_redcap-Core

R2-2109578	eDRX for RedCap UE	Huawei, HiSilicon	discussion	Rel-17	NR_redcap-Core

R2-2109649	Discussion on e-DRX for Redcap Devices	Beijing Xiaomi Mobile Softwar	discussion

R2-2109671	Leftover issues for eDRX	Intel Corporation	discussion	Rel-17	NR_redcap

R2-2109699	Further Discussion on eDRX for NR RRC Inactive and Idle	CATT	discussion	Rel-17	NR_redcap-Core

R2-2109743	Discussion on eDRX  for RedCap UEs	vivo,  Guangdong Genius	discussion	Rel-17	NR_redcap-Core

R2-2109898	Discussion on eDRX for RedCap UE	ZTE Corporation, Sanechips	discussion	Rel-17	NR_redcap-Core

R2-2110151	Leftover issues on derivation of PTW_start	DENSO CORPORATION	discussion	Rel-17	NR_redcap-Core

R2-2110331	Consideration on eDRX for RedCap UE	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2110584	Discussion on eDRX for RRC_IDLE and RRC_INACTIVE	LG Electronics UK	discussion	Rel-17

R2-2110755	Remaining issues for eDRX	MediaTek Inc.	discussion	Rel-17	NR_redcap-Core

R2-2111099	Extended DRX for Reduced Capability UEs	Ericsson	discussion	NR_redcap-Core

R2-2111129	Remaining issues in paging monitoring	Samsung	discussion	Rel-17

R2-2111335 R2-2111335-summary_offline#105_eDRX_cycles_ph1