3GPP TSG-RAN WG2 Meeting #116 Electronic	R2-2111531

Elbonia, 01 – 12 November 2021

Agenda item:	6.1.2

Source:	Nokia (Rapporteur)

Title:	Offline 005 on Stage 2

WID/SID:	NR_newRAT-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

This document is the report of the following email discussion:

[AT116-e][005][NR16] Stage-2 (Nokia)

Scope: Determine agreeable parts in a first phase, for agreeable parts agree on CRs. Treat R2-2109535, R2-2109952, R2-2110732, R2-2109459, R2-2110527

Intended outcome: Report, Agreed CRs if applicable

Deadline: Schedule 1 - A first round with Deadline for comments Thursday W1 Nov 4 1200 UTC to settle scope what is agreeable etc; A Final round with Final deadline Thursday W2 Nov 11 1200 UTC to settle details / agree CRs etc.

# 2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Nokia (Rapporteur) | Benoist Sébire | benoist.sebire@nokia.com |
| Huawei, HiSilicon | Zhenzhen Cao | caozhenzhen@huawei.com |
| Samsung | Jaehyuk Jang | jack.jang@samsung.com |
| LG Electronics | SeungJune Yi | seungjune.yi@lge.com |
| Google | Jing-Rong Hsieh | jinghsieh@google.com |
| Qualcomm Incorporated | Masato Kitazoe | mkitazoe@qti.qualcomm.com |
| vivo | Wenjuan Pu | wenjuan.pu@vivo.com |
| OPPO | Xin You | youxin@oppo.com |
| ZTE | Dong FEI | Dong.fei@zte.com.cn |
| Intel | Yi Guo | Yi.guo@intel.com |
| CATT | Pierre Bertrand | pierrebertrand@catt.cn |
| Apple | Fangli XU | fangli_xu@apple.com |
| MediaTek | Felix Tsai | chun-fan.tsai@mediatek.com |
| Sharp | Kyosuke Inoue | kyosuke_inoue@sharp.co.jp |
| ITRI | Nai-Lun Huang | NellenHuang@itri.org.tw |
| Ericsson | Cecilia Eklöf | cecilia.eklof@ericsson.com |

# 3	Discussion Phase 1

## 3.1	CRs for 38.300

Three CRs were contributed. It is suggested to discuss them individually.

R2-2109535	Corrections to early measurements in RRC INACTIVE	Samsung Electronics Co., Ltd	CR	Rel-16	38.300	16.7.0	0390	-	F	NR_Mob_enh-Core

Question 1: Do companies agree with the issue and if yes, are the suggested changes fine or does the text need to be improved or even possibly merged with another CR ?

| Answers to Question 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company |  | Yes/No |  | Technical Arguments |  |
| Huawei, HiSilicon |  | Yes |  | It may be better to put the sentence together with RRC_IDLE, because the sentence on the transmission after SMC also applies to this case. |  |
| Samsung (proponent) |  | Yes |  | Merely to add the missing scenario (i.e. RRCResumeRequest RRCSetup RRCSetupComplete with indication) which is allowed in the stage-3 specification. |  |
| LGE |  | Yes |  | Only this scenario is missing. |  |
| Google |  | Yes |  | Fine to add the missing scenario. |  |
| Qualcomm Incorporated |  | Yes, but |  | The scenario added by this CR corresponds to the behaviour in section 5.3.3.4 of RRC, where early measurement indication is controlled purely based on SIB1. Probably better suited in the previous paragraph. |  |
| vivo |  | Yes |  | The description in the proposed CR is fine to us. |  |
| Nokia |  | Yes |  | Could be merged with rapporteur CR though. Note that the CR was not checked with the rapporteur as normally required. |  |
| OPPO |  | Yes |  | We are fine with the CR. |  |
| ZTE |  | Yes |  |  |  |
| Intel |  | Yes |  |  |  |
| CATT |  | Yes |  |  |  |
| Apple |  | Yes |  | We agree with the intention. For the change, it can be in the simple way as below, and merged into the Rapporteur CR. |  |
| MediaTek |  | No strong view |  | The intention is correct. It seems not essential anyway. |  |
| Sharp |  | Yes |  |  |  |
| ITRI |  | Yes |  | Only this scenario is missing here. Fine to add it. |  |
| Ericsson |  | No |  |  |  |

Summary 1: all companies having expressed an opinion but one are fine with the intention. Two suggestions were made to simplify the text and the one from Apple seems the simplest.

Proposal 1: incorporate the change suggested by Apple into the rapporteur CR.

R2-2109952	Miscellaneous Corrections	Nokia (Rapporteur), Nokia Shanghai Bell, Sharp	CR	Rel-16	38.300	16.7.0	0391	-	F	NR_IIOT_URLLC_enh-Core

Question 2: Do companies agree with the issue and if yes, are the suggested changes fine or does the text need to be improved or even possibly merged with another CR ?

| Answers to Question 2 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Huawei, HiSilicon | Yes |  |
| Samsung | Yes | We are fine with the proposed changes, and also fine to have a separate CR. |
| LGE | Yes |  |
| Google | Yes |  |
| Qualcomm Incorporated | Yes |  |
| vivo | Yes |  |
| Nokia | Yes |  |
| OPPO | Yes |  |
| ZTE | Yes |  |
| Intel | Yes |  |
| CATT | Yes |  |
| Apple | Yes |  |
| MediaTek | Yes |  |
| Sharp | Yes |  |
| ITRI | Yes |  |
| Ericsson | Yes with a comment to the first change | “Comp. refers to header compression” has been there since the Rel-15 spec. In Rel-16, RAN2 has introduced EHC but the figure 6.1-1 and figure 6.1-2 are not updated (which has only ROHC). No strong view, but we prefer to keep “Comp. refers to header compression” and change the figure so that “ROHC” is replaced with “Comp.” There might be a need to change Rel-15 specs, but it is up to the spec rapporteur. |

Summary 2: all companies having expressed an opinion are fine with the intention. Regarding the remark from Ericsson, the rapporteur of TS 38.300 (who happens to be the same rapporteur as for this email discussion) is fine to leave Rel-15 untouched.

Proposal 2: consider R2-2109952 as baseline to incorporate further changes if any.

R2-2110732	Correction to 38300 on 2step CFRA configuration	ZTE Corporation, Sanechips	CR	Rel-16	38.300	16.7.0	0395	-	F	NR_2step_RACH-Core

Question 3: Do companies agree with the issue and if yes, are the suggested changes fine or does the text need to be improved or even possibly merged with another CR ?

| Answers to Question 3 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Huawei, HiSilicon | No | It doesn’t seem to be essential, and even if the text is needed we are not sure stage-2 is the place for such detailed information. |
| Samsung | No | We do not think that it is essential to be captured in stage-2 specification, and can leave it the sensible implementation. Note that, even in legacy, the network configures CFRA resource for 4-step RA type only on a BWP that has CBRA resource for 4-step RA type, which is not captured in stage-2 specification either. So similar change for 2 step RA seems not needed. |
| LGE | No | Not essential. If this CR is agreed, bunch of CRs specifying the restriction are expected in the future meetings. We should avoid this kind of CRs. |
| Google | Yes/No | No strong view |
| Qualcomm Incorporated | Yes |  |
| vivo | No | Prefer not to capture such detailed information in stage 2 specification. |
| Nokia | No | Could be in field description of RRC instead |
| OPPO | No | The restriction has already been captured in TS 38.331. We see no need to capture it in stage2 spec. |
| ZTE | Yes | Proponent, also, if companies think stage 3 is suitable place to put it in, we also accept. |
| Intel | Yes |  |
| CATT | No | It is not needed to capture such restriction in stage 2 specification. |
| Apple | Yes | The change can be merged into the Rapporteur CR. |
| MediaTek | Yes |  |
| Sharp | No | As some companies mention above, it is not needed to capture that in stage-2 spec. |
| ITRI | No | There seems no need to put such detailed description in stage 2 spec. It has been captured in TS 38.331. |
| Ericsson | Yes |  |

Summary 3: there is not a strong enough consensus (7 out 15) to agree the CR.

Proposal 3: R2-2110732 is not agreed.

## 3.1	CRs for 37.340

Two CRs were contributed. It is suggested to discuss them individually.

R2-2109459	Correction on conditional reconfiguration for PSCell 	Google Inc., Intel Corporation, ZTE	CR	Rel-16	37.340	16.7.0	0287	-	F	NR_Mob_enh-Core

Question 4: Do companies agree with the issue and if yes, are the suggested changes fine or does the text need to be improved or even possibly merged with another CR ?

| Answers to Question 4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Company |  | Yes/No |  | Technical Arguments |  |
| Huawei, HiSilicon |  | No |  | At least the feedback we got from RAN3 is that RAN3 did not agree on anything about this topic. |  |
| Samsung |  | No |  | No strong view but the changes seem not essential. Still, UE may fail to reply, e.g. due to radio link failure, even if it is a rare case. |  |
| LGE |  | No |  | We don’t see any problem with the current text. |  |
| Google (proponent) |  | Yes |  | In the RAN3 offline discussion (R3-214265) the moderator summary on this topic (R3-213957) is “1 see the CR is not needed. 1 has doubt if this RAN3 part. 5 are ok with the CR.“ and a way forward captured by the moderator is “RAN3 generally acknowledges this technical issue in R3-213957 and it is up to company to propose to RAN2.” As the step before the change has stated that the UE applies the new configuration and replies with the MN RRC Reconfiguration Complete message, the RLF scenario is not considered. It is misleading that the MN RRC Reconfiguration Complete message may not include a SN RRC response message. |  |
| Qualcomm Incorporated |  | No |  | The change does not seem essential. |  |
| vivo |  | No |  | The UE behaviour for sending RRC Reconfiguration complete in the mentioned cases has been clearly specified in 331 spec, so the issue raised here may not essential. |  |
| Nokia |  | Yes |  | Could be merged with another CR and cover sheet should be better formulated (impact analysis and consequences if not approved). |  |
| OPPO |  | No |  | The CR does not seem to be essential. |  |
| ZTE |  | Yes |  | Agree with Nokia. |  |
| Intel |  | Yes |  |  |  |
| CATT |  | No |  | No strong view, anyway nothing is wrong. |  |
| Apple |  | - |  | No strong view |  |
| MediaTek |  | No strong view |  | Change is correct and could be merged with Rapp’s CR. But it is not essential. |  |
| Sharp |  | No |  | No strong view, but it does not seem any essential to change. |  |
| ITRI |  | No strong view |  | The change can avoid causing misunderstanding, but nothing is wrong in the current text. |  |
| Ericsson |  | Maybe |  | The CR seems correct, but not having it will also not cause any issue. Could be included in rapporteur CR. |  |

Summary 4: there is not a strong enough consensus (5 out 15) to agree the CR.

Proposal 4: R2-2109459 is not agreed.

R2-2110527	Corrections on SCG/MCG failure handling	ZTE Corporation, Sanechips	CR	Rel-16	37.340	16.7.0	0288	-	F	NR_newRAT-Core, LTE_NR_DC_CA_enh-Core

Question 5: Do companies agree with the issue and if yes, are the suggested changes fine or does the text need to be improved or even possibly merged with another CR ?

| Answers to Question 5 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Huawei, HiSilicon | Yes |  |
| Samsung | Yes | We are fine with the proposed changes which make current descriptions clearer. |
| LGE | Comments | 1. “SCG transmission of radio bearers is not suspended”: Not needed 2. “CPCPSCell addition or PSCell change”: Agree with the intention. But, “Conditional” needs to be added. 3. “except SRB0”: Agree 4. LTE RRC message: Not needed |
| Google | Yes |  |
| Qualcomm Incorporated | Yes | For the first change in section 7.7, indeed we don’t know why only Conditional PSCell Change was added there. The behaviour should equally be applicable to “non-conditional” cases as proposed by the CR. If any company could explain the history…. |
| vivo | Yes |  |
| Nokia | Yes |  |
| OPPO | Yes |  |
| ZTE (proponent) | Yes | For LGE’s comment-1, the change is to clarify that the UE can not report measurements via SRB3 during SCG failure handling even if SRB3 is configured. For LGE’s comment-2, we used “PSCell change” to cover both CPC and legacy PSCell change in the stage-2 spec, e.g. in section 10.6. So we think no need to add CPC redundantly. For LGE’s comment-3, the LTE RRC message can be sent in case of (NG)EN-DC for fast MCG recovery. For QC’s question, the description was added for intra-SN CPC in the R16 eMob WI. But unfortunately, the behaviour for legacy case was missed. |
| Intel | See comments | Change in 7.2, do not see the need to have this additional sentence. It is general description, do not need to consider every details. Other changes Looks ok to us. |
| CATT | Yes | For the 2nd change, the CPC also needs to be added, i.e., “During the execution of PSCell addition or PSCell change or conditional PSCell change, if radio link failure is detected for MCG, the UE initiates the RRC connection re-establishment procedure”. |
| Apple | Yes | Fine with the change and the change can be merged into the Rapporteur CR. |
| MediaTek | Yes |  |
| Sharp | Yes |  |
| ITRI | Yes |  |
| Ericsson | Partly | The CR contains of 4 different changes. The first change is to mention that measurement reporting via SRB3 is only done if SCG is not suspended. We think this clarification is not needed, since it is already clear that when SCG is suspended, nothing is transmitted on SCG. If we add it here measurement reports on SRB3, we should add it also for all other cases, which seems not motivated. The rest of the changes (2-4) are to align stage-2 description with stage-3 and can be agreed. |

Summary 5: in general there seems to be a good consensus on the need and actual content of the CR. One point which remains unclear (at least to the rapporteur) is whether conditional PSCell change should be listed explicitly in 7.7.

Proposal 5: polish the wording of the R2-2110527 CR in phase 2.

# 4	Phase 2

Question 6: regarding CR R2-2110527 should conditional PSCell change be listed explicitly in 7.7

| Answers to Question 6 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| ZTE | No | In our understanding, the conditional PSCell change is defined as one of the types of PSCell change. And the current stage-2 and stage-3 specs has used “PSCell change” to refer to both CPC and legacy PSCell change. Please see the text below: - TS 37.340 section 10.6 A Conditional PSCell Change (CPC) is defined as a PSCell change that is executed by the UE when execution condition(s) is met. The UE starts evaluating the execution condition(s) upon receiving the CPC configuration, and stops evaluating the execution condition(s) once PSCell change is triggered. - TS 38.331 section 5.3.10.3 5> if neither PSCell change nor PSCell addition is ongoing (i.e. timer T304 for the NR PSCell is not running in case of NR-DC or timer T307 of the E-UTRA PSCell is not running as specified in TS 36.331 [10], clause 5.3.10.10, in NE-DC): 6> initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure. Note: the “PSCell change” in the text above covers the cases of both CPC and legacy PSCell change . So we think no need to explicitly list “conditional PSCell change” in 7.7. But if the majority wants to explicitly capture this, we can also accept it. |
| Qualcomm Incorporated | No | No strong view, but it should clear that the text “execution of PSCell change” covers the execution phase of CPC. |
| Ericsson | No | Not needed. In addition, we think the addition in 7.2 is not needed, that measurement reporting via SRB3 is only done if SCG is not suspended. It is already clear that when SCG is suspended, nothing is transmitted on SCG. |

Summary 6: Based on the feedback of three companies, conditional PSCell change does not need to be listed explicitly in 7.7.

Proposal 6: agree R2-2110527.

# 5	Conclusion

The following reflects the discussion & agreements into the minutes with revision marks:

### 6.1.2	Stage 2 corrections

You should discuss your stage 2 CRs with the specification rapporteurs before submission.

[AT116-e][005][NR16] Stage-2 (Nokia)

Scope: Determine agreeable parts in a first phase, for agreeable parts agree on CRs. Treat R2-2109535, R2-2109952, R2-2110732, R2-2109459, R2-2110527

Intended outcome: Report, Agreed CRs if applicable

Deadline: Schedule 1
Report provided in R2-2111531

#### 6.1.2.1	TS 3x.300

R2-2109535	Corrections to early measurements in RRC INACTIVE	Samsung Electronics Co., Ltd	CR	Rel-16	38.300	16.7.0	0390	-	F	NR_Mob_enh-Core

[005] proposed changes to be incorporated in Rapporteur CR

R2-2109952	Miscellaneous Corrections	Nokia (Rapporteur), Nokia Shanghai Bell, Sharp	CR	Rel-16	38.300	16.7.0	0391	-	F	NR_IIOT_URLLC_enh-Core

[005] endorsed as baseline, further updates to be provided in R2-2111470

R2-2111470	Miscellaneous Corrections	Nokia (Rapporteur), Nokia Shanghai Bell, Samsung, Sharp	CR	Rel-16	38.300	16.7.0	0391	1	F	NR_IIOT_URLLC_enh-Core, NR_Mob_enh-Core

[005] agreed

R2-2110732	Correction to 38300 on 2step CFRA configuration	ZTE Corporation, Sanechips	CR	Rel-16	38.300	16.7.0	0395	-	F	NR_2step_RACH-Core

[005] not agreed

#### 6.1.2.2	TS 37.340

R2-2109459	Correction on conditional reconfiguration for PSCell 	Google Inc., Intel Corporation, ZTE	CR	Rel-16	37.340	16.7.0	0287	-	F	NR_Mob_enh-Core

[005] not agreed

R2-2110527	Corrections on SCG/MCG failure handling	ZTE Corporation, Sanechips	CR	Rel-16	37.340	16.7.0	0288	-	F	NR_newRAT-Core, LTE_NR_DC_CA_enh-Core

[005] agreed.