3GPP TSG-RAN WG2 Meeting #116bis-e		R2-2201104

Electronic, 17 January – 25 January, 2021

Agenda item:	8.1.17

Source:	Apple

Title:	Signalling impacts of RAN1 agreements on accuracy enhancements

Document for:	Discussion and Decision

# 1	Introduction

In this paper we analyze the RAN1 agreements related to accuracy improvements, including UE Rx/Tx and/or gNB Rx/Tx timing delays and improvements for DL-AoD positioning and derive the corresponding the RAN2 signaling to support those.

Based on the analysis of the RAN1 agreements we propose enhancements to assistance data, measurements and capabilities signaling.

# 2   	Discussion

## 2.0	Background

In this section we list some background information, primarily some definitions introduced by RAN1, for the purpose of the following discussion about signalling.

For mitigation of UE Rx/Tx and/or gNB Rx/Tx timing delays, RAN1 have introduced the notion of “timing error groups” as follows:

- Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at the baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error. - Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error. - UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin. - TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin. - UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin. - TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin. - UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin. - TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

For accuracy improvements for UL-AoA, RAN1 introduced the following definitions:

- UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response. - DL PRS RSRPP is defined in a similar fashion

In this paper we discuss the following signalling enhancements that are required:

- Assistance Data

- Measurements

- Capabilities

## 2.1	Assistance Data

### 2.1.1	Mitigation of UE Rx/Tx and/or gNB Rx/Tx timing delays

For mitigation of UE Rx/Tx and/or gNB Rx/Tx timing delays, the required assistance data is the association between the DL PRS resources and the TRP Tx TEG. According to the RAN1 agreements, the following enhancements are needed:

- LMF provides the association information of DL PRS resources with TRP Tx TEG ID, to UE

In order for LMF to provide the TRP Tx TEG information, which is associated with DL PRS, LPP ProvideAssistanceData (and posSIB) signalling need to be enhanced to convey that information.

Proposal 1a: to enhance LPP ProvideAssistanceData and posSIB to convey the association information of DL PRS resources with TRP Tx TEG ID.

### 2.1.2	Accuracy improvements for DL-AoD

For Accuracy improvements for DL-AoD, according to the RAN1 agreements, the following enhancements are needed for the assistance information:

- LMF provides the TRP beam/antenna information (i.e. the quantized version of the relative Power between PRS resources per angle per TRP) to UE

  - The relative power is defined with respect to the peak power in each angle

  - For each angle, at least two PRS resources are reported.

  - Note: the peak power per angle is not provided

- UE can request, and LMF provides, indication of expected angle value and uncertainty (of the expected azimuth and zenith angle value) range(s)

  - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE

  - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

- For PRS resource reporting

  - Option 1: for each PRS resource, a subset of PRS resources for the purpose of prioritization of DL-AOD reporting

  - Option 2: for each PRS resource, the boresight direction information

Proposal 1b: to enhance LPP ProvideAssistanceData to convey: TRP beam/antenna information, indication of expected angle value and uncertainty.

With regards to the two options for PRS resource reporting (a subset of PRS resources for the purpose of prioritization of DL-AOD reporting, and boresight direction information), since RAN1 haven’t made the selection, RAN2 may need to discuss which option to support.

Proposal 1c: to discuss which option (a subset of PRS resources for the purpose of prioritization of DL-AOD reporting, and boresight direction information) of PRS resource reporting to support.

In our view, the boresight direction information is needed, however the prioritization of PRS resources can be left for UE implementation and doesn’t need to be signalled.

Proposal 1d: to enhance LPP RequestAssistanceData to allow UE to request the expected angle value and uncertainty.

## 2.2 	Measurements

### 2.2.1	Mitigation of UE Rx/Tx and/or gNB Rx/Tx timing delays

According to the RAN1 agreements, the following measurement related enhancements are needed for mitigation of UE Rx/Tx and/or gNB Rx/Tx timing delays:

- UE provides to LMF the association information of UL SRS resources for positioning with UE Tx TEGs ID

  - The association may change during the measurement reporting period and that change should be indicated by the UE

- UE measures the same DL PRS resource with N different UE Rx TEGs and report the corresponding multiple RSTD measurements and multiple UE Rx-Tx time difference measurements

  - N=[2, 3, 4, 6, 8]

  - The maximum number of reported RSTD measurements obtained from different DL PRS resources per UE Rx TEG per target TRP is 4.

  - The maximum number of reported UE Rx-Tx time difference measurements obtained from different DL PRS resources per UE Rx TEG for a TRP is 4.

- UE measures the same DL PRS resource with N different UE RxTx TEGs with the same UE Tx TEG, and report the corresponding multiple UE Rx-Tx time difference measurements

  - N=[2, 3, 4, 6, 8]

  - The maximum number of reported UE Rx-Tx time difference measurements obtained from different DL PRS resources per UE RxTx TEG for a TRP is 4.

Naturally, LPP ProvideLocationInformation needs to be enhanced to convey the information mentioned above.

Proposal 2a: to enhance LPP ProvideLocationInformation to convey the following information: association of UL SRS for positioning resources with UE Tx TEGs ID, multiple RSTD measurements (for N different UE Rx TEGs), multiple UE Rx-Tx time difference measurements (for N different UE Rx TEGs), and multiple UE Rx-Tx time difference measurements (for N different UE RxTx TEGs with the same UE Tx TEG).

NOTE: this information is only applicable to UE-assisted positioning.

Additionally, with regards to the first part (association of UL SRS for positioning resources with UE Tx TEGs ID), RAN1 discussed two signalling options:

- UE to gNB and gNB to LMF, and

- UE directly to LMF

Note that in both cases it is the LMF that makes use of this information (as per RAN1 agreement), never the gNB – in other words, even when the UE sends it to the gNB, the gNB merely forwards it to the LMF and is not expected to process or make any other use of that information. Here we would like to point out that while the information itself, which entities provide it and which entities make use of it is in RAN1’s responsibility – how the information defined in RAN1 gets to the relevant entity is signaling, which is in RAN2 domain. Furthermore, and most importantly, supporting to signaling options for exactly the same information is redundant and increases UE and network complexity without any gains or benefits.

Observation 1: RAN1 decision to support two signaling (LPP and RRC+NRPPa) options to convey the association of UL SRS resources with UE Tx TEGs ID was outside of RAN1 scope.

In other words, that was not RAN1 decision to make. Therefore, we suggest supporting only one signalling option (i.e. LPP) and to liaise that decision to RAN1.

Proposal 2b: alternative signalling for association of UL SRS resources with UE Tx TEGs ID is not needed.

### 2.2.2		Accuracy improvements for DL-AoD

For accuracy improvements for DL-AoD, according to the RAN1 agreements, the following measurement enhancements are needed:

- UE reports DL PRS RSRPP and PRS RSRP

  - The maximum number of DL PRS RSRPP M is a UE capability and its candidate values include {2,4,8,16,24}

Proposal 2c: to enhance LPP ProvideLocationInformation to convey DL PRS RSRPP.

## 2.3	Capabilities

### 2.3.1	Mitigation of UE Rx/Tx and/or gNB Rx/Tx timing delays

Based on the analysis of the RAN1 agreements for mitigation of UE Rx/Tx and/or gNB Rx/Tx timing delays, the following UE capabilities need to be introduced:

- The maximum number of UE RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT]

- The maximum number of UE TxTEGs [for UL-TDOA and/or Multi-RTT]

- The maximum number of UE-RxTx TEGs

- Capability to provide the association information of UL SRS resources for positioning with UE Tx TEGs ID

- Capability to measure the same DL PRS resource with N different UE Rx TEGs and report the corresponding multiple RSTD measurements

- Capability to measure the same DL PRS resource with N different UE Rx TEGs and report the corresponding multiple UE Rx-Tx time difference

- Capability to measure the same DL PRS resource with N different UE RxTx TEGs with the same UE Tx TEG, and report the corresponding multiple UE Rx-Tx time difference measurements

### 2.3.2	Accuracy improvements for DL-AoD

Furthermore, for accuracy improvements for DL-AoD, according to the RAN1 agreements, the following capabilities need to be introduced:

- The maximum number of DL PRS RSRPP

- Capability to receive an indication for each PRS resource, of a subset of PRS resources for the purpose of prioritization of DL-AOD reporting

- Capability to receive an indication for each PRS resource, of the boresight direction information

Proposal 3: to enhance the capability signaling with the information listed in clause 2.3 of the present contribution.

# 3	Conclusions

Proposal 1a: to enhance LPP ProvideAssistanceData and posSIB to convey the association information of DL PRS resources with TRP Tx TEG ID.

Proposal 1b: to enhance LPP ProvideAssistanceData to convey: TRP beam/antenna information, indication of expected angle value and uncertainty.

Proposal 1c: to discuss which option (a subset of PRS resources for the purpose of prioritization of DL-AOD reporting, and boresight direction information) of PRS resource reporting to support.

Proposal 1d: to enhance LPP RequestAssistanceData to allow UE to request the expected angle value and uncertainty.

Proposal 2a: to enhance LPP ProvideLocationInformation to convey the following information: association of UL SRS for positioning resources with UE Tx TEGs ID, multiple RSTD measurements (for N different UE Rx TEGs), multiple UE Rx-Tx time difference measurements (for N different UE Rx TEGs), and multiple UE Rx-Tx time difference measurements (for N different UE RxTx TEGs with the same UE Tx TEG).

Observation 1: RAN1 decision to support two signaling (LPP and RRC+NRPPa) options to convey the association of UL SRS resources with UE Tx TEGs ID was outside of RAN1 scope.

Proposal 2b: alternative signalling for association of UL SRS resources with UE Tx TEGs ID is not needed.

Proposal 2c: to enhance LPP ProvideLocationInformation to convey DL PRS RSRPP.

Proposal 3: to enhance the capability signaling with the information listed in clause 2.3 of the present contribution.