3GPP TSG-RAN WG2 Meeting #116bis-e	R2- 2201213

Online, Jan 17 – Jan 25, 2022

Agenda Item		:	8.13.2.1

Source		:	LG Electronics Inc.

Title		:	Remaining issues on SCG related MRO

Document for		:	Discussion and Decision

1. Introduction

In this contribution, we discuss remaining issues on SCG related MRO.

2. Discussion

2.1 Association between RA related information and SCG failure

RAN2 received LS from RAN3 in the meeting RAN2#115 [1] about the required information for SCG failure analysis, e.g. especially for the solution for the optimization of PScell change failure for MRO in case of MR-DC, as follows:

- CGI of the Source PSCell: the source PSCell of the last SN change. The source PSCell could be E-UTRA cell or NR cell. - CGI of the Failed PSCell: the PSCell in which SCG failure is detected or the target PSCell of the failed PScell change. The Failed PSCell could be E-UTRA cell or NR cell. - timeSCGFailure: the time elapsed since the last PSCell change initialization until SCG failure. - connectionFailureType: radio link failure or SN change failure. - random-access related information set by the PSCell

Related to above, RAN2 made the following agreements in the previous meetings RAN2#115 and RAN2#116:

1: UE reports the SN RACH report to the MN, and then MN sends the SN RACH report to SN. 2: The UE needs to include RA information in case that failureType is set to randomAccessProblem or beamFailureRecoveryFailure-r16. 3: RA-InformationCommon-r16 is used as a baseline to indicate random-access related information set by the PSCell.

Since the above agreements 2 and 3 only relates to what information the UE includes for delivering to the network not the exact signalling procedure, RAN2 have the FFS which message carries the RA-InformationCommon associated to an SCG failure. To resolve the FFS, RA-Report and SCGFailureInformation are on the table as candidates to carry the RA-InformationCommon associated to an SCG failure.

It is essential that the UE transmits the RA-Report to the network with a new triggering condition for reporting, as according to the current specification, the UE delivers the RA-Report to the network only after making a successful connection to the network (e.g. successful handover, connection establish complete, etc.) by the network initiation. Since RAN2 have to define the failure cases for initiation of reporting the RA related information by the UE, the approach using RA-Report has a bigger impact on the specification than the way the SCGFailureInformation includes the RA-InformationCommon. In our view, the RA related information associated to an SCG failure can be delivered to the network by piggybacking the RA-InformationCommon into the SCGFailureInformation, which can be simply implemented on the specification as given in Annex 5.1.

Proposal 1: The UE includes the RA-InformationCommon into the SCGFailureInformation message in case that failureType is set to randomAccessProblem or beamFailureRecoveryFailure-r16.

2.2 SCG failure type determination

In the previous meeting RAN2#116, RAN2 left the remaining issue about introduction one bit flag to indicate whether T304 is running or not in SCG failure message. In our view, this FFS comes from the need to know if an SCG failure, especially for RACH failure, occurs when T304 is running or not because the network cannot differentiate whether the UE experiences a RACH failure during handover or not by the legacy specification. Since RAN2 confirmed that the five information requested by RAN3 LS [1]‎ are needed, RAN2 need to check if the additional information can be used for the purpose of one bit to indicate whether T304 is running or not in SCG failure message. In particular, there exist the source cell related information and the target cell related information among the five information RAN3 requested, i.e. CGI of the source PSCell and CGI of the failed PSCell, which can be used for differentiating if an SCG failure occurs when T304 is running or not. For example, the network knows the UE experiences an SCG failure during handover if the SCGFailureInformation contains CGIs of the source PSCell and the failed PSCell. Otherwise, the network knows the SCG failure does not relate to handover.

Observation 1: CGI of the source PSCell and CGI of the failed PSCell in the SCGFailureInformation can be used for differentiating if an SCG failure occurs when T304 is running or not. In other words, the one bit indicating whether T304 is running or not can be replaced with the CGIs of the source PSCell and the failed PSCell in the SCGFailureInformation.

Proposal 2: There is no need to introduce one bit indicating whether T304 is running or not in SCG failure message.

In order to implement Proposal 2 and Proposal 3, we propose the text proposal given in Annex 5.1.

Proposal 3: RAN2 is asked to consider a proposed text proposal for SCG related MRO.

3. Conclusion

In this contribution, we have the following proposals:

Proposal 1: The UE includes the RA-InformationCommon into the SCGFailureInformation message in case that failureType is set to randomAccessProblem or beamFailureRecoveryFailure-r16.

Observation 1: CGI of the source PSCell and CGI of the failed PSCell in the SCGFailureInformation can be used for differentiating if an SCG failure occurs when T304 is running or not. In other words, the one bit indicating whether T304 is running or not can be replaced with the CGIs of the source PSCell and the failed PSCell in the SCGFailureInformation.

Proposal 2: There is no need to introduce one bit indicating whether T304 is running or not in SCG failure message.

Proposal 3: RAN2 is asked to consider a proposed text proposal for SCG related MRO.

4. References

[1] R3-211332 LS on information needed for MRO in SCG Failure Report

[2] TS 38.331 v16.7.0, NR; Radio Resource Control (RRC) protocol specification

5. Annex

5.1 Text proposal based on TS 38.331 [1]

5.7.3.5		Actions related to transmission of SCGFailureInformation message

The UE shall set the contents of the SCGFailureInformation message as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message due to T310 expiry:

2>	set the failureType as t310-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to T312 expiry:

2>	set the failureType as other and set the failureType-v1610 as t312-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as synchReconfigFailureSCG;

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide random access problem indication from SCG MAC:

2>	if the random access procedure was initiated for beam failure recovery:

3>	set the failureType as other and set the failureType-v1610 as beamFailureRecoveryFailure;

3> set the ra-InformationCommon as specified in subclause 5.7.10.5.

3> if the failure is detected due to reconfiguration with sync failure as described in 5.3.5.8.3:

4> set the failedPSCellCGI to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity;

4> set the previousPSCellCGI to the global cell identity and tracking area code of the PSCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

2>	else:

3>	set the failureType as randomAccessProblem;

3> set the ra-InformationCommon as specified in subclause 5.7.10.5.

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to SRB3 IP check failure:

2>	set the failureType as srb3-IntegrityFailure;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to Reconfiguration failure of NR RRC reconfiguration message:

2>	set the failureType as scg-reconfigFailure;

1>	else if the UE initiates transmission of the SCGFailureInformation message due to consistent uplink LBT failures:

2>	set the failureType as other and set the failureType-v1610 as scg-lbtFailure;

1>	else if connected as an IAB-node and the SCGFailureInformation is initiated due to the reception of a BH RLF indication on BAP entity from the SCG:

2>	set the failureType as other and set failureType-v1610 as bh-RLF;

1> include and set MeasResultSCG-Failure in accordance with 5.7.3.4;

1>	for each MeasObjectNR configured by a MeasConfig associated with the MCG, and for which measurement results are available:

2>	include an entry in measResultFreqList;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb:

3>	set ssbFrequency in measResultFreqList to the value indicated by ssbFrequency as included in the MeasObjectNR;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs:

3>	set refFreqCSI-RS in measResultFreqList to the value indicated by refFreqCSI-RS as included in the associated measurement object;

2>	if a serving cell is associated with the MeasObjectNR:

3>	set measResultServingCell in measResultFreqList to include the available quantities of the concerned cell and in accordance with the performance requirements in TS 38.133 [14];

2>	set the measResultNeighCellList in measResultFreqList to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	ordering the cells with sorting as follows:

4>	based on SS/PBCH block if SS/PBCH block measurement results are available and otherwise based on CSI-RS;

4>	using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR;

3>	for each neighbour cell included:

4>	include the optional fields that are available.

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

NOTE 2:	Field measResultSCG-Failure is used to report available results for NR frequencies the UE is configured to measure by SCG RRC signalling.

1>	if available, set the locationInfo as in 5.3.3.7.:

The UE shall submit the SCGFailureInformation message to lower layers for transmission.

–	SCGFailureInformation

The SCGFailureInformation message is used to provide information regarding NR SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCGFailureInformation message

-- ASN1START

-- TAG-SCGFAILUREINFORMATION-START

SCGFailureInformation ::=                   SEQUENCE {

criticalExtensions                           CHOICE {

scgFailureInformation                        SCGFailureInformation-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

SCGFailureInformation-IEs ::=            SEQUENCE {

failureReportSCG                         FailureReportSCG                    OPTIONAL,

nonCriticalExtension                     SCGFailureInformation-v1590-IEs     OPTIONAL

}

SCGFailureInformation-v1590-IEs ::=       SEQUENCE {

lateNonCriticalExtension                OCTET STRING                        OPTIONAL,

nonCriticalExtension                    SCGFailureInformation-v17xy-IEs OPTIONAL

}

SCGFailureInformation-v17xy-IEs ::=       SEQUENCE {

ra-InformationCommon-r17             RA-InformationCommon-r16                         OPTIONAL,

previousPSCellCGI-r17                CGI-Info-Logging-r16

failedPSCellCGI-r17 				CGI-Info-Logging-r16

nonCriticalExtension                    SEQUENCE {} 									OPTIONAL

}

FailureReportSCG ::=                       SEQUENCE {

failureType                                    ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-ReconfigFailure,

srb3-IntegrityFailure, other-r16, spare1},

measResultFreqList                          MeasResultFreqList                                                      OPTIONAL,

measResultSCG-Failure                      OCTET STRING (CONTAINING MeasResultSCG-Failure)                OPTIONAL,

...,

[[

locationInfo-r16                            LocationInfo-r16            OPTIONAL,

failureType-v1610                        ENUMERATED {scg-lbtFailure-r16, beamFailureRecoveryFailure-r16,

t312-Expiry-r16, bh-RLF-r16, spare4, spare3, spare2, spare1} OPTIONAL

]]

}

MeasResultFreqList ::=                   SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

-- TAG-SCGFAILUREINFORMATION-STOP

-- ASN1STOP

SCGFailureInformation field descriptions

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG-Failure The field contains the MeasResultSCG-Failure IE which includes available results of measurements on NR frequencies the UE is configured to measure by the NR SCG RRCReconfiguration message.

previousPSCellCGI This field is used to indicate the source PSCell of the last handover (source PSCell when the last executed RRCReconfiguration message including reconfigurationWithSync was received).

failedPSCellCGI This field is used to indicate the PSCell in which RLF is detected or the target PSCell of the failed handover.