3GPP TSG-RAN WG2 #116bis-e	R2-2201477

17th – 25th January 2022

Online

Source:                    	NTT DOCOMO, INC.

Title: 	Discussion on CPAC failure handling

Document for:        	Discussion and decision

Agenda Item:         	8.2.3.3

- Introduction

In RAN2 113e meeting, following agreement was reached

- Agreements

- 12	SCGFailureInformation procedure can be taken as the baseline for CPAC failure ‎handling in Rel-17 ‎scenarios.‎

- FFS on the exact content of the message.

- FFS if time allows on further ‎enhancements to CPAC failure handling‎

In RAN2 116e meeting, CPAC failure/recovery was discussed. Part of the enhancement of SCGFailureInformation is the scope of SON, and the other part is the scope of DC/CA, according to opinions of some companies. This contribution propose topic which should be discussed in DC/CA scope.

- Discussion

In case legacy SCGFailureInformation procedure is used for CPAC failure, one open issue is how network can distinguish between legacy PSCell addition/change and CPAC. It is preferable for network to know whether the SCG failure is CPAC failure or legacy SCG failure not only for the SON purpose, but also it allows network to decide the subsequent procedure e.g., to switch from CPAC to legacy PSCell addition change. So, it is necessary to discuss how to distinguish between CPAC failure and legacy PSCell addition/change failure in SCGFailureInformation message.

Observation1: How to distinguish between CPAC failure and legacy PSCell addition/change failure should be discussed in DC/CA scope

We propose to add a CPAC failure indication into SCGFailureInformation message as shown in Annex. Network can control subsequent procedure based on the indication (e.g., fallback from CPAC to legacy SCG addition/change with using information of UE’s measurement report). In addition, the indication is useful for improving PSCell coverage from operational perspective.

Proposal1: Introduce a CPAC failure indication to distinguish between CPAC failure and legacy PSCell addition/change failure in SCGFailureInformation message

- Conclusion

Based on the discussion in the previous sections, we made the following proposals:

Observation1: How to Identify CPAC failure and legacy PSCell addition/change failure may be DC/CA scope.

Propsal1: SCGFailureInformation include CPAC failure indication to identify with legacy PSCell adition/change failure.

- Annex

Text proposal of proposal1

### 6.2.2	Message definitions

Non-editing sections omitted.

–	SCGFailureInformation

The SCGFailureInformation message is used to provide information regarding NR SCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SCGFailureInformation message

-- ASN1START

-- TAG-SCGFAILUREINFORMATION-START

SCGFailureInformation ::=                   SEQUENCE {

criticalExtensions                           CHOICE {

scgFailureInformation                        SCGFailureInformation-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

SCGFailureInformation-IEs ::=            SEQUENCE {

failureReportSCG                         FailureReportSCG                    OPTIONAL,

nonCriticalExtension                     SCGFailureInformation-v1590-IEs     OPTIONAL

}

SCGFailureInformation-v1590-IEs ::=       SEQUENCE {

lateNonCriticalExtension                OCTET STRING                        OPTIONAL,

nonCriticalExtension                    SCGFailureInformation-v17xy-IEs                         OPTIONAL

}

SCGFailureInformation-v17xy-IEs ::=       SEQUENCE {

cpac-configured-r17                         ENUMERATED{true}                         OPTIONAL,

nonCriticalExtension                        SEQUENCE{}                                OPTIONAL

}

FailureReportSCG ::=                       SEQUENCE {

failureType                                    ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-ReconfigFailure,

srb3-IntegrityFailure, other-r16, spare1},

measResultFreqList                          MeasResultFreqList                                                      OPTIONAL,

measResultSCG-Failure                      OCTET STRING (CONTAINING MeasResultSCG-Failure)                OPTIONAL,

...,

[[

locationInfo-r16                            LocationInfo-r16            OPTIONAL,

failureType-v1610                        ENUMERATED {scg-lbtFailure-r16, beamFailureRecoveryFailure-r16,

t312-Expiry-r16, bh-RLF-r16, spare4, spare3, spare2, spare1} OPTIONAL

]]

}

MeasResultFreqList ::=                   SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

-- TAG-SCGFAILUREINFORMATION-STOP

-- ASN1STOP

SCGFailureInformation field descriptions

Cpac-configured-r17 Indicates the failure type of SCG failure is CPAC (Conditional PSCell Addition/Change) failure.

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

measResultSCG-Failure The field contains the MeasResultSCG-Failure IE which includes available results of measurements on NR frequencies the UE is configured to measure by the NR SCG RRCReconfiguration message.